| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0177 |
| --- | --- |

| Title: | AHG9: NAL unit header with layer ID partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Jeonghoon Park Maetan 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, Korea Ye-Kui Wang Ying Chen 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 82-31-279-2242 b.d.choi@samsung.com jeonghoon@samsung.com 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Samsung Electronics Co., Ltd. and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

At the 10th JCT-VC meeting, two approaches for NAL unit header and video parameter set for HEVC extension were discussed, and their “straw man” designs were described in JCTVC-J1007. This contribution proposes two alternative NAL unit header designs in the same category as approach 2 in JCTVC-J1007. The proposed design partitions a layer_id into specific scalability dimension identifiers by parsing only NAL unit headers.

# Introduction

In JCTVC-J1007, two approaches for NAL unit header and video parameter set for HEVC extension are described as follows:

- Approach 1: The mapping of each value of the layer_id (i.e., the reserved_zero_6bits in the HEVC base specification) in the NAL unit header to specific scalability dimension IDs (e.g. view_order_index, depth_flag, dependency_id and quality_id) is signaled in a look-up table in the video parameter set.

- Approach 2: The layer_id in the NAL unit header is partitioned into specific scalability dimension IDs (e.g. view_order_index, depth_flag, dependency_id and quality_id).

This contribution proposes two alternative NAL unit header designs in the same category as approach 2 in JCTVC-J1007. Both proposed designs use the same mapping method, in which a layer_id in NAL unit header is partitioned into scalability IDs according to the value of scalability_type signalled in the VPS NAL unit header. Thus, the proposed designs have different NAL unit header syntax for VPS NAL units and non-VPS NAL units.

The first design enables to utilize the reserved bits efficiently for layer_id and temporal_id by variably changing the partition length. It is a flexible design to allocate more bits to layer_id when less temporal sub-layers are needed. The bit lengths of layer_id and temporal_id are determined by the syntax element nuh_max_sub_layers in VPS NAL unit headers.

The second design focuses on providing the maximized bit length for a layer_id and a more harmonized design with approach 1 in JCTVC-J1007. It allocates 8 bits to layer_id, thus fixed-length and distinguishes base layer NAL units from non-base layer NAL units by a flag (nuh_baselayer_flag). In order to support a user-selective mapping rule, a look-up table same as approach 1 in J1007 can be signaled in the VPS when nuh_vps_skip_flag is equal to 0. When nuh_vps_skip_flag is equal to 1, parsing of VPS can be skipped. One additional benefit of this design is that there are five reserved bits in the NAL unit header for non-VPS NAL units in the base spec and can be used for other purposes.

# Proposal 1

## NAL uinit header design in HEVC base spec

Texts not highlighted are the same as in the latest HEVC WD, and those highlighted in yellow are the changed parts compared to the latest HEVC WD.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| if( nal_unit_type = = VPS_NUT ) { |  |
| nuh_reserved_zero_6bits | u(6) |
| nuh_max_sub_layers | u(3) |
| } else { |  |
| nuh_reserved_zero_nbits | u(v) |
| nuh_temporal_id_plus1 | u(v) |
| } |  |
| } |  |

nuh_reserved_zero_6bits shall be equal to 0. Other values of nuh_reserved_zero_6bits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore the value of nuh_reserved_zero_6bits.

- NOTE 1 – It is anticipated that in future scalable and/or 3D video coding extensions of this specification, this syntax element will be used to specify scalability type, which determines the partitioning of the last 9 bits of the NAL unit header in non-VPS NAL units to different scalablity dimension fields.

nuh_max_sub_layers specifies the maximum number of temporal sub-layers that may be present in the coded video sequence. The value of nuh_max_sub_layers shall be greater than 0.

nuh_reserved_zero_nbits shall be equal to 0. The number of bits used to represent nuh_reserved_zero_nbits is 9 − Ceil( Log2( nuh_max_sub_layers + 1 ) ). Other values of nuh_reserved_zero_nbits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with values of nuh_reserved_zero_nbits not equal to 0.

- NOTE 2 – It is anticipated that in future scalable and/or 3D video coding extensions of this specification, this syntax element will be used to identify additional layers that may be present in the coded video sequence, wherein a layer may be, e.g. a spatial scalable layer, a quality scalable layer, a texture view or a depth view.

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit. The number of bits used to represent nuh_temporal_id_plus1 is Ceil( Log2( nuh_max_sub_layers + 1 ) ).

The variable TemporalId is specified as

TemporalId = nuh_temporal_id_plus1 − 1		(7-1)

If nal_unit_type is in the range of 7 to 12 (coded slice of a RAP picture), inclusive, TemporalId shall be equal to 0; otherwise, when nal_unit_type is in the range of 3 to 6, inclusive (coded slice of a TSA or STSA picture), TemporalId shall not be equal to 0.

The value of TemporalId shall be the same for all VCL NAL units of an access unit. The value of TemporalId of an access unit is the value of the TemporalId of the VCL NAL units of the access unit.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If nal_unit_type is equal to SPS_NUT, EOS_NUT, or EOB_NUT, TemporalId shall be equal to 0.

–	Otherwise, if nal_unit_type is equal to AUD_NUT or FD_NUT, TemporalId shall be equal to the TemporalId of the access unit containing the non-VCL NAL unit.

–	Otherwise, when nal_unit_type is equal to SEI_NUT, TemporalId shall be equal to or greater than the TemporalId of the access unit containing the NAL unit.

When nal_unit_type is equal to VPS_NUT, nuh_temporal_id_plus1 is not present, TemporalId is derived to be 0.

- NOTE 3 – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all access units that the non-VCL NAL unit applies to. When nal_unit_type is equal to VPS_NUT or SPS_NUT, TemporalId must be equal to 0, as a sequence parameter set applies at least to one RAP access unit. When nal_unit_type is equal to AUD_NUT or FD_NUT, TemporalId must be equal to the TemporalId of the access unit containing the non-VCL NAL unit, as access unit delimiter or filler data only applies to the containing access unit. When nal_unit_type is equal to PPS_NUT, TemporalId may be less than, equal to, or greater than the TemporalId of the containing access unit, as a picture parameter set may be repeated in access units not referring to the picture parameter set (for error resilience purposes), and all picture parameter sets may be included in the beginning of a bitstream, wherein the first coded picture has TemporalId equal to 0. When nal_unit_type is equal to SEI_NUT, TemporalId may be equal to or greater than the TemporalId of the containing access unit, as an SEI NAL unit may contain a picture buffering SEI message or a picture timing SEI message that applies to a bitstream subset that includes access units for which the TemporalId values are greater than the TemporalId of the access unit containing the SEI NAL unit.

## Example design in MV-HEVC

Texts not highlighted are the same as in the latest HEVC WD, those highlighted in yellow are the changed parts compared to the latest HEVC WD, and those highlighted in green are the changed parts compared to section 3.1 above.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| if( nal_unit_type = = VPS_NUT ) { |  |
| scalability_type | u(6) |
| nuh_max_sub_layers | u(3) |
| } else { |  |
| layer_id | u(v) |
| nuh_temporal_id_plus1 | u(v) |
| } |  |
| } |  |

scalability_type specifies the scalability type of the coded video sequence. The value shall be equal to 1. Other values of scalability_type greater than 0 may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore the value of scalability_type.

nuh_max_sub_layers specifies the maximum number of temporal sub-layers that may be present in the coded video sequence. The value of nuh_max_sub_layers shall be greater than 0.

layer_id identifies the layer the NAL unit belongs to. layer_id shall be equal to 0 for any NAL unit of the base view. View order index of each NAL unit is derived to be equal to layer_id. [Ed. (YK): The part before this note is same as in the MV-HEVC WD 1 in JCT3V-A1004 d0.] The number of bits used to represent layer_id is 9 − Ceil( Log2( nuh_max_sub_layers + 1 ) ).

NOTE 1 –  NAL unit header partitioning methods specified by scalability_type, including 0 for (base) HEVC spec, 1 for MV-HEVC, and 2 for 3D-HEVC, are shown in the table below.

| scalability_type (6 bits) | nuh_temporal_id_plus1 ( 1-3 bits) | layer_id (6-8 bits) |
| --- | --- | --- |
| 0 | length: 1-3 bits value: 1-7 | length: 6-8 bits value: 0 |
| 1 | length: 1-3 bits value: 1-7 | view_order_idx: 6-8 bits |
| 2 | length: 1-3 bits value: 1-7 | depth_flag: 1 bit view_order_idx: 5-7 bits |

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit. The number of bits used to represent nuh_temporal_id_plus1 shall be equal to Ceil( Log2( nuh_max_sub_layers + 1 ) ).

The variable TemporalId is specified as

TemporalId = nuh_temporal_id_plus1 − 1		(7-2)

If nal_unit_type is in the range of 7 to 12 (coded slice of a RAP picture), inclusive, TemporalId shall be equal to 0; otherwise, when nal_unit_type is in the range of 3 to 6, inclusive (coded slice of a TSA or STSA picture), TemporalId shall not be equal to 0.

The value of TemporalId shall be the same for all VCL NAL units of an access unit. The value of TemporalId of an access unit is the value of the TemporalId of the VCL NAL units of the access unit.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If nal_unit_type is equal to SPS_NUT, EOS_NUT, or EOB_NUT, TemporalId shall be equal to 0.

–	Otherwise, if nal_unit_type is equal to AUD_NUT or FD_NUT, TemporalId shall be equal to the TemporalId of the access unit containing the non-VCL NAL unit.

–	Otherwise, when nal_unit_type is equal to SEI_NUT, TemporalId shall be equal to or greater than the TemporalId of the access unit containing the NAL unit.

When nal_unit_type is equal to VPS_NUT, nuh_temporal_id_plus1 is not present, TemporalId is derived to be 0.

- NOTE 2 – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all access units that the non-VCL NAL unit applies to. When nal_unit_type is equal to VPS_NUT or SPS_NUT, TemporalId must be equal to 0, as a sequence parameter set applies at least to one RAP access unit. When nal_unit_type is equal to AUD_NUT or FD_NUT, TemporalId must be equal to the TemporalId of the access unit containing the non-VCL NAL unit, as access unit delimiter or filler data only applies to the containing access unit. When nal_unit_type is equal to PPS_NUT, TemporalId may be less than, equal to, or greater than the TemporalId of the containing access unit, as a picture parameter set may be repeated in access units not referring to the picture parameter set (for error resilience purposes), and all picture parameter sets may be included in the beginning of a bitstream, wherein the first coded picture has TemporalId equal to 0. When nal_unit_type is equal to SEI_NUT, TemporalId may be equal to or greater than the TemporalId of the containing access unit, as an SEI NAL unit may contain a picture buffering SEI message or a picture timing SEI message that applies to a bitstream subset that includes access units for which the TemporalId values are greater than the TemporalId of the access unit containing the SEI NAL unit.

# Proposal 2

## NAL unit header design in HEVC base spec

Texts not highlighted are the same as in the latest HEVC WD, and those highlighted in yellow are the changed parts compared to the latest HEVC WD.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| if( nal_unit_type = = VPS_NUT ) { |  |
| nuh_vps_skip_flag | u(1) |
| nuh_reserved_one_8bits | u(8) |
| } else { |  |
| nuh_baselayer_flag | u(1) |
| nuh_reserved_zero_5bits | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |
| } |  |

nuh_vps_skip_flag shall be equal to 1. Other values of nuh_vps_skip_flag may be specified in the future by ITU-T | ISO/IEC.

nuh_reserved_one_8bits shall be equal to 1. Other values of nuh_reserved_one_8bits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore the value of nuh_reserved_one_8bits.

- NOTE 1 – It is anticipated that in future scalable and/or 3D video coding extensions of this specification, this syntax element will be used to specify scalability type, which determines the partitioning of the last 8 bits of the NAL unit header to different scalablity dimension fields in non-VPS NAL units.

nuh_baselayer_flag shall be equal to 1. Other values of nuh_baselayer_flag may be specified in the future by ITU-T | ISO/IEC.

nuh_reserved_zero_5bits shall be equal to 0. Other values of nuh_reserved_zero_5bits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore the value of nuh_reserved_zero_5bits.

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit.

NOTE 2 – It is anticipated that in future scalable and/or 3D video coding extensions of this specification, when nuh_baselayer_flag is equal to 0, nuh_reserved_zero_5bits and nuh_temporal_id_plus1 will be used to identify additional layers that may be present in the coded video sequence, wherein a layer may be, e.g. a spatial scalable layer, a quality scalable layer, a temporal layer, a texture view or a depth view.

The variable TemporalId is specified as

TemporalId = nuh_temporal_id_plus1 − 1		(7-3)

If nal_unit_type is in the range of 7 to 12 (coded slice of a RAP picture), inclusive, TemporalId shall be equal to 0; otherwise, when nal_unit_type is in the range of 3 to 6, inclusive (coded slice of a TSA or STSA picture), TemporalId shall not be equal to 0.

The value of TemporalId shall be the same for all VCL NAL units of an access unit. The value of TemporalId of an access unit is the value of the TemporalId of the VCL NAL units of the access unit.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If nal_unit_type is equal to SPS_NUT, EOS_NUT, or EOB_NUT, TemporalId shall be equal to 0.

–	Otherwise, if nal_unit_type is equal to AUD_NUT or FD_NUT, TemporalId shall be equal to the TemporalId of the access unit containing the non-VCL NAL unit.

–	Otherwise, when nal_unit_type is equal to SEI_NUT, TemporalId shall be equal to or greater than the TemporalId of the access unit containing the NAL unit.

When nal_unit_type is equal to VPS_NUT, nuh_temporal_id_plus1 is not present, TemporalId is derived to be 0.

- NOTE 3 – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all access units that the non-VCL NAL unit applies to. When nal_unit_type is equal to VPS_NUT or SPS_NUT, TemporalId must be equal to 0, as a sequence parameter set applies at least to one RAP access unit. When nal_unit_type is equal to AUD_NUT or FD_NUT, TemporalId must be equal to the TemporalId of the access unit containing the non-VCL NAL unit, as access unit delimiter or filler data only applies to the containing access unit. When nal_unit_type is equal to PPS_NUT, TemporalId may be less than, equal to, or greater than the TemporalId of the containing access unit, as a picture parameter set may be repeated in access units not referring to the picture parameter set (for error resilience purposes), and all picture parameter sets may be included in the beginning of a bitstream, wherein the first coded picture has TemporalId equal to 0. When nal_unit_type is equal to SEI_NUT, TemporalId may be equal to or greater than the TemporalId of the containing access unit, as an SEI NAL unit may contain a picture buffering SEI message or a picture timing SEI message that applies to a bitstream subset that includes access units for which the TemporalId values are greater than the TemporalId of the access unit containing the SEI NAL unit.

## Example design in MV-HEVC

Texts not highlighted are the same as in the latest HEVC WD, those highlighted in yellow are the changed parts compared to the latest HEVC WD, and those highlighted in green are the changed parts compared to section 3.1 above.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| if( nal_unit_type = = VPS_NUT ) { |  |
| nuh_vps_skip_flag | u(1) |
| scalability_type | u(8) |
| } else { |  |
| nuh_baselayer_flag | u(1) |
| if( nuh_baselayer_flag = = 1 ) { |  |
| nuh_reserved_zero_5bits | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| } else |  |
| layer_id_plus1 | u(8) |
| } |  |
| } |  |

nuh_vps_skip_flag equal to 0 specifies that the NAL unit header and parameter set in the VPS NAL unit should be parsed. nuh_vps_skip_flag equal to 1 specifies that parsing the VPS NAL unit can be skipped.

NOTE 1 – When nuh_vps_skip_flag is equal to 0, a look-up table mapping a layer_id into scalability identifiers can be signaled in VPS. It enables to support both approaches, a simple mapping method which partitions layer_id into scalability identifiers and a look-up table mapping method.

scalability_type specifies the scalability type of the coded video sequence. The value shall be equal to 1. Other values of scalability_type greater than 0 may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore the value of scalability_type.

nuh_baselayer_flag is equal to 1 when the NAL unit belongs to the base layer. nuh_baselayer_flag is equal to 0 when the NAL unit does not belong to the base layer.

nuh_reserved_zero_5bits shall be equal to 0 when nuh_baselayer_flag is equal to 1. Other values of nuh_reserved_zero_5bits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore the value of nuh_reserved_zero_5bits.

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit.

layer_id_plus1 minus 1 identifies the layer the NAL unit belongs to. View order index of each NAL unit is derived to be equal to layer_id − 1.

NOTE 2 – NAL unit header partitioning methods specified by scalability_type, including 0 for (base) HEVC spec, 1 for MV-HEVC, and 2 for 3D-HEVC, are shown in the table below.

| scalability_type (8 bits) | layer_id (8 bits) |
| --- | --- |
| 0 | temporal_id: 3bits |
| 1 | view_idx: 6bits temporal_id:2bits |
| 2 | depth_flag: 1 bit view_idx: 7 bits |

The variable TemporalId is specified as

TemporalId = nuh_temporal_id_plus1 − 1		(7-4)

If nal_unit_type is in the range of 7 to 12 (coded slice of a RAP picture), inclusive, TemporalId shall be equal to 0; otherwise, when nal_unit_type is in the range of 3 to 6, inclusive (coded slice of a TSA or STSA picture), TemporalId shall not be equal to 0.

The value of TemporalId shall be the same for all VCL NAL units of an access unit. The value of TemporalId of an access unit is the value of the TemporalId of the VCL NAL units of the access unit.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If nal_unit_type is equal to SPS_NUT, EOS_NUT, or EOB_NUT, TemporalId shall be equal to 0.

–	Otherwise, if nal_unit_type is equal to AUD_NUT or FD_NUT, TemporalId shall be equal to the TemporalId of the access unit containing the non-VCL NAL unit.

–	Otherwise, when nal_unit_type is equal to SEI_NUT, TemporalId shall be equal to or greater than the TemporalId of the access unit containing the NAL unit.

When nal_unit_type is equal to VPS_NUT, nuh_temporal_id_plus1 is not present, TemporalId is derived to be 0.

When nuh_baselayer_flag is equal to 0, and TemporalId is not derived from layer_id, TemporalId is set equal to the value of TemporalId of the base layer NAL unit in the same access unit.

- NOTE 3 – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all access units that the non-VCL NAL unit applies to. When nal_unit_type is equal to VPS_NUT or SPS_NUT, TemporalId must be equal to 0, as a sequence parameter set applies at least to one RAP access unit. When nal_unit_type is equal to AUD_NUT or FD_NUT, TemporalId must be equal to the TemporalId of the access unit containing the non-VCL NAL unit, as access unit delimiter or filler data only applies to the containing access unit. When nal_unit_type is equal to PPS_NUT, TemporalId may be less than, equal to, or greater than the TemporalId of the containing access unit, as a picture parameter set may be repeated in access units not referring to the picture parameter set (for error resilience purposes), and all picture parameter sets may be included in the beginning of a bitstream, wherein the first coded picture has TemporalId equal to 0. When nal_unit_type is equal to SEI_NUT, TemporalId may be equal to or greater than the TemporalId of the containing access unit, as an SEI NAL unit may contain a picture buffering SEI message or a picture timing SEI message that applies to a bitstream subset that includes access units for which the TemporalId values are greater than the TemporalId of the access unit containing the SEI NAL unit.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).