| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0178 |
| --- | --- |

| Title: | Crosscheck of JCTVC-K0139: Contouring artefact and solution |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document provides crosscheck result of JCTVC-K0139: Contouring artefact and solution by NTT DOCOMO.

# Introduction

In Fig 1.1, decoded image of “F: Kristen&Sara” (Frame #0, Intra_Main, QP=37) is shown.

Contouring artefact is typically observed at the blue background. This is caused by the propagation due to the direction of angular intra prediction. To suppress this artefact JCTVC-K0139 proposed additional method of intra smoothing.

Fig 1.1

# Simulation

## Simulation Condition

HM-8.0 is used as anchor. On top of the anchor software the proposed method has been implemented by the proponent and provided to the author. The author confirmed that the proposed method has been successfully implemented.

The following macro in “Typedef.h” is enabled to test the proposed method:

#define BI_LINEAR_REF	1

{AI, RA, LD}_{HE10, MAIN} conditions are tested with Class A to F sequences as specified by JCTVC-J1100 [2].

## Simulation Result

Summary of the result is shown in Table 2.2.1. Complete result is shown in “JCTVC-K0178.xls”. The result matches the one provided by the proponent. Impact of the proposed method on the objective quality is at ignorable level.

According to the HW and SW engineers in Sony, impact of the proposed method on the implementation cost is rather small.

Fig 2.2.1 is the decoded image of the proposed method (condition is same with Fig 1.1). As can be seen contouring artefact in the blue background as seen in Fig 1.1 is suppressed. On the other hand more mosquito noise is observed around the face and the shoulder of the left woman.

Table 2.2.1

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.07% | 0.13% | 0.09% | 0.02% | 0.05% | -0.05% |
| Class B | 0.09% | 0.14% | 0.13% | 0.01% | 0.03% | 0.04% |
| Class C | 0.02% | 0.05% | 0.06% | 0.01% | 0.02% | -0.01% |
| Class D | 0.01% | 0.04% | -0.02% | 0.00% | -0.01% | 0.01% |
| Class E | 0.12% | 0.28% | 0.23% | -0.06% | 0.10% | 0.05% |
| Overall | 0.06% | 0.12% | 0.10% | 0.00% | 0.03% | 0.01% |
|  | 0.06% | 0.12% | 0.10% | 0.00% | 0.03% | 0.00% |
| Class F | 0.04% | 0.00% | 0.06% | 0.03% | 0.01% | -0.03% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.10% | 0.01% | -0.05% | -0.05% | 0.13% | -0.46% |
| Class B | 0.05% | 0.07% | 0.01% | 0.00% | 0.08% | 0.07% |
| Class C | -0.01% | 0.15% | 0.09% | -0.01% | -0.07% | -0.08% |
| Class D | 0.02% | -0.14% | 0.13% | 0.00% | -0.08% | -0.06% |
| Class E |  |  |  |  |  |  |
| Overall | 0.04% | 0.02% | 0.04% | -0.01% | 0.02% | -0.12% |
|  | 0.04% | 0.01% | 0.03% | -0.01% | -0.02% | -0.18% |
| Class F | -0.03% | -0.08% | -0.11% | 0.06% | 0.15% | 0.11% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.02% | 0.14% | 0.17% | -0.01% | -0.07% | 0.23% |
| Class C | 0.00% | 0.07% | -0.09% | -0.04% | 0.03% | 0.17% |
| Class D | -0.07% | 0.38% | -0.38% | -0.01% | -0.09% | -0.61% |
| Class E | -0.11% | -0.03% | -0.50% | -0.15% | 0.16% | -1.24% |
| Overall | -0.03% | 0.15% | -0.16% | -0.04% | 0.00% | -0.27% |
|  | -0.03% | 0.17% | -0.18% | -0.04% | -0.01% | -0.27% |
| Class F | 0.07% | 0.35% | -0.11% | 0.01% | 0.74% | -0.44% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 98% |  |  |

Fig 2.2.1

# Conclusion

This contribution reports crosscheck result of JCTVC-K0139: Contouring artefact and solution.

The objective result provided by the proponent has been verified. The HW and SW engineers in Sony made a comment that the proposed method is feasible in terms of complexity.

The author also made comparison of the subjective quality between HM-8.0 and the proposed method. It is confirmed that the contouring artefact as seen in the flat background area is well suppressed. Side effect is also observed – more mosquito noise is observed between the flat background and the object.

The author recommends that this topic be further investigated, including non-normative solutions.

# References

[1] “High efficiency video coding (HEVC) text specification draft 8”, JCTVC-J1003, July 2012

[2] “HM 8 Common Test Conditions and Software Reference Configurations”, JCTVC-J1100, July 2012

[3] “Contouring artefact and solution”, JCTVC-K0139, October 2012

# Patent rights declaration(s)

Sony Corp. does not have any current or pending patent rights relating to the technology described in this contribution.