| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0180r1 |
| --- | --- |

| Title: | AHG9: Operation points in VPS and nesting SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Nokia Corporation, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

A problem was identified in JCTVC-J0074 that when the conformance of a bitstream containing scalable layers is checked using HEVC v1 standard or an HEVC v1 decoder decodes a bitstream containing scalable layers, the HRD parameters of the “highest” layer present in the bitstream should be used. As a solution to the problem, HEVC draft 8 includes a possibility to include multiple sets of HRD parameters for different bitstream subsets into VPS. It is asserted in this contribution that while HEVC draft 8 enables multiple sets of HRD parameters, it still lacks the capability of indicating different initial delays for bitstream subsets.

JCTVC-K0180 proposes:

- Specifying operation points as a separate loop in VPS rather than as part of the loop for the HRD parameters.

- Specifying a scalable nesting SEI message.

It is asserted that in HEVC v1, the proposed scalable nesting SEI message can be used for two purposes:

- Indicate that the nested SEI messages pertain to a range of temporal sub-layers rather than all sub-layers.

- Indicate buffering period and picture timing for any bitstream subset, such as the base layer sub-bitstream.

Revision 1 of the document attempted to revise the proposed syntax and semantics according to the comments received during the review in the JCT-VC meeting.

# Introduction

A problem was identified in JCTVC-J0074 that when the conformance of a bitstream containing scalable layers is checked using HEVC v1 standard or an HEVC v1 decoder decodes a bitstream containing scalable layers, the HRD parameters of the “highest” layer present in the bitstream should be used. As a result, JCT-VC decided to add signaling of HRD parameters for bitstream subsets into VPS on the basis of document JCTVC-J0562.

It is asserted in this contribution that while the adopted contribution JCTVC-J0562 allows sending different sets of HRD parameters, it still lacks the capability of indicating different initial delays for bitstream subsets. The buffering period and picture timing SEI messages present in the bitstream indicate the timing for the entire bitstream as decoded by HEVC v1 decoder.

This contribution proposes:

- Specifying operation points as a separate loop in VPS rather than as part of the loop for the HRD parameters. The listed operation points are indexed in their appearance order, and the index is used to refer to a particular operation point from other structures. The change enables

  - including HRD parameters for a different number of operation points than the total list of operation points, and

  - referring to operation points specified in VPS even if no HRD parameters were provided for each operation point.

- Specifying a scalable nesting SEI message which is similar to the SVC and MVC scalable nesting SEI messages. In HEVC v1, a scalable nesting SEI message can be used for two purposes:

  - Indicate that the nested SEI messages pertain to a range of temporal sub-layers rather than all sub-layers.

  - Indicate buffering period and picture timing for any operation point (or the respective bitstream subset). Hence, a HEVC v1 decoder can extract and decode the base layer sub-bitstream from a scalable HEVC bitstream with the buffering period and picture timing SEI messages indicated to pertain to base layer sub-bitstream.

# Proposal

## Video Parameter Set

The proposed changes to video_parameter_set_rbsp are indicated in red font color below.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| reserved_zero_2bits | u(2) |
| reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| next_essential_info_byte_offset //reserved_zero_12bits in the base spec | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| num_ops_minus1 | ue(v) |
| for( i = 1; i <= num_ops_minus1; i++ ) |  |
| operation_point( i ) |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| hrd_op_idx[ i ] | ue(v) |
| if( i > 0 ) |  |
| cprms_present_flag[ i ] | u(1) |
| hrd_parameters( cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

num_ops_minus1 specifies the number of operation_point( ) syntax structures present in the video parameter set.

hrd_op_idx[ i ] specifies the operation point to which the i-th hrd_parameters() syntax structure in the video parameter set applies.

cprms_present_flag[ i ] equal to 1 specifies that the HRD parameters that are common for all sub-layers are present in the i-th hrd_parameters() syntax structure in the video parameter set. cprms_present_flag[ i ] equal to 0 specifies that the HRD parameters that are common for all sub-layers are not present in the i-th hrd_parameters() syntax structure in the video parameter set and are derived to be the same as the ( i – 1 )-th hrd_parameters() syntax structure in the video parameter set. cprms_present_flag[ 0 ] is inferred to be equal to 1.

  - Scalable Nesting SEI Message

The scalable nesting SEI message provides a mechanism for associating SEI messages with bitstream subsets or with specific layers and sub-layers.

A scalable nesting SEI message contains one or more SEI messages. An SEI message contained in a scalable nesting SEI message is referred to as a nested SEI message. An SEI message not contained in a scalable nesting SEI message is referred to as a non-nested SEI message.

A buffering period SEI message and an SEI message of any other type shall not be nested in the same scalable nesting SEI message. A picture timing SEI message and an SEI message of any other type shall not be nested in the same scalable nesting SEI message.

| scalable_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| bitstream_subset_flag | u(1) |
| nesting_op_flag | u(1) |
| if( nesting_op_flag ) { |  |
| nesting_num_ops_minus1 | ue(v) |
| default_op_flag | u(1) |
| for( i = default_op_flag; i <= nesting_num_ops_minus1; i++ ) { |  |
| nesting_max_temporal_id_plus1[ i ] | u(3) |
| nesting_op_idx[ i ] | ue(v) |
| } |  |
| } else { |  |
| all_layers_flag | u(1) |
| if( !all_layers_flag ) { |  |
| nesting_no_op_max_temporal_id_plus1 | u(3) |
| nesting_num_layers_minus1 | ue(v) |
| for( i = 0; i <= nesting_num_layers_minus1; i++ ) |  |
| nesting_layer_id[ i ] | u(6) |
| } |  |
| } |  |
| while( !byte_aligned( ) ) |  |
| nesting_zero_bit /* equal to 0 */ | u(1) |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| } |  |

bitstream_subset_flag equal to 0 specifies that the nested SEI messages apply to specific layers and sub-layers. bitstream_subset_flag equal to 1 specifies that the nested SEI messages apply to one or more sub-bitstreams resulting from a sub-bitstream extraction process of subclause 10.1 with inputs specified by the syntax elements of the scalable nesting SEI message as specified below. If the nested SEI messages are picture buffereing SEI mesages, picure timing SEI messages or sub-picture timing SEI messages, bitstream_subset_flag shall be equal to 1.

Depending on the value of bitstream_subset_flag, either the layers and sub-layers or the operation points to which the nested SEI messages apply are specified by deriving the sets nestingLayedIdSet[ i ] and the variables maxTemporalId[ i ] from syntax element values as specified below.

nesting_op_flag equal to 0 specifies that the set nestingLayerIdSet[ 0 ] is specified by all_layers_flag and, when present, nesting_layer_id[ i ] for all i values in the range of 0 to nesting_num_layers_minus1, inclusive, and that the variable maxTemporalId[ 0 ] is specified by nesting_no_op_max_temporal_id_plus1. When nesting_op_flag is equal to 0, the variable nestingNumOps is set equal to 1. nesting_op_flag equal to 1 specifies that the set nestingLayerIdSet[ i ] and the variable maxTemporalId[ i ] are specified by nesting_num_ops_minus1, default_op_flag, nesting_max_temporal_id_plus1[ i ], when present, and nesting_op_idx[ i ], when present.

nesting_num_ops_minus1 plus 1 minus default_op_flag specifies the number of the following nesting_op_idx[ i ] syntax elements. The variable nestingNumOps is set equal to nesting_num_ops_minus1 + 1.

default_op_flag equal to 1 specifies that maxTemporalId[ 0 ] is equal to nuh_temporal_id_plus1 of the current SEI NAL unit minus 1 and that nestingLayerIdSet[ 0 ] contains all values in the range of 0 to nuh_reserved_zero_6bits of the current SEI NAL unit, inclusive.

nesting_max_temporal_id_plus1[ i ] is used to specify the variable maxTemporalId[ i ]. The value of nesting_max_temporal_id_plus1[ i ] shall be greater than or equal to nuh_temporal_id_plus1 of the current SEI NAL unit. The variable maxTemporalId[ i ] is set equal to nesting_max_temporal_id_plus1[ i ] – 1.

nesting_op_idx[ i ] is used to specify the set nestingLayerIdSet[ i ].

The set nestingLayerIdSet[ i ] is set to consist of op_layer_id[ nesting_op_idx[ i ] ][ j ] with all values of j in the range of 0 to op_num_layer_id_values_minus1[ nesting_op_idx[ i ] ], inclusive, where the op_layer_id[ ][ ] and op_num_layer_values_minus1[ ] values are from the active video parameter set.

all_layers_flag equal to 0 specifies that the set nestingLayerIdSet[ 0 ] is specified by nesting_num_layers_minus1 and nesting_layer_id[ i ] as specified below. all_layers_flag equal to 1 specifies that the set nestingLayerIdSet[ 0 ] consists of all values of nuh_reserved_zero_6bits present in the current access unit that are equal to or greater than nuh_reserved_zero_6bits of the current SEI NAL unit.

nesting_no_op_max_temporal_id_plus1 minus 1 specifies the value of maxTemporalId[ 0 ] when nesting_op_flag is equal to 0 and all_layers_flag is equal to 0.

nesting_num_layers_minus1 plus 1 specifies the

following nesting_layer_id[ i ] syntax elements.

nesting_layer_id[ i ] specifies the i-th nuh_reserved_zero_6bits value included in the set nestingLayerIdSet[ 0 ]. The set nestingLayerIdSet[ 0 ] is set to consist of nesting_layer_id[ i ] for all i values in the range of 0 to nesting_num_layers_minus1, inclusive.

When bitstream_subset_flag is equal to 0, the nested SEI messages apply to the sets of NAL units where each set i from 0 to nestingNumOps – 1, inclusive, consists of all NAL units with nuh_reserved_zero_6bits included in the set nestingLayerIdSet[ i ] and with nuh_temporal_id_plus1 in the range of nuh_temporal_id_plus1 of the current SEI NAL unit to maxTemporalId[ i ] + 1, inclusive.

When bitstream_subset_flag is equal to 1, the nested SEI messages apply to sub-bitstreams subBitstream[ i ] for all i values in the range of 0 to nestingNumOps – 1, inclusive, where each sub-bitstream subBitstream[ i ] is the output of the sub-bitstream extraction process of subclause 10.1 with inputs tIdTarget equal to maxTemporalId[ i ] and targetDecLayerIdSet equal to nestingLayerIdSet[ i ].

nesting_zero_bit shall be equal to 0.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).