| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0180 |
| --- | --- |

| Title: | AHG9: Operation points in VPS and nesting SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

A problem was identified in JCTVC-J0074 that when the conformance of a bitstream containing scalable layers is checked using HEVC v1 standard or an HEVC v1 decoder decodes a bitstream containing scalable layers, the HRD parameters of the “highest” layer present in the bitstream should be used. As a solution to the problem, HEVC draft 8 includes a possibility to include multiple sets of HRD parameters for different bitstream subsets into VPS. It is asserted in this contribution that while HEVC draft 8 enables multiple sets of HRD parameters, it still lacks the capability of indicating different initial delays for bitstream subsets.

JCTVC-K0180 proposes:

- Specifying operation points as a separate loop in VPS rather than as part of the loop for the HRD parameters.

- Specifying a scalable nesting SEI message.

It is asserted that in HEVC v1, the proposed scalable nesting SEI message can be used for two purposes:

- Indicate that the nested SEI messages pertain to a range of temporal sub-layers rather than all sub-layers.

- Indicate buffering period and picture timing for any bitstream subset, such as the base layer sub-bitstream.

# Introduction

A problem was identified in JCTVC-J0074 that when the conformance of a bitstream containing scalable layers is checked using HEVC v1 standard or an HEVC v1 decoder decodes a bitstream containing scalable layers, the HRD parameters of the “highest” layer present in the bitstream should be used. As a result, JCT-VC decided to add signaling of HRD parameters for bitstream subsets into VPS on the basis of document JCTVC-J0562.

It is asserted in this contribution that while the adopted contribution JCTVC-J0562 allows sending different sets of HRD parameters, it still lacks the capability of indicating different initial delays for bitstream subsets. The buffering period and picture timing SEI messages present in the bitstream indicate the timing for the entire bitstream as decoded by HEVC v1 decoder.

This contribution proposes:

- Specifying operation points as a separate loop in VPS rather than as part of the loop for the HRD parameters. The listed operation points are indexed in their appearance order, and the index is used to refer to a particular operation point from other structures. The change enables

  - including HRD parameters for a different number of operation points than the total list of operation points, and

  - referring to operation points specified in VPS even if no HRD parameters were provided for each operation point.

- Specifying a scalable nesting SEI message which is similar to the SVC and MVC scalable nesting SEI messages. In HEVC v1, a scalable nesting SEI message can be used for two purposes:

  - Indicate that the nested SEI messages pertain to a range of temporal sub-layers rather than all sub-layers.

  - Indicate buffering period and picture timing for any operation point (or the respective bitstream subset). Hence, a HEVC v1 decoder can extract and decode the base layer sub-bitstream from a scalable HEVC bitstream with the buffering period and picture timing SEI messages indicated to pertain to base layer sub-bitstream.

# Proposal

## Video Parameter Set

The proposed changes to video_parameter_set_rbsp are indicated in red font color below.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| reserved_zero_2bits | u(2) |
| max_num_layers_minus1 //reserved_zero_6bits in the base spec | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| next_essential_info_byte_offset //reserved_zero_12bits in the base spec | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| num_op_points_minus1 | ue(v) |
| for( i = 1; i <= num_op_points_minus1; i++ ) { |  |
| operation_point( i ) |  |
| } |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| hrd_op_point_idx | ue(v) |
| if( i > 0 ) |  |
| operation_point( i ) |  |
| hrd_parameters( hrd_op_point_idxi = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

num_op_points_minus1 specifies the number of operation_point syntax structures present in the video_parameter_set_rbsp syntax structure.

operation_point( i ) syntax structure in the video_parameter_set_rbsp syntax structure specifies the operation point, which has index i. operation_point( 0 ) is inferred to include op_num_layer_id_values_minus1[ 0 ] equal to 0 and op_layer_id[ 0 ][ 0 ] equal to 0.

hrd_op_point_idx specifies the index for the operation point for which the hrd_parameters syntax structure is provided.

## Scalable Nesting SEI Message

The scalable nesting SEI message provides a mechanism for associating SEI messages with subsets of a bitstream.

A scalable nesting SEI message shall contain one or more SEI messages. An SEI message contained in a scalable nesting SEI message is referred to as a nested SEI message. An SEI message not contained in a scalable nesting SEI message is referred to as a non-nested SEI message.

A buffering period SEI message and an SEI message of any other type shall not be nested in the same scalable nesting SEI message. A picture timing SEI message and an SEI message of any other type shall not be nested in the same scalable nesting SEI message.

| scalable_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| bitstream_subset_flag | u(1) |
| nesting_max_temporal_id_plus1 | u(3) |
| nesting_op_point_flag | u(1) |
| if( nesting_op_point_flag ) |  |
| nesting_op_point_idx | ue(v) |
| else { |  |
| all_layers_flag | u(1) |
| if( !all_layers_flag ) { |  |
| nesting_num_layers_minus1 | ue(v) |
| for( i = 0; i <= nesting_num_layers_minus1; i++ ) |  |
| nesting_layer_id[ i ] | u(6) |
| } |  |
| } |  |
| while( !byte_aligned( ) ) |  |
| nesting_zero_bit /* equal to 0 */ | u(1) |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| } |  |

bitstream_subset_flag equal to 0 specifies that the nested SEI messages apply to specific layers and sub-layers within the bitstream. bitstream_subset_flag equal to 1 specifies that the nested SEI messges apply to a sub-bitstream resulting from a sub-bitstream extraction process of subclause 10.1 with inputs specified by the syntax elements of the scalable nesting SEI message as specified below.

nesting_max_temporal_id_plus1 greater than 0 specifies that the variable maxTemporalId is equal to nesting_max_temporal_id_plus1 – 1. nesting_max_temporal_id_plus1 equal to 0 is reserved.

nesting_op_point_flag equal to 0 specifies that the set nestingLayerIdSet is specified by all_layers_flag and nesting_layer_id[ i ], if present. nesting_op_point_flag equal to 1 specifies that set nestingLayerIdSet is specified by nesting_op_point_idx.

nesting_op_point_idx specifies that the set nestingLayerIdSet consists of op_layer_id[ nesting_op_point_idx ][ i ] with all values of i in the range of 0 to op_num_layer_id_values_minus1[ nesting_op_point_idx ], inclusive, where op_layer_id and op_num_layer_values_minus1 values are from the active video parameter set RBSP.

all_layers_flag equal to 0 specifies that the set nestingLayerIdSet is specified by nesting_layer_id[ i ]. all_layers_flag equal to 1 specifies that the set nestingLayerIdSet consists of all values of reserved_zero_6bits present in the bitstream.

nesting_num_layers_minus1 + 1 specifies the number of values in nestingLayerIdSet.

nesting_layer_id[ i ] specifies the the set nestingLayerIdSet. nestingLayerIdSet consists of nesting_layer_id[ i ] with all values of i from 0 to nesting_num_laers_minus1, inclusive.

When bitstream_subset_flag is equal to 0, the nested SEI messages apply to NAL units with reserved_zero_6bits having one of the values in the set nestingLayerIdSet and temporal_id_plus1 in the range of temporal_id_plus1 of the SEI NAL unit to maxTemporalId + 1, inclusive.

When bitstream_subset_flag is equal to 1, the nested SEI messages apply to the sub-bitstream, whitch is the output of the sub-bitstream extraction process of subclause 10.1 with inputs tIdTarget equal to maxTemporalId and targetDecLayerIdSet equal to nestingLayerIdSet.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).