| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0181 |
| --- | --- |

| Title: | AHG7: Options present in Extended Chroma Format model |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal and Information |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Limited |  |  |

_____________________________

# Abstract

In this proposal, the 4:2:2 / 4:4:4 AHG7 model, which was previously proposed in JCT-VC J0191 and extended from the last meeting to also include the functions of JCT-VC J0357, is examined. Many of the available options are individually tested to obtain BD-rate changes and processing times, using the test material agreed upon at the previous meeting (JCT-VC J0581). Benchmarks against AVC JM18.4 are also given for comparison.

It is proposed that the committee examine these results to help agree on a base 4:2:2 and 4:4:4 (and 4:0:0) system, which may then be further developed. It is also proposed that the committee consider the operating points and test conditions for these new formats.

# Introduction

During the 10th JCT-VC conference, two proposals presented models for extending the HEVC HM 7 model to include 4:0:0, 4:2:2 and 4:4:4 chroma formats, namely JCT-VC J0191 and JCT-VC J0357. The JCT-VC J0191 model was extended to include the various features of JCT-VC J0357, and also patched to HM7.1.

The resulting model was chosen for use by AHG7, whose members were interested in the use of these additional chroma formats. Areas where the chroma and luma processing diverge had been identified in the software model, but no decision was actually made on the default choices for these areas for the new chroma formats (4:2:2, 4:4:4). However, now that an agreed initial set of sequences and test conditions have been identified (JCT-VC J0581), these options now need to be examined.

Following the meeting, the JCT-VC J0191 software was patched to HM8.0 and supplied to AHG7 via a dedicated software branch, in combination with a set of image sequences.

This document presents the results from making individual and some combined changes to the current default options in the AHG7 software.

# Methodology

The software for AHG7 was run using the standard intra_he10 and lowdelay_he10 HM profiles, and at the standard set of 4 QPs (22, 27, 32, 37). In order to get the most accurate processing times, the environment variable control method was not utilized (as this can add ~2% on to the processing times), and instead the code was recompiled having set the appropriate macro values.

The 10-bit source material was provided in both 4:2:2 and 4:4:4 formats by the AHG7 Chairman, and is illustrated in Figure 1.

Kimono	ParkScene	BirdsInCage	DucksAndLegs

Traffic	CrowdRun	OldTownCross	Seeking

Figure 1 - First frames of AHG7 test material for 4:4:4 and 4:2:2

For the simulation results presented in this proposal, only the 10-bit profiles have been run, as previous 4:2:2 and 4:4:4 simulations (which have been distributed via the reflector) indicated that there was correspondence between the 8 and 10-bit profiles. It is proposed that the committee consider the operating points for these new formats.

## Available options

The AHG7 ECF model provides 44 different option types (which are listed in categories in the coloured tables below, along with their current default values):

- 33 of these options are tested, and results reported in section 4.

- 1 of these options (highlighted in yellow) is deprecated following the removal of LMChroma from HM8.0.

- 1 of these options (highlighted in orange) relates to a new chroma scaling list, which is not tested here because a unique scaling list would need to be proposed in order for an effect to be observed.

- 1 of these options (highlighted in orange) relates to additional / alternative chroma QP mapping tables and is not considered at this stage.

- 8 of these options (highlighted in grey) relate to controlling block sizes of MDCS, but are not tested here.

# Base Results

The base AHG7 ECF model (with the current default options) was run and compared against AVC JM18.4. The JM18.4 model was run with the included 8-bit HM-like configuration files, although noting that weighted prediction (and weighted bi-prediction) had to be disabled for 4:4:4 simulations. The JM18.4 produced a decoder mismatch for Ducks-and-legs, random_access, QP27. The tables below show the comparative results, though note that the JM18.4 model was only run in 8-bit mode.

Figure 2 - Comparison of AHG7-ECF8.0 (8/10 bit) vs. AVC JM18.4 (8-bit).

# Configuration Results

The results are split into different sections below, corresponding to the tables of controls described in Section 2.1. The results are presented in a similar way to other results that have been distributed via the reflector: changes (>=0.1%) in BD-rate or run-time (>=5%) are highlighted in red or green, but it is important to note that this is only for the purposes of illustration. An accompanying spreadsheet allows these thresholds to be adjusted.

Although BD-rates and run-times are important, it should be noted that some tools may result in a complexity reduction that is not observed numerically.

Note that this revision (and the previous revision) of the document includes results from the accompanying code that corresponds to HM8.1; this revision includes the full set of 4:4:4 low-delay results. This accompanying code includes a change to RDOQ (although there is a possibility that a common issue with RDOQ remains in HM8.1 and in the ECF8.1), and a bug-fix, as documented in JCTVC-K0322, both of which affected 4:2:2 results. No significant differences are expected for 4:4:4 (or 4:2:0) simulation results. This 8.1 version of the AHG7 code also increases support for higher bit depths.

The simulation results for the 8.1 code are shown on the top half of each table (with a white background); the simulation results for the 8.0 code are shown on the bottom half of each table (with a grey background).

  - Block Structures

## Intra prediction and Intra prediction search

## Intra and inter prediction filters

## Transform and Quantizer

As can be seen in the above ECF8.0 results, there are large BD-rate changes for functions that should be almost mathematically equivalent. The causes of these are RDOQ-based encoder functions, and these have been modified in the HM8.1 equivalent release of the AHG7 code (ECF8.1).

## Entropy coding

# Combinations of different options

Looking at the results from the previous section, it can be seen that experiments 01b, 04d, 04e (which are investigations into intra-based options), 07d (which is an investigation into an inter and intra-based set of options) produce some BD-rate improvements for little effect on run-times.

However, experiment 07d increases the number of context variables required, and therefore this option is not without cost. Due to this, 07d has been broken down further into its three constituent parts.

As with the previous section, the ECF8.1 and ECF8.0 results are shown in the following tables.

# Conclusion

This document presents a comparison of the AHG7 ECF8.0 source code against AVC JM18.4. In addition many simulation results produced by varying the available options in the AHG7 ECF source code are described.

It is proposed that the committee consider these results when deciding upon the best set of options for a base 4:2:2 and 4:4:4 system, although noting the observation in Section 4.4 regarding the RDOQ functionality. It is also proposed that the committee consider the operating points for these new chroma formats.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).