# Proposed Text

Based on JCTVC-K0030_v6.

7.3.8.1	General slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| do { |  |
| coding_tree_unit( ) |  |
| end_of_slice_flag | ae(v) |
| CtbAddrTS++ |  |
| CtbAddrRS = CtbAddrTStoRS[ CtbAddrTS ] |  |
| if( !end_of_slice_flag && ( ( tiles_enabled_flag && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( entropy_coding_sync_enabled_flag && CtbAddrTS % PicWidthInCtbsY = = 0 ) ) ) { |  |
| end_of_sub_stream_one_bit /* equal to 1 */ | ae(v) |
| byte_alignment( ) |  |
| } |  |
| } while( !end_of_slice_flag ) |  |
| } |  |

7.4.8	Slice data semantics

7.4.8.1	General slice data semantics

end_of_slice_flag equal to 0 specifies that another coding tree unit is following in the slice. end_of_slice_flag equal to 1 specifies the end of the slice and that no further coding tree unit follows.

end_of_sub_stream_one_bit shall be equal to 1.

9.3.1.1	Initialization process for context variables

. . .

NOTE 1 – ctxIdxTable equal to 0 and ctxIdx equal to 0 are associated with the end_of_slice_flag, end_of_sub_stream_one_bit and pcm_flag. The decoding process specified in subclause Error: Reference source not found applies to ctxIdxTable equal to 0 and ctxIdx equal to 0. This decoding process, however, may also be implemented by using the decoding process specified in subclause Error: Reference source not found. In this case, the initial values associated with ctxIdxTable equal to 0 and ctxIdx equal to 0 are specified to be pStateIdx = 63 and valMPS = 0, where pStateIdx = 63 represents a non-adapting probability state.

9.3.1.2	Memorization process for context variables

Inputs of this process are the CABAC context variables indexed by ctxIdx.

Output of this process are variables tableStateSync and tableMPSSync containing the values of the variables pStateIdx and valMPS used in the initialization process of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found to Error: Reference source not found except for the end_of_slice_flag, end_of_sub_stream_one_bit and pcm_flag.

For each context variable, the corresponding entries pStateIdx and valMPS of tables tableStateSync and tableMPSSync are initialized to the corresponding pStateIdx and valMPS.

9.3.1.3	 Synchronization process for context variables

Inputs of this process are variables tableStateSync and tableMPSSync containing the values of the variables pStateIdx and valMPS used in the memorization process of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found to Error: Reference source not found except for the end_of_slice_flag, end_of_sub_stream_one_bit and pcm_flag.

Outputs of this process are the CABAC context variables indexed by ctxIdx..

For each context variable, the corresponding context variables pStateIdx and valMPS are initialized to the corresponding entries pStateIdx and valMPS of tables tableStateSync and tableMPSSync.

9.3.2	Binarization process

. . .

ctxIdxTable = 0 and ctxIdx = ctxIdxOffset = 0 are assigned to the syntax element end_of_slice_flag,end_of_sub_stream_one_bit and pcm_flag as further specified in subclause Error: Reference source not found. For parsing the value of the corresponding bin from the bitstream, the arithmetic decoding process for decisions before termination (DecodeTerminate) as specified in subclause Error: Reference source not found is applied.

| Table 9-1 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| . . . |  |  |  |  |  |
| end_of_slice_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| end_of_sub_stream_one_bit | all | FL, cMax = 1 | 0 | 0 | 0 |
| . . . |  |  |  |  |  |

9.3.3.2.4	Decoding process for binary decisions before termination

Inputs to this process are bits from slice data and the variables codIRange and codIOffset.

Outputs of this process are the updated variables codIRange and codIOffset, and the decoded value binVal.

This special decoding routine applies to decoding of end_of_slice_flag, end_of_sub_stream_one_bit and pcm_flag corresponding to ctxIdxTable equal to 0 and ctxIdx equal to 0. Error: Reference source not found shows the flowchart of the corresponding decoding process, which is specified as follows.

9.3.4.5	Encoding process for a binary decision before termination (informative)

This subclause does not form an integral part of this Specification.

Inputs to this process are the variables binVal, codIRange, codILow, bitsOutstanding, and BinCountsInNALunits.

Outputs of this process are zero or more bits written to the RBSP and the updated variables codILow, codIRange, bitsOutstanding, and BinCountsInNALunits.

This encoding routine shown in Error: Reference source not found applies to encoding of the end_of_slice_flag, end_of_sub_stream_one_bit and pcm_flag associated with ctxIdx equal to60.