| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0183 |
| --- | --- |

| Title: | AHG4: Dependent slices restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik Matthias Narroschke Thomas Wedi Monzastrasse 4c, 63225 Langen, Germany | Tel: Email: | +46 (0) 6103 766 - 1306 semih.esenlik@eu.panasonic.com |
| Source: | Panasonic Corp. |  |  |

_____________________________

# Abstract

The current HEVC standard includes two restrictions related to the coexistence of slices and WPP/tiles. However there is no separation between normal and dependent slices in the context of the restrictions. As a result the recently adopted dependent slices partially defeat the intended purposes of the restrictions that are imposed previously. In the document it is proposed to add limitations on the use of dependent slices in order to extend the scope of the existing two restrictions. The proposed limitations guarantee that when a parent slice and all following dependent slices are seen as a single slice, the already existing two restrictions are satisfied.

# Introduction

The dependent slices have been included in the HEVC standard during the 9th JCT-VC Meeting in Geneva [1]. Later the dependent slices are included in the “Main” profile of the HEVC during the 10th JCT-VC Meeting that was held in Stockholm.

A dependent slice is a slice that has dependencies to the previously coded slice [1]. The dependencies include the following:

- Parsing dependency: The dependent slices utilize a short slice header where the missing elements are copied from the parent slice. Moreover CABAC context model states are not initialized to defaults at the beginning of a dependent slice which introduces dependency also in the CABAC parsing process.

- Spatial prediction: The dependent slices do not prevent spatial dependencies such as motion vector prediction or intra prediction. During the pixel reconstruction step, all adjacent preceding dependent slices up to and including the first normal slice are required for the pixel reconstruction process.

According to the HEVC specification text [2] two restrictions are imposed on the joint usage of slices and tiles/WPP.

- For slices & tiles:

NOTE 1 – When tiles_enabled_flag is equal to 1 and entropy_coding_sync_enabled_flag is equal to 0, each slice must include either a subset of the coding tree units of one tile (in which case the syntax element entry_point_offset[ i ] is not present) or must include all coding tree units of an integer number of complete tiles.

- For slices & WPP:

When entropy_coding_sync_enabled_flag is equal to 1 and the first coding block in a slice is not the first coding block in the first coding tree block of a row of coding tree blocks in a tile, it is a requirement of bitstream conformance that the last coding block in the slice shall belong to the same row of coding tree blocks as the first coding block in the slice.

## Usage of Dependent slices with WPP

Figure 1: WPP with dependent slices

When the usage of dependent slices is enabled, the following ordered steps are applied for the initialization of the CABAC engine at the beginning of a CTB row according to [1].

- If the current sub-stream is the first sub-stream in a normal slice; then initialize the CABAC context model states to the default initial values.

- If the current sub-stream is the first sub-stream in a dependent slice, then check all of the slices at the upper CTB row.

  - If there is at least one slice satisfying the following conditions:

    - slice_address % picWidthInCTBs > 1

    - the slice is a normal slice

  - then initialize the CABAC context model states to the default initial values.

  - Otherwise initialize the CABAC context model states using the states of the second CTB in the upper CTB row.

- If the current sub-stream is not the first sub-stream in the current slice (dependent or normal); then initialize the CABAC context model states using the states of the second CTB in the upper CTB row.

Figure 1 exemplifies the initialization of the CABAC context model states for the dependent slice (4). Parsing of slice (4) is dependent on slice (1) (which is a normal slice) since the slice header and the CABAC contexts are inherited from it. Moreover the parsing of slice (4) also depends on slice (2) and (3) since the slice type (normal slice or dependent slice) is required for the availability checking process of the CABAC initialization operation. In the example if any of the slices (2) or (3) had been a normal slice, the CABAC engine would have been initialized to default initial states (instead of using the WPP tables from the upper right CTB).

The dependent slices introduce parsing dependency on multiple previous slices as it is described in the second step of CABAC initialization process. If a CTB row starts with a dependent slice, all of the slices in the upper CTB row are required for the initialization of the CABAC engine.

# Problem Statement

The intention of the dependent slices is to provide an efficient bitstream fragmentation method. It is particularly useful since the dependent slice header works as an entry point marker and can eliminate the need for explicit signalling of the entry point offsets in the slice header, allowing transmission without delay.

The current HEVC standard [2] includes restrictions related to the joint usage of slices together WPP/tiles. However there is no separation between the normal and dependent slices in the context of the restrictions. As a result the dependent slices partially defeat the intended purposes of the restrictions that are imposed previously.

The problems are explained in detail in the following subsections.

## Limitation on minimum latency

The joint use of WPP and dependent slices can be useful in ultra-low delay applications such as online gaming and remote surgery. The encoding (and decoding) latency can be reduced without a significant loss of coding gain since:

- WPP enables faster processing with multiple low-power encoder/decoder cores.

- Dependent slices provide a low-delay capable machanism for the fragmentation of the bitstream.

The total latency between the source and the receiver can be defined as the time difference between the first pixel of the frame appearing in the source and the first pixel of decoded frame appearing at the receiver. In practical applications the latency is the function of the following:

- Encoding/decoding latency for the first slice (or the first slice fragment) of the frame.

- Transmission latency.

Therefore in time critical applications the encoding/decoding delay can be adjusted by adjusting of the slice size and/or fragmentation of the packets before transmission.

Figure 2: Dependent slices and WPP

However the use of dependent slices limits the possibility of reducing the total latency by creating a non-causal bitstream. Figure 2 shows an example slice configuration to reduce latency. In the figure the slices (1) and (3) are available in the encoder long before the slices (2) and (4) if the encoder utilizes multiple processing cores. However the decoding of slice (3) cannot be started in parallel with slice (1), since the initialization of the CABAC engine is dependent on the types of all of the slices in the upper CTB row (The decoder has to consider the possibility that slice (2) can be a normal slice). Therefore the dependent slices introduce a fixed limit on the minimum encoding/decoding latency which is 1 CTB row.

## Error resilience problem

Figure 3: Loss resilience properties of dependent slices.

In the figure above an example is depicted where the slice (2) is assumed to be lost in the network.

In the example the slice data of slice (3) and the following dependent slices are can be parsed by the CABAC engine, since the slice header and the CABAC states are to be inherited from slice (1). Therefore the whole picture can be reconstructed with small degradation in picture quality.

However in a real scenario, it is not possible to know the type (dependent or normal) of the lost slice in the decoder. If the lost slice (slice (2) in the above figure) was a normal slice, the slice header would have been inherited from it and the CABAC states would have been initialized to default initial values. Therefore slice (3) and the following dependent slices are forced to be discarded just because the slice type (normal or dependent) of the lost slice is unknown.

## Dependent slices with tiles

Figure 4: Usage of dependent slices with tiles

The two problems that are described for the joint usage of WPP and dependent slices exist also in the case of tiles. The dependent slices introduce parsing dependency to all slices in the previous tile, therefore in order to be able to start CABAC decoding of slice (3), all of the slices in the previous tile needs to be available.

Moreover the dependent slices pose additional problems in the case of decoding a tile-partitioned frame with regular z-scan order (single core decoding). In the 8th JCT-VC meeting in San Jose a bitstream restriction has been adopted that governs the joint usage of slices and tiles. The restriction simplified the decoding of the tile-partitioned frame by the single core decoders that follow the regular z-scan order.

However the adoption of the dependent slices reintroduced the same problem. In order to start decoding slice (3) in Figure 4, either one of the following is required:

- Up-front parsing of all of the slice headers in the previous tile.

- Additional back-and-forth jumping in the bitstream in order to discover parent-dependent slice relationships.

# Proposal

In the document it is proposed to add limitations on the use of dependent slices in order to extend the scope of the existing two restrictions. The proposed limitations on the use of dependent slices guarantee that when a parent slice and all following dependent slices are seen as a single slice, the already existing two restrictions (that are presented in the Introduction Section) are satisfied.

## Dependent slices restriction for WPP

Two alternative bitstream conformance restrictions are proposed for the joint usage of WPP with dependent slices.

Alternative 1: “If WPP is enabled and if a normal slice starts after the beginning of a CTB row, then the next CTB row shall not start with a dependent slice.”

Alternative 2: “If dependent slices and WPP are both enabled, normal slices shall start only at the beginning of a CTB row.”

Both restrictions restore the causality in the bitstream for the joint usage of WPP and dependent slices, solving the problems discussed above.

According to both alternatives if a dependent slice is combined with a preceding slice in coding order (replacing the dependent slice header with entry point offset), the restriction governing the joint usage of slices and WPP is guaranteed to be satisfied.

## Dependent slices restriction for tiles

Two alternative bitstream conformance restrictions are proposed for the joint usage of tiles with dependent slices.

Alternative 1: “If the first CTB of a normal slice is not the first CTB of a tile, then the next tile shall not start with a dependent slice.”

Alternative 2: “If a dependent slice starts at the beginning of a tile, then both of the preceding and proceeding slices shall start at the beginning of a tile.”

Both restrictions restore the causality in the bitstream for the joint usage of tiles and dependent slices, solving the problems discussed above.

The first proposed alternative for tiles is essentially same as the first restriction alternative for the WPP case.

According to the second alternative if a dependent slice is combined with a preceding slice in coding order (replacing the dependent slice header with entry point offset), the restriction governing the joint usage of slices and tiles is guaranteed to be satisfied.

# Conclusion

In the document it is proposed to add limitations on the use of dependent slices in order to extend the scope of the existing two restrictions. The proposed limitations on the use of dependent slices guarantee that when a parent slice and all following dependent slices are seen as a single slice, the already existing two restrictions are satisfied.

# References

- T.	Schierl, V.	George, A.	Henkel, D.	Marpe, “Dependent Slices”, JCT-VC Document, JCTVC-I0229, 9th Meeting: Geneva, CH, April, 2012.

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 8”, JCT-VC Output Document, JCTVC-J1003_d7, 10th Meeting: Stockholm, Sweden, July, 2012.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).