| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0185 |
| --- | --- |

| Title: | Crosscheck of JCTVC-K0150 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | TK Tan | Tel: Email: | +65 97993861 tktan@m-sph.com |
| Source: |  |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck of contribution JCTVC-K0150.  The software was examined and confirmed to be implementing the proposal in JCTVC-K0150.  The simulation was conducted and objective results matching those reported in JCTVC-K0150 were obtained.  The subjective quality of the test sequences were observed to reduce the blocking artefacts found in some of the critical sequences.

# Summary

Software for the crosscheck was provided by Sony. The software was inspected.  The change to the HM8.0 was confined to the file TComLoopFilter.cpp.  However, CE1 related changes were also included in TComLoopFilter.cpp and TComPrediction.cpp but disabled in TypeDef.h.

It is confirmed that the software essentially weighted the values of β and Tc and with (Bs + 1) for the decision on using the strong filter in sub clause 8.7.2.4.3	Decision process for luma block edge, when dSam0 and dSam3 is decided.

#if SONY_DEBLOCK_K0150

Bool sw =  xUseStrongFiltering( iOffset, 2*d0, (uiBs+1)*iBeta, (uiBs+1)*iTc, piTmpSrc+iSrcStep*(iIdx*uiPelsInPart+iBlkIdx*4+0))

&& xUseStrongFiltering( iOffset, 2*d3, (uiBs+1)*iBeta, (uiBs+1)*iTc, piTmpSrc+iSrcStep*(iIdx*uiPelsInPart+iBlkIdx*4+3));

#else

Bool sw =  xUseStrongFiltering( iOffset, 2*d0, iBeta, iTc, piTmpSrc+iSrcStep*(iIdx*uiPelsInPart+iBlkIdx*4+0))

&& xUseStrongFiltering( iOffset, 2*d3, iBeta, iTc, piTmpSrc+iSrcStep*(iIdx*uiPelsInPart+iBlkIdx*4+3));

#endif

Configuration files were provided for the test1 sequences.

It was confirmed that the configuration files had the same parameter settings as the encoder_random_main.cfg and encoder_lowdelay_main.cfg of the HM8.0-tag software, with the exception of the IntraPeriod in the random access configuration, which is specific to the sequences.

# Results

The objective results for the common test sequences were obtained and found to match the results reported in the spreadsheets attached to JCTVC-K0150.  Two configurations were tested;  Part1: HM8.0 + (Bs+1) and Part2: HM8.0 + (Bs+1) + disable Intra boundary smoothing

Table 1: Results of Part1: Comparison of HM8.0 + (Bs+1) vs. HM8.0 for common test sequences

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.17% | -0.01% | -0.01% | 0.25% | 0.00% | -0.01% |
| Class B | 0.18% | -0.02% | -0.01% | 0.29% | -0.02% | -0.02% |
| Class C | 0.12% | -0.02% | -0.02% | 0.18% | -0.02% | -0.02% |
| Class D | 0.10% | -0.01% | -0.01% | 0.15% | -0.01% | -0.01% |
| Class E | 0.01% | -0.04% | -0.03% | 0.09% | -0.04% | -0.04% |
| Class F | 0.04% | -0.01% | 0.00% | 0.08% | 0.00% | -0.01% |
| Overall | 0.11% | -0.02% | -0.01% | 0.18% | -0.01% | -0.01% |
|  | 0.11% | -0.02% | -0.01% | 0.18% | -0.01% | -0.01% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.07% | -0.25% | -0.30% | 0.05% | -0.04% | -0.28% |
| Class B | 0.06% | 0.07% | -0.01% | 0.15% | -0.04% | -0.01% |
| Class C | 0.05% | -0.05% | 0.00% | 0.13% | 0.03% | -0.01% |
| Class D | 0.04% | -0.04% | -0.16% | 0.02% | -0.17% | -0.12% |
| Class E |  |  |  |  |  |  |
| Class F | 0.01% | -0.09% | -0.08% | 0.10% | 0.01% | -0.06% |
| Overall | 0.02% | -0.06% | -0.10% | 0.09% | -0.04% | -0.09% |
|  | 0.02% | -0.07% | -0.10% | 0.09% | -0.03% | -0.10% |
| Enc Time[%] | 98% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.23% | -0.07% | 0.12% | 0.09% | 0.20% |
| Class C | 0.06% | -0.06% | 0.15% | 0.09% | 0.19% | 0.12% |
| Class D | -0.02% | 0.17% | -0.17% | 0.04% | -0.28% | -0.28% |
| Class E | 0.15% | 0.32% | 0.74% | 0.28% | 0.16% | -0.20% |
| Class F | 0.00% | 0.14% | -0.22% | -0.02% | -0.04% | -0.96% |
| Overall | 0.03% | 0.15% | 0.04% | 0.09% | 0.02% | -0.20% |
|  | 0.03% | 0.18% | 0.07% | 0.09% | 0.00% | -0.14% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.17% | 0.09% | 0.05% | -0.12% | 0.18% | -0.51% |
| Class C | 0.00% | -0.03% | 0.24% | 0.02% | -0.21% | -0.12% |
| Class D | 0.00% | 0.47% | -0.04% | 0.06% | -0.18% | 0.00% |
| Class E | -0.20% | 0.23% | -0.16% | -0.21% | -0.33% | 0.03% |
| Class F | -0.09% | -0.17% | 0.75% | -0.11% | 0.99% | -0.05% |
| Overall | -0.09% | 0.11% | 0.18% | -0.07% | 0.11% | -0.16% |
|  | -0.09% | 0.06% | 0.16% | -0.07% | 0.03% | -0.18% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

Table 2: Results of Part2: Comparison of HM8.0 + (Bs+1) + disable Intra boundary smoothing vs. HM8.0 for common test sequences

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.24% | 0.13% | 0.15% | 0.34% | 0.17% | 0.19% |
| Class B | 0.16% | 0.06% | 0.06% | 0.31% | 0.07% | 0.06% |
| Class C | 0.13% | 0.04% | 0.01% | 0.19% | 0.04% | -0.02% |
| Class D | 0.11% | 0.01% | 0.01% | 0.16% | 0.04% | 0.05% |
| Class E | 0.00% | -0.01% | 0.00% | 0.09% | 0.03% | -0.01% |
| Class F | 0.01% | 0.02% | 0.01% | 0.09% | -0.10% | 0.04% |
| Overall | 0.11% | 0.05% | 0.04% | 0.21% | 0.04% | 0.05% |
|  | 0.11% | 0.04% | 0.05% | 0.21% | 0.05% | 0.05% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.07% | -0.31% | -0.06% | 0.07% | 0.00% | -0.16% |
| Class B | 0.03% | 0.02% | 0.12% | 0.15% | 0.09% | 0.09% |
| Class C | 0.06% | 0.05% | 0.15% | 0.11% | 0.11% | -0.07% |
| Class D | 0.06% | -0.11% | 0.02% | 0.03% | -0.08% | -0.26% |
| Class E |  |  |  |  |  |  |
| Class F | 0.01% | -0.05% | -0.07% | 0.07% | 0.08% | 0.13% |
| Overall | 0.02% | -0.07% | 0.04% | 0.09% | 0.04% | -0.05% |
|  | 0.02% | -0.07% | 0.04% | 0.09% | 0.03% | -0.07% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.03% | 0.36% | 0.31% | 0.13% | 0.14% | 0.35% |
| Class C | 0.08% | -0.05% | -0.10% | 0.08% | -0.01% | 0.10% |
| Class D | -0.02% | 0.50% | 0.12% | 0.06% | -0.63% | 0.29% |
| Class E | 0.17% | 0.35% | 0.87% | 0.14% | 0.16% | -0.27% |
| Class F | 0.05% | -0.03% | -0.11% | 0.11% | 0.89% | -0.80% |
| Overall | 0.04% | 0.22% | 0.19% | 0.10% | 0.11% | -0.04% |
|  | 0.04% | 0.24% | 0.19% | 0.11% | 0.06% | -0.07% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.16% | 0.33% | 0.40% | -0.13% | 0.23% | -0.10% |
| Class C | -0.01% | 0.09% | 0.17% | 0.04% | -0.09% | -0.14% |
| Class D | 0.05% | 0.20% | -0.05% | 0.05% | -0.17% | -0.03% |
| Class E | -0.15% | 1.13% | -0.36% | -0.19% | -0.23% | 0.26% |
| Class F | 0.09% | 0.14% | 0.54% | 0.15% | 0.52% | -0.22% |
| Overall | -0.04% | 0.34% | 0.18% | -0.01% | 0.08% | -0.06% |
|  | -0.04% | 0.31% | 0.22% | -0.01% | 0.01% | 0.03% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

The objective results for the test 1 sequences match the results reported in JCTVC-K0150.

Table 3: Results of Part1: Comparison of HM8.0 + (Bs+1) vs. HM8.0 for test 1 sequences

| Condition | Sequence | QP | Item | Bitrate [kbps] | Ypsnr [db] | Upsnr [dB] | Vpsnr [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LB-Main | Riverbed | 32 | hm8.0 | 5851.44 | 34.64 | 38.96 | 41.40 |
|  |  |  | Part1 | 5850.89 | 34.64 | 38.96 | 41.40 |
|  |  | 37 | hm8.0 | 2870.61 | 31.71 | 38.13 | 40.91 |
|  |  |  | Part1 | 2868.57 | 31.74 | 38.12 | 40.90 |
|  |  | 41 | hm8.0 | 1566.17 | 29.55 | 37.69 | 40.63 |
|  |  |  | Part1 | 1564.63 | 29.58 | 37.69 | 40.63 |
|  | WestWindEasy | 32 | hm8.0 | 3758.62 | 36.22 | 44.74 | 47.46 |
|  |  |  | Part1 | 3751.72 | 36.22 | 44.72 | 47.45 |
|  |  | 37 | hm8.0 | 1679.87 | 33.01 | 43.71 | 46.65 |
|  |  |  | Part1 | 1674.61 | 33.02 | 43.72 | 46.65 |
|  |  | 41 | hm8.0 | 854.87 | 30.75 | 42.93 | 45.86 |
|  |  |  | Part1 | 853.37 | 30.77 | 42.96 | 45.83 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1270.60 | 35.30 | 40.24 | 40.18 |
|  |  |  | Part1 | 1269.18 | 35.30 | 40.22 | 40.19 |
|  |  | 37 | hm8.0 | 612.05 | 31.97 | 38.75 | 38.42 |
|  |  |  | Part1 | 610.89 | 31.97 | 38.75 | 38.42 |
|  |  | 41 | hm8.0 | 316.87 | 29.38 | 37.76 | 36.85 |
|  |  |  | Part1 | 317.09 | 29.40 | 37.75 | 36.84 |
| RA-Main | DucksTakeOff | 32 | hm8.0 | 9214.38 | 29.56 | 32.50 | 36.73 |
|  |  |  | Part1 | 9215.33 | 29.57 | 32.50 | 36.73 |
|  |  | 37 | hm8.0 | 3954.07 | 27.26 | 31.55 | 36.27 |
|  |  |  | Part1 | 3954.29 | 27.27 | 31.56 | 36.27 |
|  |  | 41 | hm8.0 | 1993.55 | 25.69 | 30.96 | 35.98 |
|  |  |  | Part1 | 1993.78 | 25.70 | 30.96 | 35.98 |
|  | RedKayak | 32 | hm8.0 | 3709.59 | 35.65 | 41.82 | 44.91 |
|  |  |  | Part1 | 3707.87 | 35.65 | 41.82 | 44.91 |
|  |  | 37 | hm8.0 | 1714.38 | 32.94 | 41.01 | 44.13 |
|  |  |  | Part1 | 1713.79 | 32.95 | 41.01 | 44.14 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.42 | 43.49 |
|  |  |  | Part1 | 853.64 | 30.97 | 40.42 | 43.48 |

Table 4: Results of Part2: Comparison of HM8.0 + (Bs+1) + disable Intra boundary smoothing vs. HM8.0 for test 1 sequences

| Condition | Sequence | QP | Item | Bitrate [kbps] | Ypsnr [db] | Upsnr [dB] | Vpsnr [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LB-Main | Riverbed | 32 | hm8.0 | 5851.44 | 34.64 | 38.96 | 41.40 |
|  |  |  | Part2 | 5857.27 | 34.65 | 38.96 | 41.40 |
|  |  | 37 | hm8.0 | 2870.61 | 31.71 | 38.13 | 40.91 |
|  |  |  | Part2 | 2869.09 | 31.74 | 38.13 | 40.90 |
|  |  | 41 | hm8.0 | 1566.17 | 29.55 | 37.69 | 40.63 |
|  |  |  | Part2 | 1566.48 | 29.59 | 37.69 | 40.63 |
|  | WestWindEasy | 32 | hm8.0 | 3758.62 | 36.22 | 44.74 | 47.46 |
|  |  |  | Part2 | 3756.13 | 36.22 | 44.71 | 47.50 |
|  |  | 37 | hm8.0 | 1679.87 | 33.01 | 43.71 | 46.65 |
|  |  |  | Part2 | 1676.92 | 33.03 | 43.65 | 46.65 |
|  |  | 41 | hm8.0 | 854.87 | 30.75 | 42.93 | 45.86 |
|  |  |  | Part2 | 853.63 | 30.75 | 43.05 | 46.04 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1270.60 | 35.30 | 40.24 | 40.18 |
|  |  |  | Part2 | 1270.78 | 35.29 | 40.24 | 40.19 |
|  |  | 37 | hm8.0 | 612.05 | 31.97 | 38.75 | 38.42 |
|  |  |  | Part2 | 611.09 | 31.97 | 38.68 | 38.41 |
|  |  | 41 | hm8.0 | 316.87 | 29.38 | 37.76 | 36.85 |
|  |  |  | Part2 | 316.91 | 29.40 | 37.75 | 36.85 |
| RA-Main | DucksTakeOff | 32 | hm8.0 | 9214.38 | 29.56 | 32.50 | 36.73 |
|  |  |  | Part2 | 9218.29 | 29.57 | 32.49 | 36.73 |
|  |  | 37 | hm8.0 | 3954.07 | 27.26 | 31.55 | 36.27 |
|  |  |  | Part2 | 3957.53 | 27.27 | 31.56 | 36.27 |
|  |  | 41 | hm8.0 | 1993.55 | 25.69 | 30.96 | 35.98 |
|  |  |  | Part2 | 1997.35 | 25.70 | 30.96 | 35.97 |
|  | RedKayak | 32 | hm8.0 | 3709.59 | 35.65 | 41.82 | 44.91 |
|  |  |  | Part2 | 3713.00 | 35.66 | 41.82 | 44.91 |
|  |  | 37 | hm8.0 | 1714.38 | 32.94 | 41.01 | 44.13 |
|  |  |  | Part2 | 1715.73 | 32.95 | 41.01 | 44.13 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.42 | 43.49 |
|  |  |  | Part2 | 854.00 | 30.98 | 40.42 | 43.48 |

# Observation

The subjective quality of the test 1 sequences was observed.  It is confirmed that part1 and part2 of the proposal in JCTVC-K0150 reduces the blocking artefacts in the riverbed and ducks_take_off sequences.  However the differences for the other sequences were not as obvious.  The following table summarizes the subjective observations. The properties of part1 and part2 are similar.  Comparison to other CE1 proposal was not done.

Table 5: Subjective observations for test 1 sequences

| Condition | Sequence | QP | Subjective Observation (Part 1 & Part 2) |
| --- | --- | --- | --- |
| LB-Main | Riverbed | 32 | not significantly different |
|  |  | 37 | blockiness reduced but problem is not solved |
|  |  | 41 | blockiness reduced but problem is not solved |
|  | WestWindEasy | 32 | not significantly different |
|  |  | 37 | not significantly different |
|  |  | 41 | not significantly different |
|  | F.ChinaSpeed | 32 | not significantly different |
|  |  | 37 | not significantly different |
|  |  | 41 | not significantly different |
| RA-Main | DucksTakeOff | 32 | not significantly different |
|  |  | 37 | blockiness reduced blockness almost not visible |
|  |  | 41 | blockiness reduced but problem is not solved |
|  | RedKayak | 32 | not significantly different |
|  |  | 37 | not significantly different |
|  |  | 41 | not significantly different' |