| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0186 |
| --- | --- |

| Title: | CE1: Reduction of block artifacts in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The document studies a problem with blocking artifacts in Riverbed, West Wind Easy and China Speed similar sequences and proposes modifications to HM8.0 that fix these problems. The proposed modifications reportedly reduce blocking artifacts on Riverbed, China Speed, West Wind Easy RedKayak sequences etc. and also improve the visual quality on some sequences in the common test conditions. The proposed modifications result in the following changes in BD-rate: (0.1%, 0.0%, 0.1% and -0.1%) on Main profile and (0.2%, 0.1%, 0.5%, -0.1%) on HE10 configuration (the results for test 2), the decoding time is similar to that of the anchor. The proposed solutions do NOT require sending adaptation deblocking parameters on the sequence basis. Instead, the proposed solutions attenuate the blocking artifacts in the default operating mode.

# Introduction

During the 10th meeting in Stockholm, four proposals have been presented in order solve to the problem with deblocking filter artifacts in the default operating mode [1], [2], [3], [4] in such sequences as Riverbed, West Winds Easy and China Speed. Based on the results of the expert viewing test [5], it has been decided to test combinations of tools mostly from J0286 [1] and J0181 [2] in the mandate of Core Experiment 1: deblocking filtering [6].

This document presents the results for several combinations of the proposed techniques including the objective results and the results on additional sequences.

# Source of the blocking artifacts in the reported sequences

## Location of blocking artifacts

The sequences reported in have been studied on the topic of reducing the blocking artifacts. The sequences have been encoded and the blocking artifacts studied. It has been found that the reported artifacts are mostly caused by intra-predicted blocks of a large size (e.g. 32x32), where the transform coefficients are coarsely quantized. The blocking artifacts are quite visible even in motion since the block boundaries are usually aligned with the 32x32 sample grid. The examples of blocking such blocks are shown in Figure 1 and Figure 2.

(a)

(b)

Figure 1 West Winds Easy, Low delay configuration, QP37.  (a) Blocking artifacts, (b) Picture divided into CUs, intra-coded CU are marked by red color.

(a)

(b)

Figure 2 Riverbed, Random Access y configuration, QP37.  (a) Blocking artifacts, (b) Picture divided into CUs, intra-coded CU are marked by red color.

One can see from the figures above that the blocks are mostly created by intra-predicted blocks. The deblocking is applied for intra blocks with larger strength but it cannot, however, completely remove these blocking artifacts. The intra-predicted blocks in the pictures above are quite smooth, which means that the deblocking filtering should be applied to these block boundaries. Yet, it does not happen. After studying the problem, the reasons the deblocking does not remove those artifacts have been found.

## Why the artifacts are not removed by deblocking filtering

### Brief overview on HEVC deblocking decisions

First, the expressions below are calculated for the block boundary, where pixels pi,j belong to the block on one side of the block boundary and pixels qi,j belong to the other side of the block boundary (see Figure 3). These values are further used in the deblocking filtering decisions.

dp0 = | p2,0 − 2*p1,0 + p0,0 |;   dq0 = | q2,0 − 2*q1,0 + q0,0 |

dp3 =| p2,3 − 2*p1,3 + p0,3 | ;  dq3 = | q2,3 − 2*q1,3 + q0,3 |

dpq0 = dp0 + dq0;   dpq3 = dp3 + dq3;

dp = dp0 + dp3; 	 dq = dq0 + dq3

d = dpq0 + dpq3

p30 p20 p10 p00

p31 p21 p11 p01

p32 p22 p12 p02

A p33 p23 p13 p03

q00 q10 q20 q30

q01 q11 q21 q31

q02 q12 q22 q32

q03 q13 q23 q33  B

Figure 3 Vertical block boundary

The deblocking filtering is performed on the block boundary for lines i = 0..3  if the following condition holds:

d < β

Then there is a decision between applying a strong filter or a normal filter (which in its turn can modify from one to two pixels from the block boundary). The decisions for filtering one or two pixels from the block boundary use comparison of the values dp and dq with the threshold, approximately equal to β/6.

There is also another type of filtering, called a strong filter, which modifies three pixels from the block boundary. The following conditions shall hold in order the strong filter to be applied to the block boundary.

2*dpqi < (β>>2)					(1)

(|p3i - p0i| + |q0i – q3i|) < (β>>3)  	   	(2)

|p0i – q0i| < ((5*tC + 1)>>1)			(3)

Condition (1) checks that the signal on the sides of the block boundary is smooth, condition (2) checks that the signal on both side of the block boundary is flat for 4 pixels on from the block boundary, and the condition (3) makes sure that there is no big change in intensity between the pixels closest to the block boundary.

### Identified problems with intra-block boundaries

Intra-prediction filtering

Figure 4 shows one of the problems with deblocking of intra-blocks. Figure 4 (a) illustrates a DC intra prediction of a 32x32 block in the Riverbed sequence before adding a residual (the deblocking and SAO are turned off). Since for the large intra blocks (e.g. 32x32) the DC prediction value may be significantly different from the surrounding pixel, DC intra-filtering and intra-filtering for the vertical and horizontal modes may introduce singularities along the block boundary. These singularities may still remain after adding the reconstructed residual (see Figure 4 (b)). These singularities prevent the stronger modes of deblocking filtering from being turned on (since they are treated by deblocking as a structure at the block boundary). These artifacts are therefore still visible even after the deblocking filtering (see Figure 4(c)).

| - DC prediction mode, block 32x32, intra-filtering along a block boundary creates a singularity in prediction. | - Same block after overlaying the residual error, intra-prediction singularity creates an artifact at block boundary |
| --- | --- |
| - Remaining artifact after deblocking |  |

Figure 4 Riverbed. Example of blocking artifacts caused by intra prediction filtering.

Strong deblocking filtering decisions

The blocking artifacts visible in the reported sequences are mostly created by the 32x32 blocks. The deblocking filtering can only operate at the block boundary. It has been observed that only applying a weak deblocking filter is not enough to remove the artifacts between two 32x32 blocks with the significant difference between the reconstruction levels of the blocks, since the normal deblocking filter can only modify two samples from the block boundary. It is therefore necessary to apply a "strong" deblocking filter for such block boundaries, which modifies three pixels from the block boundary. Equations (1) – (3) show conditions necessary for applying the strong deblocking filter in HEVC.

These conditions for applying the strong filter are schematically explained in Figure 5 (a). An example of a block artifact from Riverbed is shown in Figure 5 (b). One can see that conditions (2) and (3) are not fulfilled, therefore, strong deblocking filtering will not be applied to such a block boundary, although two sides of the block boundary are quite smooth. The reason is that the strong filtering mode is only applied if the signal on the sides of the block boundary is flat and the difference between values of the pixels closest to the block boundary (p0 and q0) is small. However, these conditions are not held for the intra blocks in sequences like Riverbed, since the signal on the sides of the block boundary has a form of a ramp and the difference between p0 and p1 may be large (see Figure 5 (b)).

| (a) | (b) |
| --- | --- |

Figure 5 (a) Conditions for applying strong filter in HEVC (b) Example of blocking artifact from Riverbed. Strong filtering is not applied since two conditions do not hold (that the signal on the sides is flat and the difference in intensity between two blocks is small).

# Description of tools tested in CE1

Since the problem with blocking artifacts in the observed sequences is a complex problem, it is proposed to modify the HM and specification text as described in the following.

## Tool b.1 – Remove intra-boundary smoothing in DC, vertical, and horizontal intra prediction modes for blocks of larger size, (J0286 [1])

It has been observed that intra-prediction boundary filtering sometimes results in visually disturbing artifacts at the block boundaries. This is mostly observed for intra-predicted blocks of size 32x32 and sometimes for intra blocks of size 16x16. It is therefore proposed to reduce application of the intra-prediction filtering in DC, vertical and horizontal modes to the blocks of size 4x4 and 8x8. The simulation results show that restricting the intra prediction to smaller block sized results in BD-Rate change of 0.1% in all-intra configurations (see Table 1). Most of the gain for intra-prediction filtering comes from smaller block sizes and applying it for larger blocks sizes results in visual artifacts on certain type of content without much gain. Therefore, it is suggested to reduce the intra-prediction filtering in DC, vertical (26) and horizontal (10) modes to blocks of size 16x16, 8x8 and 4x4.

Table 1 Intra filtering in DC, vertical and horizontal modes is restricted to blocks of size 4x4 and 8x8 (the results are based on HM7.0)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.5% | 0.5% | 0.2% | 0.4% | 0.3% |
| Class B | 0.1% | 0.3% | 0.3% | 0.1% | 0.2% | 0.2% |
| Class C | 0.1% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class D | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class E | 0.0% | 0.2% | 0.1% | 0.0% | 0.2% | 0.2% |
| Overall | 0.1% | 0.3% | 0.3% | 0.1% | 0.2% | 0.2% |
|  | 0.1% | 0.3% | 0.3% | 0.1% | 0.2% | 0.2% |
| Class F | 0.0% | 0.0% | 0.1% | -0.1% | -0.3% | 0.0% |
| Enc Time[%] | 108% |  |  | 99% |  |  |
| Dec Time[%] | 107% |  |  | 93% |  |  |

Table 2 Intra filtering in DC, vertical and horizontal modes is restricted to blocks of size 4x4, 8x8, and 16x16 (the results are based on HM7.0)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class E | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 87% |  |  | 87% |  |  |
| Dec Time[%] | 82% |  |  | 79% |  |  |

## Tool b.2.1 − Modify decisions for applying strong filter (J0286 [1])

It has been demonstrated in Section 2.2.2 that current strong filtering decisions do not allow application of the strong deblocking filter to the boundaries between the large intra-predicted blocks, where it should be applied in order to improve the subjective video quality. It is therefore proposed to modify the strong filtering decisions in order for strong deblocking filter to be applied to the block boundary between the large intra-blocks.

It is proposed to modify the decisions for application of the strong filtering as shown below. The following conditions are checked for strong filter based on lines i = 0, 3.

2*dpqi < (β>>2)			          (4)

(|p3i - p0i| + |q0i – q3i|) < (β>>2)      (5) // remove a condition that 4 samples from the block boundary have the same level

dp0 = ((| p3,i − 2*p2,i + p1,i | + | q3,i − 2*q2,i + q1,i |) <<1) < (β>>2)  	(5) // instead, add a condition that 4 pixels //from the block boundary form a straight line (flat or inclined),

|p0i – q0i| < ((5*tC + 1)>>1)  (tC <<4)		      (6) //use a larger threshold in a check for the //difference between intensities of pixels p0 and p1

The proposed decisions are explained in Figure 6. The proposed decisions enable applying strong filtering also when both sides of the block boundary have a form of ramp (similar to the normal filtering mode). The threshold for evaluating pixel value difference | p0 – q0 | is also increased.

| (a) | (b) |
| --- | --- |

Figure 6 (a) Proposed decisions for strong filtering. Two checks are included: that pixel values q0, q1, q2 and q3 form a line (the same for p0, p1, p2 and p3) and that | p0 – q0 | is below threshold. (b) Block artifact from Riverbed. Filter decision matches the artifact.

## Tool b.2.2 − Modify strong filter (from J0286 [1])

Since the proposed filtering decisions turn on the strong filtering in the areas where it is not turned on in the current HEVC deblocking (e.g. on the signal that has a form of ramp), it is also necessary to modify the strong filter. Since the current HEVC filter is designed to be applied to a signal that is flat on both sides of the block boundary, the HEVC strong filter would modify a ramp when applied to it. Therefore, it is proposed to modify the filter coefficients that correspond to pixels p1, q1, p2, q2 so that the filter would not modify a ramp.

The proposed modifications to the strong filter calculations are shown below.

p0’ = Clip3( p0−tc0, p0+tc0, ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )   // same filter as in HM8.0
q0’ = Clip3( q0−tc0, q0+tc0, ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )   // same filter as in HM8.0

p1’ = Clip3( p1−2*tc, p1+2*tc, ( p2 + p1 + p0 + q0 + 2 ) >> 2 )
p2’ = Clip3( p2−2*tc, p2+2*tc, ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 ) 
q1’ = Clip3( q1−2*tc, q1+2*tc, ( p0 + q0 + q1 + q2 + 2 ) >> 2 )
q2’= Clip3( q2−2*tc, q2+2*tc, ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )

p1' = Clip3(p1-tc1, p1+tc1, ((p3 + 2*p2+ 2*p1 + p0 + 2*q0 + 4)>>3))          
q1'   = Clip3(q1-tc1, q1+tc1, ((q3 + 2*q2+ 2*q1 +   q0+2*p0 + 4)>>3))
p2' = Clip3(p2-tc3, p2+tc3, ((3*p3 + 3*p2 + p1 + q0 + 4 )>>3))
q2' = Clip3(q2-tc3, q2+tc3, ((p0 +  q1 + 3*q2 + 3*q3 +4 )>>3))

The effect of applying HEVC strong filter and the proposed filter to a ramp is shown in Figure 7. One can see from the figure that HEVC filter significantly disports the ramp when applied to it, while the proposed filter modified the ramp much less.

| - Effect of HEVC strong filter applied to a ramp | - Effect of proposed strong filter applied to a ramp |
| --- | --- |

Figure 7 Comparison of modification of ramp by HEVC filer and the proposed filter. The original signal is shown in blue, the signal after modification by a filter is shown in red.  The HEVC filter (a) modifies the ramp stronger than the proposed filter (b).

The clipping has also been modified in order to avoid blurring of the reconstructed image by applying too strong filtering (since the strong filter is applied more often). Therefore, the clipping thresholds used in the equations above are reduced relative to the current strong filter in order to reduce amount of blurriness.

The following clipping values are used: tc0 = (3*tc+1)>>1; tc1 = tc, tc2 = (tc+1)>>1.

## Tool b.3.1 − Bs modification for CU (J0286 [1])

The Bs derivation is modified. Block boundaries that have Bs equal to 2 (one of the blocks is intra) and lie on the 32x32 grid are assigned Bs value 3 if at least one of the CUs forming the boundary is equal or greater than 32x32. Since the boundary strength is used in derivation of clipping offset tc, as (Bs-1)*2, the intra-blocks edges with BS = 3, use intra_tc_offset equal to 4.

## Tool b.3.2 − Bs modification for CU (J0286 [1])

Alternatively to b.3.1, the intra_tc_offset for intra-block boundaries is increased from 2 to 3.

## Tool b.3.3 − Bs modification for max TU (Similar to J0090 [3] and J0096 [4])

Alternatively to b.3.2 and b.3.1, Bs is increased from 2 to 3 at the intra TU boundary when one of the TUs has a maximum TU size. The procedure for determining TU boundary for the maximum TU size is described in Error: Reference source not found.

## Tool a.1 – modification of deblocking β table (J0181 [3])

It is proposed to replace β values with Table 2-2 Error: Reference source not found. The values of β for QP values higher than 41 are increased relative to the current β table. Since β values are changed only for QP > 41 than there is no impact on common test conditions. Adjustments of the deblocking strength are therefore achieved by sending the deblocking filter parameters at the slice level.

# Tests to be performed in CE 1

The results for the following tests of CE1 [6] are described in this document:

test 2: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Bs modification CU (b.2.1) + Intra Smoothing Restricted (b.1)

test 4: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Intra smoothing restricted (b.1) + Intra tc_offset = 3 (b.2.2)

test 7: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Bs modification CU (b.2.1)

test 8: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Intra smoothing restricted (b.1) + Bs modification max TU (b.3.3)

test 9: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Intra smoothing restricted (b.1) + Bs modification max TU (b.3.3) + beta table modification (a.1)

# Objective performance

The tested algorithms have been implemented based on HM8.0.

The objective performance of the proposed modifications to deblocking filtering has been evaluated. The results are evaluated on the common test conditions [8]. Additionally, the results on "type 1" sequences described in CE description [6] are provided. The results for additional configurations (i.e. with transform size restricted to 16x16 and with deblocking parameters tc_offset_div2 and beta_offset_div2 set equal to 6). The results for these configurations are also provided for the "type 2" sequences, i.e. the sequences that should test the absence of degradation of quality because of the proposals.

## Performance on common test conditions

The results on the common test conditions are as in the following.

test 2: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Bs modification CU (b.2.1) + Intra Smoothing Restricted (b.1)

Table 3 BD-Rate results of Test 2 vs HM8.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.2% | 0.3% | 0.4% | 0.3% | 0.3% |
| Class B | 0.1% | 0.1% | 0.1% | 0.4% | 0.1% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% |
| Class D | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class E | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.1% |
| Overall | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | 0.0% | 0.1% | 0.1% | -0.3% |
| Class B | 0.0% | 0.1% | 0.2% | 0.2% | 0.2% | 0.3% |
| Class C | 0.0% | 0.1% | 0.2% | 0.1% | 0.1% | -0.1% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.3% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.3% |
| Overall | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.4% | 0.3% | 0.4% | 0.4% | 0.7% |
| Class C | 0.2% | 0.0% | 0.3% | 0.4% | -0.1% | 0.1% |
| Class D | -0.1% | 0.3% | -0.1% | 0.1% | 0.0% | -0.4% |
| Class E | 0.5% | 0.4% | 0.6% | 1.6% | 1.2% | 1.0% |
| Class F | 0.2% | 0.2% | 0.4% | 0.2% | 0.1% | -0.4% |
| Overall | 0.1% | 0.3% | 0.3% | 0.5% | 0.3% | 0.2% |
|  | 0.1% | 0.2% | 0.3% | 0.5% | 0.3% | 0.2% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.1% | 0.3% | -0.4% | 0.5% | 0.2% |
| Class C | 0.0% | 0.1% | 0.1% | 0.1% | -0.1% | 0.0% |
| Class D | 0.0% | 0.5% | 0.5% | 0.0% | -0.3% | 0.2% |
| Class E | -0.2% | 0.8% | -0.5% | 0.1% | -0.4% | 0.3% |
| Class F | 0.0% | 0.4% | 0.6% | 0.0% | 0.2% | -0.6% |
| Overall | -0.1% | 0.3% | 0.2% | -0.1% | 0.0% | 0.0% |
|  | -0.1% | 0.3% | 0.2% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

test 4: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Intra smoothing restricted (b.1) + Intra tc_offset = 3 (b.2.2)

Table 4 BD-Rate results of Test 4 vs HM8.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.1% | 0.1% | 0.4% | 0.2% | 0.2% |
| Class B | 0.2% | 0.1% | 0.1% | 0.4% | 0.1% | 0.1% |
| Class C | 0.2% | 0.0% | 0.0% | 0.3% | 0.1% | 0.0% |
| Class D | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% |
| Overall | 0.1% | 0.0% | 0.0% | 0.3% | 0.0% | 0.1% |
|  | 0.1% | 0.0% | 0.0% | 0.3% | 0.0% | 0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.3% | 0.0% | 0.1% | 0.0% | -0.4% |
| Class B | 0.1% | 0.2% | 0.3% | 0.3% | 0.2% | 0.2% |
| Class C | 0.1% | 0.3% | 0.4% | 0.2% | 0.2% | 0.1% |
| Class D | 0.1% | 0.0% | 0.3% | 0.0% | 0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.3% |
| Overall | 0.0% | 0.1% | 0.2% | 0.1% | 0.2% | 0.0% |
|  | 0.0% | 0.1% | 0.2% | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.5% | 0.4% | 0.4% | 0.5% | 0.7% |
| Class C | 0.1% | 0.0% | 0.2% | 0.4% | 0.3% | 0.4% |
| Class D | 0.0% | 0.1% | 0.2% | 0.2% | -0.1% | -0.1% |
| Class E | 0.5% | 0.1% | 0.4% | 1.6% | 0.9% | 0.6% |
| Class F | 0.1% | 0.1% | 0.4% | 0.2% | 0.8% | -0.8% |
| Overall | 0.1% | 0.2% | 0.3% | 0.5% | 0.5% | 0.2% |
|  | 0.1% | 0.1% | 0.3% | 0.5% | 0.4% | 0.2% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.1% | 0.2% | -0.4% | 0.4% | 0.0% |
| Class C | 0.0% | 0.2% | 0.4% | 0.1% | 0.1% | 0.0% |
| Class D | 0.0% | 0.4% | -0.2% | 0.1% | 0.1% | 0.3% |
| Class E | -0.2% | 0.4% | -0.9% | 0.1% | -0.1% | -0.1% |
| Class F | 0.0% | 0.3% | 0.4% | 0.1% | 0.6% | -0.4% |
| Overall | -0.1% | 0.3% | 0.0% | 0.0% | 0.2% | 0.0% |
|  | -0.1% | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

test 7: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Bs modification CU (b.2.1)

Table 5 BD-Rate results of Test 7 vs HM8.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% | 0.3% | 0.1% | 0.1% |
| Class B | 0.1% | 0.1% | 0.1% | 0.3% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class D | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Overall | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.1% | 0.1% | -0.2% |
| Class B | 0.0% | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.2% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | 0.0% | 0.4% | 0.4% | 0.3% |
| Class C | 0.1% | -0.1% | 0.0% | 0.3% | 0.4% | 0.4% |
| Class D | 0.0% | 0.5% | -0.2% | 0.1% | -0.1% | -0.4% |
| Class E | 0.4% | 0.2% | 0.9% | 1.6% | 1.1% | 1.1% |
| Class F | 0.1% | 0.2% | -0.1% | 0.2% | 0.6% | -0.7% |
| Overall | 0.1% | 0.2% | 0.1% | 0.5% | 0.5% | 0.1% |
|  | 0.1% | 0.2% | 0.2% | 0.5% | 0.4% | 0.2% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.0% | -0.1% | -0.4% | 0.3% | -0.3% |
| Class C | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% |
| Class D | 0.0% | 0.6% | -0.2% | 0.0% | -0.1% | -0.3% |
| Class E | -0.2% | 0.4% | -0.1% | 0.2% | 0.0% | 0.2% |
| Class F | 0.0% | 0.6% | 0.7% | 0.0% | 0.2% | -0.3% |
| Overall | -0.1% | 0.3% | 0.0% | 0.0% | 0.1% | -0.2% |
|  | -0.1% | 0.2% | 0.0% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

test 8: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Intra smoothing restricted (b.1) + Bs modification max TU (b.3.3)

Table 6  BD-Rate results of Test 8 vs HM8.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.5% | 0.5% | 0.5% | 0.5% | 0.5% |
| Class B | 0.1% | 0.3% | 0.3% | 0.4% | 0.3% | 0.3% |
| Class C | 0.1% | 0.2% | 0.1% | 0.2% | 0.2% | 0.1% |
| Class D | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% |
| Class E | 0.0% | 0.2% | 0.1% | 0.1% | 0.2% | 0.2% |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Overall | 0.1% | 0.2% | 0.2% | 0.3% | 0.2% | 0.2% |
|  | 0.1% | 0.2% | 0.2% | 0.3% | 0.2% | 0.2% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.2% | 0.2% | 0.4% | 0.5% |
| Class B | 0.0% | 0.1% | 0.3% | 0.2% | 0.2% | 0.3% |
| Class C | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | -0.2% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | 0.1% |
| Overall | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | -0.1% | 0.4% | 0.5% | 0.7% |
| Class C | 0.1% | 0.1% | 0.1% | 0.4% | 0.3% | 0.4% |
| Class D | 0.0% | 0.7% | -0.1% | 0.1% | -0.1% | -0.2% |
| Class E | 0.5% | 0.1% | 0.2% | 1.7% | 1.5% | 0.5% |
| Class F | 0.0% | 0.1% | 0.4% | 0.2% | 0.8% | -1.1% |
| Overall | 0.1% | 0.3% | 0.1% | 0.5% | 0.6% | 0.1% |
|  | 0.1% | 0.2% | 0.1% | 0.5% | 0.5% | 0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.0% | 0.1% | -0.4% | 0.4% | -0.3% |
| Class C | 0.0% | 0.2% | 0.3% | 0.1% | 0.0% | 0.2% |
| Class D | 0.0% | 0.9% | -0.5% | 0.1% | -0.2% | 0.1% |
| Class E | -0.1% | -0.3% | -0.6% | 0.2% | 0.4% | -0.1% |
| Class F | 0.0% | 0.5% | 0.7% | 0.0% | 0.5% | -0.6% |
| Overall | -0.1% | 0.3% | 0.0% | 0.0% | 0.2% | -0.2% |
|  | -0.1% | 0.2% | 0.0% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Performance on additional CE1 sequences

The following additional sequences were suggested by CE1 for participation in the subjective quality test: Riverbed, DucksTakeOff 25Hz, WestWindsEasy, RedKayak and the sequence China speed from the test set. The objective results for additional test sequences are provided below. The QPs used for these test sequences are: QP 32, QP37 and QP 41.

test 2: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Bs modification CU (b.2.1) + Intra Smoothing Restricted (b.1)

Table 7 Test 2: additional Type 1 sequences encoded with common test conditions

| RA-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DucksTakeOff, 25fps | 32 | hm8.0 | 9214.38 | 29.56 | 32.50 | 36.73 |
|  |  |  | CE1_test2 | 9216.49 | 29.57 | 32.49 | 36.73 |
|  |  | 37 | hm8.0 | 3954.07 | 27.26 | 31.55 | 36.27 |
|  |  |  | CE1_test2 | 3955.28 | 27.28 | 31.56 | 36.27 |
|  |  | 41 | hm8.0 | 1993.55 | 25.69 | 30.96 | 35.98 |
|  |  |  | CE1_test2 | 1995.73 | 25.71 | 30.96 | 35.98 |
|  | RedKayak,570fr | 32 | hm8.0 | 3709.59 | 35.65 | 41.82 | 44.91 |
|  |  |  | CE1_test2 | 3713.29 | 35.66 | 41.82 | 44.91 |
|  |  | 37 | hm8.0 | 1714.38 | 32.94 | 41.01 | 44.13 |
|  |  |  | CE1_test2 | 1716.31 | 32.96 | 41.01 | 44.12 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.42 | 43.49 |
|  |  |  | CE1_test2 | 853.91 | 30.97 | 40.42 | 43.49 |
| LB-Main | riverbed | 32 | hm8.0 | 5851.44 | 34.64 | 38.96 | 41.40 |
|  |  |  | CE1_test2 | 5852.95 | 34.65 | 38.96 | 41.40 |
|  |  | 37 | hm8.0 | 2870.61 | 31.71 | 38.13 | 40.91 |
|  |  |  | CE1_test2 | 2867.19 | 31.74 | 38.13 | 40.90 |
|  |  | 41 | hm8.0 | 1566.17 | 29.55 | 37.69 | 40.63 |
|  |  |  | CE1_test2 | 1564.68 | 29.59 | 37.69 | 40.64 |
|  | WestWindEasy | 32 | hm8.0 | 3758.62 | 36.22 | 44.74 | 47.46 |
|  |  |  | CE1_test2 | 3754.37 | 36.22 | 44.71 | 47.48 |
|  |  | 37 | hm8.0 | 1679.87 | 33.01 | 43.71 | 46.65 |
|  |  |  | CE1_test2 | 1673.36 | 33.02 | 43.64 | 46.63 |
|  |  | 41 | hm8.0 | 854.87 | 30.75 | 42.93 | 45.86 |
|  |  |  | CE1_test2 | 852.73 | 30.75 | 43.05 | 46.03 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1270.60 | 35.30 | 40.24 | 40.18 |
|  |  |  | CE1_test2 | 1268.94 | 35.30 | 40.24 | 40.19 |
|  |  | 37 | hm8.0 | 612.05 | 31.97 | 38.75 | 38.42 |
|  |  |  | CE1_test2 | 610.88 | 31.98 | 38.69 | 38.39 |
|  |  | 41 | hm8.0 | 316.87 | 29.38 | 37.76 | 36.85 |
|  |  |  | CE1_test2 | 316.63 | 29.39 | 37.75 | 36.87 |

test 4: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Intra smoothing restricted (b.1) + Intra tc_offset = 3 (b.2.2)

Table 8 Test 4: additional Type 1 sequences encoded with common test conditions

| RA-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DucksTakeOff, 25fps | 32 | hm8.0 | 9214.38 | 29.56 | 32.50 | 36.73 |
|  |  |  | CE1_test4 | 9220.11 | 29.57 | 32.49 | 36.73 |
|  |  | 37 | hm8.0 | 3954.07 | 27.26 | 31.55 | 36.27 |
|  |  |  | CE1_test4 | 3956.08 | 27.28 | 31.55 | 36.27 |
|  |  | 41 | hm8.0 | 1993.55 | 25.69 | 30.96 | 35.98 |
|  |  |  | CE1_test4 | 1995.91 | 25.71 | 30.97 | 35.98 |
|  | RedKayak,570fr | 32 | hm8.0 | 3709.59 | 35.65 | 41.82 | 44.91 |
|  |  |  | CE1_test4 | 3712.55 | 35.66 | 41.82 | 44.91 |
|  |  | 37 | hm8.0 | 1714.38 | 32.94 | 41.01 | 44.13 |
|  |  |  | CE1_test4 | 1716.38 | 32.95 | 41.01 | 44.11 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.42 | 43.49 |
|  |  |  | CE1_test4 | 854.39 | 30.98 | 40.43 | 43.50 |
| LB-Main | riverbed | 32 | hm8.0 | 5851.44 | 34.64 | 38.96 | 41.40 |
|  |  |  | CE1_test4 | 5854.49 | 34.65 | 38.95 | 41.39 |
|  |  | 37 | hm8.0 | 2870.61 | 31.71 | 38.13 | 40.91 |
|  |  |  | CE1_test4 | 2868.93 | 31.74 | 38.13 | 40.90 |
|  |  | 41 | hm8.0 | 1566.17 | 29.55 | 37.69 | 40.63 |
|  |  |  | CE1_test4 | 1565.16 | 29.59 | 37.70 | 40.60 |
|  | WestWindEasy | 32 | hm8.0 | 3758.62 | 36.22 | 44.74 | 47.46 |
|  |  |  | CE1_test4 | 3753.21 | 36.22 | 44.72 | 47.49 |
|  |  | 37 | hm8.0 | 1679.87 | 33.01 | 43.71 | 46.65 |
|  |  |  | CE1_test4 | 1675.25 | 33.03 | 43.63 | 46.63 |
|  |  | 41 | hm8.0 | 854.87 | 30.75 | 42.93 | 45.86 |
|  |  |  | CE1_test4 | 851.28 | 30.75 | 43.04 | 46.02 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1270.60 | 35.30 | 40.24 | 40.18 |
|  |  |  | CE1_test4 | 1268.88 | 35.30 | 40.25 | 40.19 |
|  |  | 37 | hm8.0 | 612.05 | 31.97 | 38.75 | 38.42 |
|  |  |  | CE1_test4 | 611.16 | 31.97 | 38.70 | 38.41 |
|  |  | 41 | hm8.0 | 316.87 | 29.38 | 37.76 | 36.85 |
|  |  |  | CE1_test4 | 317.13 | 29.40 | 37.74 | 36.82 |

test 7: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Bs modification CU (b.2.1)

Table 9 Test 7: additional Type 1 sequences encoded with common test condition

| RA-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DucksTakeOff, 25fps | 32 | hm8.0 | 9214.38 | 29.56 | 32.50 | 36.73 |
|  |  |  | CE1_test7 | 9213.32 | 29.57 | 32.49 | 36.73 |
|  |  | 37 | hm8.0 | 3954.07 | 27.26 | 31.55 | 36.27 |
|  |  |  | CE1_test7 | 3952.97 | 27.28 | 31.55 | 36.27 |
|  |  | 41 | hm8.0 | 1993.55 | 25.69 | 30.96 | 35.98 |
|  |  |  | CE1_test7 | 1992.91 | 25.70 | 30.96 | 35.98 |
|  | RedKayak,570fr | 32 | hm8.0 | 3709.59 | 35.65 | 41.82 | 44.91 |
|  |  |  | CE1_test7 | 3707.06 | 35.65 | 41.82 | 44.91 |
|  |  | 37 | hm8.0 | 1714.38 | 32.94 | 41.01 | 44.13 |
|  |  |  | CE1_test7 | 1712.61 | 32.95 | 41.01 | 44.13 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.42 | 43.49 |
|  |  |  | CE1_test7 | 852.47 | 30.97 | 40.42 | 43.47 |
| LB-Main | riverbed | 32 | hm8.0 | 5851.44 | 34.64 | 38.96 | 41.40 |
|  |  |  | CE1_test7 | 5846.52 | 34.64 | 38.96 | 41.39 |
|  |  | 37 | hm8.0 | 2870.61 | 31.71 | 38.13 | 40.91 |
|  |  |  | CE1_test7 | 2865.69 | 31.74 | 38.13 | 40.90 |
|  |  | 41 | hm8.0 | 1566.17 | 29.55 | 37.69 | 40.63 |
|  |  |  | CE1_test7 | 1562.49 | 29.58 | 37.70 | 40.64 |
|  | WestWindEasy | 32 | hm8.0 | 3758.62 | 36.22 | 44.74 | 47.46 |
|  |  |  | CE1_test7 | 3753.43 | 36.22 | 44.72 | 47.46 |
|  |  | 37 | hm8.0 | 1679.87 | 33.01 | 43.71 | 46.65 |
|  |  |  | CE1_test7 | 1673.81 | 33.02 | 43.74 | 46.67 |
|  |  | 41 | hm8.0 | 854.87 | 30.75 | 42.93 | 45.86 |
|  |  |  | CE1_test7 | 852.04 | 30.77 | 42.93 | 45.85 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1270.60 | 35.30 | 40.24 | 40.18 |
|  |  |  | CE1_test7 | 1268.94 | 35.31 | 40.23 | 40.20 |
|  |  | 37 | hm8.0 | 612.05 | 31.97 | 38.75 | 38.42 |
|  |  |  | CE1_test7 | 611.08 | 31.97 | 38.75 | 38.41 |
|  |  | 41 | hm8.0 | 316.87 | 29.38 | 37.76 | 36.85 |
|  |  |  | CE1_test7 | 317.11 | 29.39 | 37.75 | 36.84 |

test 8: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Intra smoothing restricted (b.1) + Bs modification max TU (b.3.3)

Table 10 Test 8: additional Type 1 sequences encoded with common test conditions

| RA-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DucksTakeOff, 25fps | 32 | hm8.0 | 9214.38 | 29.56 | 32.50 | 36.73 |
|  |  |  | CE1_test8 | 9223.65 | 29.57 | 32.49 | 36.73 |
|  |  | 37 | hm8.0 | 3954.07 | 27.26 | 31.55 | 36.27 |
|  |  |  | CE1_test8 | 3955.75 | 27.28 | 31.55 | 36.27 |
|  |  | 41 | hm8.0 | 1993.55 | 25.69 | 30.96 | 35.98 |
|  |  |  | CE1_test8 | 1994.89 | 25.71 | 30.96 | 35.97 |
|  | RedKayak,570fr | 32 | hm8.0 | 3709.59 | 35.65 | 41.82 | 44.91 |
|  |  |  | CE1_test8 | 3716.44 | 35.66 | 41.82 | 44.90 |
|  |  | 37 | hm8.0 | 1714.38 | 32.94 | 41.01 | 44.13 |
|  |  |  | CE1_test8 | 1717.97 | 32.96 | 41.01 | 44.12 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.42 | 43.49 |
|  |  |  | CE1_test8 | 854.16 | 30.98 | 40.42 | 43.49 |
| LB-Main | riverbed | 32 | hm8.0 | 5851.44 | 34.64 | 38.96 | 41.40 |
|  |  |  | CE1_test8 | 5857.23 | 34.65 | 38.96 | 41.39 |
|  |  | 37 | hm8.0 | 2870.61 | 31.71 | 38.13 | 40.91 |
|  |  |  | CE1_test8 | 2867.40 | 31.74 | 38.13 | 40.90 |
|  |  | 41 | hm8.0 | 1566.17 | 29.55 | 37.69 | 40.63 |
|  |  |  | CE1_test8 | 1565.39 | 29.59 | 37.69 | 40.63 |
|  | WestWindEasy | 32 | hm8.0 | 3758.62 | 36.22 | 44.74 | 47.46 |
|  |  |  | CE1_test8 | 3757.17 | 36.23 | 44.69 | 47.49 |
|  |  | 37 | hm8.0 | 1679.87 | 33.01 | 43.71 | 46.65 |
|  |  |  | CE1_test8 | 1675.42 | 33.03 | 43.61 | 46.63 |
|  |  | 41 | hm8.0 | 854.87 | 30.75 | 42.93 | 45.86 |
|  |  |  | CE1_test8 | 853.24 | 30.78 | 42.96 | 46.02 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1270.60 | 35.30 | 40.24 | 40.18 |
|  |  |  | CE1_test8 | 1268.64 | 35.31 | 40.24 | 40.16 |
|  |  | 37 | hm8.0 | 612.05 | 31.97 | 38.75 | 38.42 |
|  |  |  | CE1_test8 | 611.72 | 31.98 | 38.70 | 38.41 |
|  |  | 41 | hm8.0 | 316.87 | 29.38 | 37.76 | 36.85 |
|  |  |  | CE1_test8 | 316.90 | 29.40 | 37.71 | 36.81 |

Additional results on the tests from CE1 (such as tests with max transform size limited to 16x16 and with maximum deblocking offsets) can be found in the excel files attached to the proposal.

# Subjective results

Subjective quality of the proposal has been studied. It has been found that the proposal substantially decreases the amount of blocking artifacts on the sequences that were reported to contain a lot of blocking artifacts (West Wind Easy, Riverbed, China Speed). The proposal also attenuates the blocking artifacts on some other sequences, e.g. Type 2 sequences BasketBallDrillText, Cactus, etc.

The proposals have also been evaluated on the test sequences that belong to common test conditions. A "blind test" was used where the test person does not know the identities of the sequences. It has been found that that the proposal has same or better visual quality on most of studied sequences compared with HM8.0 on the common test conditions [8]. In general, it has been concluded that the proposal has better visual quality than HM8.0.

Examples of improvement achieved by the proposed deblocking filter are shown in Figure 8 - Figure 14.

One can see that the proposed deblocking combinations reported in this document also improve the subjective performance on some of Type 2 sequences, i.e. sequences from the common test conditions that were chosen in order to ensure there is no degradation of quality. For example, the tested schemes reduce the blockingness in the smooth areas in BasketballDrillText and Cactus sequences (see Figure 10 and Figure 11).

The proposed methods also improve the subjective quality when the maximum transform size is limited to 16x16 (see Figure 14)

| - HM8.0 | - Test 8 |
| --- | --- |

Figure 8 Comparison of HM8.0 and Test 8 on RedKayak  QP37, Random Access, Frame 396

| - HM8.0 | - Test 8 |
| --- | --- |

Figure 9 Comparison of HM8.0 and Test 8 on RedKayak  QP37, Random Access, Frame 158

| (a) HM8.0 | (b) Test 4 |
| --- | --- |

Figure 10 Comparison of HM8.0 and Test 4 on BasketBallDrillText QP37, Low DElay, Frame 42

(a) HM8.0

(b) Test 4

Figure 11 Comparison of HM8.0 and Test 4 on Cactus QP37, Low Delay, Frame 278

| (a) HM8.0 | (b) Test 4 |
| --- | --- |

Figure 12 Comparison of HM8.0 and Test 4 on China Speed QP37, Low Delay, Frame 159

| (a) HM8.0 | (b) Test 2 |
| --- | --- |

Figure 13 Comparison of HM8.0 and Test 2 on West Winds Easy QP37, Low Delay, Frame 28

| (a) HM8.0 | (b) Test 4 |
| --- | --- |

Figure 14 Comparison of HM8.0 and Test 4 on Riverbed QP37, Low Delay, Max Transform Size = 16x16, Frame 170

# Conclusions

Several tests in the scope of CE1 on Deblocking filtering have been performed. The tests include modifications to the intra-prediction filtering and to deblocking filtering. The modification significantly reduce the blocking artifacts on the reported sequences (Riverbed, WestWindEasy, China Speed) without sending additional sequence-dependent parameters to the deblocking filter. Improvements of subjective quality on other sequences have also been observed. The objective performance of the proposed algorithm is similar to HM8.0 on main profile. Some losses are reported on HE10 profile, in particular on Low-delay B configuration. The decoding time of the proposal is similar with that of the anchor.

It is therefore proposed to adopt one of the tested algorithms to the specification text and to the next version of HM software.

# References

- A. Norkin, "AHG6: Reduction of block artifacts in HEVC for large blocks", JCTVC-J0286, Stockholm, SE, July 2012.

- S. Lu, O. Nakagami, M. Ikeda, T. Suzuki, " AHG6: On deblocking filter parameters", JCTVC-J0181, Stockholm, SE, July 2012.

- G. Van der Auwera, R. Joshi, M. Karczewicz, "AHG6: Transform Dependent Deblocking Strength", JCTVC-J0090, Stockholm, SE, July 2012.

- D.-K. Kwon, M. Budagavi, "Suppression of blocking artifacts at 32x32 block boundaries", JCTVC-J0096, Stockholm, SE, July 2012.

- T. Suzuki, "BoG Report: Report of subjective test on extended adaptability range of deblocking filter", JCTVC-J0567,

- A. Norkin, T. Suzuki, M. Budagavi, G. Van der Auwera, "Description of Core Experiment 1: Deblocking filter", JCTVC-J1101, Stockholm, SE, July 2012.

- "High Efficiency Video Coding (HEVC) text specification draft 8", JCTVC-J1003, Stockholm, SE, July 2012.

- F. Bossen, "Common HM test conditions and software reference configurations", JCTVC-J1100, Stockholm, SE, July 2012.

# Specification text changes

The following changes are proposed to the specification text. The proposed modifications are shown for each tested tool separately. The specification changes for a combination of tools can be obtained as a combination of corresponding changes for each separate tool.

## Tool b.1 – Remove intra-boundary smoothing in DC, vertical, and horizontal intra prediction modes for blocks of larger size, (J0286 [1])

##### 8.4.4.2.5 Specification of Intra_DC (1) prediction mode

Inputs to this process are:

–	neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the chroma component of the current block.

Output of this process are:

–	predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as the following ordered steps:

- A variable dcVal is derived as:

dcVal = x'=0nS−1p[x'][−1]+y'=0nS−1p[−1][ y']+nS>>(k+1)	(9-1)
where k=Log2(nT)

- Depending on the chroma component index cIdx, the following applies.

- If cIdx is equal to 0 and Log2(nT) is less than X, the following applies.

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2*dcVal + p[ 0 ][ −1 ] + 2 ) >> 2	(9-2)
predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3*dcVal + 2 ) >> 2, with x = 1..nT−1	(9-3)
predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3*dcVal + 2 ) >> 2, with y = 1..nT−1	(9-4)
predSamples[ x ][ y ] = dcVal, with x, y = 1..nT−1	(9-5)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as

predSamples[ x ][ y ] = dcVal, with x, y = 0..nT−1	(9-6)

##### 8.4.4.2.6 Specification of Intra_Angular (2..34) prediction mode

…

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1 are derived as follows.

  - The index variable iIdx and the multiplication factor iFact are derived by

iIdx = ( ( y + 1 )*intraPredAngle ) >> 5		(9-7)

iFact = ( ( y + 1 )*intraPredAngle ) && 31		(9-8)

  - Depending on the value of iFact, the following applies.

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ( ( 32 – iFact )*ref[ x+iIdx+1 ] + iFact*ref[ x+iIdx+2] + 16 ) >> 5	(9-9)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ref[ x+iIdx+1 ]		(9-10)

  - When intraPartMode is equal to Intra_Angular (26), Log2(nT) is less than X, and cIdx is equal to 0, the following filtering applies with x = 0, y = 0..nT−1.

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] – p[ −1 ][ −1 ] ) >> 1 ) )	(9-11)

–	Otherwise (intraPredMode is less than 18), the following ordered steps apply.

- The reference sample array ref[ x ], with x= −nT..2*nT is specified as follows.

ref[ x ] = p[ −1 ][ −1+x ], with x=0..nT		(9-12)

- If intraPredAngle is less than 0, the main reference sample array is extented as follows.

- When ( nT*intraPredAngle ) >>5 is less than −1,

ref[ x ] = p[ −1+( ( x*invAngle+128 )>>8 ) ][ −1 ], with x=( nT*intraPredAngle ) >>5..−1	(9-13)

- Otherwise,

ref[ x ] = p[ −1 ][ −1+x ], with x=nT+1..2*nT		(9-14)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1 are derived as follows.

- The index variable iIdx and the multiplication factor iFact are derived by

iIdx = ( ( x + 1 )*intraPredAngle ) >> 5		(9-15)

iFact = ( ( x + 1 )*intraPredAngle ) && 31		(9-16)

- Depending on the value of iFact, the following applies.

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ( ( 32 – iFact )*ref[ y+iIdx+1 ] + iFact*ref[ y+iIdx+2] + 16 ) >> 5	(9-17)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ref[ y+iIdx+1 ]		(9-18)

- When intraPartMode is equal to Intra_Angular (10), Log2(nT) is less than X, and cIdx is equal to 0, the following filtering applies with x = 0..nT−1, y = 0.

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] – p[ −1 ][ −1 ] ) >> 1 )	(9-19)

## Tool b.2.1 − Modify decisions for applying strong filter (J0286 [1])

##### 8.7.2.4.6 Decision process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	variables dpq, β and tC.

Output of this process is:

–	a variable dSam containing a decision

The variable dSam is specified as follows:

–	If dpq is less than ( β >> 2 ), abs( p3 – p0 ) + abs( q0 – q3 ) is less than ( β >> 3 ) (abs(p3 – 2*p2 + p1) + abs(q3 – 2*q2 + q1))<<1  is less than ( β >> 2 ), and abs( p0 – q0 ) is less than ( 5*tC + 1 ) >> 1(tC << 4), dSam is set equal to 1.

–	Otherwise, dSam is set equal to 0.

## Tool b.2.2 − Modify strong filter (from J0286 [1])

##### 8.7.2.4.6 Filtering process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	a variable dE,

–	variables dEp1 and dEq1 containing decisions to filter samples p1 and q1 respectively,

–	a variable tC.

Output of this process is:

–	number of filtered samples nDp and nDq,

–	filtered sample values, pi’ and qj’ with i = 0..nDp − 1, j = 0..nDq − 1

Depending on dE, the following applies:

–	If the variable dE is equal to 2, the following strong filtering applies while nDp and nDq are set equal to 3:

tc0 = (3*tc+1) >> 1

tc2 = (tc +1) >> 1

p0’ = Clip3( p0−2*tc tc0, p0+2*tc tc0, ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )	(9-20)

p1’ = Clip3( p1−2*tc, p1+2*tc, ( p2 + p1 + p0 + q0 + 2 ) >> 2 )	(9-21)

p2’ = Clip3( p2−2*tc, p2+2*tc, ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 )	(9-22)

p1’ = Clip3( p1−tc, p1+tc, (p3 + 2*p2+ 2*p1 + p0 + 2*q0 + 4)>>3)	(8-23)

p2’ = Clip3( p2−tc2, p2+tc2, (3*p3 + 3*p2 + p1 + q0 + 4 )>>3 )	(8-24)

q0’ = Clip3( q0−2*tc tc0, q0+2*tc tc0, ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(9-25)

q1’ = Clip3( q1−2*tc, q1+2*tc, ( p0 + q0 + q1 + q2 + 2 ) >> 2 )	(9-26)

q2’= Clip3( q2−2*tc, q2+2*tc, ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )	(9-27)

q1’ = Clip3( q1−tc, q1+tc, (q3 + 2*q2+ 2*q1 + q0+ 2*p0 + 4)>>3)	(8-28)

q2’= Clip3( q2−tc2, q2+tc2, ( p0 + q1 + 3*q2 + 3*q3 + 4 ) >> 3 )	(8-29)

…

## Tool b.3.1 − Bs modification CU size (J0286 [1])

#### 8.7.2.3. Derivation process of boundary filtering strength

Inputs of this process are:

–	a luma picture sample array recPictureL,

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block,

–	a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

–	a two-dimensional array of size (nS)x(nS), edgeFlags.

Output of this process is:

–	a two-dimensional array of size (nS)x(nS), bS specifying the boundary filtering strength.

The boundary filtering strength array bS for the current coding unit is derived as follows.

The variables xDi, yDj, xN and yN are derived as follows.

- If edgeType is equal to EDGE_VER, xDi is set equal to ( i << 3 ), yDj is set equal to ( j << 2 ), xN is set equal to (1 << ( log2CbSize − 3 )) − 1 and yN is set equal to (1 << ( log2CbSize − 2 )) − 1.

- Otherwise (edgeType is equal to EDGE_HOR), xDi is set equal to ( i << 2 ), yDj is set equal to ( j << 3 ), xN is set equal to (1 << ( log2CbSize − 2 )) − 1 and yN is set equal to (1 << ( log2CbSize − 3 )) − 1.

For xDi with i=0..xN, the following applies.

For yDj with j=0..yN, the following applies.

- If edgeFlags[ xDi ][ yDj ] is equal to 1, the sample values are derived as follows.

- If edgeType is equal to EDGE_VER, sample p0 = recPictureL[ xC + xDi − 1 ][ yC + yDj ] and q0 = recPictureL[ xC + xDi ][ yC + yDj ].

- Otherwise (edgeType is equal to EDGE_HOR), sample p0 = recPictureL[ xC + xDi ][ yC + yDj − 1 ] and q0 = recPictureL[ xC + xDi ][ yC + yDj ].

Depending on p0 and q0, the variable bS[ xDi ][ yDj ] is derived as follows.

- If the sample p0 or q0 is in the luma coding block of a coding unit coded with intra prediction mode, the following applies

    - The variable log2CbSizeP is set equal to log2CbSize of the CU that contains pixel p0 and the variable log2CbSizeQ is set equal to log2CbSize of the CU that contains pixel q0

    - If log2CbSizeP is greater or equal 5 or log2CbSizeQ is greater or equal 5, the variable bS[ xDi ][ yDj ] is set equal to 3 provided one of the following conditions is true:

      - edgeType is equal to EDGE_VER and (((xC + xDi) >>5)<<5) is equal to (xC + xDi)

      - edgeType is equal to EDGE_HOR and (((yC + yDj) >>5)<<5) is equal to (yC + yDj)

    - Otherwise

      - the variable bS[ xDi ][ yDj ] is set equal to 2.

- Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a luma transform block which contains non-zero transform coefficient level.

- Otherwise, the following applies.

    - When edgeType is equal to EDGE_HOR and yC + yDj − 1 is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY), sample p0 = recPictureL[ xL ][ yC + yDj – 1 ] where xL is equal to ((( xC + xDi ) >> 3) << 3) + ((( xC + xDi ) >> 3) & 1) * 7.

    - If one or more of the following conditions are true, the variable bS[ xDi ][ yDj ] is set equal to 1.

    - For the prediction of the luma prediction block containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the luma prediction block containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two luma prediction blocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a luma prediction block with lop left luma sample covering ( xB, yB ), is equal to PredFlagL0[ xB, yB ] + PredFlagL1[ xB, yB ].

    - One motion vector is used to predict the luma prediction block containing the sample p0 and one motion vector is used to predict the luma prediction block containing the sample q0 and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 4 in units of quarter luma frame samples.

    - Two motion vectors and two different reference pictures are used to predict the luma prediction block containing the sample p0 and two motion vectors for the same two reference pictures are used to predict the luma prediction block containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two luma prediction blocks for the same reference picture is greater than or equal to 4 in units of quarter luma frame samples,

    - Two motion vectors for the same reference picture are used to predict the luma prediction block containing the sample p0 and two motion vectors for the same reference picture are used to predict the luma prediction block containing the sample q0 and all of the following conditions are true:

    - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two luma prediction bocks is greater than or equal to 4 in quarter luma frame samples or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two luma prediction blocks is greater than or equal to 4 in units of quarter luma frame samples,

    - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the luma prediction block containing the sample p0 and the list 1 motion vector used in the prediction of the luma prediction block containing the sample q0 is greater than or equal to 4 in units of quarter luma frame samples or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the luma prediction block containing the sample p0 and list 0 motion vector used in the prediction of the luma prediction block containing the sample q0 is greater than or equal to 4 in units of quarter luma frame samples.

    - Otherwise (none of the conditions above is true), the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise (edgeFlags[ xDi ][ yDj ] is equal to 0), the variable bS[ xDi ][ yDj ] is set equal to 0.

## Tool b.3.2 Modification of intra_tc_offset (J0286 [1])

##### 8.7.2.4.3  Decision process for luma block edge

Inputs of this process are:

–	a luma picture sample array recPictureL,

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current luma block relative to the top left sample of the current luma coding block,

–	a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

–	a variable bS specifying the boundary filtering strength.

Outputs of this process are:

–	variables dE, dEp, dEq containing decisions,

–	variables β, tC.

…

The value of the variable tC′ is determined as specified as Table 9-11 based on the luma quantization parameter Q derived as:

Q = Clip3( 0, 53, qPL + 2 3*( bS − 1) + ( tc_offset_div2 << 1) )	(9-30)

where tc_offset_div2 is the value of the variable tc_offset_div2 for the slice that contains sample q0,0.

The variable tC is derived as:

tC = tC′*(1<<(BitDepthY − 8))	(9-31)

…

##### 8.7.2.4.6 Filtering process for chroma block edge

Inputs of this process are:

–	a chroma picture sample array s’,

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current luma block relative to the top left sample of the current luma coding block,

–	a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

–	a variable bS specifying the boundary filtering strength.

Output of this process is:

–	the modified chroma picture sample array s’.

…

The value of the variable tC′ is determined as specified as Table 9-11 based on the chroma quantization parameter Q derived as:

Q = Clip3( 0, 53, QPC + 2 3*(bS − 1) + (tc_offset_div2 << 1 ) )	(9-32)

The variable tC is derived as:

tC = tC′*(1<<(BitDepthY − 8))	(9-33)

where tc_offset_div2 is the value of the variable tc_offset_div2 for the slice that contains sample q0,0.

…

## Tool b.3.3 − Bs modification max TU size (Similar to J0090 [3] and J0096 [4])

#### 8.7.2.3. Derivation process of boundary filtering strength

Inputs of this process are:

–	a luma picture sample array recPictureL,

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block,

–	a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

–	a two-dimensional array of size (nS)x(nS), edgeFlags.

–	a variable Log2MaxTrafoSize.

Output of this process is:

–	a two-dimensional array of size (nS)x(nS), bS specifying the boundary filtering strength.

The boundary filtering strength array bS for the current coding unit is derived as follows.

The variables xDi, yDj, xN and yN are derived as follows.

- If edgeType is equal to EDGE_VER, xDi is set equal to ( i << 3 ), yDj is set equal to ( j << 2 ), xN is set equal to (1 << ( log2CbSize − 3 )) − 1 and yN is set equal to (1 << ( log2CbSize − 2 )) − 1.

- Otherwise (edgeType is equal to EDGE_HOR), xDi is set equal to ( i << 2 ), yDj is set equal to ( j << 3 ), xN is set equal to (1 << ( log2CbSize − 2 )) − 1 and yN is set equal to (1 << ( log2CbSize − 3 )) − 1.

For xDi with i=0..xN, the following applies.

For yDj with j=0..yN, the following applies.

- If edgeFlags[ xDi ][ yDj ] is equal to 1, the sample values are derived as follows.

- If edgeType is equal to EDGE_VER, sample p0 = recPictureL[ xC + xDi − 1 ][ yC + yDj ] and q0 = recPictureL[ xC + xDi ][ yC + yDj ].

- Otherwise (edgeType is equal to EDGE_HOR), sample p0 = recPictureL[ xC + xDi ][ yC + yDj − 1 ] and q0 = recPictureL[ xC + xDi ][ yC + yDj ].

Depending on p0 and q0, the variable bS[ xDi ][ yDj ] is derived as follows.

- If the sample p0 or q0 is in the luma coding block of a coding unit coded with intra prediction mode, the following applies

    - The variable log2TrafoSizeQ is set equal to the log2TrafoSize of the transform units that contains the sample q0 and the variable log2TrafoSizeP is set equal to the log2TrafoSize of the transform unit that contains the sample p0

    - If one of the following conditions is true then the variable bS[ xDi ][ yDj ] is set equal to 3:

        - The sample p0 is in the luma coding block of a coding unit coded with intra prediction mode and log2TrafoSizeP is equal to Log2MaxTrafoSize

        - The sample q0 is in the luma coding block of a coding unit coded with intra prediction mode and log2TrafoSizeQ is equal to Log2MaxTrafoSize

    - Otherwise

      - the variable bS[ xDi ][ yDj ] is set equal to 2.

- Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a luma transform block which contains non-zero transform coefficient level.

- Otherwise, the following applies.

    - When edgeType is equal to EDGE_HOR and yC + yDj − 1 is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY), sample p0 = recPictureL[ xL ][ yC + yDj – 1 ] where xL is equal to ((( xC + xDi ) >> 3) << 3) + ((( xC + xDi ) >> 3) & 1) * 7.

    - If one or more of the following conditions are true, the variable bS[ xDi ][ yDj ] is set equal to 1.

    - For the prediction of the luma prediction block containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the luma prediction block containing the sample q0.

NOTE 3 – The determination of whether the reference pictures used for the two luma prediction blocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 4 – The number of motion vectors that are used for the prediction of a luma prediction block with lop left luma sample covering ( xB, yB ), is equal to PredFlagL0[ xB, yB ] + PredFlagL1[ xB, yB ].

    - One motion vector is used to predict the luma prediction block containing the sample p0 and one motion vector is used to predict the luma prediction block containing the sample q0 and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 4 in units of quarter luma frame samples.

    - Two motion vectors and two different reference pictures are used to predict the luma prediction block containing the sample p0 and two motion vectors for the same two reference pictures are used to predict the luma prediction block containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two luma prediction blocks for the same reference picture is greater than or equal to 4 in units of quarter luma frame samples,

    - Two motion vectors for the same reference picture are used to predict the luma prediction block containing the sample p0 and two motion vectors for the same reference picture are used to predict the luma prediction block containing the sample q0 and all of the following conditions are true:

    - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two luma prediction bocks is greater than or equal to 4 in quarter luma frame samples or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two luma prediction blocks is greater than or equal to 4 in units of quarter luma frame samples,

    - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the luma prediction block containing the sample p0 and the list 1 motion vector used in the prediction of the luma prediction block containing the sample q0 is greater than or equal to 4 in units of quarter luma frame samples or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the luma prediction block containing the sample p0 and list 0 motion vector used in the prediction of the luma prediction block containing the sample q0 is greater than or equal to 4 in units of quarter luma frame samples.

    - Otherwise (none of the conditions above is true), the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise (edgeFlags[ xDi ][ yDj ] is equal to 0), the variable bS[ xDi ][ yDj ] is set equal to 0.

## Tool a.1 – modification of deblocking β table (J0181 [3])

Table 9-11 – Derivation of threshold variables β′ and tC′ from input Q

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| β′ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 |
| tC′ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Q | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| β′ | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
| tC′ | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Q | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |  |  |  |
| β′ | 38 | 40 | 42 | 44 | 4652 | 4860 | 5068 | 5276 | 5484 | 5692 | 58100 | 60116 | 62132 | 64148 | - | - |  |  |  |
| tC′ | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 10 | 11 | 13 | 14 | 16 | 18 | 20 | 22 | 24 |  |  |  |

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

X is equal to 5 in tests 2 and 4 and equal to 4 in tests 8 and 9.

X is equal to 5 in tests 2 and 4 and equal to 4 in tests 8 and 9.

X is equal to 5 in tests 2 and 4 and equal to 4 in tests 8 and 9