| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0188r1 |
| --- | --- |

| Title: | On the derivation of chroma QPs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS, Patrice ONNO, Christophe GISQUET, Guillaume LAROCHE Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France | Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes a slight modification of the chroma QP derivation in order to simplify the design and to give more flexibility for controlling the chroma QP.

In a first proposal, it is suggested to replace the table used in the derivation of chroma QP values from luma QP by a generic equation which reproduces very closely the table values. This change enables to remove one table from the specification. Results on common test conditions reportedly show a small gain in chroma, with a slight loss in luma.

In a second proposal, it is suggested to make this equation more generic by introducing two new control parameters that enable to accurately control the link between luma and chroma QPs. It is reported that these parameters are not redundant with the existing parameters cb/cr_qp_offset and have a different impact on the luma-chroma QP relation. The finer chroma QP control gives more flexibility, in particular for local QP adaptation which is required for fine quality control applications. Results on common test conditions are presented to show the impact of these new control parameters.

# Introduction

In the current HEVC design, the QPs used for chroma components (QPCb and QPCr) are derived from the QP used for luma component (QPY) as follows, [1]:

- First two intermediate values QPiCb and QPiCr are derived as:

qPiCb = Clip3( −QpBdOffsetC, 57, QPY + pic_cb_qp_offset + slice_cb_qp_offset )	(8-1)

qPiCr = Clip3( −QpBdOffsetC, 57, QPY + pic_cr_qp_offset + slice_cr_qp_offset )	(8-2)

- The variables qPCb and qPCr are set equal to the value of QPC as specified in the following table (Table 8-1 in ther HEVC specification) based on the index qPi equal to qPiCb and qPiCr

Table 8-1 – Specification of QPC as a function of qPi

| qPi | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | >43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

For notation simplicity, we note qpOffsetC =  pic_cb_qp_offset + slice_cb_qp_offset.

We depict, for three values of qpOffsetC (-6, 0, 6), in Figure 1 QPc as a function of QPy and in  the ratio of luma and chroma quantization steps (QPSY/QPSC) as a function of QPy. In these examples, QpBdOffsetC=12. It can be observed that:

- There is a slope change in the curves linking luma and chroma quantization parameters at 2 given QPY values (QPY=30 and QPY=43 when qpOffsetC is equal to 0).

- The parameter qpOffsetC non-independently modifies the horizontal position of these 2 points (indicated by a diamond symbol in both figures), as well as the minimum and maximum values (range) of (QPSY/QPSC). Actually qpOffsetC has an impact simultaneously on the horizontal and vertical positions of the slope changes in the curve QPC=f(QPY).

Figure 1: QPC as a function of QPY in HM8.

Figure 2: ratio luma-chroma quantization steps as a function of QPY in HM8.

# Proposal 1: table 8-9 replacement

It is first proposed to simplify the HM design by replacing the table 8-9 of the HEVC specification [1] by the following straightforward formula:

QPC = QPI – Clip3( 0 ,  6 , (qPI – 30 ) >> 1 )		(eq.1)

which results in correspondences QPI-QPC very close to the HM8 correspondence table, as depicted in the table and figures below.

| QPI | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | >43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPC HM8 | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |
| Proposed QPC | = qPi | 30 | 31 | 31 | 32 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | = qPi − 6 |

Figure 3: QPC as a function of QPY in Proposal 1.

Figure 4: ratio luma-chroma quantization steps as a function of QPY in Proposal 1.

# Proposal 2: use of additional control parameters

## Problem of current design for local QP adaptation

Having a fine control of QP is very important to control the quality of luma and chroma of a coded picture. In particular, inside an image, the QPs are often varying in order to adapt the quality to the local content of the image.

This is illustrated in the two pictures below, previously shown in contribution JCTVC-E198 [2]. The top-picture corresponds to an encoded video slice (only top slice in shown) recorded by a camcorder currently available in the market with the AVC encoding functionality. The bottom-picture shows the QP variations inside the slice, showing that the QP is controlled at block level according to the image region characteristics.

Since the QPY may change inside the picture, the related QPC may change accordingly. Therefore being able to finely control the link between QPY and QPC across various values of QPY is very important.

This fine control is not easily achievable with the current design, since as emphasized in the introduction, the parameter qpOffsetC controls in a non-independent way the slope change starting point and the range of the curve linking the luma-chroma quantization step ratio to QPY (or equivalently QPC to QPY).

## Proposal 2 description

In order to have a finer control of the link QPY-QPC, it is proposed to generalize the equation (eq.1) as follows:

QPC = QPI – Clip3( 0 ,  6 , (qPI – cbcr_qp_start ) >> cbcr_qp_shift )		(eq.2)

where:

- cbcr_qp_start  controls the position (starting point) where the slope in the curves linking luma and chroma QPs changes;

- cbcr_qp_shift controls the inclination degree of the slope change.

These two parameters are signaled at the picture (PPS) and possibly slice (slice header) level. When they are not present in the slice header, they are enforced to the value signaled in the PPS. The syntax changes are described in Appendix. Because the slice level signaling may cause storage burden to single-pass deblocking implementations (cf JCTVC-K0145 or JCTVC-K0220), it has been preferred to signal in the software implementation the new parameters only at the picture level.

We show in Figure 5 and Figure 6 the same data as in Figure 1 and Figure 2, in which 3 additional dashed curves have been added, corresponding to Proposal 2 with the following settings:

- qpOffsetC = 0 - cbcr_qp_shift = 1 - cbcr_qp_start = 24 (dashed purple)

- qpOffsetC = 0 - cbcr_qp_shift = 1 - cbcr_qp_start = 34 (dashed blue)

- qpOffsetC = 6 - cbcr_qp_shift = 2 - cbcr_qp_start = 24 (dashed orange)

Figure 5: QPC as a function of QPY.

Figure 6: ratio luma-chroma quantization steps as a function of QPY.

Note: 	for better visual clarity, cbcr_qp_start and cbcr_qp_shift are replaced in the figures by qpStart and qpShift

It can be observed that the two new control parameters cbcr_qp_start and cbcr_qp_shift have a different impact than qpOffsetC. In particular, they have no impact on the range of the luma-chroma quantization step ratio (QPSY/QPSC).

## Min and Max values for the new parameters

The values of cbcr_qp_shift = 1 and cbcr_qp_start = 30 correspond to the Proposal 1. These values reproduce very closely the current HM table 8-9.

Considering that cbcr_qp_shift = 1 is close to the current slope change inclination, it is suggested to enable an inclination twice or half of this default one, which leads to possible values for  cbcr_qp_shift of 0 (twice of default inclination), 1 (default inclination) and 2 (half of the default inclination).

Regarding cbcr_qp_start, it is proposed to focus on the case cbcr_qp_shift = 1 and qpOffsetC=0 (default setting), and to define the limits for cbcr_qp_start in order to have either QPC=QPY or QPC=QPY-6 on the full QPY range. This leads to extreme values -12 and 50, which lead to (QPSY/QPSC)=2 and 1 respectively. This is illustrated in the Figure 8.

Figure 7: QPC as a function of QPY for qp_start=-12 and 50.

Figure 8: ratio (QPSY/QPSC) as a function of QPY for qp_start=-12 and 50.

# Advantages of the proposals

Simplification of table 8-9

Thanks to these proposals, the Table 8-9 can be replaced by a straightforward formula.

Higher flexibility to control the chroma quantization

Thanks to Proposal 2, it is possible to independently control:

- the min and max values of the range of (QPSY/QPSC) using qpOffsetC

- the starting point of the slope change using cbcr_qp_start

- the inclination degree of the slope change using cbcr_qp_shift

This gives much more flexibility to control the chroma QP related to the luma QP, which is an important feature in applications requiring local QP adaptation (CU-level).

Improved chroma deblocking

In the current HM, the QPC used for chroma deblocking is derived from neighboring CUs’ luma QP (QPQ, QPP) and from table 8-9 using qPi computed as follows:

qPi = ( ( QPQ + QPP + 1 ) >> 1 )	(8-298)

The issue is that qpOffsetC is not taken into account, which makes the QPC used for chroma deblocking different from the QPC used for chroma quantization.

Thanks to proposal 2, by replacing the table 8-9 by a simple equation (eq.2) controlled by 2 new parameters, this issue is fixed for these new control parameters. There is more coherency between the chroma quantization and deblocking processes.

# Experiments

The software patch for Proposal 2 is attached to the contribution. For Proposal 1, the change only implies 2 lines changes (modification of the values in table g_aucChromaScale).

## Proposal 1: replacement of table 8-9 by (eq.1)

The impact of proposal 1 has been evaluated according to the Common Test conditions. Intra configurations have not been tested since the change would have no impact (QPCs are unchanged in the 4 QPY cases). Separate Y, U and V BDR measure is reported, as well as the joint YUV BDR measure proposed in JCTVC-G401 [3].

The proposal 1 has a minor impact on the coding performance. A slight gain of 0.6 to 0.8% is observed on chroma, while a loss of 0.1% in luma is observed. The average YUV-BDR gain is of 0.1%. There is no impact on the encoding and decoding runtime. Full results are provided in the attached xls files.

|  | Random Access Main |  |  |  | Random Access HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | 0.0% | -0.9% | -1.0% | -0.1% | 0.0% | -1.0% | -1.2% | -0.1% |
| Class B | 0.1% | -0.5% | -0.3% | 0.0% | 0.1% | -0.6% | -0.3% | 0.0% |
| Class C | 0.1% | -0.4% | -0.5% | 0.0% | 0.1% | -0.4% | -0.5% | -0.1% |
| Class D | 0.1% | -0.7% | -0.6% | -0.1% | 0.0% | -0.7% | -0.6% | -0.1% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.1% | -0.6% | -0.6% | -0.1% | 0.1% | -0.7% | -0.6% | -0.1% |
|  | 0.1% | -0.9% | -1.0% |  | 0.1% | -1.0% | -1.0% |  |
| Class F | 0.1% | -0.2% | 0.0% | 0.0% | 0.1% | -0.2% | -0.1% | 0.1% |
|  | Low delay B Main |  |  |  | Low delay B HE10 |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.1% | -0.7% | -0.9% | -0.1% | 0.1% | -0.8% | -0.6% | -0.1% |
| Class C | 0.1% | -0.8% | -0.8% | -0.1% | 0.1% | -0.6% | -0.6% | -0.1% |
| Class D | 0.1% | -0.4% | -1.2% | -0.2% | 0.1% | -1.3% | -0.9% | -0.2% |
| Class E | 0.2% | -0.7% | -0.5% | 0.0% | 0.0% | -0.7% | -0.8% | -0.1% |
| Overall | 0.1% | -0.6% | -0.8% | -0.1% | 0.1% | -0.8% | -0.7% | -0.2% |
|  | 0.1% | -1.9% | -2.3% |  | 0.1% | -2.3% | -2.6% |  |
| Class F | 0.2% | 0.0% | -0.6% | 0.1% | 0.0% | -0.3% | -1.4% | -0.3% |

## Proposal 2: generic equation (eq.2) with 2 control parameters

We have tested different settings for the two control parameters defined in proposal 2, by using the common test conditions. The results (see also attached xls files) are depicted in the tables below. It can be observed that the balance between chroma and luma can be easily controlled by the two control parameters. Again no impact on the encoding and decoding runtime is observed.

Test 1 : cbcr_qp_start = 29 – cbcr_qp_shift = 1

|  | Random Access Main |  |  |  | Random Access HE10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U |  | V |  | YUV |
| Class A | 0.4% | -6.6% | -6.4% | -0.5% | 0.3% | -6.3% |  |  | -6.5% | -0.6% |
| Class B | 0.6% | -4.6% | -3.9% | -0.4% | 0.6% | -4.6% |  | -3.8% |  | -0.4% |
| Class C | 0.6% | -3.9% | -3.9% | -0.4% | 0.6% | -3.8% |  | -3.9% |  | -0.4% |
| Class D | 0.5% | -3.8% | -3.9% | -0.4% | 0.4% | -4.0% |  | -4.0% |  | -0.5% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.5% | -4.7% | -4.5% | -0.4% | 0.5% | -4.7% |  | -4.5% |  | -0.5% |
|  | 0.5% | -4.5% | -4.3% |  | 0.5% | -4.4% |  | -4.4% |  |  |
| Class F | 0.5% | -2.0% | -2.0% | -0.1% | 0.4% | -1.9% |  | -2.0% |  | -0.1% |
|  | Low delay B Main |  |  |  | Low delay B HE10 |  |  |  |  |  |
|  | Y | U | V | YUV | Y | U |  | V |  | YUV |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.6% | -6.9% | -6.4% | -0.8% | 0.6% | -6.7% |  | -5.8% |  | -0.8% |
| Class C | 0.7% | -5.5% | -5.4% | -0.6% | 0.6% | -5.5% |  | -5.4% |  | -0.7% |
| Class D | 0.4% | -5.5% | -6.0% | -0.7% | 0.5% | -6.4% |  | -6.2% |  | -0.8% |
| Class E | 0.4% | -5.4% | -5.5% | -0.7% | 0.4% | -4.4% |  | -4.4% |  | -0.6% |
| Overall | 0.5% | -5.9% | -5.9% | -0.7% | 0.5% | -5.9% |  | -5.5% |  | -0.7% |
|  | 0.5% | -4.5% | -4.4% |  | 0.5% | -4.6% |  | -4.0% |  |  |
| Class F | 0.7% | -3.6% | -4.1% | -0.3% | 0.5% | -3.4% |  | -4.8% |  | -0.5% |

Test 2 : cbcr_qp_start = 31 – cbcr_qp_shift = 1

|  | Random Access Main |  |  |  | Random Access HE10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U |  |  |  | V | YUV |
| Class A | -0.1% | 3.9% | 3.9% | 0.4% | -0.1% | 3.9% |  |  |  | 3.6% | 0.4% |
| Class B | -0.2% | 4.3% | 4.3% | 0.6% | -0.2% | 4.3% |  |  |  | 4.3% | 0.6% |
| Class C | -0.2% | 3.2% | 3.2% | 0.4% | -0.2% | 3.3% |  |  |  | 3.1% | 0.4% |
| Class D | -0.2% | 3.0% | 3.0% | 0.4% | -0.2% | 3.0% |  |  |  | 2.8% | 0.3% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.2% | 3.6% | 3.6% | 0.4% | -0.2% | 3.7% |  |  |  | 3.5% | 0.4% |
|  | -0.2% | 2.9% | 2.8% |  | -0.2% | 3.0% |  |  |  | 2.6% |  |
| Class F | -0.3% | 3.0% | 3.2% | 0.3% | -0.4% | 3.1% |  |  |  | 2.8% | 0.3% |
|  | Low delay B Main |  |  |  | Low delay B HE10 |  |  |  |  |  |  |
|  | Y | U | V | YUV | Y |  | U |  | V |  | YUV |
| Class A |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | 1.7% | 1.2% | 0.2% | -0.1% |  | 1.4% |  | 1.7% |  | 0.2% |
| Class C | -0.1% | 1.3% | 1.3% | 0.1% | -0.1% |  | 1.2% |  | 1.4% |  | 0.1% |
| Class D | -0.1% | 1.3% | 0.9% | 0.1% | -0.1% |  | 0.9% |  | 0.6% |  | 0.1% |
| Class E | -0.1% | 3.0% | 4.6% | 0.6% | 0.0% |  | 3.1% |  | 2.7% |  | 0.5% |
| Overall | -0.1% | 1.7% | 1.8% | 0.3% | -0.1% |  | 1.5% |  | 1.6% |  | 0.1% |
|  | -0.1% | 1.5% | 1.5% |  | -0.1% |  | 1.3% |  | 1.3% |  |  |
| Class F | 0.0% | 1.7% | 1.3% | 0.3% | -0.3% |  | 1.7% |  | -0.4% |  | -0.1% |

Test 3 : cbcr_qp_start = 22 – cbcr_qp_shift = 2

|  | Random Access Main |  |  |  | Random Access HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | 0.9% | -17.0% | -16.9% | -1.3% | 0.9% | -16.7% | -17.0% | -1.4% |
| Class B | 1.4% | -15.1% | -15.1% | -1.7% | 1.4% | -15.3% | -15.2% | -1.7% |
| Class C | 1.5% | -12.5% | -12.3% | -1.1% | 1.5% | -12.5% | -12.6% | -1.2% |
| Class D | 1.2% | -12.9% | -12.9% | -1.3% | 1.2% | -13.2% | -13.0% | -1.4% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 1.2% | -14.4% | -14.3% | -1.4% | 1.2% | -14.5% | -14.5% | -1.4% |
|  | 1.2% | -14.1% | -13.9% |  | 1.2% | -14.1% | -14.1% |  |
| Class F | 1.3% | -8.5% | -8.6% | -0.6% | 1.4% | -8.3% | -8.7% | -0.5% |
|  | Low delay B Main |  |  |  | Low delay B HE10 |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 1.2% | -16.8% | -17.0% | -1.9% | 1.2% | -16.9% | -16.6% | -1.9% |
| Class C | 1.4% | -13.6% | -13.8% | -1.3% | 1.3% | -14.2% | -14.2% | -1.5% |
| Class D | 1.1% | -13.6% | -14.3% | -1.4% | 1.2% | -15.4% | -15.4% | -1.6% |
| Class E | 0.0% | -14.9% | -14.7% | -2.0% | 0.6% | -14.6% | -15.2% | -2.1% |
| Overall | 1.1% | -14.9% | -15.1% | -1.5% | 1.1% | -15.4% | -15.5% | -1.7% |
|  | 1.1% | -14.5% | -14.6% |  | 1.1% | -15.1% | -15.0% |  |
| Class F | 1.5% | -9.9% | -10.2% | -0.7% | 1.4% | -9.5% | -11.4% | -0.9% |

# Conclusion

This contribution presents two proposals related to the chroma QP derivation.

Proposal 1 replaces the correspondence table used to derive the chroma QP (QPC) from the luma QP (QPY) by a simple formula. This change enables to remove one table from the specification. The BD-rate impact is minor and no visual impact is observed.

Proposal 2 also uses a generic formula and in addition specifies two new parameters to more finely control the relation between luma and chroma QPs. This enables to have much more flexibility to control the chroma QP based on the luma QP, which is of interest in applications requiring local QP adaptation (CU-level). It also ensures a better coherency of the chroma QP derivation in the quantization and deblocking processes.

It is recommended to adopt one of these proposals.

# References

- B. Bross et. al, “High efficiency video coding (HEVC) text specification draft 8”, JCTVC-J1003, 10th meeting, Stockholm, July 2012.

- M. Kobayashi, M. Shima, “Sub-LCU level delta QP signaling”, JCTVC-E198, 5th Meeting, Genève,　CH, 16-23 March, 2011.

- Bin Li, Gary J. Sullivan, Jizheng Xu, “RDO with weighted distortion in HEVC”, JCTVC-G401, 7th Meeting, Geneva, CH, 21-30 November, 2011.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

The changes to syntax and semantics for adding two new chroma QP control parameters at picture- or slice-level offsets are highlighted in yellow below:

## Syntax

#### 7.3.2.3		Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_cb_qp_offset | se(v) |
| pic_cr_qp_offset | se(v) |
| pic_cbcr_qp_start | se(v) |
| pic_cbcr_qp_shift | se(v) |
| pic_slice_level_chroma_qp_offsets_present_flag | u(1) |
| … |  |
| } |  |

### 7.3.3	Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| slice_qp_delta | se(v) |
| if ( pic_slice_level_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| slice_cbcr_qp_start | se(v) |
| slice_cbcr_qp_shift | se(v) |
| } |  |
| … |  |
| } |  |

## Semantics

### 7.4.2.3		Picture parameter set RBSP semantics

pic_cb_qp_offset and pic_cr_qp_offset specify offsets to the luma quantization parameter QP’Y used for deriving QP’Cb and QP’Cr, respectively. The values of pic_cb_qp_offset and pic_cr_qp_offset shall be in the range of −12 to +12, inclusive.

pic_cbcr_qp_start specifies one parameter to control the relation between the chroma quantization parameters QP’Cb and QP’Cr and the luma quantization parameter QP’Y. The value of pic_cbcr_qp_start shall be in the range of −12 to +51, inclusive.

pic_cbcr_qp_shift specifies one parameter to control the relation between the chroma quantization parameters QP’Cb and QP’Cr and the luma quantization parameter QP’Y. The value of pic_cbcr_qp_shift shall be in the range of 0 to 2, inclusive.

pic_slice_level_chroma_qp_offsets_present_flag equal to 1 indicates that the slice_cb_qp_offset, and slice_cr_qp_offset, slice_cbcr_qp_start and slice_cbcr_qp_shift syntax elements are present in the associated slice headers. pic_slice_level_chroma_qp_offsets_present_flag equal to 0 indicates that these syntax elements are not present in the associated slice headers.

### 7.4.3	Slice header semantics

slice_cb_qp_offset specifies a difference to be added to the value of pic_cb_qp_offset when determining the value of the QP’Cb quantization parameter. The value of slice_cb_qp_offset shall be in the range of −12 to +12, inclusive. When slice_cb_qp_offset is not present, it is inferred to be equal to 0. The value of pic_cb_qp_offset + slice_cb_qp_offset shall be in the range of −12 to +12, inclusive.

slice_cr_qp_offset specifies a difference to be added to the value of pic_cr_qp_offset when determining the value of the QP’Cr quantization parameter. The value of slice_cr_qp_offset shall be in the range of −12 to +12, inclusive. When slice_cr_qp_offset is not present, it is inferred to be equal to 0. The value of pic_cr_qp_offset + slice_cr_qp_offset shall be in the range of −12 to +12, inclusive.

slice_cbcr_qp_start specifies one parameter to control the relation between the chroma quantization parameters QP’Cb and QP’Cr and the luma quantization parameter QP’Y. The value of pic_cbcr_qp_start shall be in the range of −12 to +51, inclusive. When not present, slice_cbcr_qp_start is set equal to pic_cbcr_qp_start.

slice_cbcr_qp_shift specifies one parameter to control the relation between the chroma quantization parameters QP’Cb and QP’Cr and the luma quantization parameter QP’Y. The value of pic_cbcr_qp_shift shall be in the range of 0 to 2, inclusive. When not present, slice_cbcr_qp_shift is set equal to pic_cbcr_qp_shift.

deblocking_filter_override_flag equal to 0 specifies that deblocking parameters from the active picture parameter set are used for deblocking the current slice. deblocking_filter_override_flag equal to 0 specifies that deblocking parameters from the slice header are used for deblocking the current slice. When not present, the value of deblocking_filter_override_flag is inferred to be equal to 1.

### 8.6.1	Derivation process for quantization parameters

The changes in the specification are highlighted in yellow.

The variables qPCb and qPCr are set equal to the value of QPC as specified in Table 8-9 based on tThe indexes qPi equal to qPiCb and qPiCr are derived as:

qPiCb = Clip3( −QpBdOffsetC, 57, QPY + pic_cb_qp_offset + slice_cb_qp_offset )	(8-236)

qPiCr = Clip3( −QpBdOffsetC, 57, QPY + pic_cr_qp_offset + slice_cr_qp_offset )	(8-237)

The variables qPCb and qPCr are set equal to the value of QPC derived from the index qPi equal to qPiCb and qPiCr as follows:

QPC = qPi – Clip3(0, 6, (qPi- slice_cbcr_qp_start ) >> slice_cbcr_qp_shift )	(8-238) *

The chroma quantization parameters for Cb and Cr components, QP’Cb and QP’Cr are derived as:

QP’Cb = qPCb + QpBdOffsetC		(8-239)

QP’Cr = qPCr + QpBdOffsetC		(8-240)

Table 8-9 – Specification of QPC as a function of qPi

| qPi | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | >43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

* in proposal 1, slice_cbcr_qp_start is replaced by 30 and slice_cbcr_qp_shift  is replaced by 1.