| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0189 |
| --- | --- |

| Title: | On Annex A |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | TK Tan 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +65 97993861 ttk@pacific.net.sg |
| Source: | NTT DOCOMO, Inc |  |  |

_____________________________

# Abstract

This contribution highlights issues relating to the profiles, tiers and levels specified in Annex A.

# Indication of tiers

The Main and High Tiers are currently indicated by the general_tier_flag.  The general_tier_flag consumes an entire bit leaving only 2 bits for the general_profile_space.  It may be better to keep the 3 bits of the general_profile_space together and use only one pattern out of the 8 possible patterns, rather than consuming one entire bit for general_tier_flag.

| profile_tier_level( ProfilePresentFlag, MaxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( ProfilePresentFlag ) { |  |
| general_profile_space | u(2) |
| general_tier_flag | u(1) |
| general_profile_idc | u(5) |

# Step-wise MaxDPBSize

In the general level specification, the step wise MaxDPBSize is derived based on the PicSizeInSamplesY compared to the MaxLumaPS, as follows:

if ( PicSizeInSamplesY <= ( MaxLumaPS >> 2 ) )
			MaxDpbSize = Min( 4 * MaxDpbPicBuf, 16 )
		else if ( PicSizeInSamplesY <= ( MaxLumaPS >> 1 ) )
			MaxDpbSize = Min( 2 * MaxDpbPicBuf, 16 )
		else if ( PicSizeInSamplesY <= ( MaxLumaPS << 1) / 3 )
			MaxDpbSize = Min( (3 * MaxDpbPicBuf) >> 1, 16 )
		else if ( PicSizeInSamplesY <= ( ( 3 * MaxLumaPS ) >> 2 ) )
			MaxDpbSize = Min( (4 * MaxDpbPicBuf) / 3,  16 )
		else
			MaxDpbSize = MaxDpbPicBuf
where MaxLumaPS is specified in Table A-1 and MaxDpbPicBuf is equal to 6.

The possible values of MaxDPBSize are illustrated in the table below.

| PicSizeInSamplesY | MaxDPBSize = |
| --- | --- |
| <= MaxLumaPS / 4 | 16 |
| <= MaxLumaPS / 2 | 12 |
| <= 2 * MaxLumaPS / 3 | 9 |
| <= 3 * MaxLumaPS / 4 | 8 |
| otherwise | 6 |

This stepwise increment of MaxDPBSize is more complicated than necessary and the differences in number of MaxDPBSize between 6, 8 and 9 are not large.  The stepwise increment of the MaxDPBSize should be simplified by removing the non-dyadic cases of the MaxDPBSize derivation;

if ( PicSizeInSamplesY <= ( MaxLumaPS >> 2 ) )
			MaxDpbSize = Min( 4 * MaxDpbPicBuf, 16 )
		else if ( PicSizeInSamplesY <= ( MaxLumaPS >> 1 ) )
			MaxDpbSize = Min( 2 * MaxDpbPicBuf, 16 )
		else if ( PicSizeInSamplesY <= ( MaxLumaPS << 1) / 3 )
			MaxDpbSize = Min( (3 * MaxDpbPicBuf) >> 1, 16 )
		else if ( PicSizeInSamplesY <= ( ( 3 * MaxLumaPS ) >> 2 ) )
			MaxDpbSize = Min( (4 * MaxDpbPicBuf) / 3,  16 )
		else
			MaxDpbSize = MaxDpbPicBuf
where MaxLumaPS is specified in Table A-1 and MaxDpbPicBuf is equal to 6.

# Maximum level_idc value

The current definition of level_idc uses a value of 30 times the level number specified in Table A-1.

This will only give a maximum level number of 8.5.  Considering that the specification is already at level number 6.2, it may be wiser to use a smaller multiplier such as 20, which will give a maximum level number of 12.75.  Furthermore, since it is unlikely the levels x.9 is reached, we should reserve some space by specifying the level number in steps of 0.2 or 0.3 instead of 0.1 for the current levels already defined.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).