| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0197 |
| --- | --- |

| Title: | Removal of dependency between multiple PUs in a CU for parallel merging candidate list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin, Jianhua Zheng, Xiaozhen Zheng No. 156, Rd. Beiqing Haidian District, Beijing 100095 China | Tel: Email: | +8610-60612372 yblin@huawei.com |
| Source: | Hisilicon Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents an issue on parallel merging candidate list construction when a CU has multiple PUs, e.g., a CU with Nx2N partition includes two PUs, left PU and right PU. For purpose of parallelism of merging candidate list construction for the two PUs, no any dependency between the two PUs is allowed. However, in HM8 implementation, the derivation process of spatial merging candidates for the right PU still depends on the left PU, since the left PU is involved in the pruning process of merging candidate list construction for the right PU. It is proposed to completely remove the dependency between the two PUs for purpose of parallelism of PU-based merging candidate list construction. Experimental results show average coding efficiency of 0.02%(RA-MAIN), 0.00%(RA-HE10), 0.06%(LDB-MAIN) and 0.06%(LDB-HE10) with comparison to HM8.0.

# Introduction

In HM8, a restriction is applied to skip checking MVP candidate of left or above neighbour block for purpose of parallelism of PU-based merging list construction when current PU is the 2nd PU of a CU including two PUs. For example, Fig.1 shows two PUs included in a CU with partition type of Nx2N and five neighbour blocks involved in spatial merging candidate derivation for the 2nd PU, where the neighbour block A1 shall not be checked since A1 is located in the 1st PU.

Fig. 1. Neighbour blocks involved in spatial merging candidate derivation for the 2nd PU in a CU with partition type of Nx2N

However, it is observed that A1 is still used in the pruning process when checking MVP candidates of other neighbour blocks during spatial merging candidate derivation. For example, when the MVP candidate of neighbour block B1 (or A0, B2) is added into the list, A1 will be used to check whether B1 (or A0, B2) have same MV as A1 even if A1 is located in the 1st PU. It means that there still exists dependency between the 1st and 2nd PUs. Therefore the purpose of parallel merging list construction can’t be achieved.

# Proposed algorithm

To enable parallelism of merging candidate list construction, it is proposed to completely remove the dependency between the 1st PU and 2nd PU within a CU.

An additional restriction is applied to the spatial merging candidate derivation for the 2nd PU that the neighbour blocks located in the 1st PU shall not be used in the pruning process involved in checking MVP candidates of the other neighbour blocks. Specifically, the neighbour block A1 located in the 1st PU shall not be used in pruning process when current CU partition type is Nx2N, nLx2N, nRx2N, and the neighbour block B1 located in the 1st PU shall not be used in pruning process when current CU partition type is 2NxN, 2NxnU or 2NxnD. In summary, all those neighbour blocks located in the 1st PU are treated as unavailable to avoid being used in the pruning process for the 2nd PU.

# Experimental results

The proposed method has been integrated in HM8.0. The test results are shown in Table 1. BD-rate change of 0.0%, 0.1% for RA and LDB cases can be observed. The encoding/decoding running time is almost not changed.

Table 1 Test results of the proposal compared with HM8.0

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.01% | -0.09% | -0.01% | 0.00% | -0.03% | 0.03% |
| Class B | 0.02% | 0.05% | 0.02% | 0.03% | 0.04% | 0.06% |
| Class C | 0.02% | 0.09% | 0.07% | 0.01% | 0.05% | 0.00% |
| Class D | 0.04% | 0.01% | -0.03% | -0.02% | -0.09% | -0.03% |
| Class E |  |  |  |  |  |  |
| Overall | 0.02% | 0.02% | 0.01% | 0.00% | -0.01% | 0.02% |
|  | 0.02% | 0.03% | -0.01% | 0.00% | -0.01% | -0.01% |
| Class F | -0.02% | -0.06% | -0.06% | 0.01% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.06% | 0.10% | -0.11% | 0.04% | 0.16% | 0.02% |
| Class C | 0.08% | 0.12% | -0.10% | 0.03% | 0.09% | 0.15% |
| Class D | -0.02% | 0.41% | -0.25% | 0.10% | -0.29% | -0.21% |
| Class E | 0.15% | -0.15% | 0.56% | 0.07% | 0.20% | 0.19% |
| Overall | 0.06% | 0.13% | -0.02% | 0.06% | 0.03% | 0.03% |
|  | 0.07% | 0.12% | 0.01% | 0.06% | 0.04% | -0.02% |
| Class F | 0.16% | 0.22% | 0.18% | 0.00% | 0.07% | 0.02% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Conclusions

This contribution proposes to remove the dependency between multiple PUs within a CU during spatial merging candidate derivation for purpose of parallelism of PU-based merging candidate list construction. Experimental results show average BD-rate change of 0.0% & 0.1% for RA and LDB configurations, respectively, with comparison to HM8.0. It is recommended to inclusion of the proposed method into WD9.

# Text changes

##### 8.5.2.1.2 Derivation process for spatial merging candidates

Inputs to this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable singleMCLFlag,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A0, A1, B0, B1 or B2 and with X being replaced by 0 or 1)

- the availability flags availableFlagN of the neighbouring prediction units,

- the reference indices refIdxLXN of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

- the motion vectors mvLXN of the neighbouring prediction units.

For the derivation of availableFlagN, with N being A0, A1, B0, B1 or B2 and ( xN, yN ) being ( xP − 1,  yP + nPbH ), ( xP − 1,  yP + nPbH − 1 ), ( xP + nPbW,  yP − 1 ), ( xP + nPbW − 1,  yP − 1 ) or ( xP − 1,  yP − 1 ), the following applies.

–	When yP−1 is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY), the following applies.

xB0 = ((xB0>>3)<<3) + ((xB0>>3)&1)*7		(8-1)
xB1 = ((xB1>>3)<<3) + ((xB1>>3)&1)*7		(8-2)
xB2 = ((xB2>>3)<<3) + ((xB2>>3)&1)*7		(8-3)

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xN, yN ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableN.

- If singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and partIdx is equal to 1, availableB1 is set equal to FALSE.

- If singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and partIdx is equal to 1, availableA1 is set equal to FALSE.

–	If one or more of the following conditions are true with X being replaced by 0 and 1, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN is set equal to −1 and predFlagLXN is set equal to 0.

- (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xN >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yN >> (log2_parallel_merge_level_minus2 + 2)).

- N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

- availableN is equal to FALSE

- singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and partIdx is equal to 1 and N is equal to B1

- singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and partIdx is equal to 1 and N is equal to A1

- N is equal to B1 and availableA1 is equal to TRUE and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

- N is equal to B0 and availableB1 is equal to TRUE and the prediction units covering luma location ( xB1, yB1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

- N is equal to A0 and availableA1 is equal to TRUE and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

- N is equal to B2 and availableA1 is equal to TRUE and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

- N is equal to B2 and availableB1 is equal to TRUE and the prediction units covering luma location ( xB1, yB1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

–	Otherwise, availableFlagN is set equal to 1 and the following assignments are made.

mvLXN = MvLX[ xN ][ yN ]		(8-4)
refIdxLXN = RefIdxLX[ xN][ yN ]		(8-5)
predFlagLXN = PredFlagLX[ xN ][ yN ]		(8-6)

# Patent rights declaration(s)

Hisilicon Technologies, Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).