| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0198 |
| --- | --- |

| Title: | AHG7: Cross-check report of adaptive colour-space transformation of residual signals (JCTVC-K0193) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masaaki Matsumura Shohei Matsuo Hiroshi Fujii Seishi Takamura Atsushi Shimizu 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp +81-46-859-2070 matsuo.shohei@lab.ntt.co.jp +81-46-859-8452 fujii.hiroshi@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results for JCTVC-K0193 (a proposal on colour-space transformation for 4:2:2 and 4:4:4 color format sequences).

# Introduction

To support non-4:2:0 formats, AHG7 was established in the 10th JCT-VC meeting [1] and the reference software of HEVC fidelity range extension (FREXT) was released. In the 4:4:4 case, JCTVC-K0193 proposed a color space transformation method for residual of intra/inter prediction by deriving a transform matrix of color space using a block-based singular value decomposition. In addition, for U and V channel, performance of chroma QP clipping was investigated. The detail of the proposal is described in JCTVC-K0193 [2].

# Cross-checked results

The software version was HM8.0 (FrExt.). The reference was defined in AHG7 and generated by using the common test conditions [1]. The provided source code was compiled by using Microsoft Visual Studio 2010 Professional edition, and the experiments were performed on the cluster of PCs with 64-bit Windows 7, Intel Core i7-2600 (3.4GHz) and 16GB RAM. The obtained results are shown in Table 1. As an additional result, chroma QP clipped case is also described in Table 2.

| Table 1: Cross-checked results of KDDI’s proposal |  |  |  |
| --- | --- | --- | --- |
|  | All Intra HE10 (444) |  |  |
|  | Y | U | V |
| Kimono | -6.2% | 0.4% | -1.2% |
| Parkscene | -8.1% | -6.9% | -2.7% |
| BirdsInCage | -7.9% | 8.4% | -11.9% |
| DucksAndLegs | -9.4% | 5.5% | -0.2% |
| Traffic | -2.1% | 1.6% | -1.7% |
| CrowdRun | -4.5% | -7.4% | -7.7% |
| OldTownCross | -3.8% | -1.0% | -2.1% |
| Seeking | -11.5% | -17.0% | -12.6% |
| Overall | -6.7% | -2.1% | -5.0% |
| Enc Time[%] | 146% |  |  |
| Dec Time[%] | 108% |  |  |
|  | Random Access HE10 (444) |  |  |
|  | Y | U | V |
| Kimono | -6.5% | -1.7% | -0.6% |
| Parkscene | -8.1% | -8.3% | -6.0% |
| BirdsInCage | -8.4% | 27.4% | -13.7% |
| DucksAndLegs | -15.9% | 10.6% | -12.8% |
| Traffic | -4.2% | 9.2% | -0.5% |
| CrowdRun | -3.4% | -8.8% | -6.8% |
| OldTownCross | -2.5% | 4.4% | 2.3% |
| Seeking | -8.6% | -21.1% | -9.2% |
| Overall | -7.2% | 1.5% | -5.9% |
| Enc Time[%] | 111% |  |  |
| Dec Time[%] | 115% |  |  |
|  | Low delay B HE10 (444) |  |  |
|  | Y | U | V |
| Kimono | -6.8% | -0.7% | -1.0% |
| Parkscene | -9.6% | -7.0% | -6.5% |
| BirdsInCage | -16.9% | 15.2% | -32.3% |
| DucksAndLegs | -13.8% | 11.8% | -12.3% |
| Traffic | -6.0% | 9.5% | -1.8% |
| CrowdRun | -3.1% | -8.0% | -6.8% |
| OldTownCross | -5.5% | 5.2% | -3.6% |
| Seeking | -8.2% | -19.2% | -9.3% |
| Overall | -8.7% | 0.9% | -9.2% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 116% |  |  |

| Table 2: Cross-checked results of chroma QP clipped case |  |  |  |
| --- | --- | --- | --- |
|  | All Intra HE10 |  |  |
|  | Y | U | V |
| Kimono | -1.8% | -1.2% | -6.0% |
| Parkscene | -3.9% | -8.9% | -6.7% |
| BirdsInCage | -4.9% | 7.6% | -15.6% |
| DucksAndLegs | -4.1% | 4.1% | -2.7% |
| Traffic | 1.3% | -1.8% | -6.0% |
| CrowdRun | -1.8% | -10.9% | -11.8% |
| OldTownCross | -1.9% | -1.3% | -4.4% |
| Seeking | -9.0% | -17.9% | -15.4% |
| Overall | -3.3% | -3.8% | -8.6% |
| Enc Time[%] | 149% |  |  |
| Dec Time[%] | 109% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Kimono | -2.4% | -7.3% | -8.1% |
| Parkscene | -3.7% | -13.2% | -11.7% |
| BirdsInCage | -1.4% | 19.9% | -19.6% |
| DucksAndLegs | -5.2% | 9.0% | -15.9% |
| Traffic | 1.8% | 3.8% | -6.2% |
| CrowdRun | -0.4% | -16.1% | -14.5% |
| OldTownCross | 1.6% | 2.3% | -5.9% |
| Seeking | -5.4% | -25.1% | -16.3% |
| Overall | -1.9% | -3.3% | -12.3% |
| Enc Time[%] | 112% |  |  |
| Dec Time[%] | 116% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Kimono | -3.3% | -6.6% | -9.0% |
| Parkscene | -6.0% | -12.9% | -12.5% |
| BirdsInCage | -11.6% | 11.5% | -39.2% |
| DucksAndLegs | -7.0% | 9.6% | -17.7% |
| Traffic | 0.1% | 4.1% | -8.2% |
| CrowdRun | -0.5% | -15.0% | -14.5% |
| OldTownCross | -3.8% | 3.8% | -12.0% |
| Seeking | -5.6% | -22.7% | -16.3% |
| Overall | -4.7% | -3.5% | -16.2% |
| Enc Time[%] | 110% |  |  |
| Dec Time[%] | 118% |  |  |

# Conclusion

This contribution reports the test results for a modification of the color space transformation method in JCTVC-K0193. The results of coding efficiency completely matched with the proponent’s results. No mismatch between encoder’s local decoded picture and decoder’s output is observed. Relative encoding and decoding time are also confirmed. Source code was reviewed.

# References

- D. Flynn, “BoG report: Extended chroma formats,” JCTVC-H0581.pdf, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11 – 20 July 2012.

- K. Kawamura, T. Yoshino and S. Naito, “AHG7: Adaptive colour-space transformation of residual signals,” JCTVC-K0193, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012.