| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, China, 10–19 October 2012 | Document: JCTVC-K0203 M26530 |
| --- | --- |

| Title: | AHG9: A bug fix for scaling list signalling in SPS when transform skipping is enabled |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Osamoto Minhua Zhou Texas Instruments Inc. 12500 TI Boulevard, Dallas, TX-75243, USA | Tel: Email: Tel: Email: | +1 214 480 4969 osamoto@ti.com +1 214 480 3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In the current HM8.0 design, flat 4x4 default quantization matrices can be used for quantization of 4x4 TUs when transform skipping is used. However, when scaling_list_data() is called from SPS the default 4x4 quantization matrices are undefined because transform_skip_enabled_flag transmitted in PPS is unknown in SPS. In this case, when the quantization matrices (scaling_list_data()) are not carried in PPS (i.e. matrices from SPS are used) but transform skipping is enabled, the spec is broken because a decoder will inherit undefined 4x4 default quantization matrices from SPS for inverse quantization of 4x4 TUs. To fix the problem, it is proposed to insert an additional flag (sps_flat_scaling_list_enabled_flag) in SPS to enable signaling flat 4x4 default matrices in SPS.

# Problem Statement

HM8.0 adopted JCTVC-J0092, which makes 4x4 default scaling list to be flat when transform skipping is enabled in PPS.

In the text, section 7.4.2.4 says:

If transform_skip_enabled_flag is equal to 1 and SizeID is equal to 0, ScalingList[ SizeID ][ MatrixID ][ i ] is set equal to 16 for i=0..15

Otherwise, ScalingList[ SizeID ][ MatrixID ] is specified in Table 7 4 and Table 7 5.

scaling_list_data() is called from SPS as well as PPS. Default 4x4 quantization matrices are undefined in SPS because transform_skip_enabled_flag is unkown in SPS. Therefore, when quantization matrices are not carried in PPS but transform skipping is enabled, the spec is broken as 4x4 default matrices inherited from SPS are undefined.

Example

SPS:

- scaling_list_enable_flag = 1

- sps_scaling_list_data_presetnt_flag = 1

- In scaling_list_data(), intra luma 4x4 scaling list is signalled as a default list (scaling_list_pred_mode_flag = 0 and scaling_list_pred_matrix_id_delta = 0).

PPS :

- transform_skip_enabled_flag  =  1

- pps_scaling_list_data_present_flag = 0

Since pps_scaling_list_data_present_flag = 0 in PPS, we use 4x4 default quantization matrices sent in SPS. But transform_skip_enabled_flag transmitted in PPS is unknown in SPS when 4x4 default quantization matrices are reconstructed in SPS, a decoder does not have proper 4x4 default quantization matrices to do inverse quantization for 4x4 TUs.

# Current Implementation in HM8.0 software

The implementation of TS_FLAT_QUANTIZATION_MATRIX in the HM8.0 software seems incomplete. In the example above, the encoder uses the flat default list, while the decoder uses the non-flat default, so we have encoder-decoder mismatch.

# Proposals

To fix the problem we propose to insert an additional flag (sps_flat_scaling_list_enabled_flag) in SPS to enable signaling flat 4x4 default matrices in SPS. The modified SPS syntax is shown in Table 1.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| … … |  |
| seq_parameter_set_id | ue(v) |
| … …. |  |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) { |  |
| sps_flat_scaling_list_enabled_flag | u(1) |
| scaling_list_data( ) |  |
| } |  |
| } |  |
| … … |  |
| } |  |

Table 1. Modified sequence parameter set syntax

sps_flat_scaling_list_enabled_flag equal to 1 specficies that ScalingList[0][ MatrixID ][ i ] is set equal to 16 for MatrixID = 0..5 and i = 0..15 when both scaling_list_pred_mode_flag and scaling_list_pred_matrix_id_delta are set equal to 0. When not present, the value of sps_flat_scaling_list_enabled_flag is infereed to be equal to 0.

# Reference

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003, May, 2012.

- C. Lan, J. Xu, G. J. Sullivan, F. Wu, “Intra transform skipping,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0408, April, 2012.

# Patent rights declaration

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Proposed text modifications (changes marked in yellow)

#### 7.3.2.2 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … …. |  |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) { |  |
| sps_flat_scaling_list_enabled_flag | u(1) |
| scaling_list_data( ) |  |
| } |  |
| } |  |
| … … |  |
| } |  |

#### 7.4.2.2  Sequence parameter set RBSP semantics

… …

sps_scaling_list_data_present_flag equal to 1 specifies that scaling list data are present in the sequence parameter set. sps_scaling_list_data_present_flag equal to 0 specifies that scaling list data are not present in the sequence parameter set. When not present, the value of sps_scaling_list_data_present_flag is inferred to be 0.

sps_flat_scaling_list_enabled_flag equal to 1 specifies that ScalingList[0][ MatrixID ][ i ] is set equal to 16 for MatrixID = 0..5 and i = 0..15 when both scaling_list_pred_mode_flag and scaling_list_pred_matrix_id_delta are set equal to 0. When not present, the value of sps_flat_scaling_list_enabled_flag is inferred to be equal to 0.

amp_enabled_flag equal to 1 specifies that asymmetric motion partitions, i.e. PartMode equal to PART_2NxnU, PART_2NxnD, PART_nLx2N, or PART_nRx2N, may be used in coding tree blocks; amp_enabled_flag equal to 0 specifies that asymmetric motion partitions cannot be used in coding tree blocks.

… …

7.4.5 Scaling list data semantics

scaling_list_pred_matrix_id_delta specifies the target reference matrix to copy the value of scaling list. When scaling_list_pred_mode_flag is equal to 0, scaling_list_pred_matrix_id_delta specifies which matrix should be used in the current matrix by the following:

RefMatrixID = MatrixID – scaling_list_pred_matrix_id_delta	(‎0-1)

where MatrixID is specified in Table 7-4. The variable RefMatrixID shall be greater than or equal to 0. When scaling_list_pred_matrix_id_delta is equal to 0, i.e. RefMatrixID is equal to MatrixID, the scaling list is inferred from the default scaling list as follows.

–	If sps_flat_scaling_list_enabled_flag is equal to 1, or transform_skip_enabled_flag is equal to 1 and SizeID is equal to 0, ScalingList[ SizeID ][ MatrixID ][ i ] is set equal to 16 for i=0..15

–	Otherwise, ScalingList[ SizeID ][ MatrixID ] is specified in Table 7 4 and Table 7 5.