| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0204v2 |
| --- | --- |

| Title: | VPS syntax for scalable and 3D extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | +1 201 478 6145 jill@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

This attachment contains only the adopted portion of the contribution which impacts the base specification, related to operation point signaling.  It is based upon collaboration with Ye-Kui Wang and Miska Hannuksela.

The v2 revision of this attachment document includes a bug fix and editorial improvements.  Additional changes to syntax and semantics are not change marked with respect to the v1 version; instead all change marks are with respect to the base specification, for improved clarity.

In the definitions section 3.1:

operation point: An operation point is identified by a set of nuh_reserved_zero_6bits values, denoted as OpLayerIdSet, and a TemporalId value, denoted as OpTid, and the associated bitstream subset derived as the output of the sub-bitstream extraction process as specified in subclause Error: Reference source not found with OpTid and OpLayerIdSet as inputs is a conforming bitstream.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_zero_2bits | u(2) |
| vps_reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| vps_reserved_zero_12bits | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_num_hrd_parameters | ue(v) |
| vps_max_nuh_reserved_zero_layer_id | u(6) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| if( i > 0 ) |  |
| operation_point_layer_ids( i ) |  |
| hrd_parameters( i = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_max_nuh_reserved_zero_layer_id specifies that all nuh_reserved_zero_6bits values in the coded video sequence are less than or equal to vps_max_nuh_reserved_zero_layer_id.

| operation_point_layer_ids( opIdx ) { | Descriptor |
| --- | --- |
| op_num_layer_id_values_minus1[ opIdx ] | ue(v) |
| for( i = 0; i <= op_num_layer_id_values_minus1; i++ ) |  |
| op_layer_id[ opIdx ][ i ] | u(6) |
| for( i = 0; i <= vps_max_nuh_reserved_zero_layer_id ; i++ ) |  |
| op_layer_id_included_flag[ opIdx ][ i ] | u(1) |
| } |  |

The operation_point_layer_ids( opIdx ) syntax structure specifies the set of nuh_reserved_zero_6bits values included in the OpLayerIdSet of the operation points to which the opIdx-th hrd_parameters( ) syntax structure in the video parameter set applies.

op_layer_id_incuded_flag[ opIdx ][ i ] equal to 0 specifies that value of nuh_reserved_zero_6bits equal to i is not included in the OpLayerIdSet of the operation points to which the opIdx-th hrd_parameters( ) syntax structure in the video parameter set applies. op_layer_id_incuded_flag[ opIdx ][ i ] equal to 1 specifies that value of nuh_reserved_zero_6bits equal to i is included in the OpLayerIdSet of the operation points to which the opIdx-th hrd_parameters( ) syntax structure in the video parameter set applies. The sum of all op_layer_id_incuded_flag[ opIdx ][ i ], for i from 0 to vps_max_nuh_reserved_zero_layer_id, inclusive, shall be greater than 0 and less than or equal to max_num_layers_minus1.

The variable NumOpLayerIdsMinus1[ opIdx ] and the variables OpLayerId[ opIdx ][ i ], for i in the range of 0 to NumOpLayerIdsMinus1[ opIdx ], inclusive, are derived as follows.

for( i = 0, j = 0; i <= vps_max_nuh_reserved_zero_layer_id; i++ )
	if( op_layer_id_incuded_flag[ opIdx ][ i ] )
		OpLayerId[ opIdx ][ j++ ] = i
NumOpLayerIdsMinus1[ opIdx ] = j – 1

NumOpLayerIdsMinus1[ 0 ] is inferred to be equal to 0. The value of OpLayerId[ 0 ][ 0 ] is inferred to be equal to 0.

Any two sets OpLayerId[ opIdx1 ] and OpLayerId[ opIdx2 ], where opIdx1 is not equal to opIdx2, shall not include the same set of nuh_reserved_zero_6bits values.

The OpLayerIdSet of the operation points to which the opIdx-th hrd_parameters( ) syntax structure in the video parameter set applies is set to include and only include the nuh_reserved_zero_6bits values equal to OpLayerId[ opIdx ][ i ], for i in the range of 0 to NumOpLayerIdsMinus1[ opIdx ], inclusive.

# Patent rights declaration(s)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).