| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0204 |
| --- | --- |

| Title: | VPS syntax for scalable and 3D extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | +1 201 478 6145 jill@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

High level syntax for the scalable and 3D extensions is proposed.   This contribution refers to the JCTVC-J1007 output document from the Stockholm meeting, and proposes several modifications to the VPS extension design in that document.  Some of the proposed changes impact the base HEVC specification VPS.

The proposals are summarized in the list below.

- Simplification of the VPS extension to restrict same number of scalability attribute dimensions for all layers

- Renaming of num_hrd_parameters to num_hrd_operation_points

- Operation point signaling using per layer flags

- Profile & level signaling for operation points

- Profile & level signaling in SPS for enhancement layers

# Proposed VPS and SPS extension changes for scalable and multiview extensions

The proposed VPS extension and SPS syntax changes with respect to JCTVC-J1007 are described below.

## Simplification of the VPS extension to restrict same number of scalability attribute dimensions for all layers

JCTVC-J1007 described two approaches for layer_id partitioning.  In the first approach described in JCTVC-J1007 section 2.2.1, the num_dimensions_minus1[ i ] syntax element is sent for each layer i, and the dimension_type[ i ][ j ] syntax element are sent in a two-dimensional loop, for each layer i and for each dimension j.   There is an error in the syntax table for the j loop, which refers to num_dimensions_minus1 instead of num_dimensions_minus1[ i ].

We propose to simplify the syntax by sending num_dimensions_minus1 only once in the VPS extension and apply its value to all layers.  A single dimensional loop over each layer i can then be used for dimension_type[ i ] syntax element.

If a scalability attribute will be sent for at least one layer, it seems clearer to require that a value for that attribute be sent for all layers.  Allowing different layers to send a differing number of scalability attribute dimensions does not increase scalability flexibility, but may possibly under some conditions save some bits in the VPS, but also will increase the number of bits in the VPS when the number of dimensions is the same, which is assumed to be the more typical case.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_zero_bit | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| num_dimensions_minus1[ i ] | u(4) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) { |  |
| dimension_type[ i ][ j ] | u(4) |
| dimension_id[ i ][ j ] | u(8) |
| } |  |
| num_dimensions_minus1 | u(4) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) |  |
| dimension_type[ j ] | u(4) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j <= num_dimensions_minus1; j++ ) |  |
| dimension_id[ i ][ j ] | u(8) |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

## Renaming of num_hrd_parameters to num_hrd_operation_points

The operation points descriptions sent in the initial section of the VPS (prior to the extension flag) for the HRD, may be re-used in the VPS extension.  The re-naming of the syntax element clarifies that operation point descriptors, and not just HRD parameters are (optionally) present in the VPS.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| reserved_zero_2bits | u(2) |
| max_num_layers_minus1 //reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_level( 1, vps_max_sub_layers_minus1 ) |  |
| next_essential_info_byte_offset //reserved_zero_12bits | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| num_hrd_operation_points num_hrd_parameters | ue(v) |
| for( i = 0; i < num_hrd_operation_points num_hrd_parameters; i++ ) { |  |
| if( i > 0 ) |  |
| operation_point( i ) |  |
| hrd_parameters( i = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| bit_equal_to_one | u(1) |
| vps_extension( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Operation point signaling using per layer flags

A revised signaling of operation point layer signaling is proposed, which differs from the syntax function in the base specification.  The proposed method is simpler, uses only fixed length coding, and requires fewer syntax elements than the method of the base specification.  The proposed method will require fewer bits in typical usage where relatively few layers are present in the coded bitstream, but does not restrict flexibility.

| operation_point( opIdx ) { | Descriptor |
| --- | --- |
| op_num_layer_id_values_minus1[ opIdx ] | ue(v) |
| for( i = 0; i <= op_num_layer_id_values_minus1; i++ ) |  |
| op_layer_id[ opIdx ][ i ] | u(6) |
| for( i = 0; i <= max_num_layers_minus1; i++ ) |  |
| layer_present_in_op_flag[ opIdx ][ i ] | u(1) |
| } |  |

layer_present_in_op_flag[ opIdx ][ i ] equal to 1 specifies that layer i is present in operation point opIdx, equal to 0 specifies that layer i is not present in operation point opIdx.

## Profile & level signaling for operation points

In addition to re-using the HRD operation points from the initial section of the VPS, additional operation points may be sent in the VPS extension.  Profile and level indicators are optionally sent for all operation points, including both those present in the HRD in the initial section of the VPS and those present in the VPS extension.  For each operation point, a profile_present_flag is sent, and when equal to 0, instead of sending the profile related syntax elements for that operation point, a profile_op_ref syntax element is sent to indicate a reference profile from a previously sent operation point.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_zero_bit | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| num_dimensions_minus1[ i ] | u(4) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) { |  |
| dimension_type[ i ][ j ] | u(4) |
| dimension_id[ i ][ j ] | u(8) |
| } |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| // op specific information |  |
| num_additional_operation_points | u(8) |
| for (i = 0 ; i < num_additional_operation_points; i++) |  |
| operation_point( i + num_hrd_operation_points) |  |
| for( i = 1; i <= num_hrd_operation_points + num_additional_operation_points; i++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if (!vps_profile_present_flag[ i ]) |  |
| profile_op_ref[ i ] | u(8) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1) |  |
| } |  |
| } |  |

num_additional_operation_points specifies the maximum number of additional operation points present in the coded video sequences the video parameter set applies.

vps_profile_present_flag[ i ] equal to 1 specifies the profile information for operation point i is present in the profile_tier_level( ) syntax structure.  vps_profile_present_flag[ i ] equal to 0 specifies that profile information for operation point i is not present in the profile_tier_level( ) syntax structure.

When vps_profile_present_flag[ i ] equal to 0, profile information for operation point i is inferred to be equal to the profile information of operation point profile_op_ref[ i ].

## Profile & level signaling in SPS for enhancement layers

In the base HEVC specification, the VPS can be considered to be optional for the base layer, and hence the profile & level information is duplicated in the SPS.  For the enhancement layers however, the VPS is required. Hence, there is no need to replicate the profile information signaling in the SPS for enhancement layers.  A condition is proposed so that profile information is signaled only when layer_id is equal to 0.

Level information is still sent in the SPS for each layer, but in order to allow multiple layers to refer to the same SPS, the level information in the SPS for enhancement layers is interpreted as applying only to the individual layer, unlike the operation point level signaling in the VPS which corresponds to cumulative layers.

In the MVC and SVC extensions to H.264/AVC, for profiles where multiple scalable layers or views are to be decoded, the interpretation of the max MB throughput level constraint is based upon the number of layers or views to be decoded.  In MVC, because the MB throughput limit is based upon the number of views, multiple view layers whose SPS parameter values are otherwise identical, e.g. with the same image resolution and VUI data, differ in their level indicator value.   This means that they must refer to different SPS ids, and separate PPS ids.  This is inefficient in that multiple nearly identical sequence and picture parameter sets must be transmitted, plus an increase in the number of bits in each slice header to refer different picture parameter set id values, which in turn refer to the sequence parameter set id.

We propose that the pixel throughput limitation for the enhancement layers in the HEVC extensions specifies a constraint only on the individual layer, and not on the cumulative layers required for decoding that layer.

The proposed constraint definition will improve coding efficiency vs. the MVC and SVC methods by reducing the number of SPSs required to be transmitted.  Additionally, the number of bits can be reduced for the signaling of the PPS identifier in the coded slice header, because fewer different SPS id values must be signaled, and hence fewer different PPS id values.  The PPS value in the slice header is variable length entropy coded, such that higher values require more bits than smaller values.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_bit | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| profile_tier_level( layer_id = = 0, sps_max_sub_layers_minus1 ) |  |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |

# Patent rights declaration(s)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).