| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0205 |
| --- | --- |

| Title: | TL0 index SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Danny Hong Wonkap Jang | Tel: Email: | +1 201 478 6145 jill@vidyo.com danny@vidyo.com wonkap@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

It is proposed to include a TL0_index SEI message in the HEVC base specification, for improved error resiliency when temporal scalability is used.  The proposed SEI message is similar to that used in the tl0_dep_rep_index SEI message in SVC, with revised syntax and semantics in order to better align with the HEVC base specification design.

# Proposal

The proposed TL0 index SEI message syntax and semantics are similar to that used in the SVC temporal level zero dependency representation index SEI message.

The proposed syntax and semantics differ from those in SVC in the follow ways:

- use RAPs rather than IDRs to reset the TL0 value to 0

- remove references to idr_pic_id, which is not included in HEVC

- remove references to dependency_id, because spatial scalability is not included in base HEVC

- use 16 bits rather than 8 bits for tl0_idx

- use 8 bits rather than 16 bits for rap_idx

The proposed syntax and semantics are shown below, in section 1.1 as proposed, and in section 1.2 with change marks vs. the SVC syntax and semantics.

## Proposed syntax and semantics

- Temporal level zero index SEI message syntax

| tl0 _index( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| tl0_idx | 5 | u(16) |
| rap_idx | 5 | u(8) |
| } |  |  |

- Temporal level zero dependency representation index SEI message semantics

The temporal level zero index SEI message provides a mechanism for detecting whether a coded picture with TemporalId equal to 0 required for decoding the current access unit is available when NAL unit losses are expected during transport.

tl0_idx indicates the temporal level zero index for the current access unit, if TemporalId is equal to 0. Otherwise (TemporalId is greater than 0), tl0_idx indicates the temporal level zero index of the most recent access unit with TemporalId equal to 0 in decoding order.

The following applies:

–	If the current access unit contains a NAL unit with RapPicFlag equal to 1, tl0_idx shall be equal to 0.

–	Otherwise (the current access unit does not contain a NAL unit with RapPicFlag equal to 1), the following is specified:

- Let prevTL0AU be the most recent access unit in decoding order that has TemporalId equal to 0.

- Let prevTL0AUIdx be equal to the value of tl0_idx that is associated with prevTL0AU.

- Depending on TemporalId of the current access unit, the following applies:

–	If TemporalId of the current access unit is equal to 0, tl0_idx shall be equal to ( prevTL0AUIdx + 1 ) % 65536.

–	Otherwise (TemporalId of the current access unit is greater than 0), tl0_ idx shall be equal to prevTL0AUIdx.

rap_idx indicates RAP index for the current access unit.

Let prevRAP be the most recent access unit in decoding order that has RapPicFlag equal to 1.

Let prevRapIdx be equal to the value rap_idx that is associated with prevRAP.

The following applies:

–	If the current access unit contains a NAL unit with RapPicFlag equal to 1, rap_idx shall be equal to ( prevRapIdx + 1 ) % 256.

–	Otherwise (the current access unit does not contain a NAL unit RapPicFlag equal to 1), rap_idx shall be equal to prevRapIdx.

## Changes from SVC syntax and semantics

- Temporal level zero index SEI message syntax

| tl0 _index( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| tl0 _idx | 5 | u(16) |
| rap_idx | 5 | u(8) |
| } |  |  |

- Temporal level zero index SEI message semantics

The temporal level zero index SEI message provides a mechanism for detecting whether a coded picturewith TemporalId equal to 0 required for decoding the current access unit is available when NAL unit losses are expected during transport.

tl0_idx indicates the temporal level zero index for the current access unit, if TemporalId is equal to 0. Otherwise (TemporalId is greater than 0), tl0_idx indicates the temporal level zero index of the most recent access unit with TemporalId equal to 0 in decoding order.

The following applies:

–	If  the current access unit contains a NAL unit with RapPicFlag equal to 1, tl0_idx shall be equal to 0.

–	Otherwise (the  currentaccess unit does not contain a NAL unit with RapPicFlag equal to 1), the following is specified:

- Let prevTL0AU be the most recent access unit in decoding order that has TemporalId equal to 0.

- Let prevTL0AUIdx be equal to the value to tl0_idx that is associated with prevTL0AU.

- tl0_idx

- Depending on TemporalId of the current access unit, the following applies:

–	If TemporalId of the current access unit is equal to 0, tl0_ idx shall be equal to ( prevTL0AUIdx + 1 ) % 65536.

–	Otherwise (TemporalId of the current access unit is greater than 0), tl0_ idx shall be equal to prevTL0AUIdx.

rap_idx indicates RAP index for the current access unit.

The following applies:

- Let prevRAP be the most recent access unit in decoding order that has RapPicFlag equal to 1.

- Let prevRapIdx be equal to the value rap_idx that is associated with prevRAP.

The following applies:

–	If the current access unit contains a NAL unit with RapPicFlag equal to 1, rap_idx shall be equal to ( prevRapIdx + 1 ) % 256.

–	Otherwise (the current access unit does not contain a NAL unit RapPicFlag equal to 1), rap_idx shall be equal to prevRapIdx.

# Patent rights declaration(s)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).