| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0207 |
| --- | --- |

| Title: | AHG7: Update on full-chroma (YUV444) screen content test sequences of JCTVC-H0294 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuhui Wang, Tao Lin, Kailun Zhou | Tel: Email: | 86-21-5103-0961 lintao@tongji.edu.cn |
| Source: | Tongji University, Shanghai, China |  |  |

_____________________________

# Abstract

This contribution proposes five full-chroma (YUV444) screen content video sequences for AHG experiments of HEVC extension on chroma format beyond 420 and screen content coding. The sequences were originally submitted in JCTVC-H0294 and modified in response to the questions and comments raised in San Jose meeting. In particular, all copyright issues are addressed and solved

# Introduction

This contribution re-submits five screen content video sequences that will be made available to JCT-VC and parent bodies by Tongji University. Tongji University proposes to adopt these sequences for the study and improvement of the effectiveness of HEVC standard for screen content coding. YUV 4:2:0 versions of the five sequences are also provided.

# Content description

Five video sequences are provided. Table 1 shows the specifications of the test sequences.

Table 1 Specification of the test sequences

| Resolution | 1920 × 1080p |
| --- | --- |
| Frame rate | 20 fps, progressive |
| Bit depth | 8 |
| Color format | planar YUV 4:4:4 and 4:2:0 |
| Duration | 16.75 seconds (335 frames) |

The test sequences are captured from a computer graphics card HDMI output by a Tongji-customized video capture card in raw RGB 8-bit format.

The RGB sequences are converted to YUV444 format using the following formula commonly used in computer graphics with clipping to 0 or 255:

Y =  0.2990*R + 0.5870*G + 0.1140*B

U = -0.1684*R - 0.3316*G + 0.5000*B + 128

V =  0.5000*R - 0.4187*G - 0.0813*B + 128

A brief description of the test sequences is listed in Table 2, and the 1st frame is shown in Figure 1 to 5.

Table 2 Brief description of the test sequences

| No | File name | Brief description | Features |
| --- | --- | --- | --- |
| 1 | sc1CAD_waveform _1920x1080_20_335_8bit_444.yuv | CAD waveform | - Typical digital circuit design waveform with source code window and schematic window - One pixel thin lines with many colors - Scrolling - Zooming |
| 2 | sc2CAD_waveform _Street_Campus _1920x1080_20_335_8bit_444.yuv | CAD waveform plus two video clip playback windows | - All of TJU_CADwaveform.yuv - Street car/bus video clip playback window - TJU campus video clip playback window |
| 3 | sc3PCB_layout.yuv | PCB layout windows | - Typical PCB layout window - Display of multiple layers with different colors - One pixel diagonal, horizontal, vertical lines and curves with different colors - Enabling/disabling display of some layers - Moving around - Zooming in and out |
| 4 | sc4_PCB_schematic _1920x1080_20_335_8bit_444.yuv | PCB schematic windows | - Typical PCB schematic window - Schematic-layout comparison window - Component footprint display - One pixel diagonal, horizontal, vertical lines and curves - Moving around - Zooming in and out |
| 5 | sc5_ppt_doc_xls _1920x1080_20_335_8bit_444.yuv | Ppt, doc, and xls windows | - Text, graph, chart - Copy and paste between ppt, doc, and xls windows - One pixel thin lines and curves - Zooming chart and graph |

Figure 1 TJU_CAD_waveform.yuv

Figure 2 TJU_CAD_waveform_Street_Campus.yuv

Figure 3 TJU_PCB_layout.yuv

Figure 4 TJU_PCB_schematic.yuv

Figure 5 TJU_ppt_doc_xls.yuv

# Subjective Visual Quality Comparison

The five test sequences are coded using both HM8.0ECF444 (HM-8.0-dev-ahg7) software and HM8.0ECF444 Dual-coder software of JCTVC-K0134. HM8.0ECF444 Dual-coder shows quite significant subjective visual quality improvement over HM8.0ECF444 at the same very low bitrate. The following Figure 6 is a partial comparison from constructed pictures of the test sequence sc3PCB_layout.yuv.

# Conclusion

We propose to adopt these sequences for the study and improvement of the effectiveness of HEVC standard for screen content coding.

# Conditions of Use – Copyright and Release Notes

These sequences have been generated by Tongji University, and all intellectual property rights remain with Tongji University. These materials shall be used for research and development purposes only.

This copyright and permission notice shall be duplicated whenever the data are copied or distributed.

These materials cannot be distributed with charge. Tongji University makes no warranties with respect to the material and expressly disclaims any warranties regarding its fitness for any purpose. Unless the above conditions are agreed to by the recipient, no permission is granted for any use, copying, modification, and distribution of the data. By using these sequences the user agrees to the conditions of this copyright and disclaimer.

Lines & circles are broken, lose details, distorted, or discolored

No distortion

Hybrid coder: 598kbps

Dual-coder: 550kbps

z

Figure 6 Subjective visual quality comparison

# Underlying Microsoft Product Right

Microsoft Windows and Office were used in the screen shots with permission from Microsoft. See Use of Microsoft Copyrighted Content (http://www.microsoft.com/about/legal/en/us/intellectualproperty/permissions/default.aspx):

# Availability of the Sequences

Tongji University agrees to donate the test sequences to the video coding community and to upload the test sequences to JCTVC website under the Condition of Use set forth on  of this document. Contact email: vlsi@tongji.edu.cn.