| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0209 |
| --- | --- |

| Title: | AHG9: VPS and SPS design |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Tammy Lee Yongjin Cho Jeonghoon Park | Tel: Email: | b.d.choi@samsung.com tammy.lee@samsung.com yongjin9.cho@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, two VPS and SPS designs are proposed. The first design is to signal multiple cropping windows in VPS for layered streams. All possible candidates of cropping windows for all layers are signalled in VPS, and each set of cropping parameters is mapped to an index. In each SPS, the indices of cropping windows used for the layer associated with the SPS are signalled. The second design is a flexible design to remove duplicated syntax signalling between VPS and SPSs. By a flag indicating whether VPS parsing can be skipped or not, the duplicated syntax elements are signalled or not in SPS. The flag indicating the duplication has the same role as the vps_skip_flag in the second design proposed in JCTVC-K0177 [1].

# VPS and SPS design for multiple cropping windows

Output pictures can be displayed in various spatial resolutions with offsets according to the display size or the region of interests. Especially for frame packing arrangement (FPA), scalability/3D extension, and their mixtures, signalling multiple cropping offsets is necessary for various types of displays, e.g. 2D or 3D. When layered streams are extracted and displayed, each layer may have different cropping windows. For example, while one cropping window can cover all layers when the only quality scalability having the same spatial resolution in all layers is used, various cropping windows should be supported for such purposes if the spatial resolution scalability and the 3D extension are mixed. In the proposed design, all possible candidates of cropping offset parameters for all layers are signalled in VPS. Each set of cropping parameters is mapped to an index. In each SPS, the indices of cropping windows used for the layer associated with the SPS are signalled. Without explicitly signalling the current cropping window, the indexed cropping window at first is the default. To use other cropping windows instead of the default window, the indices of cropping windows should be signalled in SEI, VUI messages, or other means. In the following sub-section, a design of frame packing arrangement SEI message to explicitly signalling two target cropping window indices (left, right) is proposed.

## VPS and SPS design

Texts not highlighted are the same as in the latest HEVC WD, and those highlighted in yellow are the changed parts compared to the latest HEVC WD.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| ….. |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| vps_num_pic_cropping | u(4) |
| for( i = 0; i < vps_num_pic_cropping; i++ ) { |  |
| vps_pic_crop_left_offset[ i ] | ue(v) |
| vps_pic_crop_right_offset[ i ] | ue(v) |
| vps_pic_crop_top_offset[ i ] | ue(v) |
| vps_pic_crop_bottom_offset[ i ] | ue(v) |
| } |  |
| ….. |  |
| } |  |

vps_num_pic_cropping specifies the number of the picture cropping offset parameters in the video parameter set and the number of cropping windows in all layers associated with the current VPS. The number of num_pic_cropping shall be in the range of 0 to 15, inclusive.

vps_pic_crop_left_offset[ i ], vps_pic_crop_right_offset[ i ], vps_pic_crop_top_offset[ i ], and vps_pic_crop_bottom_offset[ i ] specify an i-th cropping rectangle of the samples of the pictures in all layers in the coded video sequence that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| … |  |
| pic_cropping_ flag | u(1) |
| If(pic_cropping_ flag ) { |  |
| sps_cropping_idx_flag | u(1) |
| sps_num_pic_cropping | u(3) |
| if (sps_cropping_idx_flag){ |  |
| for( i = 0; i < sps_num_pic_cropping; i++ ) { |  |
| cropping_idx[i] | u(4) |
| } |  |
| } else { |  |
| for( i = 0; i < sps_num_pic_cropping; i++ ) { |  |
| sps_pic_crop_left_offset[ i ] | ue(v) |
| sps_pic_crop_right_offset[ i ] | ue(v) |
| sps_pic_crop_top_offset[ i ] | ue(v) |
| sps_pic_crop_bottom_offset[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

sps_cropping_idx_flag equal to 1 specifies that the indices are signaled in the SPS. The indices indicate which cropping offset parameters in VPS will be used in the slices associated with the current SPS. sps_cropping_idx_flag equal to 0 specifies that the croppping offset parameters are signaled in the SPS.

NOTE - In HEVC base spec, sps_cropping_idx_flag is set equal to 0, when VPS parsing is skipped.

sps_num_pic_cropping specifies the number of the picture cropping offset parameters in the sequence parameter set and the number of cropping windows in all pictures associated with the current SPS. The number of num_pic_cropping shall be in the range of 0 to 3, inclusive.

sps_pic_crop_left_offset[ i ], sps_pic_crop_right_offset[ i ], sps_pic_crop_top_offset[ i ], and sps_pic_crop_bottom_offset[ i ] specify an i-th cropping rectangle of the samples of the pictures in the coded video sequence that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output.

When sps_cropping_idx_flag is equal to 1,

pic_crop_left_offset[ i] = vps_pic_crop_left_offset[cropping_idx[i]];

pic_crop_right_offset[ i] = vps_pic_crop_ right _offset[cropping_idx[i]];

pic_crop_top_offset[ i] = vps_pic_crop_top_offset[cropping_idx[i]];

pic_crop_bottom_offset[ i] = vps_pic_crop_bottom_offset[cropping_idx[i]];

When sps_cropping_idx_flag is equal to 0,

pic_crop_left_offset[ i] = sps_pic_crop_left_offset[i];

pic_crop_right_offset[ i] = sps_pic_crop_ right _offset[i];

pic_crop_top_offset[ i] = sps_pic_crop_top_offset[i];

pic_crop_bottom_offset[ i] = sps_pic_crop_bottom_offset[i];

## Design of frame packing arrangement SEI message

In this section, a design of frame packing arrangement SEI message to explicitly signalling two target cropping window indices is proposed.

| frame_packing_arrangement( payloadSize ) { | Descriptor |
| --- | --- |
| frame_packing_arrangement_id | ue(v) |
| …. |  |
| Left_TargetPicCropIdx | u(3) |
| Right_TargetPicCropIdx | u(3) |
| …… |  |
| } |  |

Left_TargetPicCropIdx specifies the index of the target output cropping window for left view.

Right_TargetPicCropIdx specifies the index of the target output cropping window for right view.

NOTE - pic_crop_left_offset[Left_TargetPicCropIdx],  pic_crop_right_offset[Left_TargetPicCropIdx], pic_crop_top_offset[ Left_TargetPicCropIdx], and pic_crop_bottom_offset[Left_TargetPicCropIdx] indicates the target window for left view. pic_crop_left_offset[Right_TargetPicCropIdx],  pic_crop_right_offset[Right_TargetPicCropIdx], pic_crop_top_offset[ Right_TargetPicCropIdx], and pic_crop_bottom_offset[Right_TargetPicCropIdx] indicates the target window for right view.

# VPS and SPS design on syntax duplication

In HEVC scalability extension spec, some parts of SPS syntax will be duplicated and redundant between layers. An efficient way to reduce the redundant bits is that those overlapped syntax elements are signalled in VPS once. However, if it is not mandatory to parse the VPS in HEVC base spec, those syntax should be duplicated between VPS and SPS.

In this section, a flag (nuh_vps_duplication_flag) is proposed in VPS NAL unit header. The flag indicates whether VPS parsing is mandatory or not. If VPS parsing is mandatory, the duplicated syntax elements in SPSs are signalled only in VPS once. Otherwise, if the VPS parsing can be skipped in HEVC base layer, those syntax elements are signalled in both VPS and SPS.

The proposed nuh_vps_duplication_flag can have the same role as the nuh_vps_skip_flag in the second design proposed in JCTVC-K0177 [1].

Texts not highlighted are the same as in the latest HEVC WD, and those highlighted in yellow are the changed parts compared to the latest HEVC WD.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| if( nal_unit_type = = VPS_NUT ) { |  |
| nuh_vps_duplication_flag | u(1) |
| nuh_reserved_one_8bits | u(8) |
| } else { |  |
| nuh_reserved_zero_6bits | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |
| } |  |

nuh_vps_ duplication _flag shall be equal to 1. Other values of nuh_vps_duplication_flag may be specified in the future by ITU-T | ISO/IEC.

- NOTE 1 – When nuh_vps_duplication_flag is equal to 1, the syntax elements in VPS are duplicated in SPS for enabling to skip VPS parsing. When nuh_vps_duplication_flag is equal to 0, the syntax elements in VPS are not duplicated in SPS. Then, parsing VPS is mandatory in the future extension.

nuh_reserved_one_8bits shall be equal to 1. Other values of nuh_reserved_one_8bits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore the value of nuh_reserved_one_8bits.

nuh_reserved_zero_6bits shall be equal to 0. Other values of nuh_reserved_zero_6bits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore the value of nuh_reserved_zero_6bits.

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_bit | u(1) |
| If( nuh_vps_ duplication _flag ) { |  |
| profile_and_level( 1, sps_max_sub_layers_minus1 ) |  |
| ……… |  |
| } |  |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |

# References

- B. Choi, J. Park, Y.-K. Wang, and Y. Chen, “AHG9: NAL unit header with layer ID partitioning”, JCTVC-K0177, 11th meeting, Shanghai, Oct. 2012.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).