| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0210 |
| --- | --- |

| Title: | AHG9: NAL unit header design for base spec |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Tammy Lee Chanyul Kim Jeonghoon Park | Tel: Email: | b.d.choi@samsung.com tammy.lee@samsung.com dionism@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the latest HEVC WD, 6bits are reserved for layer_id in NAL unit header. However, even if layer_id should be signalled in NAL unit headers in HEVC extension, it does not mean that the bits for layer_id should be reserved and be fixed to a pre-defined value in HEVC base spec. If a way to distinguish base layer NAL units from enhancement layer NAL units is present, the reserved bits can be used for other purposes than layer_id in HEVC base spec. In this document, we propose two alternative NAL unit header designs for distinguishing base layer NAL units and non-base layer NAL units. Then, how to use the reserved bits in NAL unit header in HEVC base will be introduced. Some part of this contribution overlaps the second design proposed in JCTVC-K0177 [1].

# Introduction

In the latest HEVC WD, 6bits are reserved for layer_id in NAL unit header. The generic layer_id will be mapped to several scalability identifiers in HEVC scalability extension spec. Conforming to the current NAL unit header design, we assert two points for more efficient usage of NAL unit header bits. First, even if layer_id should be signalled in NAL unit headers in HEVC extension, it does not mean that the bits for layer_id should be reserved in HEVC base spec. If a way to distinguish base layer NAL units from enhancement layer NAL units is present, the reserved bits can be used for other purposes than layer_id in the HEVC base spec. Second, 6 bits are not enough to represent all layer identifications when multiple scalability types are used. To resolve the two points mentioned above, we propose two alternative NAL unit header designs for distinguishing base layer NAL units and non-base layer NAL units. Then, how to use the reserved bits in NAL unit header in HEVC base will be introduced.

The remaining parts of this document consist of the introduction of the two alternative designs to enable signalling layer_id only in NAL unit headers of enhancement layers and some syntax elements which can replace reserved_6_zero_bits in the HEVC base spec.

# NAL unit header design for the base layer identification

In this section, two alternative ways to distinguish base layer NAL units and non-base layer NAL units are introduced. The first one is to use a bit flag in NAL unit header to differentiate base layer NAL units from enhancement layer NAL units. It is an intuitive and easy-to-parsing way. The second one is to assign new NAL unit types for non-base layer VCL NAL units. By separating base layer VCL NAL unit types and enhancement layer VCL NAL unit types, base layer NAL units can be extracted without signaling a flag. For this approach, a bit can be additionally allocated to NAL unit type bits to increase the number of NAL unit types. In both designs, by merging nuh_reserved_zero_6bits and nuh_temporal_id_plus1 to a syntax element to represent layer_id (layer_id_plus1) in non-base layers, totally 8 or 9 bits can be allocated to a generic layer_id.

## NAL unit header design with a baselayer identification flag

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| nuh_baselayer_flag | u(1) |
| if( nuh_baselayer_flag) { |  |
| nuh_reserved_zero_5bits | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| } else { |  |
| nuh_reserved_one_8bits | u(8) |
| } |  |
| } |  |

nuh_baselayer_flag is equal to 1 if the NAL unit belongs to a base layer. nuh_baselayer_flag is equal to 0 if the NAL unit does not belong to a base layer.

nuh_reserved_zero_5bits shall be equal to 0 if nuh_baselayer_flag is equal to 1. Other values of nuh_reserved_zero_5bits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore the value of nuh_reserved_zero_5bits.

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier of a NAL unit.

layer_id_plus1 minus 1 identifies the layer that a NAL unit belongs to. The scalability layer identifiers of each NAL unit is derived to be equal to layer_id − 1.

When nuh_baselayer_flag is equal to 0, and TemporalId is not derived from layer_id, TemporalId is set equal to the value of TemporalId of the base layer NAL unit in the same access unit.

In the proposed NAL unit header design, nuh_baselayer_flag is signalled in the first byte of NAL unit header. This flag indicates whether the current NAL unit belongs to a base layer or not. According to the value of this flag, the following second byte of NAL unit header will have different syntaxes. In base layers (nuh_baselayer_flag == 1), 3 bits are allocated for temporal_id and 5bits are reserved for other purposes except layer_id. In enhancement layers, all 8bits are allocated for layer_id. The temporal_id in enhancement layers can be derived from layer_id or temporal_id of the base layer as described in JCTVC-K0177.

## NAL unit header design with NUT separation

This section describes the second design about how to distinguish base layer NAL units and non-base layer NAL units. Instead of using a bit flag (nuh_baselayer_flag), base layer NAL units can be distinguished from non-base NAL units by nal_unit_type. To enable this, all VCL NAL unit types and picture types for non-base layers are defined, such as IDR_EN, CRA_EN, BLA_EN, and so on. If nal_unit_type belongs to non-base VCL NAL units, the NAL unit is classified to the enhancement layer and layer_id is signalled in NAL unit header. Otherwise, the reserved bits will be used for other purposes.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(7) |
| if(nal_unit_type >14 && nal_unit_type < 27) { |  |
| layer_id_plus1 | u(8) |
| } else { |  |
| nuh_reserved_zero_5bits | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |
| } |  |

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | UNSPEC0 | Unspecified | non-VCL |
| 1, 2 | TRAIL_R, TRAIL_N | Coded slice of a non-TSA, non-STSA trailing picture slice_layer_rbsp( ) | VCL |
| 3, 4 | TSA_R, TSA_N | Coded slice of a TSA picture slice_layer_rbsp( ) | VCL |
| 5, 6 | STSA_R, STSA_N | Coded slice of an STSA picture slice_layer_rbsp( ) | VCL |
| 7, 8, 9 | BLA_W_TFD BLA_W_DLP BLA_N_LP | Coded slice of a BLA picture slice_layer_rbsp( ) | VCL |
| 10, 11 | IDR_W_LP IDR_N_LP | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 12 | CRA_NUT | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 13 | DLP_NUT | Coded slice of a DLP picture slice_layer_rbsp( ) | VCL |
| 14 | TFD_NUT | Coded slice of a TFD picture slice_layer_rbsp( ) | VCL |
| 15, 16 | TSA_R_EN, TSA_N_EN | Coded slice of a TSA picture in enhancement layers slice_layer_extension_rbsp ( ) | VCL |
| 17, 18 | STSA_R_EN, STSA_N_EN | Coded slice of an STSA picture in enhancement layers slice_layer_extension _rbsp( ) | VCL |
| 19, 20, 21 | BLA_W_TFD_EN BLA_W_DLP_EN BLA_N_LP_EN | Coded slice of a BLA picture in enhancement layers slice_layer_extension _rbsp( ) | VCL |
| 22, 23 | IDR_W_LP_EN IDR_N_LP_EN | Coded slice of an IDR picture in enhancement layers slice_layer_extension _rbsp( ) | VCL |
| 24 | CRA_NUT_EN | Coded slice of a CRA picture in enhancement layers slice_layer_extension _rbsp( ) | VCL |
| 25 | DLP_NUT_EN | Coded slice of a DLP picture in enhancement layers slice_layer_extension _rbsp( ) | VCL |
| 26 | TFD_NUT_EN | Coded slice of a TFD picture in enhancement layers slice_layer_extension _rbsp( ) | VCL |
| 27 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| ……. |  |  |  |
| 34 | SEI_NUT | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 35..127 | RSV_NVCL33.. RSV_NVCL47 | Reserved | non-VCL |

# Proposed syntax elements in NAL unit header for HEVC base spec.

This section proposes how to use the reserved bits in NAL unit headers in the HEVC base spec, instead of layer_id. The nuh_reserved_zero_5bits in the proposed NAL unit header designs can be replaced by the following syntax elements. However, in future, other syntax elements are also possible to be considered.

1. Inter-layer prediction flag

nuh_ip_flag equal to 0 indicates that the slice is not used by the decoding of any other layer.

It enables an extractor easily tell which NAL unit should be passed and which can be discarded to save bandwidth.

2. Output flag

nuh_pic_output_flag affects the decoded picture output and removal processes. When pic_output_flag is not present, it is inferred to be equal to 1. When pic_output_flag is equal to 1, the picture is displayed. When pic_output_flag is equal to 0, the picture is not displayed.

output_flag can move to NAL unit header again, same as AVC/SVC design.

3. Prior pictures output flag

nuh_no_output_of_prior_pics_flag specifies how the previously-decoded pictures in the decoded picture buffer are treated after decoding of an IDR or a BLA picture. When the IDR or BLA picture is the first picture in the bitstream, the value of no_output_of_prior_pics_flag has no effect on the decoding process. When the IDR or BLA picture is not the picture in the bitstream and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] derived from the sequence parameter set active for the preceding picture, no_output_of_prior_pics_flag equal to 1 may (but should not) be inferred by the decoder, regardless of the actual value of no_output_of_prior_pics_flag.

Note that the values of no_output_of_prior_pics_flag need to be changed in changing of a CRA picture to a BLA picture. If no_output_of_prior_pics_flag is signalled in NAL unit header, changing the value of no_output_of_prior_pics_flag will be easier in systems.

4. Reference flag

nuh_ref_flag equal to 1 specifies that the content of the NAL unit contains a sequence parameter set, a picture parameter set, or a slice of a reference picture. nuh_ref_flag equal to 0 for a NAL unit containing a slice indicates that the slice is part of a non-reference picture.

For simplification of NAL unit types, reusing nuh_ref_flag is recommended. Please see JCTVC-K0208 for more information on NAL unit type simplification [2].

5. Discarding flag

nuh_discard_flag equal to 1 specifies that the picture in NAL unit cannot be correctly decoded and the NAL unit shall be discarded. nuh_discard_flag equal to 0 specifies that the picture in NAL unit can be correctly decoded .

For simplification of NAL unit types, using nuh_discard_flag is recommended instead of TFD-related NAL unit and picture types. Please see JCTVC-K0208 for more information on NAL unit type simplification.

6. Video codec standard type

nuh_standard_type indentifies a video codec standard (e.g. HEVC, H.264/AVC, MPEG-2). standard_type equal to 0 indicates that the current layer is conforming to this Recommendation | International Standard. Other values for standard_type are reserved for non-HEVC video coding standards.

This syntax element is helpful for multi-standard video coding.

# References

- B. Choi, J. Park, Y.-K. Wang, and Y. Chen, “AHG9: NAL unit header with layer ID partitioning”, JCTVC-K0177, 11th meeting, Shanghai, Oct. 2012.

- B. Choi, T. Lee, Y. Park, and J. Park, “AHG9: On random access point pictures”, JCTVC-K0208, 11th meeting, Shanghai, Oct. 2012.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).