| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0213-r3 |
| --- | --- |

| Title: | AHG9: On Tickets 637, 763, and 777 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 44 431 7588 chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution reviews unclosed tickets 637, 763, and 777 related to I_PCM. Tickets 637 and 763 are editorial issues. Ticket 777 addresses the potential use of wrong combinations of pcm_enable_flag, log2_min_coding_block_size_minus3, and log2_min_pcm_coding_block_size_minus3 values. The issue is an encoder-side issue but can be solved by restricting the minimum CU size less than 64 when pcm_enable_flag is equal to 1. It is recommended that the issues are resolved during the 11th JCTVC meeting.

# Ticket 637: Typos in 9.3 CABAC parsing process for slice

It is suggested that descriptions of initializing arithmetic decoding/encoding engine in 9.3.1.4 and 9.3.4.1 are improved as follows:

Replace the following first sentence of 9.3.1.4

"This process is invoked before decoding the first coding tree block of a slice."

by

"This process is invoked before decoding the first coding tree block of a slice and and after decoding of any pcm_alignment_zero_bit, num_subsequent_pcm, and all pcm_sample_luma and pcm_sample_chroma data."

Replace the following second paragraph of 9.3.4.1

"This process is invoked before encoding the first coding block of a slice, and after encoding any pcm_alignment_zero_bit and all pcm_sample_luma and pcm_sample_chroma data for a coding unit with pcm_flag equal to 1."

by

"This process is invoked before encoding the first coding block of a slice, and after encoding of any pcm_alignment_zero_bit, num_subsequent_pcm, and all pcm_sample_luma and pcm_sample_chroma data for a coding unit with pcm_flag equal to 1."

# Ticket 763: Default value for "split_cu_flag" when NumPCMBlock is not equal to 0

It is suggested that the default value for "split_cu_flag" is specified in the semantics of 7.4.8.4 Coding quadtree semantics as follows:

split_cu_flag[ x0 ][ y0 ] specifies whether a coding unit is split into coding units with half horizontal and vertical size. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When split_cu_flag[ x0 ][ y0 ] is not present, the following applies:

–	If log2CbSize is greater than Log2MinCbSizeY and the current NumPCMBlock is equal to 0, the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 1.

–	Otherwise (log2CbSize is equal to Log2MinCbSizeY or the current NumPCMBlock is not equal to 0), the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 0.

However, it was identified that the above change makes it legal to send a PCM block beyond the picture border. The example is below. (Supplemental information is provided by the attachment “JCTVC-K0213-Ticket763.xlsx”.)

Figure 1 Example of border violation.

In order to keep the above case illegal, it is suggested the following change is considered for dealing with the Ticket 763 issue.

split_cu_flag[ x0 ][ y0 ] specifies whether a coding unit is split into coding units with half horizontal and vertical size. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When split_cu_flag[ x0 ][ y0 ] is not present, the following applies:

–	If log2CbSize is greater than Log2MinCbSizeY and one or more of the following conditions are true, the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 1.

–	(x0 + ( 1 << log2CbSize )) is greater than pic_width_in_luma_samples.

–	(y0 + ( 1 << log2CbSize )) is greater than pic_height_in_luma_samples.

–	Otherwise, the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 0.

# Ticket 777: Clarification of PCM coding size

The potential use of a wrong combination of pcm_enable_flag, log2_min_coding_block_size_minus3, and log2_min_pcm_coding_block_size_minus3 values is addressed.

Although the issue is an encoder-side issue, it is suggested in the site that when pcm_enable_flag is equal to 1, constraint on log2_min_coding_block_size_minus3 is specified in 7.4.2.2 Sequence parameter set RBSP semantics as follows:

log2_min_luma_coding_block_size_minus3 specifies the minimum size of a luma coding block. It is a requirement of bitstream conformance that when pcm_enabled_flag is equal to 1, log2_min_coding_block_size_minus3 shall be less than 3.

# Recommendation

It is recommended that the issues are resolved during the 11th JCTVC meeting.

# Patent rights declaration

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).