| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0214_r2 |
| --- | --- |

| Title: | Early bumping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Jonatan Samuelsson Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46 761153563 rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents a proposed change in the output order decoder conformance section in Annex C of the HEVC specification. Currently, the bumping process is invoked after the first slice of a picture has been parsed. This contribution claims that this may cause unwanted output delay. The document discusses whether “early” picture output can be done directly after a picture has been decoded or not, but concludes that this is not possible according to the current HEVC specification since a decoder at that point in time can not know whether the following picture is a random access picture with no_output_of_prior_pics_flag set to 1 or not. To reduce the output delay in HEVC, this document proposes to point out that output of pictures directly after decoding is possible by adding the following NOTE to section "C.5.3 Picture decoding, marking and storage":

NOTE - When the number of pictures in the DPB that are marked as "needed for output" is greater than sps_max_num_reorder_pics[ HighestTid ] after the current decoded picture has been stored in the DPB, the picture in the DPB with the smallest value of PicOrderCntVal of all pictures in the DPB that are marked as "needed for output" may be cropped, output and marked as "not needed for output" by an output order conforming decoder in order to reduce output delay.

# Introduction

One of the functionalities in HEVC that was inherited from AVC was the no_output_of_prior_pics_flag which is present in the slice header in both standards. This flag tells the decoder whether pictures that are not output yet (all pictures in the DPB that are marked "needed for output") should be output or not when an IDR or BLA picture is decoded. If the flag is 1, all pictures are output and if the flag is 0, no picture is output.

One difference between HEVC and AVC is that the picture marking and bumping process in HEVC is invoked when the slice header is parsed while for AVC those processes are done after decoding of a picture, see Figure 1 below:

Figure 1 - Bumping in AVC and HEVC

If a decoder implementation is done according to Figure 1, there will be some extra delay in the bumping process of HEVC compared with AVC.

It might seem possible to perform bumping after a complete picture has been decoded also in HEVC in most cases, since you know which picture in the DPB (including the newly decoded picture) that has the lowest POC value.

Figure 2 - An early bumping example

Figure 2 shows a very simple picture structure. The idea with early bumping is to try to display each picture as soon as it has been decoded, when possible. This could potentially be done in a conforming decoder since the decoder strictly does not have to invoke the picture output process according to the HEVC specification as long as the output order is unchanged. Bumping early in Figure 2 includes outputting each picture as soon as it has been decoded.

A problem with bumping early is the definition of no_output_of_prior_pics_flag. The flag is present for random access point (RAP) pictures and controls whether the pictures in the DPB shall be output or not. If no_output_of_prior_pics_flag is set to 0 for the RAP in Figure 2, picture B is displayed. But if no_output_of_prior_pics_flag is set to 1, picture B is not displayed since it according to the current specification has not yet been output:

Figure 3 - Text from C.2.1 of the DIS

This means that output of picture B in Figure 3 depends on the value of no_output_of_prior_pics_flag of the RAP picture C. And since you do not know anything about picture C when picture B has just been decoded, you can not output picture B until the first slice of picture C has been received.

# Proposal

To address this problem, we propose to add a NOTE after a picture has been decoded. The text changes shown here are based on JCTVC-K0030_v3.

- C.5.3	Picture decoding, marking and storage

The following happens instantaneously when the last decoding unit of access unit n containing the current picture is removed from the CPB.

The current picture is considered as decoded after the last decoding unit of the picture is decoded. The current decoded picture is stored in an empty picture storage buffer in the DPB, and the following applies.

- –	If the current decoded picture has PicOutputFlag equal to 1, it is marked as "needed for output".

- –	Otherwise (the current decoded picture has PicOutputFlag equal to 0), it is marked as "not needed for output".

If the current decoded picture is a reference picture, it is marked as "used for reference", otherwise (the current decoded picture is a non-reference picture), it is marked as "unused for reference".

NOTE - When the number of pictures in the DPB that are marked as "needed for output" is greater than sps_max_num_reorder_pics[ HighestTid ] after the current decoded picture has been stored in the DPB, the picture in the DPB with the smallest value of PicOrderCntVal of all pictures in the DPB that are marked as "needed for output" may be cropped, output and marked as "not needed for output" by an output order conforming decoder in order to reduce output delay.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).