| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0216 |
| --- | --- |

| Title: | HEVC interlaced coding assessment and Chroma issue consideration |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jérôme Viéron, Pierre Larbier, Jean-Marc Thiesse, Antoine Depreux ATEME 26 Burospace, Route de Gisy, 91570 Bièvres, FRANCE | Tel: Email: | +33 16 935 8998 j.vieron@ateme.com, p.larbier@ateme.com, jm.thiesse@ateme.com |
| Source: | ATEME |  |  |

_____________________________

# Abstract

This contribution reports an assessment of interlaced sequences coding using HEVC vs AVC. Efficient field-based GOP structures are evaluated for both 8 and 16 fields using HEVC. Comparison to AVC with PAFF and MBAFF  are provided.

Moreover, two solutions for handling Chroma motion vector misalignment when encoding interlaced content (field only) are evaluated. This remains a major issue especially for low bitrates since avoiding Skip mode or using extra bits for correcting errors in chroma prediction result in high loss in coding efficiency and major subjective artifacts. Motion vector shifting during encoding or Chroma shifting before encoding are evaluated.

At this stage, HM efficiency for interlaced coding is reported to be acceptable when using efficient field only GOP structure coupled with one of the proposed solutions to deal with Chroma shift issue.

# Introduction

In previous Stockholm meeting, contribution JCTVC-J0466 [1] reported an assessment of HEVC interlaced coding considering field-only coding. Limited performance against AVC was pointed out by this contribution whereas more mitigated results were provided in [2].

In order to investigate the current state of HEVC interlace coding, we propose a more complete assessment considering adapted field GOP structure and the integration of a solution (2 tested) to deal with the Chroma motion vector misalignment currently present in HEVC.

# Contribution

## Efficient HEVC field coding

In order to rigorously evaluate the HEVC performance on interlaced material, an efficient GOP structure is proposed for field only encoding. Each pair of fields is encoded in a “frame-based” order such that a frame-field adaptive structure should be easily defined. Moreover, this GOP structure allows each field to point on both top and bottom fields. Such a structure is particularly useful for field encoding of interlaced sequences which are more efficiently coded with frame coding such as “Mobile and Calendar”.

Figures 1 and 2 represent respectively the HM reference and the proposed GOP structures considering 8 fields. In order to improve coding efficiency a 16 fields GOP is also proposed to be compared to frame coding with GOP of 8 pictures. Corresponding configuration files modifications are reported in Annex of this document.

In order to remain comparable to current configuration, same number of reference frames and same delta-QPs are used. One must note that coding efficiency should be even more improved by using more references.

Figure 1. HM reference GOP structure.

Figure 2. Proposed GOP structure.

## Chroma misalignment issue

As reported in [2, 3], current HEVC state leads to Chroma motion vector misalignment when encoding interlaced material.

Each Chroma block Inter predicted with a reference picture from a different kind of field (Top to Bottom or Bottom to Top) is systematically displaced of a 0.25 pixel position above or below its correct position. As a result, extra bits are spent when this is possible in order to compensate the misalignment whereas using Skip mode systematically leads to Chroma misalignment. None of these behaviours are desirable, especially at low bitrates where fewer bits are available for residual coefficients coding and above all, avoiding Skip mode is not possible for an efficient coding.

In particular, this issue is dramatic in a subjective point of view since this leads to multiple Chroma leaks around edges of objects highly visible for low bitrates encoding. This problem has necessarily to be considered in the current design in order to efficiently deal with interlaced content coding using field only structure.

Two solutions are proposed to fix this issue. Both are evaluated on results section.

### “AVC-like” solution

As proposed in [4], the smartest solution for dealing with this issue consists on a correction of motion vector vertical component performed during encoding when a motion vector from a top field points to a bottom field (“MV_Luma_y -= 0.25”) or inversely (“MV_Luma_y += 0.25”).

Since such a solution is not yet considered by the group, we proposed another solution with more limited impact on code.

### Non-intrusive solution

As described in [3], we propose to align both top and bottom fields chroma samples before encoding. The related chroma displacements have then to be signaled in the bitstream, in order, at the decoding side, to realign the chroma samples in a proper way (as in the original source content for instance).

To validate our proposal, Lanczos 3 filtering (6 taps), has been used to shift the chroma components. Hence, a phase shifting of ¼ pixels down on the top field and of ¼ pixels up on the bottom field are considered.

With such an approach, the encoding input chroma samples have the same relative phase with regard to luma samples, as for progressive material. Moreover, such phasing is fully in line with the so-called “nominal” chroma samples locations in progressive mode knowing.

Thus prior to encode, we recommend to perform a phase shift of ¼ pixels down on the top field and a phase shift of ¼ pixels up on the bottom field when encoding interlaced content with HEVC. Hence, based on the phase information signalled in the bitstream, the decoder side can properly de-shift the chroma samples in a suitable way.

On signalization

For signalization, we proposed in [2] to add three new syntax elements chroma_sample_loc_delta_flag, chroma_sample_loc_delta_x and chroma_sample_loc_delta_y related to chroma samples location in the Field indication SEI message.

However, it was invoked by the group that with the current design, chroma phase positions can be signaled using the VUI syntax element in a dedicated way when encoding interlaced data. Hence, in the current WD (as in H.264/MPEG-4 AVC) alternative chroma sample relative locations may be indicated in case of 4:2:0 content in VUI using the so-called chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field syntax elements as described in Figure 3.

Figure 3 – Location of chroma samples for top and bottom fields for 4:2:0 format as a function of chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field

Thus, when dealing with 4:2:0 interlaced content, we can signal a chroma phase position of ¼ pixels down on the top field and of ¼ pixels up on the bottom field as follows.

chroma_loc_info_present_flag 		set to 1

chroma_sample_loc_type_top_field 		set to 4

chroma_sample_loc_type_bottom_field 	set to 2

However, one must remark that using VUI we are not describing a Chroma phase shifting but only a Chroma phase position. Actually, when using VUI in such a way we lose the original Chroma phase position of the source. Hence, such an usage is not in line with the original philosophy of the VUI. However, the current VUI definition is not clear enough and we propose here to clarify its definition in order to propose an usage in line with the standard definition.

# Results

## Experimental settings

In order to make relevant experiments regarding current broadcast applications of legacy and interlaced content, both HD and SD sequences have been considered in the experiments described in the sequel. Hence, four HD (1920x1080/30i) sequences specified in ITU-R BT.1120 [5] have been used:

#13 "Church HD",

#16 "Whale show HD"

#22 "Marching in HD".

# "Walk HD ".

Moreover, two SD (720x480 and 720x576/30i) sequences extracted from classical test sequences test sets have been considered:

# "Mobile and Calendar SD" (first 300 frames).

# "EBU scene3 SD " (third scene of EBU test sequence with dancing childs in yellow costume).

In order to assess current HEVC efficiency against H.264/AVC, a benchmark has been performed on interlaced sequences using HM8.0.

HEVC results are depicted in Table 3 and 5:

- HEVC results of first and second column have been generated using HM8.0 with original configuration file for respectively 8 frames and 8 fields.

- HEVC results of third column have been generated using HM8.0 and proposed configuration file (see section 2.1 and section 7).

- HEVC results of fourth column have been generated using previous configuration and the proposed Chroma shift before encoding.

- HEVC results of fifth column have been generated using previous configuration and Chroma MV shifting during encoding.

ATEME H.264/AVC encoder has been used for this evaluation with all ATEME “encoder only” specific tools disabled and representative GOP structure conditions as defined in following Table 2 and 4. H.264/AVC interlaced tools such as MBAFF and PAFF are activated in order to challenge HEVC performance. Finally quantization parameters leading to realistic bitrates target for broadcast applications have been considered.

## GOP of 8 fields

Table 2 Coding configuration.

|  | H.264/AVC | HEVC |
| --- | --- | --- |
| FrameRate | {25, 30} | {50, 60} |
| SourceHeight | 1088 | 544 |
| FrameToBeEncoded | 450 | 900 |
| IntraPeriod | 48 | 96 |
| GOPSize | 4 (frames) | 8 (fields) |

Table 3 Results of BD-Rate comparing the current proposal to H.264/AVC with both PAFF and MBAFF.

|  | BD-rate (HM8.0 frame)* |  |  | BD-rate (HM8.0 field) |  |  | BD-rate (HM8.0 field optimized GOP) |  |  | BD-rate (HM8.0 field optimized GOP + chroma shift before coding) |  |  | BD-rate (HM8.0 field optimized GOP + chroma shift inside HM) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Video Sequences | Y | U | V | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Church_HD | -28.2 | -62.0 | -44.0 | -51.7 | -51.2 | -31.6 | -53.8 | -48.8 | -29.9 | -53.8 | -50.0 | -30.7 | -53.8 | -52.3 | -33.3 |
| Whale_Show_HD | -6.4 | -47.5 | -35.7 | -18.9 | -44.4 | -11.2 | -19.8 | -35.2 | -26.8 | -20.8 | -37.1 | -29.4 | -20.8 | -39.2 | -31.9 |
| Marching_in_HD | -21.9 | -81.5 | -78.7 | -16.8 | -57.7 | -50.8 | -13.2 | -43.0 | -32.2 | -13.3 | -45.9 | -35.9 | -13.3 | -49.5 | -40.8 |
| Walk_HD | -34.8 | -66.3 | -57.6 | -31.9 | -42.2 | -31.7 | -32.1 | -37.8 | -26.9 | -32.1 | -39.4 | -29.1 | -32.2 | -44.7 | -35.8 |
| Mobile_and_calendar_SD | -34.9 | -72.5 | -70.1 | 7.8 | -16.6 | 0.7 | 1.4 | -2.4 | 14.8 | 1.1 | -7.2 | 7.8 | 1.1 | -11.4 | 1.4 |
| EBU_scene3_SD | 40.4 | 24.2 | 30.3 | -22.8 | -33.8 | -26.8 | -24.7 | -32.0 | -24.6 | -25.1 | -33.7 | -25.6 | -25.2 | -35.2 | -27.6 |
| Overall | -14.3 | -50.7 | -42.6 | -22.4 | -41.0 | -25.2 | -23.8 | -33.2 | -20.9 | -24.0 | -35.5 | -23.8 | -24.0 | -38.7 | -28.0 |

* NOTE – Frame coding has been performed with a 8 frame GOP, it cannot rigorously be compared to 8 fields GOP. For a more correct comparison of frame coding, look for table 4.

## GOP of 16 fields

Table 4 Coding configuration.

|  | H.264/AVC | HEVC |
| --- | --- | --- |
| FrameRate | {25, 30} | {50, 60} |
| SourceHeight | 1088 | 544 |
| FrameToBeEncoded | 450 | 900 |
| IntraPeriod | 48 | 96 |
| GOPSize | 8 (frames) | 16 (fields) |

Table 5 Results of BD-Rate comparing the current proposal to H.264/AVC with both PAFF and MBAFF.

|  | BD-rate (HM8.0 frame) |  |  | BD-rate (HM8.0 field)** |  |  | BD-rate (HM8.0 field optimized GOP) |  |  | BD-rate (HM8.0 field optimized GOP + chroma shift before coding) |  |  | BD-rate (HM8.0 field optimized GOP + chroma shift inside HM) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Video Sequences | Y | U | V | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Church_HD | -23.9 | -56.2 | -37.0 | -49.2 | -44.4 | -23.7 | -56.2 | -55.3 | -39.7 |  |  |  | -56.3 | -58.4 | -43.0 |
| Whale_Show_HD | -6.0 | -45.2 | -34.0 | -22.1 | -50.6 | -47.0 | -21.6 | -33.9 | -25.4 |  |  |  | -21.6 | -36.8 | -29.2 |
| Marching_in_HD | -20.9 | -80.3 | -77.8 | -15.7 | -52.3 | -45.6 | -17.3 | -48.1 | -39.4 | -17.3 | -50.1 | -41.9 |  |  |  |
| Walk_HD | -32.3 | -63.4 | -55.1 | -29.9 | -37.6 | -27.6 | -36.0 | -43.8 | -34.3 | -36.1 | -46.4 | -37.8 | -36.1 | -50.0 | -42.5 |
| Mobile_and_calendar_SD | -32.1 | -70.2 | -67.5 | 12.5 | -6.5 | 12.5 | 0.1 | -8.2 | 6.2 | -0.2 | -14.7 | -2.1 | -0.2 | -18.3 | -8.0 |
| EBU_scene3_SD | 39.2 | 23.4 | 28.1 | -23.4 | -34.2 | -27.9 | -24.7 | -31.7 | -23.7 | -25.1 | -33.3 | -24.1 | -25.2 | -34.8 | -26.4 |
| Overall | -14.3 | -50.9 | -42.6 | -21.3 | -37.6 | -26.5 | -26.0 | -36.8 | -26.1 |  |  |  |  |  |  |

** NOTE – Field coding using reference HM with a GOP of 8 fields not optimized.

## Analysis

From results provided above, many conclusions can be drawn.

First of all, the proposed GOP structure provides a quasi-systematic performance gain for sequences that have to be encoded in field only. However, this new GOP structure leads to an increase of reference kind of fields crossing (top to bottom and bottom to top) and consequently strengthening the Chroma misalignment issue.

Moreover, as can be seen regarding both SD sequences, efficient interlaced content encoding implies that some scene have to be encoded in field only and others in progressive only. Otherwise, losses in performance can be very important. Consequently, efficient adaptive frame/field decision can help to improve performances as studied in [7].

Finally, it is demonstrated that the Chroma misalignment issue has impacts on both objective and subjective performance (chroma leaks). At least one solution has to be considered in the current design.

# Conclusion

HM efficiency for interlaced coding could be considered as acceptable when using efficient field only GOP structure coupled with one of the proposed solutions to deal with Chroma shift issue.

We recommend to use the proposed GOP structure for efficient field encoding of interlace material.

About Chroma issue, the authors recommend to adopt a Chroma shifting approach in order to effectively correct the visual default while improving objective results.

Finally, we proposed to integrate our modifications (SEI or VUI message and Interlaced input management) to the HM in order to fully support the encoding of interlaced content.

# Reference(s)

[1]	A. Luthra, and D. Baylon, “Performance of HEVC for Interlaced Video”, JCTVC-J0466, July, 2012, Stockholm, SE.

[2]	Jérôme Viéron, Jean-Marc Thiesse and Pierre Larbier, “On Chroma considerations for Interlaced material coding”, JCTVC-I0502, May, 2012, Geneva, CH.

[3]	Jumpei Koyama, Akihiro Yamori, Kimihiko Kazui, Satoshi Shimada and Akira Nakagawa, “Modification of derivation process of motion vector information for interlace format”, JCTVC-G196, November, 2011, Geneva, CH.

[4]	Jérôme Viéron, Jean-Marc Thiesse and Pierre Larbier, “Interlaced coding performance and Chroma consideration”, JCTVC- J0258, July, 2012, Stockholm, SE.

[5]	Rec. ITU-R BT.1210-3, "Test materials to be used in subjective assessment".

[6]	B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, "High efficiency video coding (HEVC) text specification draft 8", JCTVC-J1003, July 2012, Stockholm, SE.

[7]	D. Hoang, J. Xin, “Coding interlaced video using sequence-adaptive field-frame”, JCTVC-K0250, October, 2012, Shanghai, CH.

# Patent rights declaration(s)

ATEME may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable an non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex

## Proposed configuration file modification:  GOP of 8 fields

#======== Coding Structure =======

IntraPeriod                   : 96           # Period of I-Frame ( -1 = only first)

DecodingRefreshType           : 1           # Random Accesss 0:none, 1:CDR, 2:IDR

GOPSize                       : 8           # GOP Size (number of B slice = GOPSize-1)

Frame1:  B    1   1        0.442   0           3                1       3         -1 -3 -4               0

Frame2:  B    8   1        0.442   0           4                1       4         -7 -8 -10 -11          1      -7        4         1 1 1 1

Frame3:  B    4   2        0.3536  0           2                1       4         -3 -4 -6 4             1       4        5         1 1 1 0 1

Frame4:  B    5   2        0.3536  0           2                1       4         -1 -4 -5 3             1      -1        5         1 1 0 1 1

Frame5:  B    2   3        0.3536  0           2                1       5         -1 -2  2 3 6           1       3        5         1 1 1 1 1

Frame6:  B    3   4        0.68    0           2                0       5         -1 -2  1 2 5           1      -1        6         1 0 1 1 1 1

Frame7:  B    6   3        0.3536  0           2                1       4         -1 -2 -4 2             1      -3        6         1 0 1 1 1 0

Frame8:  B    7   4        0.68    0           2                0       4         -1 -2 -3 1             1      -1        5         1 1 0 1 1

ListCombination               : 1           # Use combined list for uni-prediction in B-slices

## Proposed configuration file modification:  GOP of 16 fields

#======== Coding Structure =======

IntraPeriod                   : 96          # Period of I-Frame ( -1 = only first)

DecodingRefreshType           : 1           # Random Accesss 0:none, 1:CDR, 2:IDR

GOPSize                       : 16          # GOP Size (number of B slice = GOPSize-1)

#        Type POC QPoffset QPfactor temporal_id #ref_pics_active ref_pic #ref_pics reference pictures     predict deltaRIdx-1 deltaRPS #ref_idcs reference idcs

Frame1:  B    1   1        0.442    0           3                1       3         -1 -3 -4               0

Frame2:  B    16  1        0.442    0           4                1       4         -15 -16 -18 -19        1      -15       4         1 1 1 1

Frame3:  B    8   2        0.3536   0           2                1       3         -7 -8 8                1       8        5         1 1 0 0 1

Frame4:  B    9   2        0.3536   0           2                1       4         -1 -8 -9 7             1      -1        4         1 1 1 1

Frame5:  B    4   3        0.3536   0           2                1       5         -3 -4 4 5 12           1       5        5         1 1 1 1 1

Frame6:  B    5   3        0.3536   0           2                1       5         -1 -4 3 4 11           1      -1        6         1 0 1 1 1 1

Frame7:  B    2   3        0.3536   0           2                1       6         -1 2 3 6 7 14          1       3        6         1 1 1 1 1 1

Frame8:  B    3   4        0.68     0           2                0       7         -1 -2 1 2 5 6 13       1      -1        7         1 1 1 1 1 1 1

Frame9:  B    6   3        0.3536   0           2                1       6         -1 -2 -5 2 3 10        1      -3        8         0 1 1 1 1 1 1 0

Frame10: B    7   4        0.68     0           2                0       6         -1 -2 -3 1 2 9         1      -1        7         1 1 0 1 1 1 1

Frame11: B    12  3        0.3536   0           2                1       4         -3 -4 -7 4             1      -5        7         0 1 0 1 1 1 0

Frame12: B    13  3        0.3536   0           2                1       4         -1 -4 -5 3             1      -1        5         1 1 0 1 1

Frame13: B    10  3        0.3536   0           2                1       5         -1 -2 2 3 6            1       3        5         1 1 1 1 1

Frame14: B    11  4        0.68     0           2                0       6         -1 -2 -3 1 2 5         1      -1        6         1 1 1 1 1 1

Frame15: B    14  3        0.3536   0           2                1       4         -1 -2 -5 2             1      -3        7         0 1 0 1 1 1 0

Frame16: B    15  4        0.68     0           2                0       5         -1 -2 -3 -6 1          1      -1        5         1 1 1 1 1

ListCombination               : 1           # Use combined list for uni-prediction in B-slices