| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0218 |
| --- | --- |

| Title: | Tile-based region-of-interest signalling with sub-picture SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | R. Skupin, V. George, T. Schierl Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 314 23881 robert.skupin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The partitioning of pictures into tiles in HEVC provides a new way to define regions of interest (ROIs) within coded pictures. This contribution proposes two SEI messages that allow assignment of priorities to tiles and provide the image dimensions of tiles in pixel. In combination, the two new SEI messages allow for a comprehensive ROI signalization to the system or transport layer. Application examples and benefits of the proposal are reported.

# Introduction

In SVC, region-of-interest (ROI) signalling based on macro blocks is embedded into the scalability information SEI message. ROIs are relevant in many applications, e.g. in a video conference application with multiple speakers in a single picture or for signalling of background and foreground within a coded picture. Such signalling can facilitate system and transport layers, e.g. MANEs can begin discarding non-ROI NAL units during network congestion events or video conferencing systems can zoom in on the active speaker among the people in a given picture. With the picture partitioning methods available in HEVC [1], new means to define ROIs in a given coded picture can be established, i.e. facilitate the tile setup in a coded picture and accordingly assigning priorities to tiles.

Apart from the priority of certain regions within the picture, there are use cases that require information on the image dimensions of tiles within a coded video sequence. This information is provided in the associated picture parameters sets but requires parameter set activation. Readily accessible information about tile dimensions on a higher level eases the parsing overhead in this regard for various applications, e.g. when assigning tiles to displays in a multi monitor environment.

The combination of the two signalling capabilities (tile priority and tile image dimension) allows for comprehensive ROI description within a coded picture. The current HEVC draft [1] allows Supplemental Enhancement Information NAL units that contain a sub-picture timing SEI message to be interleaved with the VCL NAL of an access unit. Whenever SEI messages apply only to certain picture partitions such as tiles sub-picture SEI messages can be advantageous. The following bullets give some benefits of sub-picture SEI messages in context of tile-based ROI signalling:

- In scenario with constrains on latency, sub-picture SEI messages allow to remove decoding units from the coded picture buffer before all decoding units have been processed. An encoder does not have to process all decoding units before it can compose the related SEI messages to place them at the beginning of an access unit. In applications such as image analysis in automotive and high speed industrial applications, ROI signalling per sub-picture SEI message allows lowering processing delays on de/encoders through early picture buffer removal of decoding units.

- With sub-picture SEI messages, the picture partition(s) to which a SEI message applies are implicitly signalled through the position of the sub-picture SEI message within the access unit. When signalling ROIs through sub-picture SEI messages, explicit signalling mechanisms to reference tiles, e.g. by identifiers and indirection, are not needed. Such mechanisms may even be impractical in scenarios with dynamic picture partitioning, i.e. change of tile dimensions and/or number of tiles during a coded video sequence.

# Proposed syntax for new SEI messages

## Sub-picture ROI info SEI message

The sub-picture ROI SEI message signals a region-of-interest through a ROI identifier and an associated priority of the tile following in decoding order. It can be located in front of any VCL NAL unit but cannot follow the last VCL NAL unit within an access unit.

| subpic_roi_info( payloadSize ) { | Descriptor |
| --- | --- |
| reserved_one_8bits | u(8) |
| roi_id | ue() |
| roi_priority | ue() |
| } |  |

roi_id indicates the region-of-interest identifier of the tile following in decoding order.

roi_priority indicates the priority of the decoding unit following in decoding that belong to the region-of-interest with ROI identifier equal to the value of roi_id. A roi_priority value of 0 indicates the lowest priority.

## Sub-picture tile dimensions info SEI message

The sub-picture tile dimensions info SEI message provides information on the image dimension of one or more tiles following the sub-picture tile dimensions info SEI message in decoding order.

| subpic_tile_dimensions_info( payloadSize ) { | Descriptor |
| --- | --- |
| reserved_one_7bits | u(7) |
| multiple_tiles_flag | u(1) |
| num_tiles_minus1 = 0 |  |
| if( multiple_tiles_flag ) |  |
| num_tiles_minus1 | ue() |
| for( i = 0; i <= num_tiles_minus1 ; i++ ) { |  |
| tile_width[ i ] | u(16) |
| tile_height[ i ] | u(16) |
| } |  |
| } |  |

multiple_tiles_flag with a value of 1 indicates that the current sub-picture tile dimensions info SEI message applies to more than one tile. multiple_tiles_flag with a value of 0 indicates that the current sub-picture tile dimensions info SEI message applies to a single tile only.

num_tiles_minus1 indicates the number of tiles following in decoding order for which image dimensions are signalled in the sub-picture tile dimensions info SEI message

tile_width[ i ] indicates the width of the i-th tile following in decoding order in pixels.

tile_height[ i ] indicates the height of the i-th tile following in decoding order in pixels.

## Changes to the existing draft syntax

SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| … |  |
| else if( payloadType = = 132 ) |  |
| subpic_timing( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| subpic_tile_dimensions_info( payloadSize ) |  |
| else if( payloadType = = 134 ) |  |
| roi_info ( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( !byte_aligned( ) ) { |  |
| bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

# References

- JCT-VC, "High-Efficiency Video Coding (HEVC) text specification draft 8", JCTVC-J1003, July 2012.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).