| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0220 |
| --- | --- |

| Title: | Use of Chroma QP offsets in Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sandeep Kanumuri Gary J. Sullivan One Microsoft Way Redmond WA 98052-6399 USA | Tel: Email: | +1-425-707-3140 skanumu@microsoft.com garysull@microsoft.com |
| Source: | Microsoft Corporation |  |  |

_____________________________

# Abstract

This contribution proposes that the effect of chroma QP offsets be taken into account when determining the parameter tC for the deblocking filtering of chroma components. It is asserted that the current derivation of this parameter in the current HEVC design does not properly reflect the actual QP used for chroma when non-zero QP offsets are used. At the preceding JCT-VC meeting of July 2012 in Stockholm, some concern was expressed about the complexity impact of using the chroma QP offsets, due to the potential need to store the chroma QP offset values on a CTU basis. This contribution asserts that this complexity impact is relatively small and thus does not necessarily justify the current design. However, this contribution also suggests that if that complexity impact is considered important, a compromise approach that accounts for picture-level chroma QP offsets without applying the slice-level QP offsets would eliminate that memory requirement and provide better performance than the current scheme of completely ignoring the chroma QP offset values in the deblocking filter process.

# Introduction

In the current HEVC design [1], the filter “strength” (tC parameter) used while deblocking a block edge of a chroma component is determined using a value QPC obtained as follows:

The variable QPC is determined as specified in Error: Reference source not found based on the index qPi derived as:

qPi = ( ( QPQ + QPP + 1 ) >> 1 )	(8-1)

Here QPQ and QPP represent the luma QP values for the blocks present on either side of the edge.

The general idea is to adjust the filter strength based on the QP values used to quantize the samples around the edge. While Equation 8-298 makes sense when the chroma QP offsets (pic_cb_qp_offset+slice_cb_qp_offset and pic_cr_qp_offset+slice_cr_qp_offset) are equal to zero, it does not make sense when they are not equal to zero. For different values of offsets, the QP used for chroma components would be different, but the filter strength would remain the same.

Thus the current deblocking filter design does not provide component-specific customization of the strength of the deblocking filter process.

We note that this issue was discussed in several previous contributions (see [2]–[4]).

# Proposed modification

We propose to modify Equation 8-298 such that the effect of chroma QP offsets is taken into account using one of the two alternative solutions described below.

## Solution A

For this solution, the effect of chroma QP offsets at both the picture level and the slice level are taken into account. The proposed modification is highlighted in green as follows:

qPI = ( (( QPQ + QPP + 1 ) >> 1) + cqp_offset )	(8-298)

Here cqp_offset represents pic_cb_qp_offset+slice_cb_qp_offset and pic_cr_qp_offset+slice_cr_qp_offset for components Cb/U and Cr/V respectively.

## Solution B

For this solution, unlike Solution A, the effect of chroma QP offsets specified only at the picture level are taken into account. The proposed modification is highlighted in green as follows:

qPI = ( (( QPQ + QPP + 1 ) >> 1) + cqp_offset )	(8-298)

Here cqp_offset represents pic_cb_qp_offset and pic_cr_qp_offset for components Cb/U and Cr/V respectively.

## Importance of adjusting for picture-level offsets

While Solution A is capable of adjusting for both picture-level and slice-level offsets, Solution B is a simplification that accounts for offsets established at the picture level but not at the slice level. With Solution B, for the operation of deblocking filter, the offsets do not need to be stored on a CTU or slice basis – rather, only the picture-level offsets (only 2 per picture) need to be stored.

The encoder can still use slice-level offsets for the purpose of rate-control, but even in such situations, picture-level offsets alone may be adequate for adjusting the deblocking strength – especially, for example, in situations where the chroma plane quality is considered at equal or greater importance to luma plane quality (where offsets need to be non-zero to match the luma QP to chroma QP). Some rate control schemes may also make use of picture-level offsets without using slice-level offsets. Hence it is important that the effect of picture-level offsets be taken into account for the purpose of deblocking – even if the slice-level offsets are not used for that purpose.

# Conclusion

We consider this issue to be a “design bug” in the current HEVC design [1], and we recommend fixing it, at the very least, for picture-level offsets.

We recommend the adoption of one of the two proposed solutions into the current HEVC design.

# References

- B. Bross et. al, “Proposed Editorial Improvements for High efficiency video coding (HEVC) Text Specification Draft 8”, JCTVC-K0030, 11th meeting, Shanghai, October 2012.

- G. V. Auwera and M. Karczewicz, “AHG6: Chroma QP Offsets and Chroma Deblocking Filter”, JCTVC-J0091, 10th Meeting, Stockholm, July 2012.

- G. V. Auwera, X. Wang, M. Karczewicz, “AHG6: Chroma QP Offsets and Chroma Deblocking Filtering”, JCTVC-I0283, 9th Meeting, Geneva, April 2012.

- S. Kanumuri and G. J. Sullivan, “Use of Chroma QP offsets in Deblocking”, JCTVC-J0343, 10th Meeting, Stockholm, July 2012.

# Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).