| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10 –19 October, 2012 | Document: JCTVC-K0222 WG11 Number: m26553 |
| --- | --- |

| Title: | On Reference Picture Set Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes an extension field in the short-term reference picture set. This field enables extension of the short-term reference picture sets in future extensions of HEVC.

Revision of this document adds a modified syntax for extension signaling.

# Introduction

HEVC Draft 8 [1] includes the syntax and semantics for signalling short-term reference picture set (RPS).

It is anticipated that the future extensions of HEVC such as the scalable extension of HEVC [2] will need to signal additional syntax elements related to the short-term reference picture set. As an example, information regarding layer_id may need to be signaled in short-term RPS.

This document proposes an extension field in the short-term reference picture set. This field enables extension of the short-term reference picture sets in future extensions of HEVC.

It is asserted that the proposed extension mechanism is simpler and cleaner approach when signaling additional syntax elements related to short-term reference picture sets in future extensions of HEVC. It has following advantages:

- Currently short-term reference picture sets can be signaled in sequence parameter set (SPS) and/or in slice header. If the additional syntax elements need to be signaled for short-term RPS in future extension of HEVC then using the currently available mechanism in HEVC draft 8, additional syntax needs to be defined in two places: one in sps_extension and another in slice header extension. These two extensions syntax will be different because the slice header extension syntax is defined differently than SPS extension syntax.

Instead of this the technique in this document proposes to define extension field directly inside the short-term reference picture set to provide the future extensibility. It is asserted that this is cleaner and simpler approach compared to defining such an extension inside SPS and Slice header.

- The proposed short-term RPS extension mechanism can convey all the syntax elements and extensions related to short-term RPS including (delta_poc elements, used_by_curr_pic elements and as an example layer_id elements) together, which is advantageous. Instead if SPS extension mechanism is used to signal the new short-term RPS extension related syntax elements then a decoder needs to parse many additional syntax elements after parsing short_term_ref_pic_set which includes the short-term RPS related syntax elements defined currently (delta_poc elements, used_by_curr_pic elements) to reach and obtain other necessary short-term RPS syntax elements related to extension (e.g. layer_id elements). This would require the decoder to keep track of num_negative_pics and num_positive_pics for each of the short-term RPS for a total of num_short_term_ref_pic_sets short-term reference picture sets.

# Proposed Change

We propose following syntax and semantics. The proposed changes are highlighted below compared to HEVC Draft 8 [1].

We provide syntax for the HEVC Base specification, which includes the proposed short-term RPS extension.

We also provide and propose example syntax for HEVC extension, which utilizes the proposed short-term RPS extension mechanism.

## Syntax and Semantics:

#### Short-term reference picture set syntax – HEVC Base Specification

This section describes the syntax and semantics for short-term RPS extension for HEVC base specification [1].

| short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( inter_ref_pic_set_prediction_flag ) { |  |
| if( idx = = num_short_term_ref_pic_sets ) |  |
| delta_idx_minus1 | ue(v) |
| delta_rps_sign | u(1) |
| abs_delta_rps_minus1 | ue(v) |
| for( j = 0; j <= NumDeltaPocs[ RIdx ]; j++ ) { |  |
| used_by_curr_pic_flag[ j ] | u(1) |
| if( !used_by_curr_pic_flag[ j ] ) |  |
| use_delta_flag[ j ] | u(1) |
| } |  |
| } |  |
| else { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| rps_extension_flag | u(1) |
| if( rps_extension_flag ) { |  |
| rps_extension_length | ue(v) |
| for(i=0; i<rps_extension_length; i++) |  |
| rps_extension_data_byte[i] | u(8) |
| } |  |
| } |  |

rps_extension_flag equal to 0 specifies that no rps_extension_data_flag syntax elements are present in the short-term reference picture set RBSP syntax structure. rps_extension_flag shall be equal to 0 in bitstreams conforming to this Specification. The value of 1 for rps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the extension data that follow the value 1 for rps_extension_flag.

rps_extension_length specifies the length of RPS extension data in bytes not including the bits used for signaling rps_extension_length element. The value of rps_extension_length shanll be in the range of 0 to 256 inclusive.

rps_extension_data_byte may have any value. Its value does not affect decoder conformance to profiles specified in this Specification.

In a variant approach the syntax element rps_extension_flag could be omitted and the value of rps_extension_length syntax element could be checked if it is greater than 0 to indicate signaling of RPS extension data.

#### Short-term reference picture set syntax ( Example and Proposal for HEVC extension)

This section describes an example syntax and semantics for short-term RPS extension for use for a scalable extension of HEVC [2].

| short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( inter_ref_pic_set_prediction_flag ) { |  |
| if( idx = = num_short_term_ref_pic_sets ) |  |
| delta_idx_minus1 | ue(v) |
| delta_rps_sign | u(1) |
| abs_delta_rps_minus1 | ue(v) |
| for( j = 0; j <= NumDeltaPocs[ RIdx ]; j++ ) { |  |
| used_by_curr_pic_flag[ j ] | u(1) |
| if( !used_by_curr_pic_flag[ j ] ) |  |
| use_delta_flag[ j ] | u(1) |
| } |  |
| } |  |
| else { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_equal_to_one | u(1) |
| rps_extension_length | ue(v) |
| rps_extension( ) |  |
| } |  |

| rps_extension() { |  |
| --- | --- |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| layer_id_s0[i] | u(6) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| layer_id_s1[i] | u(6) |
| } |  |
| } |  |

layer_id_s0[ i ] specifies the value of layer_id for the i-th reference picture. This value together with the value of delta_poc_s0_minus1[ i ] is used to identify the reference picture.

layer_id_s1[ i ] specifies the value of layer_id for the i-th reference picture. This value together with the value of delta_poc_s1_minus1[ i ] is used to identify the reference picture.

Additional Variant Signaling:

Additionally for bit savings for HEVC base specification the presence of the proposed extension field in the short-term reference picture set can be conditioned on a flag in the SPS. Syntax and semantics for this are shown below.

#### SPS Syntax and semantics:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_bit | u(1) |
| … |  |
| strps_extension_present_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |

strps_extension_present_flag equal to 0 specifies that no short-term reference picture set extension syntax elements are present in the short-term reference picture set syntax. strps_extension_present_flag  shall be equal to 0 in bitstreams conforming to this Specification. The value of 1 for strps_extension_present_flag is reserved for future use by ITU-T | ISO/IEC.

In this case the proposed short-term RPS extension syntax elements are signalled only when strps_extension_present_flag has a value of 1 for future extension of HEVC.

# Conclusion

The document proposed an extension field in the short-term reference picture set. This field enables extension of the short-term reference picture set in future extensions of HEVC. We propose to adopt the proposed approach in HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Bros, W-J. Han, J-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 8,” JCTVC-J1003_d7, Stockholm, July 2012.

[2] A. Luthra, “Joint Preliminary Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC),” ISO/IEC JTC1/SC29/WG11 M25345, Stockholm, July, 2012