| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0224 |
| --- | --- |

| Title: | Syntax cleanup for reference picture list modification signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Sandeep Kanumuri One Microsoft Way Redmond WA 98052-6399 USA | Tel: Email: | +1-425-703-5308 garysull@microsoft.com skanumu@microsoft.com |
| Source: | Microsoft Corporation |  |  |

_____________________________

# Abstract

The syntax for reference picture list modification, as found in the ref_pic_list_modification() syntax structure, is asserted to have unnecessary redundancies for the signalling of some of its syntax elements. This contribution proposes modifications that are asserted to remove the redundancies.

# Introduction

This contribution proposes a set of signalling enhancements related to reference picture list modification. In particular, a few redundancies are identified in the signalling of ref_pic_list_modification_flag_lX and list_entry_lX. These redundancies are avoided by the proposed modifications.

# Proposed enhancements

## Signalling of ref_pic_list_modification_flag_lX (X = 0 or 1)

In the syntax for ref_pic_list_modification(), it is proposed that ref_pic_list_modification_flag_l0 and ref_pic_list_modification_flag_l1 only be sent when NumPocTotalCurr is greater than 1. When NumPocTotalCurr is less than or equal to 1, there is no possibility for modification and hence no need to send these flags. This can save one or two flags for every slice when the condition is fulfilled. The proposed syntax modification is indicated in green below:

| ref_pic_list_modification( ) { | Descriptor |
| --- | --- |
| if( NumPocTotalCurr > 1 ) |  |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| list_entry_l0[ i ] | u(v) |
| if( slice_type = = B ) { |  |
| if( NumPocTotalCurr > 1 ) |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| list_entry_l1[ i ] | u(v) |
| } |  |
| } |  |

## Signalling of list_entry_lX (X = 0 or 1)

### Option A

In the syntax for ref_pic_list_modification(), it is proposed that list_entry_lX[0] not be sent when NumPocTotalCurr is equal to 2 and num_ref_idx_lX_active_minus1 is equal to 0. In such a case, list_entry_lX[0] can be inferred based on ref_pic_list_modification_flag_lX since there are only two choices possible (default value of 0 or the non-default value of 1). The proposed syntax and semantics modifications are indicated in green below:

| ref_pic_list_modification( ) { | Descriptor |
| --- | --- |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 && !( NumPocTotalCurr = = 2 && num_ref_idx_l0_active_minus1 = = 0 ) ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| list_entry_l0[ i ] | u(v) |
| if( slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 && !( NumPocTotalCurr = = 2 && num_ref_idx_l1_active_minus1 = = 0 ) ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| list_entry_l1[ i ] | u(v) |
| } |  |
| } |  |

list_entry_l0[ i ] specifies the index of the reference picture in RefPicListTemp0 to be placed at the current position of reference picture list 0. The length of the list_entry_l0[ i ] syntax element is Ceil( Log2( NumPocTotalCurr) ) bits. The value of list_entry_l0[ i ] shall be in the range of 0 to NumPocTotalCurr − 1, inclusive. If NumPocTotalCurr is equal to 2 and num_ref_idx_l0_active_minus1 is equal to 0, the syntax element list_entry_l0[0] is inferred to be equal to ref_pic_list_modification_flag_l0. Otherwise, when the syntax element list_entry_l0[ i ] is not present, it is inferred to be equal to 0.

list_entry_l1[ i ] specifies the index of the reference picture in RefPicListTemp1 to be placed at the current position of reference picture list 1. The length of the list_entry_l1[ i ] syntax element is Ceil( Log2( NumPocTotalCurr ) ) bits. The value of list_entry_l1[ i ] shall be in the range of 0 to NumPocTotalCurr − 1, inclusive. If NumPocTotalCurr is equal to 2 and num_ref_idx_l1_active_minus1 is equal to 0, the syntax element list_entry_l1[0] is inferred to be equal to ref_pic_list_modification_flag_l1. Otherwise, when the syntax element list_entry_l1[ i ] is not present, it is inferred to be equal to 0.

### Option B

In addition to Option A, for P slices with weighted_pred_flag equal to 0 or for B slices with weighted_bipred_flag equal to 0, it is proposed that list_entry_lX[ 0 ] and list_entry_lX[ 1 ] not be sent when NumPocTotalCurr is equal to 2 and num_ref_idx_lX_active_minus1 is equal to 1. In such a case, list_entry_lX[ 0 ] and list_entry_lX[ 1 ] are inferred to be 1 and 0 respectively since reference picture list modification would not have been needed for the only other possibility (list_entry_lX[0] and list_entry_lX[ 1 ] being equal to 0 and 1 respectively). Furthermore, for P slices with weighted_pred_flag equal to 0 or for B slices with weighted_bipred_flag equal to 0, it is proposed that the length of list_entry_lX[ i ] syntax element be limited to Ceil( Log2( NumPocTotalCurr − i ) ) bits, since in this case, it is only useful to place each reference picture once in the list and thus the number of useful possibilities decreases as the index ‘i’ increases. The proposed syntax and semantics modifications are indicated in green below:

| ref_pic_list_modification( ) { | Descriptor |
| --- | --- |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 && !( NumPocTotalCurr = = 2 && num_ref_idx_l0_active_minus1 = = 0 ) && !( NumPocTotalCurr = = 2 && num_ref_idx_l0_active_minus1 = = 1 && ( ( weighted_pred_flag != 1 && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag != 1 && slice_type = = B ) ) ) ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| list_entry_l0[ i ] | u(v) |
| if( slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 && !( NumPocTotalCurr = = 2 && num_ref_idx_l1_active_minus1 = = 0) && !( NumPocTotalCurr = = 2 && num_ref_idx_l1_active_minus1 = = 1 && weighted_bipred_flag != 1 ) ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| list_entry_l1[ i ] | u(v) |
| } |  |
| } |  |

list_entry_l0[ i ] specifies the index of the reference picture in RefPicListTemp0 to be placed at the current position of reference picture list 0. For P slices with weighted_pred_flag equal to 0 and for B slices with weighted_bipred_flag equal to 0, the length of list_entry_l0[ i ] syntax element is Ceil( Log2( NumPocTotalCurr – i ) ) bits. Otherwise, the length of the list_entry_l0[ i ] syntax element is Ceil( Log2( NumPocTotalCurr) ) bits. If NumPocTotalCurr is equal to 2 and num_ref_idx_l0_active_minus1 is equal to 0, the syntax element list_entry_l0[ 0 ] is inferred to be equal to ref_pic_list_modification_flag_l0. Otherwise, if NumPocTotalCurr is equal to 2, num_ref_idx_l0_active_minus1 is equal to 1 and weighted prediction is disabled (when weighted_pred_flag is equal to 0 and the current slice is a P slice or weighted_bipred_flag is equal to 0 and the current slice is a B slice), the syntax elements list_entry_l0[ 0 ] and list_entry_l0[ 1 ] are inferred to be equal to 1 and 0 respectively. Otherwise, when the syntax element list_entry_l0[ i ] is not present, it is inferred to be equal to 0.

If weighted prediction is disabled (when weighted_pred_flag is equal to 0 and the current slice is a P slice or weighted_bipred_flag is equal to 0 and the current slice is a B slice) the value of list_entry_l0[ i ] shall be in the range of 0 to NumPocTotalCurr − (i + 1), inclusive, and the list RefPicListTemp0 is shortened by removal of each entry list_entry_l0[ i ] from the list RefPicListTemp0 after the entry value is parsed. Otherwise, the value of list_entry_l0[ i ] shall be in the range of 0 to NumPocTotalCurr − 1, inclusive.

list_entry_l1[ i ] specifies the index of the reference picture in RefPicListTemp1 to be placed at the current position of reference picture list 1. If weighted_bipred_flag is equal to 0, the length of list_entry_l1[ i ] syntax element is Ceil( Log2( NumPocTotalCurr − i ) ) bits. Otherwise, the length of the list_entry_l1[ i ] syntax element is Ceil( Log2( NumPocTotalCurr ) ) bits. If NumPocTotalCurr is equal to 2 and num_ref_idx_l1_active_minus1 is equal to 0, the syntax element list_entry_l1[ 0 ] is inferred to be equal to ref_pic_list_modification_flag_l1. Otherwise, if NumPocTotalCurr is equal to 2, num_ref_idx_l1_active_minus1 is equal to 1 and weighted prediction is disabled (weighted_bipred_flag is equal to 0 and the current slice is a B slice), the syntax elements list_entry_l1[ 0 ] and list_entry_l1[ 1 ] are inferred to be equal to 1 and 0 respectively. Otherwise, when the syntax element list_entry_l1[ i ] is not present, it is inferred to be equal to 0.

If weighted prediction is disabled (when weighted_pred_flag is equal to 0 and the current slice is a P slice or weighted_bipred_flag is equal to 0 and the current slice is a B slice) the value of list_entry_l1[ i ] shall be in the range of 0 to NumPocTotalCurr − (i + 1), inclusive, and the list RefPicListTemp1 is shortened by removal of each entry list_entry_l1[ i ] from the list RefPicListTemp1 after the entry value is parsed. Otherwise, the value of list_entry_l0[ i ] shall be in the range of 0 to NumPocTotalCurr − 1, inclusive.

# Conclusion

We recommend the proposed improvements to the reference picture list modification syntax and its semantics to eliminate redundancies in the syntax signalling.

# References

- B. Bross et. al, “Proposed Editorial Improvements for High efficiency video coding (HEVC) Text Specification Draft 8”, JCTVC-K0030, 11th meeting, Shanghai, October 2012.

# Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).