- Proposed Changed Text for Annex A.

  - Tiers and levels

    - General level specifications

For purposes of comparison of tier capabilities, the tier with general_tier_flag equal to 0 shall be considered to be a lower tier than the tier with general_tier_flag equal to 1.

For purposes of comparison of level capabilities, a particular level shall be considered to be a lower level than some other level if the level is listed a higher row of Error: Reference source not found than the other level. [Ed. (GJS): If we follow the guidance in the note above, comparison of level values will not necessarily work properly when a reserved level value is used in the future.]

The following is specified for expressing the constraints in this annex.

- Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0.

- Let picture n be the coded picture or the corresponding decoded picture of access unit n.

- Let the variable fR be set to 1  300.

Bitstreams conforming to a profile at a specified level shall obey the following constraints:

- The nominal removal time of access unit n (with n > 0) from the CPB as specified in subclause Error: Reference source not found, satisfies the constraint that tr,n( n ) − tr( n − 1 ) is greater than or equal to Max( PicSizeInSamplesY  MaxLumaSR, fR ) for the value of PicSizeInSamplesY of picture n − 1, where MaxLumaSR is the value specified in Error: Reference source not found that applies to picture n − 1.

- The difference between consecutive output times of pictures from the DPB as specified in subclause Error: Reference source not found, satisfies the constraint that to,dpb( n ) >= Max( PicSizeInSamplesY  MaxLumaSR, fR ) for the value of PicSizeInSamplesY of picture n, where MaxLumaSR is the value specified in Error: Reference source not found for picture n, provided that picture n is a picture that is output and is not the last picture of the bitstream that is output.

- PicSizeInSamplesY <= MaxLumaPS, where MaxLumaPS is specified in Error: Reference source not found.

- pic_width_in_luma_samples <= Sqrt( MaxLumaPS * 8 )

- pic_height_in_luma_samples <= Sqrt( MaxLumaPS * 8 )

- sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] <= MaxDpbSize, where MaxDpbSize is derived as specified by the following:

For the byte aligned case

bdY = 8*ceil(BitDepthY 8), bdCb = 8*ceil(BitDepthC bdCr = 8*ceil(BitDepthC 8).

For the packed case

bdY= BitDepthY, bdCb= BitDepthC, bdCr= BitDepthC.

The scale factors for Y, Cb, Cr components factor with respect to 4:2:0 subsampling are sY, sCB, sCR.

|  | 4:2:0 | 4:2:2 | 4:4:4 |
| --- | --- | --- | --- |
| sY | 1 | 1 | 1 |
| sCb | 1 | 2 | 4 |
| sCr | 1 | 2 | 4 |

An overall scale factor is computed as

sF=((4*sY*bdY+sCb*bdCb+sCr*bdCy) (6*8)

if ( PicSizeInSamplesY <= ( MaxLumaPS >> 2 ) )
			MaxDpbSize = Min( 4 * MaxDpbPicBuf/sF, 16 )
		else if ( PicSizeInSamplesY <= ( MaxLumaPS >> 1 ) )
			MaxDpbSize = Min( 2 * MaxDpbPicBuf/sF, 16 )
		else if ( PicSizeInSamplesY <= ( MaxLumaPS << 1) / 3 )
			MaxDpbSize = Min( (3 * MaxDpbPicBuf/sF) >> 1, 16 )
		else if ( PicSizeInSamplesY <= ( ( 3 * MaxLumaPS ) >> 2 ) )
			MaxDpbSize = Min( (4 * MaxDpbPicBuf/sF) / 3,  16 )
		else
			MaxDpbSize = MaxDpbPicBuf/sF
where MaxLumaPS is specified in Error: Reference source not found and MaxDpbPicBuf is equal to 6.

Error: Reference source not found specifies the limits for each level. The use of the MinCR parameter column of Error: Reference source not found is specified in subclause Error: Reference source not found.

A tier and level to which the bitstream conforms shall be indicated by the syntax element general_level_idc as follows.

–	general_tier_flag equal to 0 indicates conformance to the Main tier, and general_tier_flag equal to 1 indicates conformance to the High tier, according to the tier constraint specifications in Error: Reference source not found. general_tier_flag shall be equal to 0 for levels below level 4 (corresponding to the entries in Error: Reference source not found marked with "-").

–	level_idc shall be set equal to a value of 30 times the level number specified in Error: Reference source not found.