| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10–19 October 2012 | Document: JCTVC-K0225 WG11 Number: m26556 |
| --- | --- |

| Title: | On DPB Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Louis Kerofsky, Sachin Deshpande 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-360-817-87644 lkerofsky@sharplabs.com sdeshpande@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

The HEVC design includes a decoded picture buffer (DPB) for holding pictures for reference and reordering. The value of MaxDpbSize influences memory needs of a decoder and is defined through level parameters. Given a maximum DPB size parameter (MaxDpbPicBuf), maximum luma picture size (MaxLumaPS), and size of a picture in the current sequence (PicSizeInSamplesY), the maximum number of reference pictures supported (MaxDpbSize) is determined. Current calculation in HEVC Draft 8 for MaxDpbSize does not account for different bit-depth or chroma subsampling other than 8-bit 4:2:0. This proposal describes extending the current design to support higher bit-depths, alternate chroma subsampling formats. The proposed MaxDpbSize calculation accounts for memory use due to bit-depth and/or chroma components.

# Current MaxDpbSize calculation

The current HEVC draft 8 [1] defines the calculation of MaxDpbSize in Annex A as follows.

Bitstreams conforming to a profile at a specified level shall obey the following constraints:

a)	The nominal removal time of access unit n (with n > 0) from the CPB as specified in subclause C.2.2 satisfies the constraint that tr,n( n ) − tr( n − 1 ) is equal to or greater than Max( PicSizeInSamplesY  MaxLumaSR, fR ) for the value of PicSizeInSamplesY of picture n − 1, where MaxLumaSR is the value specified in Table A 1 that applies to picture n − 1.

b)	The difference between consecutive output times of pictures from the DPB as specified in subclause C.3.2 satisfies the constraint that to,dpb( n ) >= Max( PicSizeInSamplesY  MaxLumaSR, fR ) for the value of PicSizeInSamplesY of picture n, where MaxLumaSR is the value specified in Table A 1 for picture n, provided that picture n is a picture that is output and is not the last picture of the bitstream that is output.

c)	PicSizeInSamplesY <= MaxLumaPS, where MaxLumaPS is specified in Table A 1.

d)	pic_width_in_luma_samples <= Sqrt( MaxLumaPS * 8 )

e)	pic_height_in_luma_samples <= Sqrt( MaxLumaPS * 8 )

f)	sps_max_dec_pic_buffering[ HighestTid ] <= MaxDpbSize, where MaxDpbSize is derived as specified by the following:

if ( PicSizeInSamplesY <= ( MaxLumaPS >> 2 ) )

MaxDpbSize = Min( 4 * MaxDpbPicBuf, 16 )

else if ( PicSizeInSamplesY <= ( MaxLumaPS >> 1 ) )

MaxDpbSize = Min( 2 * MaxDpbPicBuf, 16 )

else if ( PicSizeInSamplesY <= ( MaxLumaPS << 1) / 3 )

MaxDpbSize = Min( (3 * MaxDpbPicBuf) >> 1, 16 )

else if ( PicSizeInSamplesY <= ( ( 3 * MaxLumaPS ) >> 2 ) )

MaxDpbSize = Min( (4 * MaxDpbPicBuf) / 3,  16 )

else

MaxDpbSize = MaxDpbPicBuf

where MaxLumaPS is specified in Table A 1 and MaxDpbPicBuf is equal to 6.

This calculation can be directly applied for high bit-depth or alternate chroma formats however since the results are based only on the luma picture sizes the necessary memory for an implementation must assume the pictures consume the most memory allowed consistent with the level i.e. highest bit-depth and most chroma samples per luma sample.  In the case where the current sequence does not use these most memory consuming frames, the MaxDpbSize is lower than could be supported by the available memory.

# Proposed extensions to MaxDpbSize calculation

The existing process for computing MaxDpbSize is extended by introducing scaling factors which account for bit-depth and chroma subsampling. These factors are designed to equal 1 for 8-bit samples and 4:2:0 chroma subsampling and hence agree with the existing calculation of MaxDpbSize for 4:2:0, 8 bits per channel.

If frame packing is used, the actual bit-depth values are used to compute the overall scale factor directly. If data is stored byte aligned, the bit-depth values are rounded up to the nearest multiple of 8.

For the byte aligned case

bdY = 8*ceil(BitDepthY 8), bdCb = 8*ceil(BitDepthC bdCr = 8*ceil(BitDepthC 8).

For the packed case

bdY= BitDepthY, bdCb= BitDepthC, bdCr= BitDepthC.

Note for one channel images bdCb=bdCr=0 in this calculation

We define the scale factors for Y, Cb, Cr components factor with respect to 4:2:0 subsampling as sY, sCB, sCR.

|  | 4:2:0 | 4:2:2 | 4:4:4 |
| --- | --- | --- | --- |
| sY | 1 | 1 | 1 |
| sCb | 1 | 2 | 4 |
| sCr | 1 | 2 | 4 |

Then an overall scale factor is computed as follows:

sF=((4*sY*bdY+sCb*bdCb+sCr*bdCy) (6*8)

It can be noted that sF will be 1 for 8-bit per channel, 4:2:0

If an alpha channel is included the scale factor has an additional term based on scale factor of alpha channel (sA) and its bit depth (bdA). Where value sA is relative to the number of luma samples i.e. sA=4 if there is an alpha sample for each luma sample and bdA is the bit-depth of the alpha channel. In this case the sale factor is computed as follows:

sF=((4*sY*bdY+sCb*bdCb+sCr*bdCy+sA*bdA) (6*8)

Then the MaxDpbSize is calculated as:

sps_max_dec_pic_buffering[ HighestTid ] <= MaxDpbSize, where MaxDpbSize is derived as specified by the following 
		if ( PicSizeInSamplesY <= ( MaxLumaPS >> 2 ) )
			MaxDpbSize = Min( 4 * MaxDpbPicBuf/sF, 16 )
		else if ( PicSizeInSamplesY <= ( MaxLumaPS >> 1 ) )
			MaxDpbSize = Min( 2 * MaxDpbPicBuf/sF, 16 )
		else if ( PicSizeInSamplesY <= ( MaxLumaPS << 1) / 3 )
			MaxDpbSize = Min( (3 * MaxDpbPicBuf/sF) >> 1, 16 )
		else if ( PicSizeInSamplesY <= ( ( 3 * MaxLumaPS ) >> 2 ) )
			MaxDpbSize = Min( (4 * MaxDpbPicBuf/sF) / 3,  16 )
		else
			MaxDpbSize = MaxDpbPicBuf/sF
where MaxLumaPS is specified in Error: Reference source not found and MaxDpbPicBuf is equal to 6.

# Results of proposed extension of MaxDpbSize

Results for MaxDpbSize from our proposed formula are shown below for some examples values of chroma format and bitdepth.

Table 1 - Example Scale Factors for various bit depth and chroma formats

| Chroma | sY | sCb | sCr | bY | bCb | bCr | sF (byte-aligned) | sF (packed) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4:2:0 | 1 | 1 | 1 | 8 | 8 | 8 | 1 | 1 |
| 4:2:0 | 1 | 1 | 1 | 10 | 10 | 10 | 2 | 5 |
| 4:2:0 | 1 | 1 | 1 | 12 | 12 | 12 | 2 | 3 |
| 4:2:0 | 1 | 1 | 1 | 8 | 10 | 10 | 3 | 13 |
| 4:2:2 | 1 | 2 | 2 | 8 | 8 | 8 | 3 | 3 |
| 4:2:2 | 1 | 2 | 2 | 10 | 10 | 10 | 3 | 5 |
| 4:4:4 | 1 | 4 | 4 | 8 | 8 | 8 | 2 | 2 |
| 4:4:4 | 1 | 4 | 4 | 10 | 10 | 10 | 2 | 5 |

Table 2- MaxDpbSize for various scale factors

| PicSizeInSamplesY | sF=1 (HEVC Draft 8) | sF=13 | sF=5 | sF=3 | sF=2 |
| --- | --- | --- | --- | --- | --- |
| PicSizeInSamplesY ≤ 0.25 * MaxLumaFS | 16 | 16 | 16 | 16 | 12 |
| 0.25 * MaxLumaFS < PicSizeInSamplesY ≤ 0.5 * MaxLumaFS | 12 | 11 | 9 | 8 | 6 |
| 0.5 * MaxLumaFS < PicSizeInSamplesY ≤ 0.667 * MaxLumaFS | 9 | 8 | 7 | 6 | 4 |
| 0.667 * MaxLumaFS < PicSizeInSamplesY ≤ 0.75 * MaxLumaFS | 8 | 7 | 6 | 5 | 4 |
| 0.75 * MaxLumaFS < PicSizeInSamplesY ≤ 1.0 * MaxLumaFS | 6 | 5 | 4 | 4 | 3 |

# Conclusions

Current calculation in HEVC Draft 8 for MaxDpbSize assumes 8-bit 4:2:0 chroma subsampling. This proposal describes extending the current design to support higher bit-depths, alternate chroma subsampling formats. We propose to adopt the proposed approach in HEVC.

# References

[1] B. Bros, W-J. Han, J-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 8,” JCTVC-J1003_d7, Stockholm, July 2012.

# Patent rights declaration

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).