| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 October 2012 | Document: JCTVC-K0226 |
| --- | --- |

| Title: | Confidence level and mixed-content indicators for field indication SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Yongjun Wu One Microsoft Way Redmond WA 98052-6399 USA | Tel: Email: | +1-425-703-5308 garysull@microsoft.com yongjunw@microsoft.com. |
| Source: | Microsoft Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to add an indication of a degree of confidence to the field indication SEI message. It is asserted that some encoders may not be able to determine with certainty an accurate value for the progressive_source_flag and duplicate_flag, and may also encounter video content that has mixed  progressive/interlace characteristics. Adding an indicator to express a degree of confidence in the indicated values will assist decoders in determining how best to use the received video. Additionally, we propose to add the ability to indicate that the content has mixed characteristics rather than exhibiting purely-interlaced or purely-progressive source characteristics.

# Introduction

In the current HEVC draft standard [1], there are two syntax elements in the field indication SEI message that can be used to describe the properties of the source video content.

"progressive_source_flag equal to 1 indicates that the scan type of the associated picture should be interpreted as progressive. progressive_source_flag equal to 0 indicates that the scan type of the associated picture should be interpreted as interlaced. When the field indication SEI message is not present, the value of progressive_source_flag should be inferred to be equal to 1."

…

"duplicate_flag equal to 1 indicates that the current picture is indicated to be a duplicate of a previous picture in output order. duplicate_flag equal to 0 indicates that the current picture is not indicated to be a duplicate picture."

However, in some application scenarios, an HEVC encoding system might not have enough information to determine a correct value for these syntax elements, for various reasons. For example, the system might simply receive fields or frames as input video data and may have limitations of its computation power, memory capacity, or delay characteristics that do not enable it to perform a deep analysis of the source content characteristics. The system generating the SEI message might also only have limited access to the information of the uncompressed pictures. In general, it can be difficult to determine the true source characteristics.

Moreover, it is also possible that the source content can exhibit mixed characteristics. A field-based text or graphics overlay applied to progressive-scan video is one way this can occur.

In such situations, it would be helpful for the encoder to be able to indicate the degree of confidence that it has in the values of these indicated syntax elements. A decoder or display subsystem could use the indicated degree of confidence to control subsequent processes such as controlling de-interlace processing or controlling whether or not it should attempt to detect the source video properties for itself rather than relying on those indicated by the SEI message.

It may additionally be helpful to enable an explicit indication of mixed progressive-interlaced content characteristics.

# Proposed modification

We propose a small modification of the existing field indication SEI message to indicate a confidence level for the syntax elements of the field indication information that indicate source video properties – specifically, the progressive_source_flag and the duplicate_flag. The modification is highlighted in green below.

Field indication SEI message syntax

| field_indication( payloadSize ) { | Descriptor |
| --- | --- |
| field_pic_flag | u(1) |
| progressive_source_flag | u(1) |
| mixed_characteristics_flag | u(1) |
| duplicate_flag | u(1) |
| if( field_pic_flag ) |  |
| bottom_field_flag | u(1) |
| else if( !progressive_source_flag ) |  |
| top_field_first_flag | u(1) |
| else |  |
| reserved_zero_1bit /* equal to 0 */ | u(1) |
| confidence_level | u(2) |
| reserved_zero_bit /* equal to 0 */ | u(1) |
| } |  |

mixed_characteristics_flag equal to 1 indicates that the video content has somewhat mixed progressive and interlaced scan characteristics – for example, as may be generated by the use of field-based graphics overlays wither otherwise-progressive-scan video content. mixed_characteristics_flag equal to 0 indicates that the video content is not indicated to have mixed characteristics.

…

confidence_level equal to 3 indicates a high degree of assurance that the progressive_source_flag, mixed_characteristics_flag, and duplicate_flag are correct, and the decoder may confidently rely on this information; equal to 2 indicates a reasonable degree of confidence that these syntax elements are correct and that it is recommended for subsequent processes to honor the information unless substantial capabilities are available to conduct further analysis of the content characteristics; equal to 1 indicates that further analysis of the content characteristics should be conducted if feasible; and equal to 0 indicates that subsequent processes should not rely on the correctness of these syntax elements.

# Conclusion

We recommend extending the field indication SEI message in the HEVC draft design [1] as proposed. The extension can indicate the level of confidence in the syntax elements of the field indication SEI message that describe the source characteristics of the video content, which can be useful in some application scenarios in which the encoder has limited information about the origin of the video content, limited analysis resources, and limited access to uncompressed pictures.

Note: The proposals to add the mixed_characteristics_flag and to add the confidence_level indication, although related to each other, can also be considered separately. If the confidence_level indication is used without adding the mixed_characteristics_flag, the semantics of the confidence_level indication should be modified by removing the mentions of the mixed_characteristics_flag from its semantics.

# References

- B. Bross et. al, “High efficiency video coding (HEVC) text specification draft 8”, JCTVC-J1003_d7, 10th meeting, Stockholm, July 2012.

# Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).