| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10–19 October, 2012 | Document: JCTVC-K0229 |
| --- | --- |

| Title: | Improvement of the rate control for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonghun Yoo Junghak Nam Hyomin Choi Donggyu Sim | Email: | mmyjh@kw.ac.kr qejixfyza@kw.ac.kr hyomin06@kw.ac.kr dgsim@kw.ac.kr |
| Source: | Kwangwoon University (KWU) |  |  |

_____________________________

# Abstract

This contribution proposes a way to improve compression performance of the HM8.0 rate control model for the random access (RA) and to minimize fluctuation for low delay (LD). For the random access case,  weighting factors to assess frame complexity are determined with distortion and bitrates for accurate target-bit computation. In addition, buffer control is performed for all the random access points in order to improve RD performance. For the low delay case, the rate control is performed for even the first coded frame in a GOP and a single lambda value for the rate distortion optimization (RDO) is employed regardless of the temporal layer ID (TID) in order to reduce bitrate fluctuation. With the proposed modification of the rate control, average BD-PSNR increases by 0.74 dB for the RA case. For LD case, the variation of bits fluctuation is reduced by 50.88%.

# Introduction

The rate control model proposed by JCTVC-J0023 was adopted at the San Jose meeting. The rate control based on the unified RQ model (URQ) can be used regardless of variable CU block sizes, slice boundary, tile boundary, and frame boundary. Despite of the advantages, the average BD-Bitrate loss of HM8.0 with the rate control is up to 31.51% for RA, 27.46% for LB, and 21.82% for LP, respectively.  The average difference between predicted target-bit and generated-bit is observed by 50.01% and 51.74% for LB and LP, respectively. This contribution proposes modification of the rate control to improve compression performance for RA and target-bit prediction accuracy for LD.

For the RA case, a frame target-bit should be precisely predicted, since the frame target-bits influences the RQ model, which directly result in compression performance for the subsequent frame. For the HM8.0 rate control, a frame complexity which determines the frame target-bit is measured with generated bits and a quantization parameter (QP). We found that it is not sufficient to evaluate the frame complexity with only the generated-bit and the QP. In the proposal, the frame complexity is precisely calculated by using distortion and Lagrange multiplier (lambda). In the HM8.0 rate control, two virtual buffers (VB) which indicate accumulated status of buffer occupancy are employed for the firstly coded frame and remainder frames, respectively. In this contribution, we also employ two VBs. However, one is for the every IDR frames and the other is for the rest of them.

In the LD case, a rate control algorithm designed for real-time systems should minimize bitrate fluctuation which causes transmission delay. With the HM8.0 rate control algorithm, any RQ models are not used for the first frame in a GOP, resulting that it makes persistent fluctuation. In this proposal, the unified RQ model is applied to the every first frames from the second GOP to the rest GOPs. As a result, RQ model is applied to all the frames except for the first frame in a sequence. The hierarchical QP scheme is used to enhance compression performance with TIDs in the HM8.0. The lambda of RDO depends on the TID and that affects actual generated bits. For LD applications with a rate control, it is likely to be more important to meet the target bit requirement rather than to improve coding gain. It is desirable to control the target bit with only rate control algorithm not by other factors like TID. In the proposal, the lambda for RDO is determined regardless of the TID, as a result, the TID does not influence actual generated bits with the proposed rate control algorithm.

# Proposed rate control algorithms

2.1. Modification of the complexity weighting factors for RA

The initial QP for a target frame is determined by a RQ-model, given a target bit for the frame. For decision of the target-bit, the HM8.0 rate control considers two factors, which are the VB based on remaining budget and buffer occupancy. The target-bit allocated for the jth frame in the ith GOP is computed by a weighted combination of Ti(j) and Ti(j):

Ti(j)=α×Ti(j)+β×Ti(j)                           (1)

where α and β are set to 0.6 and 0.4, respectively. Ti(j) is calculated by

Tij=Wref,i(j−1)×Bi(j)Wref,i(j−1)×Nleftref+Wnonref,i(j−1)×Nleftnonref                (2)

where Bi(j) is the remaining total budget in the GOP. As a result, Ti(j) is determined with the reference-complexity weighting factor, Wref,i(j)for the reference frames and the non-reference-complexity weighting factor, Wnonref,i(j) for the non-reference frames. For the HM8.0 rate control, they are defined by

Wref,ij=bij×Qij8+7×Wref,ij−18                      (3)

Wnon−ref,ij=bi(j)×Qi(j)8+7×Wnon−ref,ij−18

where Qi(j) is the QP and bi(j) is the actual generate bits of the jth frame in the ith GOP.

The weighting factors deal with the generated bits and the QP for estimation of frame complexity. Although the QP and the generated-bit could evaluate the frame complexity, it is not sufficient to predict the accurate frame complexity. In this proposal, the complexity weighting factors are estimated with the distortion and the actual generated bit. The proposed complexity weighting factors are defined by

Wref,ij=di(j)+bij×λij8+7×Wref,ij−18                    (4)

Wnon−ref,ij=di(j)+bij×λij8+7×Wnon−ref,ij−18

where di(j) is MAD between the reconstructed frame and original frame. Note that the MAD is used for MAD model update and we can reuse it for the proposed complexity weighting factors. With the proposed algorithm, we can computer more proper target bit for a frame and we found that it can improve RD performance.

2.2. Virtual buffer management at the random access points for RA

This contribution employs two types of VBs, a short-term VB and a long-term VB. The short-term VB immediately allocates the exceeded bits to the next frame in the same way as the HM8.0 rate control. Since the IDR frame is coded with a large amount of bits, the short-term VB reduces the bits for the next frame, which causes the quality degradation. Therefore, this contribution proposes the long-term VB which distributes the exceeded bits to the consecutive frames until the next IDR frame is coded. Figure 1 shows examples for the long-term VB and the short term-VB.

- (b)

Figure 1. (a) short-term VB scheme and (b) long-term VB scheme

2.3. The rate control of firstly coded frame in a GOP for LD

Since the HM8.0 rate control is not applied to first frames in all GOPs, bit fluctuation cannot be controlled appropriately. Figure 2 shows that fluctuation is caused by the first frame not being the RQ-model in a GOP.

Figure 2. fluctuation effect of Class D BasketballPass for LP 424Kbps for the first frame In the HM8.0

This contribution proposes that the RQ model is applied to the every first frames except for the first frame of a sequence to reduce bit fluctuation. Table 1 presents the method for determining a firstly coded frame QP, compare to the HM8.0.

Table 1. QP derivation process for first frame in a GOP

| HM8.0 with the rate control | HM8.0 with the proposed rate control |
| --- | --- |
| QP  average of a previous GOP IF previous frame QP + 2 < QP THEN QPQP+2 ELIF previous frame QP – 2 > QP THEN QPQP-2 ENDIF | Predict a target-bit IF No remaining-bit in GOP THEN QP  previous frame QP + 2 ELSE invoke the RQ model ENDIF |

2.4. The rate control regardless of the temporal layer ID for LD

In HM8.0, hierarchical QP scheme is employed to improve coding efficiency. The hierarchical GOP structure allocates different QPs and lambdas depending on the TIDs. The lambda value is computed by

λ=QPfactor×2QP−123×max2,min4,QP−126 if TID > 0        (5)

The lambda is influenced by the TID and different lambda again influences actual generated bits in RDO process. Figure 3 presents persistent fluctuation effect according to the TID having zero TID.

Figure 3. fluctuation effect of Class D BasketballPass for LP 424Kbps for the TID

For the low-delay applications with a rate control algorithm, it is frequently desirable to control properly actual bitrate with only rate control algorithm not the other factors. We found that the different lambda depending on the TIDs affect actual generated bits, especially, for the frames having zero TID. In this contribution, we employ a single lambda regardless of TID in order to consistently control the actual generated bitrate for low-delay applications. In the proposed modification, the first one of Eq. (5) not the second one is used.

# Experiments

The proposed rate control is evaluated with the CBR condition. The target bitrates are set to the output bitrates generated by HM8.0 with JCTVC-H1100 common test conditions. To shows how many bits apart from the target bitrates is defined by

ΔKbps='bitsRC'−'bitsHM8.0''bitsHM8.0'×100                        (6)

ΔPSNR is absolute difference between the proposed rate control and the HM8.0:

ΔPSNR='PSNRRC'−'PSNRHM8.0'                         (7)

Table 2. BD-Rate performance of the proposed rate control and the HM8.0 rate control, compared to the HM8.0 without rate control

|  | The HM8.0 RC |  | The proposed RC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Δkbps | ΔPSNR | Δkbps | ΔPSNR | BD-PSNR | BD-BR |
| Class A | 1.04 % | -1.44 dB | 1.32 % | -1.05 dB | 0.31 dB | -6.24 % |
| Class B | 0.81 % | -1.22 dB | 1.25 % | -0.59 dB | 0.55 dB | -17.37 % |
| Class C | 0.66 % | -1.46 dB | 0.26 % | -0.34 dB | 1.16 dB | -23.58 % |
| Class D | 0.70 % | -1.17 dB | 0.36 % | -0.34 dB | 0.89 dB | -19.54 % |
| Class E |  |  |  |  |  |  |
| average | 0.80 % | -1.32 dB | 0.80 % | -0.58 dB | 0.73 dB | -16.68 % |

Table 2 shows changing-range of Δkbps and ΔPSNR for RA case. The average over-, under fluctuation ranges of Δkbps is about 0.8% and ΔPSNR degradation was about 0.58dB, respectively. It is noticeable that the compression performance of the RA with the proposed algorithms, modification of complexity weighting factors and the management strategy of virtual buffer, increases by 0.73db, compare to the HM8.0 rate control.

The figure 3 shows the difference between the predicted target-bit and the actual generated bits in the low delay scheme of the HM8.0 rate control.

(a)

(b)

Figure 4. The gaps of (a) Class B PartyScene for LB 3,440Kbps and Class D BasketballPass for LP 424Kbps between actual bits and target-bit in the HM8.0 RC

The figure 4 shows the relation between the predicted target-bit and the actual generated bits for the proposed rate control in LD

(a)

(b)

Figure 5. The gaps of (a) Class A PartyScene for LB 3,440Kbps and (b) BasketballPass for LP 424K bps between actual bits and target-bit in the proposed RC

Table 3. Performance evaluation of the proposed rate control for LB, compared with HM8.0 with the rate control

|  | The HM8.0 RC |  | The proposed RC (LB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Δkbps | ΔPSNR | Δkbps | ΔPSNR | BD-PSNR | BD-BR |
| ClassA |  |  |  |  |  |  |
| ClassB | 0.26% | -0.93 dB | 0.34% | -2.07 dB | -1.08 dB | 40.62 % |
| ClassC | 0.38% | -0.56 dB | 0.19% | -1.26 dB | -0.63 dB | 16.96 % |
| ClassD | 0.13% | -0.45 dB | 0.17% | -0.92 dB | -0.44 dB | 11.62 % |
| ClassE | 0.50% | -0.77 dB | 0.72% | -1.48 dB | -0.72 dB | -1.45 % |
| Average | 0.32% | -0.68 dB | 0.35% | -1.43 dB | -0.72 dB | 16.94 % |

Table 4. Performance evaluation of the proposed rate control for LP, compared with HM8.0 with the rate control

|  | The HM8.0 RC |  | The proposed RC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Δkbps | ΔPSNR | Δkbps | ΔPSNR | BD-PSNR | BD-BR |
| ClassA |  |  |  |  |  | % |
| ClassB | 0.19% | -0.86 dB | 0.29% | -1.87 dB | -1.23 dB | 47.73 % |
| ClassC | 0.26% | -0.53 dB | 0.18% | -1.16 dB | -1.23 dB | 18.65 % |
| ClassD | 0.14% | -0.45 dB | 0.19% | -0.87 dB | -0.72 dB | 13.41 % |
| ClassE | 0.50% | -0.71 dB | 0.73% | -1.42 dB | -0.51 dB | 30.46 % |
| Average | 0.27% | -0.64 dB | 0.35% | -1.33 dB | -0.92 dB | 27.56 % |

ΔVAR is calculated with variance by

ΔVAR='VARRC'−'VARHM8.0''VARRC'×100

Table 5. Evaluation of bit fluctuation

| Class | HM8.0 RC |  | Proposed RC |  | ΔBITS |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | LB | LP | LB | LP | LB | LP |
| Class B | 70460 | 70722 | 32029 | 38191 | 54.54% | 46.00% |
| Class C | 15734 | 17265 | 2692 | 2864 | 82.89% | 83.41% |
| Class D | 2785 | 2876 | 1581 | 1744 | 43.23% | 39.36% |
| Class E | 7778 | 9377 | 6270 | 6026 | 19.39% | 35.74% |
| average | 24189 | 25060 | 10643 | 12206 | 50.01% | 51.13% |

(9)

Although average BD-PSNR is reduced by 0.92db for LB and 0.72 for LP, compare to the HM8.0 with rate control, the average absolute difference for bits fluctuation is reduced by 50.01% for LB and 51.74 for LP, respectively.

(a)                                    (b)

(c)                                   (d)

Figure 6. Accumulated occupancy of virtual buffer of (a) Class B ‘BQTerrace’ 1,986K bps, (b) Class C BQMall 4,184K bps, (c) Class D ‘BlowingBubble‘ 821K bps, and (d) Class E Johnny 1,506K bps

Figure 6 shows status of accumulated occupancy of virtual buffer. The average occupancy of virtual buffer stays near the 0 on X-axis.

# References

- J. Si, S. Ma, W. Gao, “Adaptive rate control for HEVC,” JVT of ISO/IEC MPEG and ITU-T VCEG, JVT-J0057, Stockholm, SE, July. 2012

- H. Choi, J. Nam, J. Yoo, D. Sim, I. V. Bajić, “Rate control based on unified RQ model for HEVC,” JVT of ISO/IEC MPEG and ITU-T VCEG, JVT-H023, San José, CA, Feb. 2012

- Y. Liu, Z.G. Li, and Y.C. Soh, “A novel rate control scheme for low delay video communication of H.264/AVC standard,” IEEE Trans. on Circuits and Syst. Video Tech., vol. 17, no. 1, pp. 68-78, Jan. 2007

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCT-VC G1200 (m22869), Geneva, CH, Nov. 2011

# Patent rights declaration(s)

KWU does not have any current or pending patent rights relating to the technology described in this contribution.