| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0230 |
| --- | --- |

| Title: | Non-CE1: Simplification of strong filter decisions in CE1 deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda, Ali Tabatabai 1730 N. First Street San Jose, CA 95112 USA Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo 141-8610 Japan | Tel: Email: | +1-408-352-4082 Masaru.Ikeda@am.sony.com Ali.Tabatabai@am.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution addresses simplification of strong filter decisions in CE1 deblocking. In current CE1, some items are discussed to increase the strength of deblocking filter. One of them is the modification of strong filter decisions (b.2.1). In this contribution, 2 parts of those strong filter decisions are proposed to be combined to one part, and then the number of the comparison reduces 3 to 2 comparisons per line and totally 6 to 4 for 4 lines.

It is reported that for “CE1 b.2.1 + b.2.2” vs. “CE1 b.2.2 + Proposed simplification” the average results are provided as follows: AI-Main: 0.0%, RA-Main: 0.0%, LB-Main: -0.1%, LP-Main: 0.0%, AI-HE10: 0.0%, RA-HE10: 0.0%, LB-HE10: -0.1% and LP-HE10: 0.0%. In addition for “CE1 test3” vs. “CE1 test3 + Proposed simplification”, the average results are provided as follows: AI-Main: 0.0%, RA-Main: 0.0%, LB-Main: -0.1%, LP-Main: 0.1%, AI-HE10: 0.0%, RA-HE10: 0.0%, LB-HE10: -0.1% and LP-HE10: 0.0%. And also for “CE1 test5” vs. “CE1 test5 + Proposed simplification”, the average results are provided as follows: AI-Main: 0.0%, RA-Main: 0.0%, LB-Main: 0.0%, LP-Main: 0.0%, AI-HE10: 0.0%, RA-HE10: 0.0%, LB-HE10: -0.1% and LP-HE10: 0.0%.

# Introduction

In CE1 [1], some improvements for deblocking filter are proposed to remove residual blocking artifacts. One of them addresses to strong filter decisions (b.2.1 [2]). In HM8, deblocking decisions largely consist of three parts, which are Bs decision, on/off decision and strong/weak filter decision. In on/off decision, below expressions are calculated for pixels p0i-p2i/q0i-q2i on both sides of block boundary. Those expressions evaluate if the pixels p0i-p2i/q0i-q2i have the continuity (a straight line) or not as shown in Figure 1.

dp0 = | p20 − 2*p10 + p00 |; 		dq0 = | q20 − 2*q10 + q00 |;			(1)

dp3 = | p23 − 2*p13 + p03 |; 		dq3 = | q23 − 2*q13 + q03 |;			(2)

dpq0 = dp0 + dq0; 			dpq3 = dp3 + dq3; 				(3)

dp = dp0 + dp3; 			dq = dq0 + dq3;					(4)

d = dpq0 + dpq3;									(5)

Figure 1: Vertical block boundary

Figure 2: Evaluation of pixels on both sides of block boundary

The on/off decision is done as “deblocking on for the boundary” if the following condition holds:

d < β										(6)

Then strong filter decision is done for the 4 lines. In HM8, the following three conditions ((7), (8) and (9)) for lines i = 0, 3 are checked. Strong filter is applied for the 4 lines if all conditions hold

2*dpqi < (β>>2)						(corresponding to green)	(7)

| p3i – p0i | + | q3i – q0i | < (β>>3)				(corresponding to red)		(8)

| p0i – q0i | < ((5*tc + 1)>>1)				(corresponding to purple)	(9)

The expression (7) checks the continuity (straight line or not) for pixels p0-p2/q0-q2. The expression (8) checks the flatness for pixels p0-p3/q0-q3. The expression (9) checks the gap between p0 and q0.

Figure 3: Strong filter decisions in HM8

In CE1 b.2.1, the strong filter decisions are modified to increase the chance to apply strong filter as follows.

2*dpqi < (β>>2)						(corresponding to green)	(10)

((| p3i − 2*p2i + p1i | + | q3i − 2*q2i + q1i |)<<1) < (β>>2)	(corresponding to red)		(11)

| p0i – q0i | < (tc<<4)					(corresponding to purple)	(12)

The expression (10) is the same one as (7) in HM8. The expression (11) checks the continuity for pixels p1-p3/q1-q3 as with (10). The expression (12) is basically the same one as (9) in HM8 and the difference is the threshold. As is already known, both expression (10) and (11) check the continuity for pixels on both sides of block boundary. Therefore those expressions are redundant.

Figure 4: Strong filter decisions in CE1 b.2.1

# Proposed simplification

In this contribution, the following strong filter decision is proposed to solve the redundant.

(| p3i – p2i – p1i + p0i | + | q3i – q2i − q1i + q0i |) < (β>>3)	(corresponding to red)		(13)

| p0i – q0i | < (tc<<4)					(corresponding to purple)	(14)

The expression (14) is the same one as (12) in CE1 b.2.1. The expression (13) combines comparison of (10) and (11) into a single comparison and checks the continuity for pixels p0-p3/q0-q3 at one time as shown in Figure 5. This simplification can reduce number of comparison 3 to 2 per line, and totally 6 to 4 comparisons per 4 lines.

Figure 5: Proposed strong filter decisions

# Experimental results

The experiment is done for mandatory conditions based on JCTVC-J1101 with using HM-8.0. The detail of test sequences, configurations and cases is described in JCTVC-J1100 [3]. Both encoding and decoding time are measured with using the same machine as anchor and the decoding one is done without YUV output file.

## On common test conditions

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.2% | 0.0% | 0.1% | -0.1% |
| Class B | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% | -0.1% | 0.0% | -0.2% |
| Class C | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% |
| Class D | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | -0.2% |
| Class E | -0.4% | 0.2% | -0.4% | -0.8% | -0.7% | -0.3% |
| Class F | -0.1% | 0.4% | 0.6% | 0.1% | 0.1% | -0.3% |
| Overall | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% | -0.2% |
|  | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 97% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.4% | 0.1% | -0.1% | -0.2% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | -0.2% |
| Class D | 0.0% | -0.2% | -0.1% | 0.0% | 0.2% | 0.0% |
| Class E | 0.1% | 0.1% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class F | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 96% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 1: “CE1 b.2.1 + b.2.2” vs. “CE1 b.2.2 + Proposed simplification”

Table 1 shows the comparison between CE1 b.2.1 and this proposal on top of HM8 with CE1 b.2.2. Those decisions can provide effective results with blocking artifacts combined with strong filtering modification (b.2.2). The average results are provided as follows: AI-Main: 0.0%, RA-Main: 0.0%, LB-Main: -0.1%, LP-Main: 0.0%, AI-HE10: 0.0%, RA-HE10: 0.0%, LB-HE10: -0.1% and LP-HE10: 0.0%.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | -0.1% | 0.2% | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F | -0.1% | -0.2% | -0.1% | 0.1% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% |
| Class C | -0.1% | 0.0% | -0.1% | 0.0% | 0.3% | 0.2% |
| Class D | 0.0% | -0.2% | 0.0% | 0.0% | -0.2% | 0.1% |
| Class E | -0.4% | 0.0% | 0.0% | -0.8% | -0.7% | -0.3% |
| Class F | 0.1% | 0.6% | 0.3% | 0.0% | -0.2% | -0.6% |
| Overall | -0.1% | 0.1% | 0.0% | -0.1% | -0.2% | -0.1% |
|  | -0.1% | 0.1% | -0.1% | -0.1% | -0.2% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | -0.1% | 0.1% | -0.2% | -0.6% |
| Class C | 0.0% | -0.2% | 0.3% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | -0.3% | -0.8% | 0.1% | 0.2% | -0.2% |
| Class E | 0.1% | -0.4% | -0.2% | -0.2% | 0.2% | -0.3% |
| Class F | 0.1% | 0.0% | 0.5% | 0.1% | -0.4% | 0.3% |
| Overall | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% |
|  | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2: “CE1 test3” vs. “CE1 test3 +Proposed simplification”

Table 2 shows the comparison between CE1 b.2.1 and this proposal on top of CE1 test3. The average results are provided as follows: AI-Main: 0.0%, RA-Main: 0.0%, LB-Main: -0.1%, LP-Main: 0.1%, AI-HE10: 0.0%, RA-HE10: 0.0%, LB-HE10: -0.1% and LP-HE10: 0.0%.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.4% |
| Class B | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.3% | -0.1% | -0.2% | -0.5% |
| Class C | 0.0% | 0.2% | -0.2% | -0.1% | -0.2% | -0.3% |
| Class D | 0.0% | 0.2% | -0.2% | 0.0% | 0.1% | 0.1% |
| Class E | -0.2% | 0.2% | -0.1% | -0.7% | -0.6% | -0.2% |
| Class F | 0.1% | 0.4% | -0.2% | 0.1% | -0.4% | 0.3% |
| Overall | 0.0% | 0.2% | -0.2% | -0.1% | -0.3% | -0.1% |
|  | 0.0% | 0.2% | -0.1% | -0.1% | -0.3% | -0.1% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | -0.3% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | 0.4% | 0.0% | -0.4% | 0.0% |
| Class E | 0.1% | -0.3% | 0.1% | 0.0% | -0.2% | 0.4% |
| Class F | 0.0% | 0.1% | 0.1% | 0.1% | -0.4% | 0.7% |
| Overall | 0.0% | 0.0% | 0.2% | 0.0% | -0.2% | 0.1% |
|  | 0.0% | 0.1% | 0.2% | 0.0% | -0.2% | 0.1% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Table 3: “CE1 test5” vs. “CE1 test5 + Proposed simplification”

Table 3 shows the comparison between CE1 b.2.1 and this proposal on top of CE1 test5. The average results are provided as follows: AI-Main: 0.0%, RA-Main: 0.0%, LB-Main: 0.0%, LP-Main: 0.0%, AI-HE10: 0.0%, RA-HE10: 0.0%, LB-HE10: -0.1% and LP-HE10: 0.0%.

## On additional CE1 sequences

The following additional sequences were suggested by CE1 for participation in the subjective quality test riverbed, DucksTakeOff, WestWindsEasy, RedKayak and the sequence ChinaSpeed from the test set. The experimental results for the additional sequences are provided below. The QPs used for the experiment are QP 32, 37 and 41.

| RA-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DucksTakeOff, 25fps | 32 | hm8.0 | 9214.38 | 29.56 | 32.50 | 36.73 |
|  |  |  | CE1_b.2.1+b.2.2 | 9213.04 | 29.57 | 32.50 | 36.73 |
|  |  |  | Proposal | 9215.11 | 29.57 | 32.50 | 36.73 |
|  |  | 37 | hm8.0 | 3954.07 | 27.26 | 31.55 | 36.27 |
|  |  |  | CE1_b.2.1+b.2.2 | 3953.52 | 27.28 | 31.56 | 36.27 |
|  |  |  | Proposal | 3953.06 | 27.27 | 31.56 | 36.27 |
|  |  | 41 | hm8.0 | 1993.55 | 25.69 | 30.96 | 35.98 |
|  |  |  | CE1_b.2.1+b.2.2 | 1992.29 | 25.70 | 30.96 | 35.97 |
|  |  |  | Proposal | 1993.23 | 25.70 | 30.96 | 35.97 |
|  | RedKayak,570fr | 32 | hm8.0 | 3709.59 | 35.65 | 41.82 | 44.91 |
|  |  |  | CE1_b.2.1+b.2.2 | 3710.86 | 35.65 | 41.82 | 44.92 |
|  |  |  | Proposal | 3709.06 | 35.65 | 41.82 | 44.92 |
|  |  | 37 | hm8.0 | 1714.38 | 32.94 | 41.01 | 44.13 |
|  |  |  | CE1_b.2.1+b.2.2 | 1713.69 | 32.95 | 41.01 | 44.13 |
|  |  |  | Proposal | 1713.78 | 32.95 | 41.01 | 44.13 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.42 | 43.49 |
|  |  |  | CE1_b.2.1+b.2.2 | 853.12 | 30.97 | 40.42 | 43.49 |
|  |  |  | Proposal | 852.45 | 30.97 | 40.43 | 43.47 |
| LB-Main | riverbed | 32 | hm8.0 | 5851.44 | 34.64 | 38.96 | 41.40 |
|  |  |  | CE1_b.2.1+b.2.2 | 5848.49 | 34.64 | 38.96 | 41.40 |
|  |  |  | Proposal | 5848.46 | 34.64 | 38.96 | 41.40 |
|  |  | 37 | hm8.0 | 2870.61 | 31.71 | 38.13 | 40.91 |
|  |  |  | CE1_b.2.1+b.2.2 | 2865.04 | 31.74 | 38.13 | 40.90 |
|  |  |  | Proposal | 2866.50 | 31.74 | 38.13 | 40.90 |
|  |  | 41 | hm8.0 | 1566.17 | 29.55 | 37.69 | 40.63 |
|  |  |  | CE1_b.2.1+b.2.2 | 1563.65 | 29.58 | 37.69 | 40.62 |
|  |  |  | Proposal | 1562.98 | 29.58 | 37.70 | 40.63 |
|  | WestWindEasy | 32 | hm8.0 | 3758.62 | 36.22 | 44.74 | 47.46 |
|  |  |  | CE1_b.2.1+b.2.2 | 3751.34 | 36.21 | 44.72 | 47.44 |
|  |  |  | Proposal | 3752.09 | 36.22 | 44.71 | 47.44 |
|  |  | 37 | hm8.0 | 1679.87 | 33.01 | 43.71 | 46.65 |
|  |  |  | CE1_b.2.1+b.2.2 | 1674.34 | 33.02 | 43.73 | 46.68 |
|  |  |  | Proposal | 1675.13 | 33.02 | 43.72 | 46.66 |
|  |  | 41 | hm8.0 | 854.87 | 30.75 | 42.93 | 45.86 |
|  |  |  | CE1_b.2.1+b.2.2 | 852.43 | 30.76 | 42.92 | 45.86 |
|  |  |  | Proposal | 852.66 | 30.76 | 42.93 | 45.84 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1270.60 | 35.30 | 40.24 | 40.18 |
|  |  |  | CE1_b.2.1+b.2.2 | 1268.25 | 35.30 | 40.24 | 40.19 |
|  |  |  | Proposal | 1269.16 | 35.30 | 40.23 | 40.20 |
|  |  | 37 | hm8.0 | 612.05 | 31.97 | 38.75 | 38.42 |
|  |  |  | CE1_b.2.1+b.2.2 | 610.91 | 31.98 | 38.75 | 38.40 |
|  |  |  | Proposal | 610.67 | 31.98 | 38.73 | 38.39 |
|  |  | 41 | hm8.0 | 316.87 | 29.38 | 37.76 | 36.85 |
|  |  |  | CE1_b.2.1+b.2.2 | 317.14 | 29.39 | 37.76 | 36.82 |
|  |  |  | Proposal | 317.14 | 29.39 | 37.76 | 36.82 |

Table 4: CE1 b.2.2 + Proposed simplification

| RA-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DucksTakeOff, 25fps | 32 | hm8.0 | 9214.38 | 29.56 | 32.50 | 36.73 |
|  |  |  | CE1_test3 | 9216.49 | 29.57 | 32.49 | 36.73 |
|  |  |  | Proposal | 9222.36 | 29.57 | 32.49 | 36.73 |
|  |  | 37 | hm8.0 | 3954.07 | 27.26 | 31.55 | 36.27 |
|  |  |  | CE1_test3 | 3955.28 | 27.28 | 31.56 | 36.27 |
|  |  |  | Proposal | 3956.72 | 27.28 | 31.55 | 36.27 |
|  |  | 41 | hm8.0 | 1993.55 | 25.69 | 30.96 | 35.98 |
|  |  |  | CE1_test3 | 1994.15 | 25.70 | 30.96 | 35.98 |
|  |  |  | Proposal | 1996.34 | 25.71 | 30.96 | 35.97 |
|  | RedKayak,570fr | 32 | hm8.0 | 3709.59 | 35.65 | 41.82 | 44.91 |
|  |  |  | CE1_test3 | 3713.29 | 35.66 | 41.82 | 44.91 |
|  |  |  | Proposal | 3713.68 | 35.66 | 41.82 | 44.91 |
|  |  | 37 | hm8.0 | 1714.38 | 32.94 | 41.01 | 44.13 |
|  |  |  | CE1_test3 | 1716.31 | 32.96 | 41.01 | 44.12 |
|  |  |  | Proposal | 1716.29 | 32.96 | 41.01 | 44.12 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.42 | 43.49 |
|  |  |  | CE1_test3 | 853.40 | 30.97 | 40.42 | 43.48 |
|  |  |  | Proposal | 854.12 | 30.98 | 40.42 | 43.47 |
| LB-Main | riverbed | 32 | hm8.0 | 5851.44 | 34.64 | 38.96 | 41.40 |
|  |  |  | CE1_test3 | 5852.95 | 34.65 | 38.96 | 41.40 |
|  |  |  | Proposal | 5855.27 | 34.65 | 38.96 | 41.39 |
|  |  | 37 | hm8.0 | 2870.61 | 31.71 | 38.13 | 40.91 |
|  |  |  | CE1_test3 | 2867.19 | 31.74 | 38.13 | 40.90 |
|  |  |  | Proposal | 2869.04 | 31.75 | 38.13 | 40.91 |
|  |  | 41 | hm8.0 | 1566.17 | 29.55 | 37.69 | 40.63 |
|  |  |  | CE1_test3 | 1563.64 | 29.58 | 37.70 | 40.64 |
|  |  |  | Proposal | 1563.72 | 29.59 | 37.69 | 40.63 |
|  | WestWindEasy | 32 | hm8.0 | 3758.62 | 36.22 | 44.74 | 47.46 |
|  |  |  | CE1_test3 | 3754.37 | 36.22 | 44.71 | 47.48 |
|  |  |  | Proposal | 3755.79 | 36.23 | 44.72 | 47.49 |
|  |  | 37 | hm8.0 | 1679.87 | 33.01 | 43.71 | 46.65 |
|  |  |  | CE1_test3 | 1673.36 | 33.02 | 43.64 | 46.63 |
|  |  |  | Proposal | 1675.33 | 33.02 | 43.62 | 46.64 |
|  |  | 41 | hm8.0 | 854.87 | 30.75 | 42.93 | 45.86 |
|  |  |  | CE1_test3 | 851.80 | 30.76 | 43.03 | 45.99 |
|  |  |  | Proposal | 851.96 | 30.75 | 43.02 | 45.97 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1270.60 | 35.30 | 40.24 | 40.18 |
|  |  |  | CE1_test3 | 1268.94 | 35.30 | 40.24 | 40.19 |
|  |  |  | Proposal | 1268.87 | 35.31 | 40.24 | 40.19 |
|  |  | 37 | hm8.0 | 612.05 | 31.97 | 38.75 | 38.42 |
|  |  |  | CE1_test3 | 610.88 | 31.98 | 38.69 | 38.39 |
|  |  |  | Proposal | 610.19 | 31.97 | 38.69 | 38.42 |
|  |  | 41 | hm8.0 | 316.87 | 29.38 | 37.76 | 36.85 |
|  |  |  | CE1_test3 | 316.61 | 29.39 | 37.75 | 36.85 |
|  |  |  | Proposal | 316.52 | 29.39 | 37.75 | 36.82 |

Table 5: CE1 test3 + Proposed simplification

| RA-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DucksTakeOff, 25fps | 32 | hm8.0 | 9214.38 | 29.56 | 32.50 | 36.73 |
|  |  |  | CE1_test5 | 9220.11 | 29.57 | 32.49 | 36.73 |
|  |  |  | Proposal | 9219.54 | 29.57 | 32.49 | 36.73 |
|  |  | 37 | hm8.0 | 3954.07 | 27.26 | 31.55 | 36.27 |
|  |  |  | CE1_test5 | 3956.08 | 27.28 | 31.55 | 36.27 |
|  |  |  | Proposal | 3957.35 | 27.28 | 31.55 | 36.27 |
|  |  | 41 | hm8.0 | 1993.55 | 25.69 | 30.96 | 35.98 |
|  |  |  | CE1_test5 | 1994.57 | 25.71 | 30.97 | 35.98 |
|  |  |  | Proposal | 1994.54 | 25.70 | 30.96 | 35.98 |
|  | RedKayak,570fr | 32 | hm8.0 | 3709.59 | 35.65 | 41.82 | 44.91 |
|  |  |  | CE1_test5 | 3712.55 | 35.66 | 41.82 | 44.91 |
|  |  |  | Proposal | 3713.02 | 35.66 | 41.81 | 44.90 |
|  |  | 37 | hm8.0 | 1714.38 | 32.94 | 41.01 | 44.13 |
|  |  |  | CE1_test5 | 1716.38 | 32.95 | 41.01 | 44.11 |
|  |  |  | Proposal | 1716.61 | 32.96 | 41.01 | 44.12 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.42 | 43.49 |
|  |  |  | CE1_test5 | 853.50 | 30.97 | 40.43 | 43.49 |
|  |  |  | Proposal | 853.53 | 30.97 | 40.43 | 43.47 |
| LB-Main | riverbed | 32 | hm8.0 | 5851.44 | 34.64 | 38.96 | 41.40 |
|  |  |  | CE1_test5 | 5854.49 | 34.65 | 38.95 | 41.39 |
|  |  |  | Proposal | 5855.32 | 34.65 | 38.96 | 41.40 |
|  |  | 37 | hm8.0 | 2870.61 | 31.71 | 38.13 | 40.91 |
|  |  |  | CE1_test5 | 2868.93 | 31.74 | 38.13 | 40.90 |
|  |  |  | Proposal | 2867.50 | 31.74 | 38.12 | 40.89 |
|  |  | 41 | hm8.0 | 1566.17 | 29.55 | 37.69 | 40.63 |
|  |  |  | CE1_test5 | 1564.80 | 29.58 | 37.69 | 40.64 |
|  |  |  | Proposal | 1563.78 | 29.59 | 37.69 | 40.61 |
|  | WestWindEasy | 32 | hm8.0 | 3758.62 | 36.22 | 44.74 | 47.46 |
|  |  |  | CE1_test5 | 3753.21 | 36.22 | 44.72 | 47.49 |
|  |  |  | Proposal | 3755.02 | 36.22 | 44.70 | 47.48 |
|  |  | 37 | hm8.0 | 1679.87 | 33.01 | 43.71 | 46.65 |
|  |  |  | CE1_test5 | 1675.25 | 33.03 | 43.63 | 46.63 |
|  |  |  | Proposal | 1676.73 | 33.03 | 43.62 | 46.62 |
|  |  | 41 | hm8.0 | 854.87 | 30.75 | 42.93 | 45.86 |
|  |  |  | CE1_test5 | 851.60 | 30.76 | 43.02 | 46.02 |
|  |  |  | Proposal | 853.41 | 30.76 | 43.04 | 46.01 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1270.60 | 35.30 | 40.24 | 40.18 |
|  |  |  | CE1_test5 | 1268.88 | 35.30 | 40.25 | 40.19 |
|  |  |  | Proposal | 1269.55 | 35.30 | 40.24 | 40.19 |
|  |  | 37 | hm8.0 | 612.05 | 31.97 | 38.75 | 38.42 |
|  |  |  | CE1_test5 | 611.16 | 31.97 | 38.70 | 38.41 |
|  |  |  | Proposal | 611.82 | 31.97 | 38.70 | 38.40 |
|  |  | 41 | hm8.0 | 316.87 | 29.38 | 37.76 | 36.85 |
|  |  |  | CE1_test5 | 316.84 | 29.39 | 37.76 | 36.84 |
|  |  |  | Proposal | 316.28 | 29.38 | 37.76 | 36.83 |

Table 6: CE1 test5 + Proposed simplification

# Conclusions

This contribution presents simplification of strong filter decisions in CE1 deblocking. The simplification can reduce the number of comparison is from 3 to 2 per line and from 6 to 4 per 4 lines. For “CE1 b.2.1 + b.2.2” vs. “CE1 b.2.2 + Proposed simplification”, the average results are provided as follows: AI-Main: 0.0%, RA-Main: 0.0%, LB-Main: -0.1%, LP-Main: 0.0%, AI-HE10: 0.0%, RA-HE10: 0.0%, LB-HE10: -0.1% and LP-HE10: 0.0%. In addition for “CE1 test3” vs. “CE1 test3 + Proposed simplification”, the average results are provided as follows: AI-Main: 0.0%, RA-Main: 0.0%, LB-Main: -0.1%, LP-Main: 0.1%, AI-HE10: 0.0%, RA-HE10: 0.0%, LB-HE10: -0.1% and LP-HE10: 0.0%. And also for “CE1 test5” vs. “CE1 test5 + Proposed simplification”, the average results are provided as follows: AI-Main: 0.0%, RA-Main: 0.0%, LB-Main: 0.0%, LP-Main: 0.0%, AI-HE10: 0.0%, RA-HE10: 0.0%, LB-HE10: -0.1% and LP-HE10: 0.0%.

It is proposed to discuss this simplification of strong filter decisions together with other CE1 proposals, and adopt this technique into next HM.

# Draft text

In this proposal, the following modifications with yellow highlight are presented.

##### 8.7.2.4.6 Decision process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	variables dpq, β and tC.

Output of this process is:

–	a variable dSam containing a decision

The variable dSam is specified as follows:

–	If dpq is less than ( β >> 2 ), abs( p3 – p0 ) + abs( q0 – q3 ) is less than ( β >> 3 ) (abs(p3 – p2 – p1 + p0) + abs(q3 – q2 – q1 + q0))<<1  is less than ( β >> 2 ), and abs( p0 – q0 ) is less than ( 5*tC + 1 ) >> 1(tC << 4), dSam is set equal to 1.

–	Otherwise, dSam is set equal to 0.

# References

- JCT-VC, “Description of Core Experiment 1: Deblocking filter”, JCTVC-J1101, 10th. JCT-VC Meeting, Stockholm, SE, July, 2012

- JCT-VC, “AHG6: Reduction of block artifacts in HEVC for large blocks”, JCTVC-J0286, 10th. JCT-VC Meeting, Stockholm, SE, July, 2012

- JCT-VC, “HM8 Common Test Conditions and Software Reference Configurations”, JCTVC-J1100, 10th. JCT-VC Meeting, Stockholm, SE, July, 2012

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).