| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, China, 09–19 October 2012 | Document: JCTVC-K0231 WG11 Number: m26563 |
| --- | --- |

| Title: | VPS syntax re-ordering for easy access of extension parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
|  | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Electronics Inc., Sony Corp. |  |  |

_____________________________

# Abstract

This contribution presents re-ordering of the syntax parameters for the VPS and VPS Extension Syntax structures available in the "straw man" designs for HEVC extensions. This is to allow easy access of some of the syntax parameters with fixed lengths by external network devices for better communication purposes.

- On HEVC VPS Syntax Structure

The VPS syntax structure in the current DIS specification [2] contains three undefined syntaxes of reserved_zero bits - vps_reserved_zero_2bits, vps_reserved_zero_6bits and vps_reserved_zero_12bits.

In the "straw man" designs [1] of VPS and its extensions, the last two undefined syntaxes above are utilized as max_num_layers_minus1 and next_essential_info_byte_offset, shown in the next sub-section. Also in [1], two design approaches are considered for the VPU Extension syntax structures, out of which either one or both may be considered for HEVC extensions.

## On VPS Syntax structure changes

The following VPS syntax structure is available in [1], where the vps_reserved_zero_12bits in DIS [2] is replaced by the syntax parameter next_essential_info_byte_offset.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| reserved_zero_2bits | u(2) |
| max_num_layers_minus1 //reserved_zero_6bits in DIS | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_level( 1, vps_max_sub_layers_minus1 ) // should be profile_tier_level |  |
| next_essential_info_byte_offset //reserved_zero_12bits in DIS | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| num_hrd_parameters | ue(v) |
| for( i = 0; i < num_hrd_parameters; i++ ) { |  |
| if( i > 0 ) |  |
| op_point( i ) |  |
| hrd_parameters( i = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| bit_equal_to_one | u(1) |
| vps_extension( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The syntax parameter next_essential_info_byte_offset is defined in [1]  as follows:

"specifies the byte offset of the next set of fixed-length coded information in the video parameter set NAL unit, starting from its beginning of the NAL unit".

However, the "next set of" syntax parameters of "pics" information (buffering, reorder, latency) present after this next_essential_info_byte_offset are not of fixed-length sizes in the above syntax structure, but of unsigned integer 0-th order Exp-Golomb-codes, ue(v).

Also in Note-3 of [1], it is mentioned that such fixed coded length information is "essential for session negotiation and/or capability exchange", especially in network related middle-box or other simpler devices. This will enable easy access of the essential information that VPS and VPS extension provides in their respective syntax parameters without the need of any entropy decoding function.

It may also be noted that the above VPS syntax structure from [1] also contains a new syntax structure extension of vps_extension().

## Addition of VPS Extension Syntax structure

In [1], the VPS Extension syntax structure has two "strawman" design approaches listed, and both of them contain syntax parameters with fixed-length information. Also in our contribution [3] for a new VPS extension structure, all such syntax parameters contain fixed-length information.  As noted in the previous section, many of these VPS Extension syntax parameters will also be valuable for external network devices to browse and extract useful information for better communication purposes. Also the "profile_tier_level( 1, vps_max_sub_layers_minus1 )" syntax structure contains Fixed-length syntax parameters.

However the positioning of such VPS extension information in the VPS syntax structure of [1], as shown above, needs to be changed as follows to enable easy access of these extension parameters:

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| reserved_zero_2bits | u(2) |
| max_num_layers_minus1 //reserved_zero_6bits in the base spec | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| next_essential_info_byte_count //reserved_zero_12bits in the base spec | u(12) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) // change in the name |  |
| next_essential_info_byte_offset //reserved_zero_12bits in the base spec | u(12) |
| bit_equal_to_one | u(1) |
| vps_extension( ) |  |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| num_hrd_parameters | ue(v) |
| for( i = 0; i < num_hrd_parameters; i++ ) { |  |
| if( i > 0 ) |  |
| op_point( i ) |  |
| hrd_parameters( i = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| bit_equal_to_one | u(1) |
| vps_extension( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

In this syntax parameter re-ordering, the VPS Extension information is placed near the beginning of the VPS NALU payload for easy access. This is because many VPS Extension information for HEVC extensions may be directly useable by middle-boxes and other network devices. In the above syntax structures, the previously defined next_essential_info_byte_offset in [1] is renamed as next_essential_info_byte_count.

Also its semantics description is changed as follows:

next_essential_info_byte_count specifies the total number of bytes occupied by the optional fields of profile_tier_level(.), vps_extension() and any stuffing bits including the bits of the syntax parameters bit_equal_to_one and   vps_extension_byte_alignment_reserved_one_bit.

Note1:  The byte-offset from the start of the VPS NAL Unit packet to next_essential_info_byte_count is already known as 4.

# Conclusion

In this contribution, the HEVC VPS Syntax Structure is modified with re-ordering of some syntax parameters for the future HEVC Extensions in scalable video coding, multi-view coding and 3D video coding areas. This re-ordering will enable the external network devices for easy access of some of the useful parameters in VPS and its extension, that can be used for better communication and system integration purposes.

# Patent rights declaration(s)

Sony Electronics Inc. / Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Ye-Kui Wang, “Solutions considered for NAL unit header and video parameter set for HEVC extensions”, JCTVC-J1007, Stockholm, Sweden, July 2012.

- B. Bross, et. al., “High Efficiency Video Coding (HEVC) text specification draft 8,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J1003_d7, Stockholm, Sweden, July 2012.

- M. Haque, K. Sato and A. Tabatabai, "VPS Extension updates in Strawman Design Approach 1 using a modified version of Scalability Mapping Table", JCTVC-K0233, Shanghai, China, October 2012.