| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, China, 09–19 October 2012 | Document: JCTVC-K0232 WG11 Number: m26564 |
| --- | --- |

| Title: | On support of mixed video sequences in High Level Syntaxes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
|  | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc., Sony Corp. |  |  |

_____________________________

# Abstract

This contribution suggests some options to include support for mixed sequences and mixed content coding in High Level Syntaxes of the current HEVC version 1 parameter sets as well as in its extensions.

- On Mixed Contents and Coding support

The current high level syntax structures of Video Parameter Set (VPS) and Sequence parameter Set (SPS) in DIS specification [1]  do not address any support for mixed video sequences of progressive, interlaced and 3-D view pictures. It is also anticipated that in HEVC extensions, some of these sequences may contain coded contents with mixed or hybrid coding type of HEVC, AVC, etc., either for scalable layers (Base and/or Enhancement) or in 3D views (stereo or multi-views). This is also addressed in a current contribution [4].

This contribution first identifies some undefined syntax parameters in the existing parameter sets to include possible support for such mixed coding contents as well as mixed sequences of progressive, interlace and 2D/3D frames. In case such changes are not possible with the undefined syntax parameters, we also suggest to create some additional parameters for support of such mixed sequences near their respective syntax extensions areas (*_extension_flag and *extension_data_flag) at the end of such parameter sets. This later case will follow the same path as in the first case while using the corresponding syntax parameter definitions.

The following table shows which existing parameters could possibly be utilized to include support for mixed content coding and mixed sequences:

| Parameter Set | Undefined Syntaxes in Version-1 | Descriptions with new suggestions |  |
| --- | --- | --- | --- |
| Case-1: Existing syntax parameters |  |  |  |
| video_parameter_set_rbsp( ) / VPS | vps_reserved_zero_2bits, u(2) |  | JCTVC-K0232: vps_sequence_type |
|  | vps_reserved_zero_6bits, u(6) |  | JCTVC-J1007: vps_max_num_layers_minus1 |
|  | vps_reserved_zero_12bits, u(12) |  | JCTVC-J1007: next_essential_info_byte_offset |
| vps_extension() | scalability_type, u(4) sub_scalability_type, u(4) |  | JCTVC-K0233: coding_type, interlace parameter (top_field_first), etc. |
| sequence_parameter_set_rbsp() / SPS | sps_reserved_zero_bit, u(1) |  | JCTVC-K0232: sps_sequence_type |
|  | pic_height_in_luma_samples, ue(v) |  | Height for field = ½ Height for frame |
| picture_parameter_set_rbsp() / PPS | - |  | - |
| Case-2: Future extensions to define |  |  |  |
| VPS, SPS and PPS | Any other syntax for Interlace Support ? | JCTVC-K0232: To define |  |
|  | *_extension_flag, u(1) | Undefined |  |
|  | *_extension_data_flag, u(1) | Undefined |  |

Case-1: Existing syntax parameters

In the VPS syntax structure of DIS [1], there are three parameters of “reserved bits” as shown in yellow-highlighted areas below - vps_reserved_zero_2bits, vps_reserved_zero_6bits and vps_reserved_zero_12bits that could be utilized for new parameters definition in VPS.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_sequence_type // vps_reserved_zero_2bits in DIS | u(2) |
| vps_reserved_zero_6bits // used in JCTVC-J1007 | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_and_level( 1, vps_max_sub_layers_minus1 ) |  |
| vps_reserved_zero_12bits // used in JCTVC-J1007 | u(12) |
| ……………….. DELETED …………………………….. |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

The use of the last two parameters of vps_reserved_zero_6bits and vps_reserved_zero_12bits is already addressed in the last meeting's contribution JCTVC-J1007 [2].

Semantics of vps_sequence_type

vps_sequence_type replaces the vps_reserved_zero_2bits syntax parameter in VPS to indicate the mixed sequence types as follows:

| vps_sequence_type | Sequence type in VPS |
| --- | --- |
| 00 | Frame pictures only |
| 01 | Mixed Progressive and Interlace pictures |
| 10 | Mixed 2D and 3D Frame pictures |
| 11 | Mixed 2D and 3D Frame-Compatible pictures |

When the value of vps_sequence_type is 00, it indicates the coded sequences contains Frame pictures only.

When the value of vps_sequence_type is 01, it indicates the coded sequences contains mixed sequences of Progressive and Interlace pictures.

When the value of vps_sequence_type is 10, it indicates the coded sequences contains mixed sequences of 2D and 3D Frame pictures.

When the value of vps_sequence_type is 11, it indicates the coded sequences contains mixed sequences of 2D and 3D Frame-Compatible (Stereo-view) pictures.

In the SPS syntax structure of DIS [1], there are two parameters as shown in yellow-highlighted areas below - sps_reserved_zero_bit and pic_height_in_luma_samples that could be of some relevance for Interlace coding support in SPS.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_sequence_type_flag // sps_reserved_zero_bit in DIS | u(1) |
| …………….. DELETED ………………………….. |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| …………….. DELETED ………………………….. |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The use of the last parameter of pic_height_in_luma_samples is obvious where field_height is half of the frame_height in case of Interlaced content versus similar progressive contents.

Semantics of sps_sequence_type_flag

sps_sequence_type_flag renames the sps_reserved_zero_bit syntax parameter in SPS to indicate the coded sequence types with the following choices as follows for vps_sequence_type of 0, 1 or 2:

| sps_sequence_type_flag | Sequence type |
| --- | --- |
| 0 | Frame (2-D) |
| 1 | Frame / Field / 3-D Frame / 3-D Frame-Compatible |

When the value of sps_sequence_type_flag is 0, it indicates "Frame" sequences (2D), otherwise it indicates either a "Frame" or "Field" or "3-D (stereo, multi-view)" or "3-D Frame-Compatible (stereo-view)" sequence.

The combined effect of vps_sequence_type and sps_sequence_type_flag is shown in the table below:

| VPS syntax |  | SPS syntax |  |
| --- | --- | --- | --- |
| vps_sequence_type | Sequence type in VPS | sps_sequence_type_flag | Sequence type in SPS |
| 00 | Frame pictures only | 0 | Frame |
|  |  | 1 | Frame |
| 01 | Mixed Progressive and Interlace pictures | 0 | Frame |
|  |  | 1 | Field |
| 10 | Mixed 2D and 3D Frame pictures | 0 | 2D Frame |
|  |  | 1 | 3D Frame (stereo, multi-view) |
| 11 | Mixed 2D and 3D Frame-Compatible pictures | 0 | 2D Frame |
|  |  | 1 | 3D Frame-Compatible (stereo-view) |

Case-2: Extension syntax parameters

If the syntax parameters of vps_reserved_zero_2bits and sps_reserved_zero_bit cannot be utilized as mentioned in Case-1 above, we can perhaps extend the VPS and SPS by adding such new syntax parameters near the existing syntax parameters of extension_flag and extension_data_flag at the end of the respective VPS and SPS parameter sets. The red high-lighted parts in the syntax structures for the edited versions of VPS and SPS show these new syntax parameters to address mixed picture sequences.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_zero_2bits | u(2) |
| vps_reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_and_level( 1, vps_max_sub_layers_minus1 ) |  |
| vps_reserved_zero_12bits | u(12) |
| ……………….. DELETED …………………………….. |  |
| vps_sequence_type_flag // for mixed sequence type in VPS | u(1) |
| if(vps_sequence_type_flag) { |  |
| vps_sequence_type | u(2) |
| vps_coding_type | u(2) |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Semantics of vps_sequence_type_flag is a 1-bit flag that indicates whether the information regarding mixed sequences (vps_seqeuence_type, vps_coding_type) are present (=1) in VPS or not (=0).

vps_sequence_type is already defined earlier in Option 1.

Semantics of vps_coding_type

vps_coding_type indicate the sequences coded with the following coding types:

| vps_coding_type | Coding type in VPS |
| --- | --- |
| 00 | HEVC |
| 01 | AVC |
| 10 | reserved |
| 11 | reserved |

When the value of vps_coding_type is 0, it indicates HEVC coding. When it is 1,  it indicates AVC coding and other values of 2 and 3 are reserved.

Application of the vps_coding_type may be restricted as an option for the Base Layer (View) pictures of all sequences falling under a VPS. In the more general case, vps_coding_type may also be applicable for some Enhancement Layers (Views) where Inter-layer or Inter-view predictions are not present.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_bit | u(1) |
| …………….. DELETED ………………………….. |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| …………….. DELETED ………………………….. |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_sequence_type_flag // for mixed sequence type | u(1) |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

It may be added that for such extension of VPS or SPS, these newly defined syntax parameters of vps_sequence_type, vps_coding_type and sps_seqeuence_type_flags can be assigned more bits to include many other variations, as necessary for the future HEVC extensions in the same spirit.

# Conclusion

In this contribution, we have added support for mixed sequences of progressive, interlace, 3-D (stereo, multi-view), 3-D Frame-Compatible (stereo-view) video sequences in the high level syntax structures of VPS and SPS without affecting the existing version-1 syntax structures in option 1. It also suggested for adding different coding type as an extension syntax parameter in option 2. The target goal is to indicate such mixed sequences by using the remaining values of the reserved bits in option 1 and also possibly adding the extension syntax for coding type in option 2. In case, such changes are not possible, we also suggest for inclusion of new syntax parameters as extensions at the end of the corresponding parameter sets with the same definitions as outlined.

# Patent rights declaration(s)

Sony Electronics Inc. / Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- B. Bross, et. al., “High Efficiency Video Coding (HEVC) text specification draft 8,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J1003_d7, Stockholm, Sweden, July 2012.

- Ye-Kui Wang, “Solutions considered for NAL unit header and video parameter set for HEVC extensions”, JCTVC-J1007, Stockholm, Sweden, July 2012.

- M. Haque, K. Sato and A. Tabatabai, "VPS Extension updates in Strawman Design Approach 1 using a modified version of Scalability Mapping Table", JCTVC-K0233, Shanghai, China, October 2012.

- K. Sato, "On inter-layer prediction enabling/disabling for HEVC scalable extensions", JCTVC-K0175, Shanghai, China, October 2012.