| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, China, 09–19 October 2012 | Document: JCTVC-K0233 WG11 Number: m26565 |
| --- | --- |

| Title: | VPS Extension updates in Strawman Design Approach 1 using a modified version of Scalability Mapping Table |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
|  | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc., Sony Corp. |  |  |

_____________________________

# Abstract

This contribution first presents updates of the scalability_type mapping table in the approach 2 VPS Extension design. It then utilizes this table in the approach 1 VPS Extension design for the final version. It also extends this mapping table by defining another mapping table of sub_scalability_type to support mixed video sequences for various HEVC extensions of scalable video coding, multi-view coding, 3D video coding, etc.

- On scalability_type mapping table

The “strawman design” for VPS extension in approach 2 of JCTVC-J1007 [1] utilizes a scalability_type mapping table that specifies the dimensions signaled through layer_id in the NAL unit header. The layer_id (6 bits of NAL Unit Header data) is partitioned into specific scalability dimension IDs, as shown in the syntax structure for vps_extension() in approach 2 design below:

| vps_extension( ) { // approach 2 design | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| // scalability type and layer_id partitioning method |  |
| scalability_type | u(4) |
| for( i = 0; i < MaxDim( scalability_type ); i++ ) |  |
| layer_id_dim_len[ i ] | u(3) |
| // layer specific information |  |
| for( i = 0; i <= max_num_layers_minus1; i++ ) { |  |
| vps_layer_id[ i ] | u(6) |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

However, the scalability_type mapping table in [1], as shown below, can be enhanced further by incorporating additional changes for an optimized and flexible vps_extension() syntax structure design.

| scalability_type | MaxDim(scalability_type) | Scalability dimensions |
| --- | --- | --- |
| 0 | 1 | none (base HEVC) |
| 1 | 2 | spatial and quality |
| 2 | 3 | spatial, quality, reserved |
| 3 | 4 | spatial, quality, reserved, reserved |
| 4 | 2 | multiview and depth |
| 5 | 3 | multiview, depth, reserved |
| 6 | 4 | multiview, depth, reserved, reserved |
| 7 | 4 | multiview, spatial, quality and depth |
| 8 | 5 | multiview, spatial, quality, depth, reserved |
| 9 | 6 | multiview, spatial, quality, depth, reserved, reserved |
| 10...15 | reserved | reserved |

If we compare the two design approaches in [1] for vps_extension() syntax structure, it is found that the design approach 2 has some limitation in its basic definition, as it restricts the bit-allocations for various dimesnion_ids when it defines the syntax parameter, layer_id_dim_len[i] as follows:

"layer_id_dim_len[ i ] specifies the length, in bits, of the i-th scalability dimension ID. The sum of the values layer_id_dim_len[ i ] for all i values in the range of 0 to 7 shall be less than or equal to 6."

| for( i = 0; i < MaxDim( scalability_type ); i++ ) |  |
| --- | --- |
| layer_id_dim_len[ i ] | u(3) |

It means we may be forced to assign a particular scalability layer with a very limited set of values for all dimesnion_ids. Such limitation may not be a suitable design choice.

In this regard, the vps_extension() syntax structure of approach 1 design in [1], does not have such limitation in its syntax parameter definition. Hence the approach 1 design is considered for the final design choice for vps_extension() syntax structure.

But this approach 1 design as available in [1], may require further improvement in its design optimization as well as adding some new requirements for HEVC extensions of scalable video coding, multi-view coding and 3D video coding, etc. So the yellow -highlighted color parts below will be changed according to descriptions in the next sub-section.

| vps_extension( ) { // approach 1 design | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| num_dimensions_minus1[ i ] | u(4) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) { |  |
| dimension_type[ i ][ j ] | u(4) |
| dimension_id[ i ][ j ] | u(8) |
| } |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

| dimension_type[ i ][ j ] | dimension_id[ i ][ j ] |
| --- | --- |
| 0 | view order idx |
| 1 | depth flag |
| 2 | dependency ID |
| 3 | quality ID |
| 4..15 | reserved |

As seen from the syntax structure above for approach 1 design that uses the above table of dimension_type[i][j] and dimension_id[i][j], we may observe the following:

a) First, it is quite possible to remove the syntax parameter of dimension_type[i][j], when we consider a re-mapped table of scalability_type from approach 2 design.

b) Second, the bit-assignments of dimension_id[[i][j], 8 bits unsigned each, can also be compacted further by properly allocating the dimension_ids with appropriate number of bits via a Look-Up-Table (LUT) approach in the new scalability_type mapping table, as described in the later sub-sections.

The following sub-section presents such design by re-defining the scalability_type mapping table and the corresponding vps_extension() syntax structure to replace the original vps_extension() syntax structure of approach 1 design.

## VPS Extension

In the new scalability_type mapping table as shown below, two new dimension_ids are added -  coding_type, sub_scalability_type. Also in the table below the corresponding names of dimension_id and their LUT-type bit-assignments dimension_id_len are added.

| scalability_type | MaxDim (scalability_type) | Scalability dimensions | dimension_id | dimension_id_len |
| --- | --- | --- | --- | --- |
| 0 | 0 | none (base HEVC) | - | - |
| 1 | 4 | coding_type, sub_scalability_type, spatial, quality | coding_type, sub_scalability_type, dependencyID, qualityID | 2, 4, 4, 4 |
| 2 | 5 | coding_type, sub_scalability_type, spatial, quality, reserved | coding_type, sub_scalability_type, dependencyID, qualityID, reserved | 2, 4, 4, 4, 4 |
| 3 | 6 | coding_type, sub_scalability_type, spatial, quality, reserved, reserved | coding_type, sub_scalability_type, dependencyID, qualityID, reserved, reserved | 2, 4, 4, 4, 4, 4 |
| 4...15 | - | reserved |  |  |

| coding_type | Video coding type |
| --- | --- |
| 0 | HEVC (default) |
| 1 | AVC |
| 2 .. 3 | reserved |

In addition, the new dimesnsion_id parameter, sub_scalability_type, uses a mapping  table, also defined below.

It may be noted such new additions of scalability syntax parameter and sub_scalability dimension_id parameters addresses the same issues for support of mixed sequences to be present in a VPS and the corresponding picture sequences [3][4].

| sub_scalability_type | sub_scalability dimension_type | MaxDimSub (sub_scalability_type) | diemension_id | dimensionSub_id_len |
| --- | --- | --- | --- | --- |
| 0 | mono-view (2D) | 0 | - | - |
| 1 | interlace (2D) | 1 | top_field_first | 1 |
| 2 | frame-compatible (3D) | 3 | depth_flag, left_view_first, fc_format | 1, 1, 4 |
| 3 | stereo-view (3D) | 3 | depth_flag, view1_id, view2_id | 1, 4, 4 |
| 4 | multi-view (3D) | 1 | depth_flag | 1 |
| 5 ... 15 | reserved |  |  |  |

It may also be added that a fixed number of bits are assigned for each dimension_id in a Look-Up-Table (LUT) method (last columns in the mapping tables). Here dimension_id_len and dimensionSub_id_len represent the number of bits assigned for a dimension_id in the above two mapping tables of scalability_type and sub_scalability_type respectively.

Instead of using dimension_id_len or dimensionSub_id_len as syntax parameters, as done in the original approach 1 design [1] with more bits assignment, a fixed size of bit-lengths is assigned for each dimension_id, that may result in more compact tables.

It may be added such bit-length assignments are made based upon past examples for such dimension_ids from SVC, MVC and 3D Video coding specifications and this may be open for further discussions to change such bit-allocation assignments.

The following changes (yellow-highlighted) are made for the new vps_extension() syntax structure that is different from approach 1 design as available in [1].

| vps_extension( ) { // approach 1 design modified |  | Descriptor |
| --- | --- | --- |
| while( !byte_aligned( ) ) |  |  |
| vps_extension_byte_alignment_reserved_one_bit |  | u(1) |
| // layer specific information |  |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |  |
| // mapping of layer ID to scalability dimension IDs |  |  |
| scalability_type[ i ] | u(4) |  |
| num_dimensions1 = MaxDim ( scalability_type[i] ) |  |  |
| for( j = 0; j < num_dimensions1; j++ ) { |  |  |
| nb = dimension_id_len[j] |  |  |
| dimension_id[ i ][ j ] | u(nb) |  |
| } |  |  |
| if (num_dimesnions1 > 0) { |  |  |
| sub_scalability_type[ i ] = dimension_id[ i][1] |  |  |
| num_dimensions2 = MaxDimSub ( sub_scalability_type[i] ) |  |  |
| for( j = 0; j < num_dimensions2; j++ ) { |  |  |
| nb = dimensionSub_id_len[j] |  |  |
| dimension_id[ i ][ num_dimensions1 + j ] | u(nb) |  |
| } |  |  |
| if (sub_scalability_type[i] == 4) { // multi-view |  |  |
| num_views[ i ] // views number | u(4) |  |
| for(j=0; j< num_views[i]; j++) |  |  |
| view_id[ i ][ j ] // view_ids | u(4) |  |
| } |  |  |
| } |  |  |
| // layer dependency |  |  |
| num_direct_ref_layers[ i ] |  | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |  |
| ref_layer_id[ i ][ j ] |  | u(6) |
| } |  |  |
| } |  |  |

Semantics descriptions of new syntax parameters, dimension_ids and new terms

scalability_type[ i ] specifies the scalability types for the i-th scalability layer in use in the coded video sequence and the dimensions mapped through layer_id in the NAL unit header. As mentioned in [1], "when scalability_type[i] is equal to 0, the coded video sequence conforms to the base HEVC specification, thus layer_id of all NAL units is equal to 0 and there are no NAL units belonging to an enhancement layer or view".

sub_scalability_type[ i ] specifies the sub_scalability types for the i-th scalability layer in use in the coded video sequence and the dimensions mapped are the extended part of scalability_type[i] dimensions  to consider mixed video sequences in content type.

dimension_id_len[ i ] specifies the length, in bits, of the i-th scalability_type dimension ID in the scalability_type mapping table.

dimensionSub_id_len[ i ] specifies the length, in bits, of the i-th sub_scalability_type dimension ID in the sub_scalability_type mapping table.

num_views[ i ] specifies the  total number of views for the i-th scalability layer coded in the 3D  video bitstream with multi-view coding.

view_id[ i ][ j ] specifies the ID value for the j-th view of the i-th scalability layer in the 3D  video bitstream with multi-view coding.

view1_id, view2_id specifies the ID values for the stereo view in the 3D  video bitstream with stereo-view coding (2 views)

left_view_first and fc_format are dimesnion_ids for Frame-compatible 3D video.

left_view_first being 1 means that the reconstructed sequence starts with the frame-compatible format having a left view as the first view output during the decoding process,  else it is a right_view_first for frame-compatible video coding.

fc_format is a 4-bit data element indicating the format for multiplexing of two views into a single frame or sequence of frames for frame-compatibe 3D setreo-views. Examples for fc_format values are Top_Bottom (0), Side_by_Side (1), Row Interleaved (2), Column Interleaved (3), Checkerboard (4), etc.

coding_type indicates the video coding type in the coded video bitstreams - 0 (HEVC), 1 (AVC), etc. Optionally, application of the coding_type may be restricted for the Base Layer (View) pictures of all sequences, and also it could be applicable for some Enhancement Layers (Views) where Inter-layer or Inter-view predictions are not present.

dependencyID specifies a dependency identifier for the NAL unit. dependencyID shall be equal to 0 in prefix NAL units.  (see AVC specification for dependency_id)

qualityID specifies a quality identifier for the NAL unit. qualityID shall be equal to 0 in prefix NAL units.   (see AVC specification for quality_id)

top_field_first being 1 means that the reconstructed sequence starts with a top field as the first field output during the decoding process,  else it is a bottom_field_first for interlace video coding.

depth_flag a 1-bit flag indicates whether depth-view bistsream exists for 3D video coding extension.

# Conclusion

In this contribution, the scalability_type mapping table is enhanced with a new sub_scalability_type table addition for the VPS extension design approach 2. Then these tables are suitably utilized in the VPS extension syntax structure design of approach 1 as the final design version.

# Patent rights declaration(s)

Sony Electronics Inc. / Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Ye-Kui Wang, “Solutions considered for NAL unit header and video parameter set for HEVC extensions”, JCTVC-J1007, Stockholm, Sweden, July 2012.

- B. Bross, et. al., “High Efficiency Video Coding (HEVC) text specification draft 8,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J1003_d7, Stockholm, Sweden, July 2012.

- M. Haque, K. Sato and A. Tabatabai, “On support of mixed video sequences in High Level Syntaxes”, JCTVC-K0232, October 2012

- K. Sato, "On inter-layer prediction enabling/disabling for HEVC scalable extensions", JCTVC-K0175, Shanghai, China, October 2012.