| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, China, 09–19 October 2012 | Document: JCTVC-K0234 WG11 Number: m26566 |
| --- | --- |

| Title: | HEVC VUI Parameters Extension beyond Version 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
|  | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Electronics Inc., Sony Corp. |  |  |

_____________________________

# Abstract

This contribution presents updates of the current HEVC VUI Parameters Syntax structure to consider the HEVC extensions of scalable video coding, multi-view coding, 3D video coding, etc. A few new syntax parameters are introduced by using examples from a modified version of scalability_type mapping table of VPS Extension design approach 2.

- On HEVC VUI Syntax Structure

The VUI syntax structure in the current DIS Draft [2] does not address the HEVC extensions of scalable video coding, multi-view coding, 3D video coding, etc.  In our contribution, the existing VUI syntax structure is extended to include such new VUI syntax parameters needed for the HEVC extensions.

The new VUI syntax parameters for HEVC extensions are borrowed from the VPS extensions as defined in [1] and [3]. This contribution contains 2 options of VUI extension designs, as illustrated in the sub-sections below. The first option considers the scalability_type mapping tables for VPS Extension design in [1]. The second option is added for VUI Extension by using the syntax parameters of VPS Extension design of Approach 1 in [3].

## HEVC VUI Parameters Syntax Structure

In the VUI syntax structure as available in [2], a hevc_extension_flag syntax parameter is added.

When the value of hevc_extension_flag  is set to 1, an extension syntax structure for VUI is introduced for HEVC extensions. The yellow-highlighted parts in the VUI parameters syntax structure below show these new syntaxes.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = Extended_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| video_signal_type_present_flag | u(1) |
| if( video_signal_type_present_flag ) { |  |
| video_format | u(3) |
| video_full_range_flag | u(1) |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coefficients | u(8) |
| } |  |
| } |  |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| neutral_chroma_indication_flag | u(1) |
| field_seq_flag | u(1) |
| hevc_extension_flag | u(1) |
| if( hevc_extension_flag ) |  |
| vui_parameters_ext_layer( sps_max_layers_minus1 ) |  |
| hrd_parameters_present_flag | u(1) |
| if( hrd_parameters_present_flag ) |  |
| hrd_parameters( 1, sps_max_sub_layers_minus1 ) |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

## HEVC VUI Parameters Extension Layer

In Option 1 of this syntax structure shown below, the syntax parameters are borrowed from the VPS Extension design and the corresponding scalability_type mapping tables in [1].

| vui_parameters_ext_layer( MaxNumLayersMinus1) { | Descriptor |
| --- | --- |
| for( i = 0; i <= MaxNumLayersMinus1; i++ ) { |  |
| // vui scalability type and dimension IDs |  |
| vui_scalability_type[ i ] | u(4) |
| num_dimensions1 = MaxDim (vui_scalability_type[i]) |  |
| for( j = 0; j < num_dimensions1; j++ ) { |  |
| nb = dimension_id_len[j] |  |
| vui_dimension_id[ i ][ j ] | u(nb) |
| } |  |
| if(num_dimensions1 > 0) { |  |
| vui_sub_scalability_type[ i ] = dimension_id[ i ] [ 1 ] |  |
| num_dimensions2 = MaxDimSub (vui_sub_scalability_type[i]) |  |
| for( j = 0; j < num_dimensions2; j++ ) { |  |
| nb = dimensionSub_id_len[j] |  |
| vui_dimension_id[ i ][ num_dimensions1 + j ] | u(nb) |
| } |  |
| if (vui_sub_scalability_type[i] == 4) { // multi-view |  |
| vui_num_views[ i ] // views number | u(4) |
| for(j=0; j< vui_num_views[i]; j++) |  |
| vui_view_id[ i ][ j ] // view_ids | u(4) |
| } |  |
| } |  |
| } |  |
| } |  |

## HEVC VUI Parameters Extension Layer: Option 2

In Option 2 of this syntax structure shown below, the syntax parameters are borrowed from the VPS Extension parameters of Approach 1 in [3].

| vui_parameters_ext_layer( MaxNumLayersMinus1) { | Descriptor |
| --- | --- |
| for( i = 0; i <= MaxNumLayersMinus1; i++ ) { |  |
| // vui dimension IDs |  |
| vui_num_dimensions_minus1[ i ] | u(4) |
| for( j = 0; j <= vui_num_dimensions_minus1; j++ ) { |  |
| vui_dimension_type[ i ][ j ] | u(4) |
| vui_dimension_id[ i ][ j ] | u(8) |
| } |  |
| } |  |
| } |  |

It may be added that the following table is already available in [3]:

| vui_dimension_type[ i ][ j ] | vui_dimension_id[ i ][ j ] |
| --- | --- |
| 0 | view order idx |
| 1 | depth flag |
| 2 | dependency ID |
| 3 | quality ID |
| 4..15 | reserved |

# Conclusion

In this contribution, the HEVC VUI Syntax Structure is extended to deal with coded video contents beyond HEVC Version-1 for the HEVC Extensions in scalable video coding, multi-view video coding, 3D video coding, etc.

# Patent rights declaration(s)

Sony Electronics Inc. / Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- M. Haque, K. Sato and A. Tabatabai, "VPS Extension updates in Strawman Design Approach 1 using a modified version of Scalability Mapping Table", JCTVC-K0233, Shanghai, China, October 2012.

- B. Bross, et. al., “High Efficiency Video Coding (HEVC) text specification draft 8,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J1003_d7, Stockholm, Sweden, July 2012.

- Ye-Kui Wang, “Solutions considered for NAL unit header and video parameter set for HEVC extensions”, JCTVC-J1007, Stockholm, Sweden, July 2012.