| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC- K0235 m26567 |
| --- | --- |

| Title: | Single inclusion of long-term reference pictures in RPS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Rickard Sjöberg | Tel: Email: | +46761263591 jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution contains a proposal for how to avoid that the same long-term reference picture is included more than once in the same reference picture set (RPS). At the Stockholm meeting the possibility to signal long-term reference pictures in the sequence parameter set (SPS) was adopted. It is therefore possible to signal the least significant bits (LSB) of long-term reference pictures both in the SPS and in the slice header and the semantics makes it possible to include the same long-term reference picture more than once in the same RPS even though that has no practical usage. This contribution proposes that the LSB that are signalled in the sequence parameter set (SPS) and the LSB that are signalled in the slice header are put in a joint ordered (non-increasing) list and that for elements with the same LSB, the most significant bits (MSB) are signalled in strictly increasing order. The contribution claims that the proposed changes makes it impossible to include the same long-term reference picture more than once in the same RPS, that ambiguity in the decoding process is removed and that redundancy in the signalling of delta_poc_msb_cycle_lt is removed.

After offline discussions regarding one case that was not handled, an updated text has been included as an attachment to this document: “K0235_proposed_text_changes.doc”.

# Introduction

All since the adoption of RPS to the HEVC specification the syntax for signalling which pictures to be included in the RPS together with the decoding process for RPS has prohibited the same picture from being included more than once in an RPS. This is a design feature motivated by:

- Making the signalling more efficient.

- Avoid having a decoding process that handles multiple inclusions of the same picture in an RPS.

- Avoid having bitstream restrictions that encoders would have to check and that might be unintentionally violated.

There is no usage of including the same picture more than once in an RPS. On the contrary it is very ambiguous for a decoder how to handle a bitstream that includes the same picture more than once in an RPS. The purpose of the RPS is to provide a set of what pictures to keep in the DPB and provide a set of what pictures that can be used for reference by the current picture. Having the same reference picture more than once in the sets does not make sense, especially if the used_by_curr_pic_flag is set differently for the different occurrences. The decoding process does not specify how to handle that. An encoder that would like to have the same picture more than once in the reference picture lists accomplishes that through reference picture list modifications.

# Problem Statement

Recent changes to the long-term reference picture part of the RPS have unfortunately broken the design principle that the syntax prohibits the inclusion of the same reference picture more than once. Specifically:

- When there are multiple long-term reference picture with the same PocLsbLt, they are signalled in increasing DeltaPocMSBCycleLt order. However a “+1” is missing in the equation making it possible to include the same long-term picture more than once (by signalling “0” for delta_poc_msb_cycle_lt).

- When long-term reference pictures are signalled both in the SPS and in the slice header the long-term reference pictures are signalled in non-increasing order of PocLsbLt, but in two independent lists, making it possible to include the same picture in both lists.

# Proposed solution

For the first problem we simply propose to add the “+1” in the equation:

if( i  = =  0  | |  i = =  num_long_term_sps  | |  PocLsbLt[ i − 1 ]  ! =  PocLsbLt[ i ] )
	DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ]
else							(7-1)
	DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ] + DeltaPocMSBCycleLt[ i − 1 ] + 1

For the second problem we propose to create only one sorted list of PocLsbLt of long-term reference pictures instead of two separate independent lists. This is accomplished through moving the derivation of PocLsbLt, UsedByCurrPicLt and DeltaPocMSBCycleLt from the semantics section to the decoding process. During the construction of PocLsbLt both the values from the SPS signalling and from the slice header signalling are evaluated. Iteratively, the highest value from the lists is inserted in the PocLsbLt list:

      - General slice header semantics

…

used_by_curr_pic_lt_flag[ i ] equal to 0 specifies that the i-th candidate long-term reference picture to be included in the long-term reference picture set of the current picture is not used for reference by the current picture.

The variables PocLsbLt[ i ] and UsedByCurrPicLt[ i ] are derived as follows.

–	If i is less than num_long_term_sps, PocLsbLt[ i ] is set equal to lt_ref_pic_poc_lsb_sps[ lt_idx_sps[ i ] ] and UsedByCurrPicLt[ i ] is set equal to used_by_curr_pic_lt_sps_flag[ lt_idx_sps[ i ] ].

–	Otherwise, PocLsbLt[ i ] is set equal to poc_lsb_lt[ i ] and UsedByCurrPicLt[ i ] is set equal to used_by_curr_pic_lt_flag[ i ].

delta_poc_msb_present_flag[ i ] equal to 1 specifies that delta_poc_msb_cycle_lt[ i ] is present. delta_poc_msb_present_flag[ i ] equal to 0 specifies that delta_poc_msb_cycle_lt[ i ] is not present. delta_poc_msb_present_flag[ i ] shall be equal to 1 when there is more than one reference picture in the decoded picture buffer with the least significant bits of the picture order count value equal to poc_lsb_lt[ i ].

delta_poc_msb_cycle_lt[ i ] is used to determine the value of the most significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture.

The variable DeltaPocMSBCycleLt[ i ] is derived as follows.

if( i  = =  0  | |  i = =  num_long_term_sps  | |  PocLsbLt[ i − 1 ]  ! =  PocLsbLt[ i ] )
	DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ]
else							(7-2)
	DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ] + DeltaPocMSBCycleLt[ i − 1 ]

8.3.2 Decoding process for reference picture set

…

The variables PocLsbLt[ i ], UsedByCurrPicLt[ i ] and DeltaPocMSBCycleLt[ i ] are derived as follows.

j = 0
k = num_long_term_sps
for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) {
	if( ( k = = num_long_term_sps + num_long_term_pics ) | | 
		 ( ( j < num_long_term_sps ) && 
		 ( lt_ref_pic_poc_lsb_sps[ lt_idx_sps[ j ] ] > poc_lsb_lt[ k ] ) ) ) {
		PocLsbLt[ i ] = lt_ref_pic_poc_lsb_sps[ lt_idx_sps[ j ] ]
		UsedByCurrPicLt[ i ] = used_by_curr_pic_lt_sps_flag[ lt_idx_sps[ j++ ] ]
	} else {					(8-X)
		PocLsbLt[ i ] = poc_lsb_lt[ k ] 
		UsedByCurrPicLt[ i ] = used_by_curr_pic_lt_flag[ k++ ]
	}
	if( delta_poc_msb_present_flag[ i ] ) {
		if( i  = =  0  | |  PocLsbLt[ i − 1 ]  ! =  PocLsbLt[ i ] )
			DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ]
		else
			DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ] + DeltaPocMSBCycleLt[ i − 1 ] + 1
	}
}

For each i, in the range of 1 to num_long_term_sps + num_long_term_pics − 1, inclusive, when PocLsbLt[ i ] is equal to PocLsbLt[ i − 1 ], delta_poc_msb_present_flag[ i ] and delta_poc_msb_present_flag[ i − 1 ] shall both be equal to 1.

# Conclusions

With the proposed changes to the derivation of long-term reference pictures in RPS it is impossible to include the same long-term reference picture more than once in the same RPS. The proposal removes redundancy in the signalling of long-term reference pictures and removes ambiguity in the decoding process.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).