# Proposed syntax and semantics additions to Annex E

### VUI parameters syntax

| vui_parameters( ) { | Descriptor |
| --- | --- |
| … |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| min_spatial_segmentation_idc | u(8) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

Semantics

min_spatial_segmentation_idc indicates the maximum size of the spatial segments in the pictures in the coded video sequence. The variable MinSpatialSegmentation is calculated as:

MinSpatialSegmentation = ( min_spatial_segmentation_idc  4) + 1

A slice is said to contain a specific luma sample if the coding block that contains the luma sample is contained in the slice. Correspondingly, a tile is said to contain a specific luma sample if the coding block that contains the luma sample is contained in the tile.

Depending on the value of min_spatial_segmentation_idc, the following applies:

–	If min_spatial_segmentation_idc is equal to 0, no limit on the maximum size of spatial segments is indicated.

–	Otherwise (min_spatial_segmentation_idc is not equal to 0), exactly one of the following conditions shall be fulfilled:

–	tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 0, in each picture parameter set activated within the coded video sequence and there is no slice in the coded video sequence that contains more than PicSizeInSamplesY / MinSpatialSegmentation number of luma samples.

–	tiles_enabled_flag is equal to 1 and entropy_coding_sync_enabled_flag is equal to 0, in each picture parameter set activated within the coded video sequence and there is no tile in the coded video sequence that contains more than PicSizeInSamplesY / MinSpatialSegmentation number of luma samples.

–	tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 1, in each picture parameter set activated within the coded video sequence and the syntax elements pic_width_in_luma_samples, pic_height_in_luma_samples and the variable CtbSize are restricted such that:

( 2 * pic_height_in_luma_samples + pic_width_in_luma_samples ) * CtbSize 
≤ PicSizeInSamplesY / MinSpatialSegmentation

NOTE – The syntax element min_spatial_segmentation_idc can be used by a decoder to calculate the maximum number of luma samples to be processed by one thread, making the assumption that the decoder maximally utilizes the parallel decoding information. It can be noted that there might be inter-dependencies between the different threads e.g. deblocking filtering across tile and slice boundaries or entropy synchronization. In order to aid decoders in planning the decoding workload distribution, encoders are encourage set the value of min_spatial_segmentation_idc to the highest possible value for which one of the three conditions above is fulfilled. For the case when tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 1 that means setting:

min_spatial_segmentation_idc =
		4 * PicSizeInSamplesY / ( ( 2 * pic_height_in_luma_samples + pic_width_in_luma_samples ) * CtbSize ) − 4

When min_spatial_segmentation_idc is not present, it is inferred to be equal to 0.