| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0236 m26568 |
| --- | --- |

| Title: | Decoder parallelism indication |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Rickard Sjöberg Magnus Westerlund Bo Burman Per Fröjdh Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46761263591 jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document proposes a parameter; Threads to be used for indicating how many parallel decoding threads a bitstream is encoded for. The parameter also represents an approximation of the speed-up that can be achieved when exploiting the parallel decoding possibilities to its maximum. The parameter is proposed to be represented by the syntax element threads_idc in the optional video usability information (VUI) part of the sequence parameter set (SPS), together with the other bitstream restrictions. Any value of threads_idc greater than 0 imposes restrictions on the bitstream.

The proposed parameter can be used together with profile and level to indicate the capabilities of a decoder: “For profile P1, when Threads is greater than or equal to T1, I can decode all bitstreams up to and including level L1”. The proposed parameter is also intended to be used as a declaration from an encoder of the level of parallelism in the bitstreams it produces.

In essence this proposal is a resubmission of JCTVC-J0249 with one major difference: The advocated parameter is in this document proposed as a VUI syntax element whereas in JCTVC-J0249 it was proposed as a mandatory SPS syntax element.

# Introduction

The HEVC profiles, levels and tiers impose bitstream restrictions. When it is indicated in a bitstream that the bitstream conforms to a certain level, limitations are put on the bitstream, for example when it comes to picture resolution. The value of level_idc does not affect the decoding process and may in that sense be regarded as supplemental information. However, this information is extremely important for decoders to deduce what bitstreams they are capable of decoding. profile, level and tier also provides the means for a decoder to express its capabilities without having to list different individual parameters (such as resolution, framerate, bitrate etc).

The VUI part of the SPS contains syntax elements which allow encoders to indicate that the bitstream obeys further restrictions, additional to those imposed by profile, level and tier. Examples of such restrictions are; max_bits_per_mincu_denom which indicates an upper bound for the number of coded bits of coding_unit( ) data for any coding block and log2_max_mv_length_vertical which indicates the maximum absolute value of a decoded vertical motion vector component.

For a decoder that contains multiple cores or processing units (e.g. a multi-core CPU) the performance and the capabilities when decoding a bitstream on one core compared to dividing the decoding workload on all available cores might be very different. Some bitstreams are suitable for parallel decoding and some are not. Specifically, bitstreams that use wavefront parallel processing (WPP) or a certain number of tiles or slices per picture are easily parallelizable. However, there is no general parameter to describe or represent the level of parallelism of a bitstream. Likewise, there is no general parameter that a decoder can use to indicate what level of parallelism the decoder needs in order to be capable of decoding a bitstream of a certain level and profile.

We propose to introduce Threads to represent the level of parallelism in a bitstream.

The proposed Threads does not affect the decoding process. Instead it imposes restrictions on the bitstream that are of very high importance for a decoder. In multi-threaded software decoders with flexible workload assignment it is generally the number of luma samples processed per thread that is the bottleneck. We propose restriction for the number of luma samples per thread for the three different parallelization schemes: WPP, tiles and slices.

Since Threads puts a limit on the number of luma samples per thread it also gives an approximation of the speed-up that can be achieved when exploiting the parallel decoding possibilities assuming that the decoding time per luma sample is constant. Threads will give a lower bound for the number of threads (number of parallel decodable parts) but does not specify the upper bound. Theoretically it is always possible to reach the indicated speed-up even when the number of threads is not a multiple of the number of cores in the decoder (using time-division multiplexing), when assuming constant processing time per luma sample. In practice a decoder has to take into account its own limitations when it comes to scheduling as well as memory access effects of different partitioning schemes and unequal processing time of luma samples, and report its capabilities according to the worst case performance.

# Proposed syntax, semantics and additions to Annex A

### VUI parameters syntax

| vui_parameters( ) { | Descriptor |
| --- | --- |
| … |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| threads_idc | u(8) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

Semantics

threads_idc is used to indicate the number of threads the bitstream is constructed for. The variable Threads is calculated as:

Threads = (threads_idc / 4) + 1

When Threads is greater than 1, it provides an indication of the level of parallelism in the bitstream and the following restrictions applies.

Let PicSizeLuma be pic_width_in_luma_samples * pic_height_in_luma_samples. A luma sample is said to be contained in a slice if the coding block that contains the luma sample is contained in the slice. Correspondingly, a luma sample is said to be contained in a tile if the coding block that contains the luma sample is contained in the tile. When Threads is greater than 1, exactly one of the following conditions shall be fulfilled:

- tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 0, in each picture parameter set activated within the coded video sequence and there is no slice in the coded video sequence that contains more than Floor( PicSizeLuma / Threads ) number of luma samples.

- tiles_enabled_flag is equal to 1 and entropy_coding_sync_enabled_flag is equal to 0, in each picture parameter set activated within the coded video sequence and there is no tile in the coded video sequence that contains more than Floor( PicSizeLuma / Threads ) number of luma samples.

- tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 1, in each picture parameter set activated within the coded video sequence and the syntax elements pic_width_in_luma_samples, pic_height_in_luma_samples and the variable CtbSize are restricted such that:

( 2 * pic_height_in_luma_samples + pic_width_in_luma_samples ) * CtbSize 
≤ Floor( PicSizeLuma / Threads )

NOTE – The variable Threads can be used by a decoder to calculate the maximum number of luma samples to be processed by one thread, making the assumption that the decoder maximally utilizes the parallel decoding information. It can be noted that there might be inter-dependencies between the different threads e.g. deblocking across tile and slice boundaries or entropy synchronization. To aid decoders in planning the decoding workload distribution it is recommended that encoders set the value of threads_idc to the highest possible value for which one of the three conditions above is fulfilled. For the case when tiles_or_entropy_coding_sync_idc = 2 that means setting:

threads_idc = Floor( 4 * PicSizeLuma / (( 2 * pic_height_in_luma_samples + pic_width_in_luma_samples ) * CtbSize )) − 4

## Regarding the syntax

The proposed restrictions represent limits to the number of luma samples that needs to be processed by each thread (as a function of PicSizeLuma and Threads).

With the proposed syntax Threads can have any value from 1 to 64.75, inclusive. If Threads is equal to 1, no additional restrictions apply. Non-integer values can be used when the encoder does not divide each picture into exactly equally large parts. For example, an encoder that encodes each picture with 4 slices but cannot guarantee that they are of equal size can signal Threads to be 3.75 meaning that no slice can be larger than (1/3.75) of the total picture size. Even though there are no decoders with 3.75 cores this could be very useful information for a multi-core decoder.

# Usage of threads_idc

In H.264/AVC and the current design of HEVC it is possible for a decoder to express its capabilities in terms of profile and level (and in HEVC tier), saying for example “For profile P1, I can decode all bitstreams up to and including level L1”, “For profile P2, I can decode all bitstreams up to and including level L2”.

With the proposed Threads parameter a decoder may in addition to the sequential performance indicate its capabilities as a function of the parallelism in the bitstream. For example “For profile P1, when Threads is greater than or equal to T1, I can decode all bitstreams up to and including level L1”, “For profile P2, when Threads is greater than or equal to T2, I can decode all bitstreams up to and including level L2” etc.

Encoders that would like to declare that they are encoding bitstreams with a certain level of parallelism will set Threads to an appropriate value, preferably the highest possible value for which one of the three conditions is fulfilled.

Encoders that do not want to indicate the level of parallelism in their bitstreams may chose not to signal the VUI bitstream restrictions or to set Threads equal to 1 regardless of if WPP, tiles or slices are used.

# Details and an example

Consider the following example in which a Main tier decoder contains 4 processing cores each capable of decoding 220 million luma samples per second with a maximum picture resolution of 3.6 million luma samples and max bitrate 60 mbps. That is more than enough to decode level 4.1 (e.g. 1920x1080p60Hz). However, it is not sufficient for decoding level 5.1 which allows up to 535 million luma samples per second, a picture size of ~9 million luma samples and max bitrate 40 mpbs.

If a 4096x2160p60Hz sequence (which is the maximum allowed in level 5.1) has Threads higher than or equal to 4 the decoder knows that the luma samples of each picture will be divided so that each thread does not have to process more than ~2.2 million luma samples. Multiplying that by the frame rate (60 Hz) gives us the maximum number of luma samples each core has to process per second (133 million).

However, Threads only specifies the upper limit for how many luma samples each thread has to process. The division of the picture might be into an arbitrary number of parallel decodable parts. If the number of such parts is larger than the number of cores at the decoder, one or more of the decoding cores has to decode multiple parts. Worst case for a decoder with X cores that requires Threads to be at least X, and does not support time-division multiplexing, is when there are X+1 equally large parts. One core must then handle 2/(X+1) of the total picture size.

For this example were the decoder has 4 cores the worst case is when the picture is divided into 5 parts each having ~1.77 million luma samples. One core would then have to handle ~3.54 million luma samples per picture and ~212 million luma samples per second. As can be seen from the numbers above each core is capable of handling more than this and the decoder can declare its capabilities as:

“I can decode all bitstreams up to and including level 4.1. Additionally, when Threads is greater than or equal to 4, I can decode all bitstreams up to and including level 5.1”.

# Discussion

It is our opinion that the parallelization tools in HEVC are very important for the adoption of HEVC in various applications. Especially in an environment where more and more platforms and devices contains multiple processing units. Parallelizing the decoding process can increase performance and/or reduce battery consumption. Having a single parameter for representing the parallelism would enable capability indication to include parallelism and would improve the possibilities for negotiation of parameters in unicast scenarios (such as video conferencing).

The three different conditions that an encoder can chose from (restriction on slices, tiles or WPP) means that a decoder must be prepared for any of these cases when indicating its capabilities in terms of Threads. It is our opinion that the three specific cases are reasonable to handle by a decoder that exploits parallelism and that the three cases provides a good trade-off between flexibility at the encoder side and demands on the decoder side.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).