| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0237 |
| --- | --- |

| Title: | Rotation of Residual block for Transform Skipping in HM8 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dake He Jing Wang Gaelle Martin-Cocher | Tel: Email: | +1-519-5978760 {dhe, wangj, gmartincocher}@rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

This proposal reports the performance of the technique described in JCTVC-J0093 in HM8.0, which rotates residual blocks by 180 degrees when transform is skipped. In common test conditions, the average BD-rates resulting from this change against HM8.0 anchor are (-0.2%, -0.1%, 0.0%, -0.2%, -0.1%, -0.1%) for (AI-Main, RA-Main, LD-Main, AI-HE10, RA-HE10, LD-HE10). For Class F sequences, the average BD-rates are (-1.4%, -1.0%, -0.5%, -1.4%, -0.9%, -0.6%).

# Introduction

A simplified design of intra transform skipping [1], [2], [3] was adopted into the HEVC working draft [4]. It was shown in [1] that the proposed design of intra transform skipping substantially improves the coding efficiency for Class F sequences while having little impact on the coding efficiency for sequences in Classes A-E.

In [5], a change that effectively rotates a block of residual samples 180 degrees in transform skipping (see Figure 1) was described. Since residual data from intra prediction with transform skipped typically have nonzero values concentrated at position near bottom-right corner, as they are farthest from the samples used prediction, such rotation will align data for subsequent quantization and entropy coding, consistent with the data output from transform.

This change is replicated when the lossless mode is enabled by the cu_transquant_bypass_flag [4], where all transforms are skipped.

NZ

180 degrees

NZ

0

0

Fig. 1 Rotating a block of residual samples 180 degrees

# Proposed Changes

## Intra Transform Skipping

In HM8.0, the following two lines of code in xITransformSkip

pResidual[j * uiStride + k] =  (plCoef[j*width+k] + offset) >> transformSkipShift;

pResidual[j * uiStride + k] =  plCoef[j*width+k] << transformSkipShift;

are proposed to be changed to the following lines, respectively.

pResidual[j * uiStride + k] =  (plCoef[(height-j-1)*width+width-k-1] + offset) >> transformSkipShift;

pResidual[j * uiStride + k] =  plCoef[(height-j-1)*width+width-k-1] << transformSkipShift;

Effectively, the changes to the indices rotate the residual block 180 degrees in the decoder. Correspondingly, changes to rotate the residual blocks 180 degrees in the encoder are implemented in xTransformSkip.

## Lossless Mode

To implement the change in the lossless mode in HM8.0 decoder	, the following line of code in invtransformNxN

rpcResidual[k*uiStride+j] = pcCoeff[k*uiWidth+j];

is proposed to be changed to

rpcResidual[k*uiStride+j] = pcCoeff[(uiHeight-1-k)*uiWidth+(uiWidth-1-j)];

Corresponding changes are implemented in transformNxN for the encoder.

Changes in Sections 2.1 and 2.2 above are provided in a single patch file named “jctvc-k0000.patch”.

# Results

Table 1 below shows the performance of the proposed change against HM8.0 anchor in common test conditions [6]. Note that the timing results are not reliable as they were obtained in a mixed computing cluster.

Table 1. BD-rates against HM8.0 Anchor

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class C | -0.3% | -0.2% | -0.3% | -0.2% | -0.2% | -0.3% |
| Class D | -0.4% | -0.4% | -0.5% | -0.3% | -0.3% | -0.4% |
| Class E | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
| Overall | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% |
|  | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% |
| Class F | -1.4% | -1.5% | -1.7% | -1.4% | -1.6% | -1.8% |
| Enc Time[%] | 125% |  |  | 126% |  |  |
| Dec Time[%] | 115% |  |  | 113% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | 0.0% | -0.1% | 0.0% | -0.3% |
| Class B | 0.0% | -0.2% | 0.1% | 0.0% | -0.1% | 0.1% |
| Class C | -0.1% | -0.3% | -0.1% | -0.2% | -0.3% | -0.6% |
| Class D | -0.2% | -0.4% | -0.7% | -0.2% | -0.4% | -0.9% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.3% | -0.2% | -0.1% | -0.2% | -0.4% |
|  | -0.1% | -0.3% | -0.2% | -0.1% | -0.2% | -0.3% |
| Class F | -1.0% | -1.1% | -1.2% | -0.9% | -1.1% | -1.3% |
| Enc Time[%] | 123% |  |  | 116% |  |  |
| Dec Time[%] | 110% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | -0.3% |
| Class C | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% |
| Class D | -0.1% | 0.1% | -0.2% | 0.0% | -0.4% | -0.1% |
| Class E | -0.1% | 0.1% | 0.0% | -0.2% | 0.4% | -0.8% |
| Overall | 0.0% | 0.1% | -0.1% | -0.1% | -0.1% | -0.3% |
|  | 0.0% | 0.1% | -0.1% | -0.1% | -0.1% | -0.4% |
| Class F | -0.5% | -0.6% | -0.8% | -0.6% | -0.6% | -1.3% |
| Enc Time[%] | 117% |  |  | 115% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |

Table 2 below shows the performance of the proposed change against HM8.0 anchor in lossless mode, using the following encoding options,

--TransquantBypassEnableFlag=1 --CUTransquantBypassFlagValue=1

and setting QP to 0 and -12 for main and HE10 configurations, respectively.

Table 2. Rate savings in lossless mode against HM8.0 anchor

|  | All Intra Main | All Intra HE10 |
| --- | --- | --- |
| Class A | -2.1% | -1.1% |
| Class B | -1.3% | -0.8% |
| Class C | -2.0% | -1.2% |
| Class D | -2.3% | -1.2% |
| Class E | -2.0% | -1.1% |
| Overall (w/o F) | -1.9% | -1.1% |
| Overall (w/ F) | -1.9% | -1.1% |
| Class F | -2.0% | -1.1% |
|  | Random Access Main | Random Access HE10 |
| Class A | -0.5% | -0.2% |
| Class B | -0.2% | -0.1% |
| Class C | -0.4% | -0.3% |
| Class D | -0.4% | -0.2% |
| Class E |  |  |
| Overall (w/o F) | -0.4% | -0.2% |
| Overall (w/ F) | -0.5% | -0.3% |
| Class F | -1.0% | -0.7% |
|  | Low delay B Main | Low delay B HE10 |
| Class A |  |  |
| Class B | -0.2% | -0.1% |
| Class C | -0.4% | -0.2% |
| Class D | -0.3% | -0.1% |
| Class E | -0.1% | 0.0% |
| Overall (w/o F) | -0.2% | -0.1% |
| Overall (w/ F) | -0.3% | -0.2% |
| Class F | -0.8% | -0.5% |

# Conclusions

Experimental results in common test conditions against HM8.0 anchor show that rotation of residual blocks when transform is skipped effectively improves the coding efficiency not only for Class F sequences, but also for Classes C, D, and E sequences, in particular in intra settings. A similar change is further proposed for the lossless coding mode in HEVC where improvement in coding efficiency is also observed. It is recommended that these changes are adopted into the HEVC working draft.

# References

[1] C. Lan, J. Xu, G. J. Sullivan, and F. Wu, “Intra transform skipping,” JCTVC-I0408.

[2] X. Peng, C. Lan, J. Xu, and G. J. Sullivan, “Inter transform skipping,” JCVC-J0237.

[3] A. Gabriellini, M. Mrak, D. Flynn, and M. Naccari, “Transform skipping for inter predicted coding units,” JCTVC-J0077.

[4] B. Boss, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. and Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 8,” JCTVC-J1003.

[5] D. He, J. Wang, and G. Martin-Cocher, “Rotation of residual block for transform skipping,” JCTVC-J0093.

[6] F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-J1100.

# Proposed Change to HEVC Working Draft

The change to intra transform skipping in the HEVC working draft is highlighted below.

### Scaling and transformation process

…

The (nW)x(nH) array of residual samples r are derived as specified as follows:

- If cu_transquant_bypass_flag is equal to 1, the (nT)x(nT) array r is set equal to the (nT)x(nT) array of transform coefficients TransCoeffLevel[ xT ][ yT ][ cIdx ].    The residual sample values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are set to TransCoeffLevel[ xT ][ yT ][ cIdx ][ nT-1-x ][ nT-1-y ].

- Otherwise, the following ordered steps apply:

- …

- The (nT)x(nT) array of residual samples r is derived as follows.

- If transform_skip_flag[ xT ][ yT ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are derived as:

r[ nT-1-x ][ nT-1-y ] = ( d[ x ][ y ] << 7 )	(8-1)

- Otherwise (transform_skip_flag[ xT ][ yT ][ cIdx ] is equal to 0), …

- …

# Patent rights declaration(s)

Research In Motion Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).