| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10–19 October 2012 | Document: JCTVC-K-0241 WG11 Number: m26579 |
| --- | --- |

| Title: | Color Gamut Scalable Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Louis Kerofsky, Andrew Segall, Seung-Hwan Kim, and Kiran Misra 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-360-817-87644 lkerofsky@sharplabs.com asegall@sharplabs.com kimse@sharplabs.com kmisra@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This document proposes a scalable extension for HEVC that supports different color gamuts in enhancement and base layers.  Here, the emphasis is on supporting the UHDTV (BT.2020) color gamut in an enhancement layer and the BT.709 color gamut in a base layer.  This is motivated by a need to support both HD and UHDTV devices in the near future.  The document proposes a color gamut prediction tool for converting between BT.709 and BT.2020 with a series of multiplies and adds, while also accounting for a change in bit-depth.

# Color Gamut Scalability

The upcoming deployment of UHDTV devices and content will use a different color gamut than legacy devices.  Specifically, HD uses the BT.709 [1] recommendation, while UHDTV will use the BT.2020 [2] recommendation.  A key difference between these systems is that the color gamut of UHDTV is significantly larger than HD.  It is asserted that will provide a more “life like” viewing experience, which is consistent with other UHDTV characteristics, such as high resolution.

With the scalable extension design beginning within JCTVC, it is important to completely address the requirements of future applications that require scalability.  As captured in previous documents, a primary motivator for the spatial extension work in JCTVC is to support UHDTV spatial resolutions with HD resolution baselayers.  Moreover, the project requirements document capture that an enhancement layer and a baselayer in the developing system should support different color gamuts.  Here, we propose a system to support color gamut scalability to achieve these goals.

The diagram of a general scalable encoder is shown in Figure 1.  The key features illustrated are that the inputs for the base and enhancement layers are represented in different color gamuts, and that the enhancement layer encoder is able to use reconstructed frame from the decoder during encoding the enhancement layer.

Figure 1 - Scalable Encoder

We note that our system completely supports a change in bit-depth between the enhancement and base layer color gamuts as well, which is motivated by the definition of UHDTV as either a 10-bit or 12-bit color gamut, as compared to the common use of 8-bit BT.709 in consumer applications.  Moreover, we want to clarify that “color gamut scalability” differs from the idea of “color format scalability” previously addressed by the video coding community i.e. [3].  This previous work is concerned with the change in the chroma subsampling format, or resolution.

# Background: Comparing BT.709 vs. BT.2020 (UHDTV)

Existing HD content is represented in the BT.709 color space that defines the spatial resolution, RGB primaries and white point, nonlinearity, chroma components and sample bit-depth.  Recently, the ITU has ratified a similar description for higher resolution content, BT.2020.  This content is referred to as UHDTV in the following.

The UHDTV recommendation differs from the BT.709 color space description in several areas, including:

- UHDTV supports 4K and 8K resolution.

- UHDTV has wider primaries but the same white point as BT.709

- UHDTV uses the same nonlinearity as BT.709.

- UHDTV has different definition of Chroma components than BT.709

- UHDTV supports only 10-bit and 12-bit samples and not 8-bit samples as in BT.709.

## Color Gamut differences

The difference in color gamut is shown on the chromaticity diagram in Figure 2.  The wider gamut of UHDTV is seen in the triangle connecting the primaries.  The green and red primaries are particularly expanded as seen in this diagram.  As a remark, we note that if the same fidelity of BT.709 8-bit quantization is desired over the region of overlap of the triangles, the UHDTV components will require quantization to more than 8-bits to reflect the fact that they cover a larger range of values.  This is one motivation for the higher bit-depth of the BT.2020 recommendation.

Figure 2 - Chromaticity of BT.709 and UHDTV

We illustrate the relation between the BT.709 and UHDTV color spaces by taking existing content and mapping into UHDTV using a chain of steps on each pixel:

- Conversion from YCbCr to nonlinear rgb representation using a transform matrix (chroma upsampling)

- Conversion from nonlinear rgb to linear RGB components.

- Conversion from linear RGB to linear XYZ using the definition of primaries and white point of BT.709

- Conversion from linear XYZ to linear RGB using the definition of primaries and white point of UHDTV

- Conversion from linear RGB to nonlinear rgb components.

- Conversion from nonlinear rgb to YCbCr using the definitions of UHDTV followed by chroma subsampling to 4:2:0.

We illustrate the result of this conversion with a sample image pair, Figure 3 and Figure 4.  The nonlinear rgb values of each point of the conversion are shown.  Although the data represents the same pixel value, the images appear different since they are representation in two different color spaces.  The red color for instance looks more saturated in the BT.709 image than in the UHDTV image. This is due to the fact that the UHDTV red primary is farther from the white point than that of BT.709.

Figure 3 - Basketball Drive BT.709

Figure 4 - Basketball Drive UHDTV (shown as BT.709)

## Data analysis

While the conversion between BT.709 and BT.2020 was described above, we consider the possibility to perform the conversion directly in the codeword (non-linear YCbCr) domain instead.  This has the large advantage of reducing the complexity of a conversion process, which is appealing for codec implementation.  To study this aspect, scatter plots of each color channel of UHDTV versus the corresponding channel of BT.709 were produced.  These plots show an affine relationship between the color gamuts, and different models are shown to relate this data.  As a broad statement, the conversion of color space conversion roughly scales the luma channel by 4, the Cb channel by 3.25, and the Cr channel by 2.0.

We summarize the results in Figures 5-7, which show the original data compared to three different prediction models.  A summary of the prediction models is given below:

- Bit-depth difference predictor: for each sample, the UHDTV sample is predicted from the corresponding BT.709 sample value using the bit-depth difference.  In the examples below, this consists of adding two least significant bits corresponding to an 8-bit to 10-bit conversion

- Gain-Offset predictor: for each sample, the UHDTV sample is predicted from the corresponding BT.709 sample using a gain and offset model where the gain and offset are allowed to be different per color channel but are constant for the entire frame.

YUHDTV=g1⋅YBT.709+o1

CbUHDTV=g2⋅CbBT.709+o2

CrUHDTV=g3⋅CrBT.709+o3

- Cross Channel predictor: for each sample, the UHDTV sample is predicted from the BT.709 samples corresponding to the same pixel using a matrix and offset model.  The matrix and offset are different per color channel but constant for the entire picture.  For the case of chroma sub-sampling, the corresponding samples between luma and chroma must be identified.  We advocate a simple approach.  Specifically, for luma prediction, the chroma values correspond to four luma samples.  For chroma prediction, the upper left luma sample of a 2x2 group is used to form the predictor.

YCbCrUHDTV=m11m12m13m21m22m23m31m32m33⋅YCbCrBT.709+o1o2o3

The Gain Offset and Cross Channel predictors contain parameters which can be designed to minimize the MSE between the UHDTV image data and the prediction from BT.709.  In these plots, the parameters were designed to minimize MSE between the UHDTV reference and the predictor values. It is noted that the BD predictor is a special case of the Gain Offset predictor which is a special case of the Cross Channel predictor.  The luma to chroma weights of the Cross Color predictor of often seen nearly zero.  In this proposal, we recommend imposing these values to be zero to eliminate the dependence of Cb and Cr components of UHDTV on the Y component of BT.709 input.

Figure 5 Y-Channel UHDTV vs. BT.709

Figure 6 Cb-Channel UHDTV vs. BT.709

Figure 7 Cr-Channel UHDTV vs. BT.709

The MSE prediction error for each color channel and each predictor is summarized in the plot below.  Note for this image, the bit-depth predictor has a high prediction error.  The deviation between the data and the BD predictor is seen in the plot.  The Cross Color predictor is effective at lowering the MSE of the predictor significantly in the luma channel.

Figure 8 - Preditor MSE comparison

# Color Gamut Scalable Video Coding System

An important element of a scalable encoder is exploiting relation between the base and enhancement layers.  We introduce a color gamut prediction module into the generic scalable encoder presented above to give the color gamut scalable video encoder shown in Figure 9.  The key element is the addition of the Color Space Predictor (or Color Gamut Predictor) function.  We anticipate the use of both default, fixed parameters, as well as parameters that are signaled in the bit-stream, and note that the method can be used with both HEVC and non-HEVC base layer bit-streams.

Figure 9 Color Gamut Scalable Encoder

In the next sections, we provide more information about the Color Gamut Predictor.

## Prediction Details

The Color Gamut Predictor operates on reconstructed base layer pictures to provide a prediction picture to the enhancement layer. The predictor operates on Y, Cb and Cr samples associated with each pixel to produce a prediction image in the enhancement layer color space.  This image may then be upsampled to the enhancement layer resolution.  (Or conversely, it may be upsampled prior to the predictor.)

Here, we consider three candidates for the Color Gamut Predictor.  The first is a simple bit-depth predictor that accounts for any change in bit-depth between layers but not a color gamut change.  The second is a Gain-Offset predictor model, and the third is the Cross-Color predictor.  Each is defined below:

Bit-Depth predictor

Predictor operates on each color channel individually and scales the data to account for any bit-depth difference between the base and enhancement layers.  For 8-bit to 10-bit scalability this is simply a left shift by 2.

SampleUHDTV = SampleBT.709 << 2

Gain-Offset predictor

Slight generalization of the bit-depth predictor allows separate gain and offset values to be specified per color channel.  This is in contrast to the bit-depth predictor that uses a gain of 4 and an offset of zero.  The six parameters {gi ,oi} are specified as integers with a number of fraction bits given by nFractionBits

YUHDTV=(g1⋅YBT.709+o1+(1<<(nFractionBits−1))>>nFractionBits

CbUHDTV=g2⋅CbBT.709+o2+(1<<(nFractionBits−1))>>nFractionBits

CrUHDTV=g3⋅CrBT.709+o3(1<<(nFractionBits−1))>>nFractionBits

Cross-Color predictor

Each sample is computed using as a weighted combination of samples of all color space elements corresponding to the pixel.  The gains and offset may differ per color component but are otherwise constant i.e. not spatially varying.  Note due to 4:2:0 chroma subsampling, the correspondence between luma and chroma samples must be established.  For an output luma sample, the chroma sample associated to the luma sample is taken directly for use in the predictor. This corresponds to upsampling the chroma by sample replication.  For each chroma sample there are four assocaiated luma samples. The upper right luma sample is taken for use in the prediction of chroma values. This corresponds to decimation of the luma to the chroma resolution.  With these understandings, the Cross Color predictor can be described in equations below where the 12 prediction parameters {mij,oi} are specified as integers with a number of fraction bits given by nFractionBits.  As before though, we recommend restricting the luma to chroma weights to be zero.

YUHDTV= (m11⋅YBT.709+m12⋅CbBT.709+m13⋅CrBT.709+o1+(1<<(nFractionBits−1))>>nFractionBits

CbUHDTV= (m21⋅YBT.709+m22⋅CbBT.709+m23⋅CrBT.709+o2+(1<<(nFractionBits−1))>>nFractionBits

CrUHDTV= (m31⋅YBT.709+m32⋅CbBT.709+m33⋅CrBT.709+o3+(1<<(nFractionBits−1))>>nFractionBits

## Prediction Parameter Design

An important issue for the predictor is parameter selection.  Here, we describe the use MSE between the UHDTV data and the predicted data to design the parameters.  This becomes a traditional MSE minimization design which can be solved with conventional methods.  Details are provided below for the Cross-Color prediction parameter design.

Given two frames a base layer picture in BT.709 that may be the result of reconstruction in the base layer encoder or the base layer input for example and an enhancement layer picture in the UHDTV color space, we first assume that the resolutions are the same thought the bit-depths will differ in general.  The prediction design problem is then to minimize the MSE between the enhancement layer picture and the predictor output.  Consider the design of the Y channel predictor.  Let YUHDTV and YBT.709 denote vectors of luma samples and CbBT.709 and CrBT.709 vectors of upsampled chroma samples.  The cross color prediction parameters {m1j,o1} are determined by:

This can be equivalently formulated using a vector of parameters {m11, m12, m13, o1} and an Nx4 matrix M whose columns are the BT.709 samples and a column of 1s where N is the number of luma samples.

Equation 1 Matrix of BT.709 Samples

Equation 2 MSE minimization problem

This is a conventional MSE minimization problem to determine vector x given the matrix M and the vector YUHDTV.  The solution is given by the pseudo inverse:

Equation 3 Y prediction parameters

Recall that the vectors CbBT.709 and CrBT.709 vectors of upsampled chroma samples.  A similar expression holds for the chroma components where the luma data is decimated to the chroma resolution.  The solution is of the same from with the definition of matrix M depending on the decimated luma.

Equation 4 Matrix of BT.709 samples

Equation 5 Cb prediction parameters

Equation 6 Cr prediction parameters

Similar method can be used to design the gain offset only models.

## Signaling Prediction Parameters

The predictor uses a few parameters which could be carried in various portions of the bitstream i.e. SPS, PPS, slice header or lower layers.  The set of parameter we use is described below.  Our current implementation carries these in the slice header.  The parameters used are summarized below.

# Conclusions

A scalable system for support different color gamuts in different layers of a bit-stream is proposed.  The system considers the case of a BT.709 color gamut in the baselayer, and a BT.2020 color gamut in the enhancement layer.  This addresses the issue of efficiently supporting a UHDTV and HD device in the same bit-stream.  Two color gamut predictors are derived, consisting of a gain-offset and cross-channel model, and each supports color gamut with a sequence of multiplies and adds.  This is significantly lower complexity than a full color gamut conversion, which requires mapping a signal to the linear RGB domain and then forward to the new color gamut.

We propose study of the color space prediction tool in a core experiment.

# References

- ITU-R Recommendation BT.709 “Parameter values for the HDTV standards for production and international programme exchange”

- ITU-R Recommendation BT.2020  “Parameter values for UHDTV systems for production and international programme exchange”

- [1] B. Bros, W-J. Han, J-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 8,” JCTVC-J1003_d7, Stockholm, July 2012.

- Wei Dai et al, “Scalability, 3DV, and Higher Chroma Coding,” JCTVC-J0127, Stockholm, July 2012

# Patent rights declaration

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).