| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0245r1 |
| --- | --- |

| Title: | Fix for residual coding of transform skipped blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Jacob Ström Färögatan 6 16480 Stockholm | Tel: Email: | +46 10 7141538 kenneth r andersson @ ericsson com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

This contribution presents fixes in the residual coding of transform-skipped blocks to combat the different nature of the transform skip residual compared to transform coefficients: The addressing of the context for the coding of the significance flags is adjusted, the start-up context model for non-zero coefficient greater than 1 flags is adjusted and the coding of the reference position for the last coefficient position is adjusted. The contribution claims that the average gains for common conditions (excluding class F) is 0.0%, and that the average BDR gain for class F is 0.8%. By also turning on RDOQ for transform skipped blocks, the contribution claims that the gains become 0.1% (excluding class F) and 1.5% for class F.

The contribution also presents an alternative, namely to reverse the scan directions for coding of transform skip blocks. This is same as is done in JCTVC-K0237. Results are reported both for the case when RDOQ is turned off for transform skipped blocks and when it is turned on. For RDOQ turned off, the contribution reports that the gains for common conditions (excluding class F) is 0.1% and that the average BDR gain for class F is 1.0%. If RDOQ is turned on, the gains are reported to be 0.2% for common conditions (excluding class F), and 1.7% for class F.

The contribution also presents a combination of the alternatives, namely to reverse the scan directions for transform-skipped blocks and adjust the start-up context model for non-zero coefficient greater than 1 flags coding. With RDOQ turned off for transform skipped blocks, results are reported to be 0.1% for the common conditions (excluding class F) and that the average BDR gain for class F (only) is 1.3%. With RDOQ turned on for transform skipped blocks, results are reported to be 0.2% for the common conditions (excluding class F) and that the average BDR gain for class F is 2.0%.

The contribution also presents a combination of the alternatives with adjustment for lossless coding, namely to reverse the scan directions for lossless coded blocks (same as in JCTVC-J0237) and the coding of the reference position for the last coefficient position is adjusted. Results are reported for the common conditions (excluding class F) to be 0.9% and that the average BDR gain for class F is 1.1%. Corresponding gains in JCTVC-K0237 are 0.6% (excluding class F) and 1.0% for class F.

The contribution also presents an encoder-only modification to turn on RDOQ for transform skip. The gains are reported to be 0.1% compared to common conditions (excluding class F) and the average BDR gain for class F is reported to be 0.6%.

# Background

Transform skip for intra [3] was adopted at the second last meeting and for inter [4] at the last meeting.

The transform coefficients are typically larger in the top left corner than in the bottom right. This is exploited in the coefficient coding in HEVC, see Figure 1.

Figure 1 Left: Block after transform and quantization. Right: Diagonal scanning order.

However when the transform is skipped it is the quantized residual that is coded and the characteristic of that in intra prediction is that the residual typically is larger further away from the reference samples, see Figure 2. In inter prediction the residual can appear generally anywhere in the transform skipped block.

Figure 2 Block after quantization of residual from intra prediction. Note that now the large coefficients will be in the beginning of the scan.

At last meeting there were two proposals to have reverse scan orders for residual coding of intra transform skipped blocks [5][6].  In this proposal an alternative 1 solution is presented which keeps the same scan orders as in HM8.0 but instead adjusts the residual coding when the block to be coded use transform skip. An alternative 2 solution is also presented, same as proposed for intra in [5][6] that here have reverse scan orders for both intra and inter transform skipped blocks.

# Alternative 1

Proposed changes to the HEVC text is shown in section 9 (high-lighted in yellow).

## Adjustment of coding of last nonzero coefficient position

The last significant coefficient position describe where the first nonzero position is in the scan order and the decoder can thus start decoding coefficients from this position and towards the end of the scan.  It is encoded with respect to origin in the top left of the block. Coefficients after the last significant coefficient are zero. When transform is used the coefficients are typically distributed with bias towards the top left of the block as shown in Figure 1 and thus the last significant coefficient position for transform blocks will typically have a relatively small value. When transform skip is used the larger coefficients will typically occur in the beginning of the scan as illustrated in Figure 2 and thus the last significant coefficient position will typically have a large value.

To avoid distorting the context model for coding of last nonzero coefficient positions with typically large last nonzero positions when transform skip is used and to reduce the overhead for coding last coefficient position when transform skip is used it is proposed to adjust the coding of the last coefficient position so that it references the bottom right corner of the block instead of the top left corner when transform skip is used.

Decoder software change (high-lighted in yellow):

UInt uiBlkPosLast;

if(isTransformSkip)

{

uiBlkPosLast      = 15 - (uiPosLastX + (uiPosLastY<<uiLog2BlockSize));

}

else

{

uiBlkPosLast      = uiPosLastX + (uiPosLastY<<uiLog2BlockSize);

}

## Adjustment of context addressing for coding of significant_coeff_flag’s

With respect to Figure 1 one can see that the significant coefficient flags for transform blocks with high probability to be nonzero are in the end of the scan and low probability to be nonzero in the beginning of the scan. When transform skip is used the larger coefficients will typically be in the beginning of the scan and the small values in the end of the scan as illustrated in Figure 2.

To avoid distorting the context for coding of significance coefficient flags when transform skip is used and reduce the overhead for coding of significance flags when transform skip is used it is proposed to use the context for the scan position (3- uiPosX,3- uiPosY) instead of (uiPosX, uiPosY) when transform skip is used.

Decoder software change (high-lighted in yellow):

if(isTransformSkip)

uiCtxSig  = TComTrQuant::getSigCtxInc( patternSigCtx, uiScanIdx, 3-uiPosX, 3-uiPosY, blockType, uiWidth, uiHeight, eTType );

else

uiCtxSig  = TComTrQuant::getSigCtxInc( patternSigCtx, uiScanIdx, uiPosX, uiPosY, blockType, uiWidth, uiHeight, eTType );

## Adjustment of context initialization for coding of coeff_abs_level_greater1_flag’s

The coeff_abs_level_greater1_flag’s are determined for up to the first eight nonzero coefficients in the scan. Each flag indicates if a corresponding coefficient has a magnitude larger that 1. First flag is decoded with one context (greater1Ctx=1). If a previous flag is one all remaining flags will be coded with a context with high probability of being larger than 1 (greater1Ctx=0). Again due to the nature of transform coefficients the first flag will have a relatively high probability of being equal or smaller than one. Transform skip blocks on the other hand will typically have large values in the beginning and thus distorting the context of the first flag in the scan. To avoid distorting the context of the first flag and to reduce the overhead for coding of it the proposal is to initialize the greater1Ctx to 0 when transform skip is used.

Decoder software change (high-lighted in yellow):

if(isTransformSkip&&numNonZero)

{

c1 = 0;

}

else

{

c1 = 1;

}

ContextModel *baseCtxMod = ( eTType==TEXT_LUMA ) ? m_cCUOneSCModel.get( 0, 0 ) + 4 * uiCtxSet : m_cCUOneSCModel.get( 0, 0 ) + NUM_ONE_FLAG_CTX_LUMA + 4 * uiCtxSet;

for ( Int i = 0; i < numNonZero; i++) absCoeff[i] = 1;

Int numC1Flag = min(numNonZero, C1FLAG_NUMBER);

Int firstC2FlagIdx = -1;

for( Int idx = 0; idx < numC1Flag; idx++ )

{

m_pcTDecBinIf->decodeBin( uiBin, baseCtxMod[c1] );

if( uiBin == 1 )

{

c1 = 0;

if (firstC2FlagIdx == -1)

{

firstC2FlagIdx = idx;

}

}

else if( (c1 < 3) && (c1 > 0) )

{

c1++;

}

absCoeff[ idx ] = uiBin + 1;

}

## Results

Encoding timings are from a linux cluster.

Table 1Anchor HM8.0 versus Alternative 1 with RDOQ turned on for transform skipped blocks.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class C | -0,1% | -0,1% | -0,1% | -0,1% | -0,1% | -0,1% |
| Class D | -0,1% | 0,0% | -0,1% | -0,1% | 0,0% | -0,1% |
| Class E | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class F | -1,6% | -1,7% | -2,1% | -1,6% | -1,7% | -2,2% |
| Enc Time[%] | 105% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,2% | -0,2% | 0,0% | 0,1% | -0,1% |
| Class B | 0,0% | 0,1% | 0,2% | 0,0% | 0,1% | 0,2% |
| Class C | -0,2% | -0,2% | 0,0% | -0,2% | -0,3% | -0,4% |
| Class D | -0,2% | -0,3% | -0,3% | -0,3% | -0,2% | -0,4% |
| Class E |  |  |  |  |  |  |
| Overall | -0,1% | -0,1% | -0,1% | -0,1% | 0,0% | -0,2% |
|  | -0,1% | -0,2% | -0,1% | -0,1% | -0,1% | -0,2% |
| Class F | -1,6% | -1,5% | -2,1% | -1,5% | -1,3% | -2,2% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,1% | 0,2% | 0,0% | 0,0% | 0,3% |
| Class C | -0,1% | 0,0% | 0,1% | -0,2% | 0,3% | 0,2% |
| Class D | -0,3% | 0,0% | -0,4% | -0,2% | -0,4% | 0,1% |
| Class E | 0,1% | -0,3% | 0,1% | 0,1% | 0,9% | -1,2% |
| Overall | -0,1% | 0,0% | 0,0% | -0,1% | 0,1% | -0,1% |
|  | -0,1% | 0,0% | -0,1% | -0,1% | 0,1% | -0,1% |
| Class F | -1,3% | -0,9% | -1,2% | -1,5% | -0,7% | -3,1% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Y | U | V |
| --- | --- | --- | --- |
| Avg exl classF | -0,1% | 0,0% | -0,1% |
| Avg class F | -1,5% | -1,3% | -2,2% |

Below both encoding and decoding time is from linux cluster.

Table 2Anchor HM8.0 versus Alternative 1 with RDOQ turned off for transform skipped blocks.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class C | -0,1% | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class F | -1,1% | -1,0% | -1,5% | -1,1% | -1,0% | -1,5% |
| Enc Time[%] | 98% |  |  | 96% |  |  |
| Dec Time[%] | 106% |  |  | 102% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,2% | 0,1% |
| Class B | 0,0% | 0,0% | 0,1% | 0,0% | 0,1% | 0,1% |
| Class C | 0,0% | 0,0% | 0,0% | -0,1% | -0,1% | -0,2% |
| Class D | 0,0% | 0,0% | -0,2% | -0,1% | 0,0% | -0,3% |
| Class E |  |  |  |  |  |  |
| Overall | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% |
|  | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | -0,1% |
| Class F | -0,8% | -0,6% | -1,1% | -0,7% | -0,4% | -1,2% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 97% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,2% | 0,0% | 0,0% | 0,0% | 0,3% |
| Class C | 0,0% | 0,1% | 0,2% | -0,1% | 0,1% | 0,5% |
| Class D | -0,1% | 1,1% | -0,6% | 0,0% | -0,5% | -0,2% |
| Class E | 0,0% | -0,1% | -0,1% | 0,0% | 0,6% | -0,5% |
| Overall | 0,0% | 0,3% | -0,1% | 0,0% | 0,0% | 0,1% |
|  | 0,0% | 0,3% | -0,1% | 0,0% | 0,0% | 0,0% |
| Class F | -0,6% | 0,0% | -1,1% | -0,7% | -0,1% | -1,8% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |

|  | Y | U | V |
| --- | --- | --- | --- |
| Avg exl classF | 0,0% | 0,1% | 0,0% |
| Avg class F | -0,8% | -0,5% | -1,4% |

# Alternative 2

Reverse scans for both intra and inter transform skipped blocks. Proposed changes to the HEVC text are shown in section 10. Below encoding timings are from a linux cluster.

Table 3Anchor HM8.0 versus Alternative 2 with RDOQ turned on for transform skipped blocks.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | -0,1% | -0,1% | 0,0% | -0,1% | -0,1% |
| Class C | -0,3% | -0,3% | -0,3% | -0,3% | -0,3% | -0,4% |
| Class D | -0,4% | -0,4% | -0,6% | -0,4% | -0,5% | -0,6% |
| Class E | -0,2% | -0,1% | -0,1% | -0,2% | -0,1% | -0,1% |
| Overall | -0,2% | -0,2% | -0,2% | -0,2% | -0,2% | -0,2% |
|  | -0,2% | -0,2% | -0,2% | -0,2% | -0,2% | -0,2% |
| Class F | -1,9% | -2,2% | -2,4% | -1,9% | -2,3% | -2,4% |
| Enc Time[%] | 97% |  |  | 79% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,4% | -0,3% | 0,0% | -0,1% | -0,2% |
| Class B | 0,0% | -0,1% | 0,1% | 0,0% | 0,0% | -0,1% |
| Class C | -0,3% | -0,5% | -0,4% | -0,3% | -0,4% | -0,6% |
| Class D | -0,4% | -0,7% | -0,7% | -0,5% | -0,8% | -0,9% |
| Class E |  |  |  |  |  |  |
| Overall | -0,2% | -0,4% | -0,3% | -0,2% | -0,3% | -0,4% |
|  | -0,2% | -0,4% | -0,3% | -0,2% | -0,4% | -0,4% |
| Class F | -1,8% | -2,2% | -2,3% | -1,8% | -2,0% | -2,3% |
| Enc Time[%] | 95% |  |  | 84% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,2% | 0,1% | 0,0% | -0,3% | 0,1% |
| Class C | -0,2% | -0,3% | -0,2% | -0,2% | 0,0% | 0,2% |
| Class D | -0,3% | -0,2% | -0,8% | -0,2% | -0,7% | -0,3% |
| Class E | -0,1% | 0,3% | 0,2% | -0,3% | 0,1% | -0,6% |
| Overall | -0,1% | 0,0% | -0,2% | -0,1% | -0,3% | -0,1% |
|  | -0,1% | 0,0% | -0,2% | -0,1% | -0,3% | -0,1% |
| Class F | -1,3% | -1,3% | -1,6% | -1,4% | -1,2% | -1,5% |
| Enc Time[%] | 96% |  |  | 82% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Y | U | V |
| --- | --- | --- | --- |
| Avg exl classF | -0,2% | -0,2% | -0,2% |
| Avg class F | -1,7% | -1,8% | -2,1% |

Below both encoding and decoding timings are from a linux cluster.

Table 4 Anchor HM8.0 versus Alternative 2 with RDOQ turned off for transform skipped blocks

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% | 0,0% |
| Class C | -0,3% | -0,2% | -0,3% | -0,2% | -0,2% | -0,3% |
| Class D | -0,4% | -0,4% | -0,5% | -0,3% | -0,3% | -0,4% |
| Class E | -0,1% | 0,0% | 0,0% | -0,1% | 0,0% | -0,1% |
| Overall | -0,2% | -0,1% | -0,2% | -0,2% | -0,1% | -0,1% |
|  | -0,2% | -0,1% | -0,2% | -0,2% | -0,1% | -0,2% |
| Class F | -1,4% | -1,5% | -1,7% | -1,4% | -1,6% | -1,8% |
| Enc Time[%] | 97% |  |  | 92% |  |  |
| Dec Time[%] | 102% |  |  | 85% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,3% | 0,0% | -0,1% | 0,0% | -0,3% |
| Class B | 0,0% | -0,2% | 0,1% | 0,0% | -0,1% | 0,1% |
| Class C | -0,1% | -0,3% | -0,1% | -0,2% | -0,3% | -0,6% |
| Class D | -0,2% | -0,4% | -0,7% | -0,2% | -0,4% | -0,9% |
| Class E |  |  |  |  |  |  |
| Overall | -0,1% | -0,3% | -0,2% | -0,1% | -0,2% | -0,4% |
|  | -0,1% | -0,3% | -0,2% | -0,1% | -0,2% | -0,3% |
| Class F | -1,0% | -1,1% | -1,2% | -0,9% | -1,1% | -1,3% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 88% |  |  | 86% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,2% | 0,0% | 0,0% | -0,1% | -0,3% |
| Class C | 0,0% | -0,1% | -0,1% | -0,1% | -0,2% | -0,2% |
| Class D | -0,1% | 0,1% | -0,2% | 0,0% | -0,4% | -0,1% |
| Class E | -0,1% | 0,1% | 0,0% | -0,2% | 0,4% | -0,8% |
| Overall | 0,0% | 0,1% | -0,1% | -0,1% | -0,1% | -0,3% |
|  | 0,0% | 0,1% | -0,1% | -0,1% | -0,1% | -0,4% |
| Class F | -0,5% | -0,6% | -0,8% | -0,6% | -0,6% | -1,3% |
| Enc Time[%] | 98% |  |  | 94% |  |  |
| Dec Time[%] | 91% |  |  | 87% |  |  |

|  | Y | U | V |
| --- | --- | --- | --- |
| Avg exl classF | -0,1% | -0,1% | -0,2% |
| Avg class F | -1,0% | -1,1% | -1,3% |

# Combination - Alternative 3

As was seen in the previous alternative, reversing the scan order for transform-skipped blocks helps coding efficiency, see also in [7], since we are changing from a scan order as shown in Figure 2, with big coefficients in the beginning, to a scan order more like the one in Figure 1, with small coefficients in the beginning. However, even after reversal, the coefficients resulting from a transform skipped block differ from those resulting from a regular, transformed block: A transformed block will have high energy compaction, meaning that the coefficients in the beginning of the scan order will be quite small. For a transform skipped block, although the coefficients in scan order will still be sorted from small to big, the first coefficients will still be relatively large. Therefore, it makes more sense to initialize the context coeff_abs_level_greater1_flag to 0 as described in Section 2.3.

Hence, in alternative 3, we combine alternative 2 with adjustment of context initialization for coding of coeff_abs_level_greater1_flag’s as in section 2.3. Proposed changes to the HEVC text are shown in section 11.

Below we show the BDR performance versus HM-8.0. Both encoding and decoding timings are from a linux cluster.

Table 5 Anchor HM8.0 versus combination with RDOQ turned on for transform skipped blocks

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | -0,1% | -0,1% | 0,0% | -0,1% | -0,1% |
| Class C | -0,3% | -0,3% | -0,4% | -0,3% | -0,3% | -0,4% |
| Class D | -0,4% | -0,5% | -0,6% | -0,4% | -0,5% | -0,5% |
| Class E | -0,2% | -0,1% | -0,1% | -0,2% | -0,1% | -0,1% |
| Overall | -0,2% | -0,2% | -0,2% | -0,2% | -0,2% | -0,2% |
|  | -0,2% | -0,2% | -0,2% | -0,2% | -0,2% | -0,2% |
| Class F | -2,4% | -2,4% | -2,8% | -2,4% | -2,5% | -2,9% |
| Enc Time[%] | 82% |  |  | 80% |  |  |
| Dec Time[%] | 69% |  |  | 70% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,3% | -0,3% | 0,0% | 0,1% | -0,2% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | 0,0% |
| Class C | -0,3% | -0,3% | -0,3% | -0,3% | -0,3% | -0,6% |
| Class D | -0,4% | -0,6% | -0,4% | -0,5% | -0,7% | -1,0% |
| Class E |  |  |  |  |  |  |
| Overall | -0,2% | -0,3% | -0,2% | -0,2% | -0,2% | -0,4% |
|  | -0,2% | -0,3% | -0,3% | -0,2% | -0,2% | -0,4% |
| Class F | -2,0% | -1,8% | -2,3% | -2,0% | -1,9% | -2,4% |
| Enc Time[%] | 83% |  |  | 84% |  |  |
| Dec Time[%] | 74% |  |  | 73% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,1% | 0,1% | 0,0% | 0,0% | 0,2% |
| Class C | -0,1% | -0,2% | 0,1% | -0,2% | -0,1% | 0,0% |
| Class D | -0,3% | 0,6% | -0,6% | -0,2% | -0,9% | -0,4% |
| Class E | 0,0% | 0,3% | 1,1% | 0,0% | -0,8% | -1,0% |
| Overall | -0,1% | 0,2% | 0,1% | -0,1% | -0,4% | -0,2% |
|  | -0,1% | 0,2% | 0,0% | -0,1% | -0,3% | -0,3% |
| Class F | -1,4% | -1,4% | -2,0% | -1,6% | -1,1% | -2,9% |
| Enc Time[%] | 81% |  |  | 83% |  |  |
| Dec Time[%] | 70% |  |  | 74% |  |  |

|  | Y | U | V |
| --- | --- | --- | --- |
| Avg exl classF | -0,2% | -0,2% | -0,2% |
| Avg class F | -2,0% | -1,8% | -2,6% |

Table 6 Anchor HM8.0 versus combination with RDOQ turned off for transform skipped blocks

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% | 0,0% |
| Class C | -0,3% | -0,2% | -0,3% | -0,3% | -0,2% | -0,3% |
| Class D | -0,3% | -0,3% | -0,4% | -0,4% | -0,3% | -0,4% |
| Class E | -0,1% | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% |
| Overall | -0,2% | -0,1% | -0,2% | -0,2% | -0,1% | -0,2% |
|  | -0,2% | -0,1% | -0,2% | -0,2% | -0,1% | -0,2% |
| Class F | -1,8% | -1,7% | -2,0% | -1,8% | -1,7% | -2,1% |
| Enc Time[%] | 94% |  |  | 85% |  |  |
| Dec Time[%] | 94% |  |  | 79% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,1% | -0,2% | -0,1% | 0,2% | -0,1% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class C | -0,1% | -0,2% | 0,0% | -0,1% | -0,2% | -0,3% |
| Class D | -0,2% | -0,5% | -0,4% | -0,2% | -0,5% | -0,6% |
| Class E |  |  |  |  |  |  |
| Overall | -0,1% | -0,2% | -0,1% | -0,1% | -0,1% | -0,2% |
|  | -0,1% | -0,2% | -0,2% | -0,1% | -0,1% | -0,2% |
| Class F | -1,2% | -0,9% | -1,2% | -1,2% | -0,9% | -1,3% |
| Enc Time[%] | 92% |  |  | 89% |  |  |
| Dec Time[%] | 90% |  |  | 78% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,0% | 0,1% | 0,0% | -0,2% | 0,4% |
| Class C | 0,0% | 0,1% | -0,1% | -0,1% | -0,1% | 0,2% |
| Class D | 0,0% | 0,6% | 0,2% | 0,0% | 0,1% | -0,9% |
| Class E | 0,0% | -0,1% | 0,7% | -0,2% | -0,1% | 0,3% |
| Overall | 0,0% | 0,2% | 0,2% | 0,0% | -0,1% | 0,0% |
|  | 0,0% | 0,1% | 0,1% | 0,0% | -0,1% | 0,0% |
| Class F | -0,7% | -0,6% | -0,9% | -0,8% | -0,8% | -1,5% |
| Enc Time[%] | 93% |  |  | 87% |  |  |
| Dec Time[%] | 87% |  |  | 83% |  |  |

|  | Y | U | V |
| --- | --- | --- | --- |
| Avg exl classF | -0,1% | -0,1% | -0,1% |
| Avg class F | -1,3% | -1,1% | -1,5% |

# Combination – Alternative 3 extended for lossless coding

Since losslessly coded blocks are not undergoing transform, it makes sense to reverse the scan order when coding them, just as is done for transform-skipped blocks in [7]. This makes the scan order more similar to the one in Figure 1 (with coefficients typically sorted from small to large) and hence better suited to the subsequent coefficient coding. However, the reversal of the scan order does not help the encoding of the last non-zero coefficient position, uiBlkPosLast. The reason for this is the fact that, in lossless mode, so many of the coefficients are non-zero. Assume for example that all coefficients in the block are nonzero. The last non-zero coefficient will therefore be in the bottom right corner. Since the current HEVC draft uses the top-left corner as the reference point, uiBlkPosLast will then be a large number, e.g., 15 if we have a 4x4 block. Reversing the scan order as in alternative 2 above will not help – even after the rotation, the last non-zero coefficient will still be 15 (since all coefficients are non-zero).

Therefore, in lossless coding, we propose to reverse the scan direction and also encode uiBlkPosLast with reference to the bottom right of the residual block instead of the top left. Proposed changes to the HEVC text are shown in section 12.

Decoder software change (high-lighted in yellow):

if(isTransquantBypass)

uiBlkPosLast      = uiWidth*uiHeight -1 - (uiPosLastX + (uiPosLastY<<uiLog2BlockSize));

else

uiBlkPosLast      = uiPosLastX + (uiPosLastY<<uiLog2BlockSize);

Void TComTrQuant::invtransformNxN( Bool transQuantBypass, TextType eText, UInt uiMode,Pel* rpcResidual, UInt uiStride, TCoeff*   pcCoeff, UInt uiWidth, UInt uiHeight,  Int scalingListType, Bool useTransformSkip )

{

if(transQuantBypass)

{

for (UInt k = 0; k<uiHeight; k++)

{

for (UInt j = 0; j<uiWidth; j++)

{

rpcResidual[k * uiStride + j] =  pcCoeff[(uiHeight-1-k)*uiWidth+(uiWidth-1-j)];

}

}

return;

}

Below we show the BDR performance versus HM-8.0 lossless coding. Both encoding and decoding timings are from a linux cluster.

Table 7 HM-8.0 lossless versus Alternative 3 lossless

|  | All Intra Main | All Intra HE10 |
| --- | --- | --- |
| Class A | -2,2% | -1,4% |
| Class B | -1,5% | -1,1% |
| Class C | -2,1% | -1,4% |
| Class D | -2,3% | -1,4% |
| Class E | -2,2% | -1,3% |
| Overall (w/o F) | -2,0% | -1,3% |
| Overall (w/ F) | -2,0% | -1,3% |
| Class F | -2,0% | -1,2% |
|  | Random Access Main | Random Access HE10 |
| Class A | -0,8% | -0,4% |
| Class B | -0,6% | -0,3% |
| Class C | -0,7% | -0,5% |
| Class D | -0,6% | -0,4% |
| Class E |  |  |
| Overall (w/o F) | -0,7% | -0,4% |
| Overall (w/ F) | -0,8% | -0,5% |
| Class F | -1,2% | -0,8% |
|  | Low delay B Main | Low delay B HE10 |
| Class A |  |  |
| Class B | -0,5% | -0,3% |
| Class C | -0,7% | -0,4% |
| Class D | -0,5% | -0,3% |
| Class E | -0,5% | -0,2% |
| Overall (w/o F) | -0,6% | -0,3% |
| Overall (w/ F) | -0,6% | -0,4% |
| Class F | -1,0% | -0,6% |

| Avg excl ClassF | -0,9% |
| --- | --- |
| Avg ClassF | -1.1% |

# Encoder only modification to turn on RDOQ for transform skip

RDOQ is a useful tool for bitrate reduction in residual coding but is not used for transform skip in HM-8.0. Here we show the effect of turning on RDOQ for transform skip. Below both encoding and decoding time is from linux cluster.

Table 8Anchor HM8.0 versus RDOQ turned on for transform skipped blocks

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class C | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class F | -0,4% | -0,5% | -0,6% | -0,3% | -0,5% | -0,5% |
| Enc Time[%] | 95% |  |  | 81% |  |  |
| Dec Time[%] | 89% |  |  | 70% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | -0,1% | -0,1% | 0,0% | -0,1% | -0,4% |
| Class B | 0,0% | 0,0% | 0,1% | 0,0% | 0,0% | 0,1% |
| Class C | -0,1% | -0,1% | 0,0% | -0,1% | 0,0% | -0,1% |
| Class D | -0,2% | 0,0% | -0,1% | -0,2% | 0,0% | 0,0% |
| Class E |  |  |  |  |  |  |
| Overall | 0,0% | -0,1% | 0,0% | -0,1% | 0,0% | -0,1% |
|  | 0,0% | -0,1% | 0,0% | -0,1% | 0,0% | -0,1% |
| Class F | -0,8% | -0,9% | -0,9% | -0,7% | -0,7% | -0,8% |
| Enc Time[%] | 93% |  |  | 84% |  |  |
| Dec Time[%] | 84% |  |  | 72% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,1% | 0,1% | -0,3% | 0,0% | 0,0% | 0,1% |
| Class C | -0,1% | 0,0% | 0,1% | -0,2% | 0,1% | 0,2% |
| Class D | -0,3% | -0,2% | -0,5% | -0,2% | -0,5% | -0,6% |
| Class E | 0,0% | 0,1% | 0,9% | -0,1% | -0,2% | 0,2% |
| Overall | -0,1% | 0,0% | 0,0% | -0,1% | -0,1% | 0,0% |
|  | -0,1% | 0,0% | -0,1% | -0,1% | -0,2% | -0,1% |
| Class F | -0,7% | -0,4% | -0,9% | -0,9% | 0,2% | -2,1% |
| Enc Time[%] | 94% |  |  | 82% |  |  |
| Dec Time[%] | 89% |  |  | 73% |  |  |

|  | Y | U | V |
| --- | --- | --- | --- |
| Avg exl classF | -0,1% | 0,0% | 0,0% |
| Avg class F | -0,6% | -0,5% | -1,0% |

# Conclusions

The contribution has shown two alternatives to fix the coding of the residual of transform skipped blocks given that the typical appearance of the residuals is different from transform coded residual. Both approaches improve coding efficiency of class F sequences where transform skip is used frequently while keeping decoding and encoding time unchanged.

Alternative 1 shows that it is possible without reversing the scan directions as in alternative 2. Instead, in alternative 1 only minor fixes of the residual coding are necessary.

We also show a combination of alternative 2 with adjustment of the start-up context model for coding of greater than 1 flags which gives the best coding efficiency.

We also show an extension for lossless coding with reversing the scan directions and with adjustment of the last coefficient coding which gives gains for all classes and conditions compared to HM-8.0 lossless coding.

It is therefore proposed to adopt one of the alternatives to have a residual coding design that also manages transform skip residuals and lossless coded residuals adequately and to turn on RDOQ for transform skipped blocks.

# References

- B. Bross, W-J. Han et al, “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-J1003_d8, Stockholm, Sweden, July 2012

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-J1100, Stockholm, Sweden, July 2012

- C. Lan, J. Xu, G. Sullivan, F. Wu, “Intra transform skipping”, JCTVC-I0408, Geneva, Switzerland, April 2012

- X. Peng, C. Lan, J. Xu, G. Sullivan,  “Inter transform skipping”, JCTVC-J0207, Geneva, Switzerland, April 2012

- J. An, L. Zhao, Y. Huang, S. Lei, “Residue scan for intra transform skip mode”, JCTVC-J0053, Stockholm, Sweden, July 2012

- D. He, J. Wang, G. Martin-Cocher ,“Rotation of Residual Block for Transform Skipping”, JCTVC-J0093, Stockholm, Sweden, July 2012

- D. He, J. Wang, G. Martin-Cocher , “Rotation of Residual block for Transform Skipping in HM8”, JCTVC-K0237, Shanghai, China, October 2012

# Proposed text modifications alternative 1

last_significant_coeff_x_suffix specifies the suffix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_x_suffix shall be in the range from 0 to ( 1 << ( ( last_significant_coeff_x_prefix >> 1 ) − 1 ) ) − 1, inclusive.

The column position of the last significant coefficient in scanning order within a transform block LastSignificantCoeffX is derived as follows where ( x0, y0 ) specifies the transform block location and cIdx specifies an indicator for the colour component.

- If last_significant_coeff_x_suffix is not present, the following applies.

If transform_skip_flag[x0][y0][cIdx] is equal to 1

LastSignificantCoeffX  =  3−last_significant_coeff_x_prefix	(7-54)

Otherwise

LastSignificantCoeffX  =  last_significant_coeff_x_prefix	(7-54)

- Otherwise (last_significant_coeff_x_suffix is present), the following applies.

LastSignificantCoeffX  =  (1 << ((last_significant_coeff_x_prefix >> 1) − 1))  *  	
				(2 + (last_significant_coeff_x_prefix & 1))  +  	(7-55)
				last_significant_coeff_x_suffix

last_significant_coeff_y_suffix specifies the suffix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_y_suffix shall be in the range from 0 to ( 1 << (  ( last_significant_coeff_y_prefix >> 1 ) − 1 ) ) − 1, inclusive.

The row position of the last significant coefficient in scanning order within a transform block LastSignificantCoeffY is derived as follows where ( x0, y0 ) specifies the transform block location  and cIdx specifies an indicator for the colour component.

- If last_significant_coeff_y_suffix is not present, the following applies.

If transform_skip_flag[x0][y0][cIdx] is equal to 1

LastSignificantCoeffY  =  3−last_significant_coeff_y_prefix	(7-56)

Otherwise

LastSignificantCoeffY  =  last_significant_coeff_y_prefix	(7-56)

- Otherwise (last_significant_coeff_y_suffix is present), the following applies.

LastSignificantCoeffY  =  (1 << ((last_significant_coeff_y_prefix >> 1) − 1))  *  	
				(2 + (last_significant_coeff_y_prefix & 1))  +  	(7-57) 
				last_significant_coeff_y_suffix

When scanIdx is equal to 2, the coordinates are swapped as follows.

( LastSignificantCoeffX, LastSignificantCoeffY ) = Swap( LastSignificantCoeffX, LastSignificantCoeffY )	(7-58)

##### 9.3.3.1.4 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC, yC ), the scan order index scanIdx, the transform block size log2TrafoSize, the transform block location ( x0, y0 ) and the colour component index cIdx.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element coded_sub_block_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoSize is equal to 2, sigCtx is derived using ctxIdxMap[ ] specified in Table 9-39 as follows..

If transform_skip_flag[x0][y0][cIdx] is equal to 1

sigCtx  =  ctxIdxMap[ ((3−yC) << 2) + 3−xC ]	(9-15)

Otherwise

sigCtx  =  ctxIdxMap[ (yC << 2) + xC ]	(9-15)

##### 9.3.3.1.5 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag

Inputs to this process are the colour component index cIdx, the transform block position ( x0, y0 ), the current sub-block scan index i and the current coefficient scan index n within the current sub-block.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and for its derivation the following applies.

- If this process is invoked for the first time for the current sub-block scan index i, the following applies.

- The variable ctxSet is initialized as follows.

- If the current sub-block scan index i is equal to 0 or cIdx is greater than 0, the following applies.

ctxSet  =  0		(9-30)

- Otherwise (i is greater than 0 and cIdx is equal to 0), the following applies.

ctxSet  =  2		(9-31)

- The variable lastGreater1Ctx is derived as follows.

- If the current sub-block with scan index i is the first one to be processed in this subclause for the current transform block, the variable lastGreater1Ctx is set equal to 0.

- Otherwise, the variable lastGreater1Ctx is set equal to the value of greater1Ctx that has been derived during the last invocation of the process specified in this subclause for the syntax element coeff_abs_greater1_flag for the previous sub-block with scan index i + 1.

- When lastGreater1Ctx is equal to 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet  +  1		(9-32)

- The variable greater1Ctx is initialized as follows.

  - If transform_skip_flag[x0][y0][cIdx] is equal to 1

The variable greater1Ctx is set equal to 0.

  - Otherwise

The variable greater1Ctx is set equal to 1.

# Proposed text modifications alternative 2

The (nT)x(nT) array of residual samples r is derived as specified as follows:

- If cu_transquant_bypass_flag is equal to 1, the (nT)x(nT) array r is set equal to the (nT)x(nT) array of transform coefficients TransCoeffLevel[ xT ][ yT ][ cIdx ].

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause 8.6.3 is invoked with the transform block location ( xT, yT ), the size of the transform block nT, the (nT)x(nT) array of transform coefficients, the chroma component variable cIdx and the quantization parameter qP as the inputs and the output is a (nT)x(nT) array of scaled transform coefficients d.

- The (nT)x(nT) array of residual samples r is derived as follows.

- If transform_skip_flag[ xT ][ yT ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are derived as:

r[ x ][ y ] = ( d[nT-1-x ][ nT-1-y ] << 7 )	(8-243)

# Proposed text modifications alternative 3

The (nT)x(nT) array of residual samples r is derived as specified as follows:

- If cu_transquant_bypass_flag is equal to 1, the (nT)x(nT) array r is set equal to the (nT)x(nT) array of transform coefficients TransCoeffLevel[ xT ][ yT ][ cIdx ].

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause 8.6.3 is invoked with the transform block location ( xT, yT ), the size of the transform block nT, the (nT)x(nT) array of transform coefficients, the chroma component variable cIdx and the quantization parameter qP as the inputs and the output is a (nT)x(nT) array of scaled transform coefficients d.

- The (nT)x(nT) array of residual samples r is derived as follows.

- If transform_skip_flag[ xT ][ yT ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are derived as:

r[ x ][ y ] = ( d[nT-1-x ][ nT-1-y ] << 7 )	(8-243)

##### 9.3.3.1.5 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag

Inputs to this process are the colour component index cIdx, the transform block position ( x0, y0 ), the current sub-block scan index i and the current coefficient scan index n within the current sub-block.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and for its derivation the following applies.

- If this process is invoked for the first time for the current sub-block scan index i, the following applies.

- The variable ctxSet is initialized as follows.

- If the current sub-block scan index i is equal to 0 or cIdx is greater than 0, the following applies.

ctxSet  =  0		(9-30)

- Otherwise (i is greater than 0 and cIdx is equal to 0), the following applies.

ctxSet  =  2		(9-31)

- The variable lastGreater1Ctx is derived as follows.

- If the current sub-block with scan index i is the first one to be processed in this subclause for the current transform block, the variable lastGreater1Ctx is set equal to 0.

- Otherwise, the variable lastGreater1Ctx is set equal to the value of greater1Ctx that has been derived during the last invocation of the process specified in this subclause for the syntax element coeff_abs_greater1_flag for the previous sub-block with scan index i + 1.

- When lastGreater1Ctx is equal to 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet  +  1		(9-32)

- The variable greater1Ctx is initialized as follows.

  - If transform_skip_flag[x0][y0][cIdx] is equal to 1

The variable greater1Ctx is set equal to 0.

  - Otherwise

The variable greater1Ctx is set equal to 1.

# Proposed text modifications alternative 3 lossless

The (nT)x(nT) array of residual samples r is derived as specified as follows:

- If cu_transquant_bypass_flag is equal to 1, the (nT)x(nT) array r is set equal to the (nT)x(nT) array of transform coefficients TransCoeffLevel[ xT ][ yT ][ cIdx ]. The residual sample values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are set to TransCoeffLevel[ xT ][ yT ][ cIdx ][ nT−1−x ][ nT−1−y ].

last_significant_coeff_x_suffix specifies the suffix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_x_suffix shall be in the range from 0 to ( 1 << ( ( last_significant_coeff_x_prefix >> 1 ) − 1 ) ) − 1, inclusive.

The column position of the last significant coefficient in scanning order within a transform block LastSignificantCoeffX is derived as follows.

- If last_significant_coeff_x_suffix is not present, the following applies.

If cu_ transquant_bypass _flag is equal to 1

LastSignificantCoeffX  =  (1<<log2TrafoSize) −1−last_significant_coeff_x_prefix	(7-54)

Otherwise

LastSignificantCoeffX  =  last_significant_coeff_x_prefix	(7-54)

- Otherwise (last_significant_coeff_x_suffix is present), the following applies.

If cu_ transquant_bypass _flag is equal to 1

LastSignificantCoeffX  =  (1<<log2TrafoSize) −1−  (1 << ((last_significant_coeff_x_prefix >> 1) − 1))  *  	
				(2 + (last_significant_coeff_x_prefix & 1))  +  	(7-55)
				last_significant_coeff_x_suffix

Otherwise

LastSignificantCoeffX  =  (1 << ((last_significant_coeff_x_prefix >> 1) − 1))  *  	
				(2 + (last_significant_coeff_x_prefix & 1))  +  	(7-55)
				last_significant_coeff_x_suffix

last_significant_coeff_y_suffix specifies the suffix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_y_suffix shall be in the range from 0 to ( 1 << (  ( last_significant_coeff_y_prefix >> 1 ) − 1 ) ) − 1, inclusive.

The row position of the last significant coefficient in scanning order within a transform block LastSignificantCoeffY is derived as follows.

- If last_significant_coeff_y_suffix is not present, the following applies.

If cu_ transquant_bypass _flag is equal to 1

LastSignificantCoeffY  =  (1<<log2TrafoSize) −1−last_significant_coeff_y_prefix	(7-56)

Otherwise

LastSignificantCoeffY  =  last_significant_coeff_y_prefix	(7-56)

- Otherwise (last_significant_coeff_y_suffix is present), the following applies.

If cu_ transquant_bypass _flag is equal to 1

LastSignificantCoeffY  =  (1<<log2TrafoSize) −1− (1 << ((last_significant_coeff_y_prefix >> 1) − 1))  *  	
				(2 + (last_significant_coeff_y_prefix & 1))  +  	(7-57) 
				last_significant_coeff_y_suffix

Otherwise

LastSignificantCoeffY  =  (1 << ((last_significant_coeff_y_prefix >> 1) − 1))  *  	
				(2 + (last_significant_coeff_y_prefix & 1))  +  	(7-57) 
				last_significant_coeff_y_suffix

# Patent rights declaration(s)

Ericsson AB may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)