| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0247 |
| --- | --- |

| Title: | Region of interest (ROI) SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Goksel Dedeoglu Madhukar Budagavi | Email: Email: | goksel@ti.com madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution reports that the region of interest (ROI) based video coding is used in practice to improve video quality in pre-determined regions of videos such as human faces, text, and graphical elements such as logos. It is observed that a content analysis block that uses image analysis and understanding techniques such as face detection, text detection, foreground-background segmentation, etc. is typically used to determine ROIs and their relative priorities. This contribution asserts that ROI-based video coding runs into a fundamental problem in cloud computing and video infrastructure applications, where it is common to chain multiple encode-decode modules to prepare video content for delivery to end-users. In such cascaded video pipelines it is reported that there is no ready mechanism to preserve ROIs throughout the pipeline leading to video quality degradation in the ROIs. This contribution proposes a Supplemental enhancement information (SEI) message for transmitting ROI information. It is asserted that by using this SEI message, ROI information calculated in upstream modules can be propagated to downstream encoders to help preserve the ROI information and quality throughout the pipeline. The ROI information signaled in SEI consists of number of ROIs, top left coordinate of ROIs in pixels, width of ROIs in pixels, priority of ROIs, type of ROIs, label/index of ROIs.

# Introduction

Region of Interest (ROI) coding is used to improve video quality in pre-determined regions of videos such as human faces, text, and graphical elements such as logos. Video coding with ROI involves two steps as shown in Figure 1: (a) Video content analysis to determine where the ROIs are, and (b) Video coder that takes as input all available ROI information and the video frame to generate a compressed bitstream. The content analysis block uses image understanding and content analysis techniques such as face detection, text detection, foreground-background segmentation, etc. to determine ROIs and their relative priorities. The video codec can then use the ROI information to modulate the quantization step size according to ROI priorities: for instance, an ROI with higher priority can be quantized using small quantization step size (yielding higher fidelity reconstructions), while an ROI with lower priority can be quantized using larger quantization step size (yielding larger compression artifacts).

Figure 1: Video encoders with Region of Interest (ROI) functionality can preserve the visual quality of select regions such as faces, text, and logos. ROI can be either provided by the user, or detected automatically through content analysis.

ROI-coding runs into a fundamental problem in cloud computing and video infrastructure applications, where it is common to chain multiple encode-decode modules to prepare video content for delivery to end-users. Examples include, multipoint video conferencing, overlaying text or logos on top of existing video content, server based video stabilization and quality enhancement, trans-rating between bit-rate/quality options, and transcoding from one video standard to another. Such cascaded video pipelines offer no ready mechanism to preserve ROIs throughout the chain. Figure 2 illustrates this problem in a very simple pipeline, where face ROIs are paid attention to in the beginning, but forgotten about as soon as the video needs to be re-processed for an overlay function.

Figure 2: Example of multiple (chained) encodings that maybe carried out in cloud computing or video infrastructure applications. V-CA denotes video content analysis to select ROIs, V-ENC denotes video encoder, and V-DEC denotes video decoding. Observe how the face ROIs will be subject to usual compression artifacts as soon as they get processed in the second module, which overlays text or logos onto the video stream.

One way to preserve all the ROIs of interest would be to replicate the content analysis capability in each and every video module. This is illustrated in Figure 3, where the text/logo overlay module of Figure 2 is augmented with content analysis capability so that it can detect faces again and implement face-ROI coding accordingly. It is clearly an expensive proposition to embed content analysis solutions in every module.

Figure 3: One way to deal with the “lost ROI information” problem is to embed content analysis solutions throughout the entire chain of video decode-encode modules. In this example, we added face detection capability to the overlay module of Figure 2. Considering the space of all possible ROIs of interest, this approach quickly become prohibitively expensive.

It is therefore desirable for the ROI information calculated in upstream blocks to be propagated to downstream encoders. An immediate advantage of this would be reduction of system-level costs, as ROIs need not be rediscovered over and over throughout the chain of encode-decode steps. Second, video information around ROIs would be better preserved, since each round of encoding introduces quality loss, causing the very ROI estimation to degrade.

We propose transmission of ROI information as Supplemental enhancement information (SEI) message embedded in the bitstream from one encoder to the next. Figure 4 shows the block diagram of a video encoder that embeds ROI information in a separate SEI message. Such an SEI message could be transmitted once every picture, or once every N pictures depending on whether the ROI information changes significantly from one picture to the next.

Figure 4: A video encoder transmits ROI as meta-data embedded in a SEI message. In contrast to Figure 1, the ROI meta-data can now be re-used and modified by subsequent video modules in the video processing chain.

Figure 5 illustrates potential benefits of communicating ROI meta-data in a system with cascaded video encoders. Contrasting with Figure 3, the content analysis block (V-CA) is no longer needed. The following (and all subsequent) encoders are simply provided the ROI information to use during encoding, via, for instance, SEI messages. This will reduce the overall system cost since V-CA block is not needed in every encoder. Note that the intermediate encoders are allowed to modify the ROI SEI message if they introduce a new ROI, such as a new logo/text region.

Figure 5: Example of multiple encodings that maybe carried out in cloud computing or video infrastructure applications when ROI meta-data is embedded in the bitstream using SEI message. Observe how the face ROIs are communicated to the “overlay” module so that they can be preserved.

There can be multiple ROIs in a picture. The following parameters are defined for ROI:

- roi_window_start_xpos: Specifies the starting x-position of the ROI in pixels.

- roi_window_start_ypos: Specifies the starting y-position of the ROI in pixels.

- roi_window_width: Specifies the width of the ROI in pixels.

- roi_window_height: Specifies the height of the ROI in pixels

- roi_priority: Specifies the priority for the ROI.

- roi_type: Specifies the type for the ROI. Some examples of ROI types: facial region, text region, license place region, watermark region, sky region, etc. The ROI type is helpful during video coding to determine quantization step sizes and coding mode to use  (e.g. text region could use transform skipping, sky region could use finer quantization step size etc.). The ROI type is also useful for post-processing decoded video; for instance, the coordinates of the facial regions transmitted in the SEI message could be re-used during video stabilization in the video server to center the video frame around the face. The facial region coordinates could also be input to a face recognition module and used video indexing purposes. In general, the ROI type can be used for domain specific post-processing.

- roi_label: Specifies the label for the ROI. roi_label can be used to track ROIs from one frame to another: for instance, if they are used to track a person’s face in the video sequence, the person’s face will be assigned the same label through the whole video sequence.

# ROI SEI message definition

Modifications to HEVC draft specification are highlighted below.

D.1 SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 2 ) |  |
| pan_scan_rect( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 6 ) |  |
| recovery_point( payloadSize ) |  |
| else if( payloadType = = 9 ) |  |
| scene_info( payloadSize ) |  |
| else if( payloadType = = 15 ) |  |
| full_frame_snapshot( payloadSize ) |  |
| else if( payloadType = = 16 ) |  |
| progressive_refinement_segment_start( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 19 ) |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 20 ) |  |
| deblocking_filter_display_preference( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 23 ) |  |
| tone_mapping_info( payloadSize ) |  |
| else if( payloadType = = 45 ) |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 47 ) |  |
| display_orientation( payloadSize ) |  |
| else if( payloadType = = 128 ) |  |
| sop_description( payloadSize ) |  |
| else if( payloadType = = 129 ) |  |
| field_indication( payloadSize ) |  |
| else if( payloadType = = 130) |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 131) |  |
| active_parameter_sets( payloadSize ) |  |
| else if( payloadType = = 132 ) |  |
| subpic_timing( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| roi( payloadSize ) |  |
| Else |  |
| reserved_sei_message( payloadSize ) |  |
| if( !byte_aligned( ) ) { |  |
| bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

D.1.23 Region of interest SEI message syntax

| roi( payloadSize ) { | Descriptor |
| --- | --- |
| num_rois_minus1 | ue(v) |
| for (i = 0; i <= num_rois_minus1; i++ ) { |  |
| roi_window_start_xpos [ i ] | ue(v) |
| roi_window_start_ypos [ i ] | ue(v) |
| roi_window_width [ i ] | ue(v) |
| roi_window_height [ i ] | ue(v) |
| roi_priority [ i ] | ue(v) |
| roi_type [ i ] | ue(v) |
| roi_label [ i ] | ue(v) |
| } |  |
| } |  |

D.2 SEI payload semantics

D.2.23 Region of interest SEI message semantics

num_rois_minus1 plus 1 specifies the number of region of interest.

roi_window_start_xpos [i] specifies start x-position of the region of interest i in pixels.

roi_window_start_ypos [i] specifies start y-position of the region of interest i in pixels.

roi_window_width [i] specifies width of the region of interest i in pixels.

roi_window_height [i] specifies height of the region of interest i in pixels.

roi_priority [i] specifies the priority for the region of interest i. roi_priority varies from 0 to 255 with 0 implying largest priority and 255 implying lowest priority.

roi_type [i] specifies the type for the region of interest i. roi_type varies from 0 to 63. Table xx list the roi_type mapping.

Table xx: roi_type mapping

| roi_type | Type of the region of interest |
| --- | --- |
| 0 | Face region |
| 1 | Text region |
| 2 | License plate region |
| 3 | Watermark region |
| 4 | Sky region |
| 5 - 63 | Undefined |

roi_label [i] specifies the label for the region of interest i. roi_label is used to track ROIs from one frame to another. E.g. they can be used to track a person’s face in the video sequence, the person’s face will be assigned the same label through the whole video sequence.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).