| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0249 |
| --- | --- |

| Title: | AhG6: SAO offset coding for higher bit-depth video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Madhukar Budagavi Vivienne Sze | Tel: Email: | +1-214-480-1562 wskim@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In HM-8.0, SAO offset range depends on bit-depth, and the offset value is bypass coded using the truncated unary binarization, where the maximum number of bins is 7, 15, and 31 for 8-bit, 9-bit, and 10- to 14-bit video, respectively. There are 12 offsets per LCU (4 for each color component). It is reported that in the worst case, SAO offsets will generate around 729 Mbins/sec for 10-bit video at 3840x2176@60fps assuming 16x16 LCU size. Considering that bypass bins throughput could be around only 2 times that of context coded bins at typically used processor frequencies, it is asserted that SAO offset bins can be burdensome for UHD real-time decoder implementation. In this contribution, it is proposed to use a combination of the truncated unary (prefix, cMax=7) and fixed length (FL) or 0th exp-Golomb (EG0) binarization (suffix) for higher bit-depth video (9-bit and above) to reduce the worst case number of bins for SAO offset coding. The Main Profile (8-bit video) SAO offset coding remains unchanged. Experimental results are reportedly shown that there is 0.0/0.0/0.0% gain for Y/Cb/Cr, while the maximum number of bins for 10-bit video is reduced from 31 to 12 with FL suffix or 16 with EG0 suffix, respectively.

# Introduction

In HM-8.0 [1], SAO offset range is restricted to [0, 7] for 8 bit video, [0, 15] for 9 bit video, and [0, 31] for 10 bit video, respectively. SAO offset is coded using the truncated unary binarization, and it depends on the bit-depth of video. Fig. 1 shows the binarization for 8-bit, 9 bit, and 10-bit video. There will be 7 bins, 15 bins, and 31 bins, respectively for 8-bit, 9-bit, and 10-bit video, in the worst case as shown in Fig. 1. Note that video with bit-depth larger than 10 bit have the same SAO offset limits as 10-bit video and use the same binarization as 10-bit video.

CABAC is a throughput bottleneck for a hardware implementation. Therefore, it is desirable to reduce the worst case number of bins. Usually context coded bins affect throughput more than bypass coded bins. But, bypass coded bins also affect throughput, roughly speaking, by half of that of the context coded bins. Even though bins for SAO offsets are bypass coded, 31 bins per offset can be too much burdensome.

For example, it is 729 Mbins/sec with 3840x2176@60fps 10 bit video, 16x16 LCU size only for SAO offsets (4 for each color compoment, 12 for each LCU). Therefore, it will be desirable to reduce the number of bins to ease implementation complexity.

Note that no change is proposed for Main Profile, which uses bit-depth equal to 8.

< 8 bit video>

0: 0

1: 10

2: 110

3: 1110

…

6: 1111110

7: 1111111

< 9 bit video >

0: 0

1: 10

2: 110

3: 1110

…

6: 1111110

7: 11111110

8: 111111110

9: 1111111110

…

14: 111111111111110

15: 111111111111111

< 10 bit video >

0: 0

1: 10

2: 110

3: 1110

…

6: 1111110

7: 11111110

8: 111111110

9: 1111111110

…

30: 1111111111111111111111111111110

31: 1111111111111111111111111111111

Fig. 1. Generated bins to code SAO offset using truncated unary binarization in case of 8 bit, 9 bit and 10 bit video in HM-8.0 (offset value: binarized bins). Bit-depths of 11-14 bits use the same binarization as 10-bit video.

# Proposed methods

We propose a new binarization scheme for SAO offset to reduce the maximum number of bins when bit-depth is larger than 8. Note that the binarization scheme for 8 bit video is not changed (no change for Main Profile). The new binarization consists of the truncated unary code prefix with cMax=7 and a suffix. For a suffix, we have tested two different schemes – first, fixed length (FL) binarization, and second, 0th exp-Golom (EG0).

In case of FL suffix, cMax is set to ( 1 << (Min( bitDepth, 10) – 5)) – 1, which is consistent to the offset range. Note that the suffix is only present when bitDepth is larger than 8 and prefix equals to a string of 7 bins equal to 1. When only prefix exists, sao_offset_abs is set to the decoded value of prefix. When both prefix and FL suffix exist, sao_offset_abs is set to the decoded value of suffix.

This is the same for EG0 suffix, except that the encoded and decoded value of suffix is (sao_offset_abs – 6). Therefore, when both prefix and EG0 suffix exist, sao_offset_abs is set to the decoded value of suffix plus 6.

Fig. 2 compares the binarization of proposed method versus HM-8.0 for bit-depth equal to 10. All bins are bypass coded as in HM-8.0. By using the proposed method, the worst case number of bins for 10-bit video is reduced from 31 to 12 in case of TU+FL, and 31 to 16 in case of TU+EG0, respectively.

We have shown two approaches. Note that only one of them shall be selected. The advantage of TU+FL is that it has less number of bins in the worst case than TU+EG0, while the advantage of TU+EG0 is that it uses the existing binarization scheme in HM-8.0, and does not depend on bit-depth.

0: 0

1: 10

2: 110

3: 1110

…

6: 1111110

7: 11111110

8: 111111110

9: 1111111110

…

14: 111111111111110

15: 111111111111111

< HM-8.0 >

0: 0

1: 10

2: 110

3: 1110

…

6: 1111110

7: 11111110111

8: 11111111000

9: 11111111001

…

14: 11111111110

15: 11111111111

< TU + FL >

0: 0

1: 10

2: 110

3: 1110

…

6: 1111110

7: 11111110

8: 1111111100

9: 1111111101

…

14: 111111111100

15: 111111111101

< TU + EG0 >

- 9 bit video

0: 0

1: 10

2: 110

3: 1110

…

6: 1111110

7: 11111110

8: 111111110

9: 1111111110

…

30: 1111111111111111111111111111110

31: 1111111111111111111111111111111

< HM-8.0 >

0: 0

1: 10

2: 110

3: 1110

…

6: 1111110

7: 111111100111

8: 111111101000

9: 111111101001

…

30: 111111111110

31: 111111111111

< TU + FL >

0: 0

1: 10

2: 110

3: 1110

…

6: 1111110

7: 11111110

8: 1111111100

9: 1111111101

…

30: 1111111111101000

31: 1111111111101001

< TU + EG0 >

- 10 bit video

Fig. 2. Binarization of SAO offset in HM-8.0 and in proposed technique for 9 bit and 10-bit video (offset value: binarized bins). Bit-depths of 11-14 bits use the same binarization as 10-bit video. Left: HM-8.0; center: TU+FL; right: TU+EG0.

# Experimental results

To evaluate the coding efficiency of the proposed methods, the common test conditions [2] are used, and HM-8.0 is used as the anchor.

Results of the proposed method of TU+FL are summarized in Table 1, and those of TU+EG0 in Table 2, respectively. Compared to HM-8.0 anchor, BD-rate results of 0.0/0.0/0.0 % (Y/Cb/Cr) is resulted for TU+FL, and 0.0/0.1/0.0 for TU+EG0, respectively, on average.

Table 1. BD-rate results summary of the proposed TU+FL binarization method.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.1% | -0.1% |
| Class B | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% |
| Class D | -0.1% | 0.0% | 0.0% |
| Class E |  |  |  |
| Overall | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% |
| Class C | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.2% | -0.2% |
| Class E | -0.1% | 0.4% | -0.2% |
| Overall | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.1% | -0.1% |
| Class F | -0.2% | 0.0% | -1.2% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 0.2% | -0.5% |
| Class C | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | 0.1% |
| Class E | 0.1% | -0.5% | 0.1% |
| Overall | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.2% | -0.1% |
| Class F | -0.2% | -0.3% | 0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2. BD-rate results summary of the proposed TU+EG0 binarization method.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | 0.2% |
| Class D | -0.1% | -0.1% | -0.3% |
| Class E | -0.1% | -0.2% | 0.1% |
| Overall | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | -0.1% |
| Class F | -0.2% | -0.5% | -1.6% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.2% | -0.4% |
| Class C | 0.0% | -0.2% | 0.1% |
| Class D | 0.1% | -0.5% | 0.2% |
| Class E | 0.0% | -0.1% | 0.4% |
| Overall | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% |
| Class F | -0.2% | -0.3% | -0.3% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 104% |  |  |

# Conclusion

In this contribution, new SAO offset binarization schemes are proposed for bit depth larger than 8, which consists of the truncated unary prefix code with cMax=7 and a suffix code (FL or EG0). These reduce the maximum number of bins for 10 bit video from 31 to 12 (TU+FL) or 16 (TU+EG0). Experimental results show that there is no coding efficiency loss from this simplification. Nothing is modified for Main profile (bit depth = 8) in this proposal.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 8,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-J1003, Jul. 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-J1100, Jul. 2012.

# Proposed Text Changes

## TU+FL binarization case

Table 9-32 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| --- | --- | --- | --- | --- | --- |
| sao_offset_abs | na | TU, cMax = ( 1<< (Min( bitDepth, 10) − 5)) − 1 as specified in subclause 9.3.2.x | na | na | na, (Bypass) |

#### 9.3.2.x Binarization process for sao_offset_abs

Input to this process is a request for a binarization for the syntax element sao_offset_abs and bitDepth.

Output of this process is the binarization of the syntax element.

When bitDepth is equal to 8, the binarization of sao_offset_abs is specified by invoking the TU binarization process as specified in subclause 9.3.2.2 with cMax = 7.

When bitDepth is greater than 8, the binarization of sao_offset_abs consists of a prefix part and (when present) a suffix part. The prefix of the binarization is specified by invoking the TU binarization process for the prefix part of the value specified by Min( synVal, 7 ) with cMax = 7. When prefix is greater than 6, the suffix bin string is derived using the FL binarization process as specified in subclause 9.3.2.5 for the value specified by synVal with cMax = ( 1<< (Min( bitDepth, 10) – 5)) – 1.

## TU+EG0 binarization case

Table 9-32 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| --- | --- | --- | --- | --- | --- |
| sao_offset_abs | na | TU, cMax = ( 1<< (Min( bitDepth, 10) − 5)) − 1 as specified in subclause 9.3.2.x | na | na | na, (Bypass) |

#### 9.3.2.x Binarization process for sao_offset_abs

Input to this process is a request for a binarization for the syntax element sao_offset_abs and bitDepth.

Output of this process is the binarization of the syntax element.

When bitDepth is equal to 8, the binarization of sao_offset_abs is specified by invoking the TU binarization process as specified in subclause 9.3.2.2 with cMax = 7.

When bitDepth is greater than 8, the binarization of sao_offset_abs consists of a prefix part and (when present) a suffix part. The prefix of the binarization is specified by invoking the TU binarization process for the prefix part of the value specified by Min( synVal, 7 ) with cMax = 7. When sao_offset_abs is greater than 6, the suffix bin string is derived using the EGk binarization as specified in subclause 9.3.2.4 for the value specified by (synVal – 6 ) with the Exp-Golomb order k set equal to 0.

# Patent rights declaration

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).