| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0253 |
| --- | --- |

| Title: | AHG7: The performance of extended chroma mode for non 4:2:0 format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Joon-Young Park, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Email: | jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution reports the performance of chroma prediction which predicts chroma samples using linear combination of luma samples. The average gains of HEVC-inherited LM chroma are -2.8%, -5.8%, -5.0% for the case of all intra high efficiency in chroma 4:2:2 sampling and -3.1%, -4.4%, -5.1% in chroma 4:4:4 sampling. With HEVC-inherited LM chroma, the average gains for the case of all intra main are -2.5%, -5.3%, -4.4% in chroma 4:2:2 sampling and -2.8%, -4.1%, -4.5% in chroma 4:4:4 sampling. It also proposes that the parameters for the linear combination are acquired by each PU to reduce the number of the derivation to get them. It reports additional gain of -0.1%, -0.3%, -0.3% for 4:2:2 sampling and 0.0%, 0.0%, -0.1% for 4:4:4 sampling in the case of all intra high efficiency and -0.1%, -0.3%, -0.3% for 4:2:2 sampling and 0.0%, 0.0%, 0.0% for 4:4:4 sampling in the case of all intra main.

# Introduction

For HEVC, chroma prediction based on the linear combination of luma samples which was called LM chroma in short was proposed [1-4]. It utilized the relations between chroma samples and corresponding luma samples and estimated the chroma samples by linear combination of the luma samples. Since the chroma samples for the current block are not known for decoder, the parameters for the linear combination are calculated with the neighbouring samples.

In case of 4:2:0 sampling, chroma samples are predicted as as follows.

predC[x,y]=α×(PL[2x,2y]+PL[2x,2y+1])>>1+β	(1),

where PL indicates luma samples.

The derivation of the parameter α and β can be described as follows.

α=R(PL,P'C)R(PL,PL)β=M(P'C)−α×M(PL)	(2),

where P'c indicates reconstructed chroma samples around the target block. And PL indicates the luma samples considering the phase difference between luma and chroma samples, which is derived by

PL[x,y]=(PL[2x,2y]+PL[2x,2y+1])>>1	(3).

Fig. 1 depicts chroma samples and luma samples used for the prediction and the calculation in the case of 4:2:0 sampling. The locations of the samples used for the derivation of α and β are shown as gray circles and the location of luma samples used for the prediction of chroma samples are shown as red circles.

Fig 1 chroma and luma samples involved in LM chroma prediction

Since chroma prediction is done by TU, these parameters α and β are also derived by each TU with the neighbours around the TU.

# Proposed Methods

## Extended chroma prediction using linear combination of luma samples

We propose to use the extended chroma prediction using linear combination of luma samples (LM chroma) for non 4:2:0 sampling. The basic algorithm remains as before regardless of the samplings but the samples involved should be changed.

For the case of 4:4:4 sampling, all of luma and chroma neighbours are used for the parameter calculation and all of luma samples in the block are used for the prediction of chroma samples.

For the case of 4:2:2 sampling, downsampling of luma samples is required but it is not need to consider the phase difference between luma and chroma samples unlike 4:2:0 sampling. The locations of the samples used for the derivation of α and β are shown as gray circles and the location of luma samples used for the prediction of chroma samples are shown as red circles in Fig. 2.

Fig 2 chroma and luma samples involved in LM chroma prediction for 4:2:2 sampling

## Derivation of the parameters

We propose that the parameters are derived per each CU not per each TU to reduce the number of calculation. Once parameters are calculated, they are used for all TUs in the CU. If the parameters are calculated per each TU, the calculation could be invoked 256 times for 64x64 CU with 256 4x4 TUs in worst case.

The proposed method reduces total number of derivation and total number of access to memories which stores luma samples without changing the calculation itself. Moreover it may help to get more reliable relationship between luma and chroma samples since it uses a larger number of training samples in average. The difference in process from the current method is depicted in Fig. 3.

Fig 3 Process of parameter derivation

# Experimental Results

The experimental result is shown below. The proposed method is implemented on top of HM8.0-dev-ahg7 provided. The detailed results including RA and LD cases are described in the accompanied excel files. The anchor of the test is HM8.0-dev-ahg7 and the chroma prediction is used in both MAIN configuration and HE10 configuration. Applying extended chroma prediction brings -2.5%, -5.3%, -4.4% gain for AI Main and -2.8%, 5.8%, -5.0% gain for AI HE10 in the case of 4:2:2 sampling. And -2.8%, -4.1%, -4.5% gain for AI Main and -3.1%, 4.4%, -5.1% gain for AI HE10 in the case of 4:4:4 sampling.

The parameter derivation per CU brings another gains of -0.1%, -0.3%, -0.3% and -0.1%, -0.3%, -0.3% in respect to AIMAIN and AIHE10 in the case of 4:2:2 sampling. And 0.0%, 0.0%, 0.0% and 0.0%, 0.0%, -0.1% in respect to AIMAIN and AIHE10 in the case of 4:4:4 sampling.

It is noted that chroma components are set to have one prediction mode for the CU corresponding 8x8 luma CU in the current base software.

## Performance of extended chroma prediction

### 4:2:2 sampling

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | -2.5% | -3.4% | -2.8% | -2.8% | -4.1% | -3.5% |
| Parkscene | -3.3% | -6.3% | -3.4% | -3.6% | -6.9% | -3.5% |
| Birds-in-cage | -2.6% | -3.2% | -7.7% | -3.0% | -3.8% | -9.4% |
| Ducks-and-legs | -1.0% | -1.0% | -1.2% | -1.0% | -1.1% | -1.4% |
| Traffic | -1.2% | -2.1% | -3.0% | -1.3% | -2.2% | -3.7% |
| Crowd-run | -1.8% | -6.2% | -4.0% | -2.0% | -6.5% | -4.3% |
| Old-town | -1.7% | -4.4% | -2.9% | -1.9% | -4.9% | -3.1% |
| Seeking | -6.2% | -15.7% | -10.2% | -6.5% | -16.6% | -10.8% |
| Overall | -2.5% | -5.3% | -4.4% | -2.8% | -5.8% | -5.0% |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |

### 4:4:4 sampling

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | -2.3% | -2.2% | -2.9% | -2.6% | -2.7% | -3.5% |
| Parkscene | -3.6% | -4.1% | -3.7% | -3.9% | -4.6% | -3.9% |
| Birds-in-cage | -2.2% | -2.5% | -8.0% | -2.4% | -2.7% | -9.6% |
| Ducks-and-legs | -1.1% | -1.0% | -1.5% | -1.2% | -1.1% | -1.6% |
| Traffic | -1.0% | -1.7% | -3.1% | -1.1% | -1.8% | -3.7% |
| Crowd-run | -2.5% | -5.6% | -4.7% | -2.6% | -5.9% | -5.2% |
| Old-town | -1.9% | -2.9% | -2.5% | -2.1% | -3.1% | -2.7% |
| Seeking | -8.1% | -12.5% | -9.5% | -8.5% | -13.4% | -10.3% |
| Overall | -2.8% | -4.1% | -4.5% | -3.1% | -4.4% | -5.1% |
| Enc Time[%] | 107% |  |  | 107% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

## Performance of parameter calculation per CU

### 4:2:2 sampling

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | -0.1% | -0.3% | -0.3% | -0.1% | -0.3% | -0.3% |
| Parkscene | -0.1% | -0.4% | -0.3% | -0.1% | -0.4% | -0.3% |
| Birds-in-cage | 0.0% | -0.1% | -0.3% | 0.0% | -0.1% | -0.4% |
| Ducks-and-legs | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% |
| Traffic | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% | -0.3% |
| Crowd-run | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.2% |
| Old-town | 0.0% | -0.2% | -0.1% | 0.0% | -0.3% | -0.3% |
| Seeking | -0.1% | -0.7% | -0.5% | -0.1% | -0.7% | -0.5% |
| Overall | -0.1% | -0.3% | -0.3% | -0.1% | -0.3% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

### 4:4:4 sampling

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Parkscene | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% |
| Birds-in-cage | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% |
| Ducks-and-legs | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% |
| Traffic | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Crowd-run | 0.3% | 0.4% | 0.3% | 0.2% | 0.4% | 0.3% |
| Old-town | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
| Seeking | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

### Number of computation in a CU

#### 64x64 CU with 4 32x32 TUs

|  |  | HM8.0_dev_AHG7 with LMon | Proposed |
| --- | --- | --- | --- |
| number of parameter derivation |  | 4 | 1 |
| L & C | addition | 4*(32*2) + 4*(32*2) | 64*2 + 64*2 |
| LL & LC | addition | 4*(32*2) + 4*(32*2) | 64*2 + 64*2 |
|  | multiplication | 4*(32*2) + 4*(32*2) | 64*2 + 64*2 |
| Etc. | addition | 4*14 | 14 |
|  | multiplication | 4*5 | 5 |
| Total | addition | 1080 | 526 |
|  | multiplication | 532 | 261 |

#### 64x64 CU with 256 4x4 TUs

|  |  | HM8.0_dev_AHG7 with LMon | Proposed |
| --- | --- | --- | --- |
| number of parameter derivation |  | 256 | 1 |
| L & C | addition | 256*(4*2) + 256*(4*2) | 64*2 + 64*2 |
| LL & LC | addition | 256*(4*2) + 256*(4*2) | 64*2 + 64*2 |
|  | multiplication | 256*(4*2) + 256*(4*2) | 64*2 + 64*2 |
| Etc. | addition | 256*14 | 14 |
|  | multiplication | 256*5 | 5 |
| Total | addition | 11776 | 526 |
|  | multiplication | 5376 | 261 |

# References

- J. Kim, S. Park, Y. Choi, Y. Jeon, B. Jeon, “New intra chroma prediction using inter-channel correlation”, JCTVC-B021, 2nd JCT-VC Meeting, Geneva, Switzerland, July 2010

- J. Chen, V. Seregin, “Chroma intra prediction by reconstructed luma samples”, JCTVC-C206, 3rd Meeting, Guangzhou, China, Oct. 2010

- J. Chen, V. Seregin, S. Lee, W-J Han, J. Kim, B. Jeon, “CE6.a: Chroma intra prediction by reconstructed luma samples”, JCTVC-D350, 4th Meeting, Daegu, Korea, Jan. 2011

- J. Chen, V. Seregin, W-J Han, J. Kim, B. Jeon, “CE6.a.4: Chroma intra prediction by reconstructed luma samples”, JCTVC-E266, 5th Meeting, Geneva, Switzerland, Mar. 2011

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).