| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0255 |
| --- | --- |

| Title: | Cleanup of reference picture list modification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto, Takeshi Tsukuba, Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes two changes on reference picture list modification signaling. The first change removes redundant modification flag signaling. The second change is to make the text of initialization process clearer.

# Introduction

Study of reference picture signaling in the latest proposed draft (JCTVC-K0030_v3) reveals that there are redundancies of reference picture list (RPL) modification signaling. This contribution proposes to remove those redundancies to clean up the text and avoid design confusion. This contribution also proposes to make the text of initialization process clearer.

# Redundant signaling of RPL modification information

## Current RPL modification (JCTVC-K0030_v3)

The current RPL modification syntax is shown in Table 1 and 2. In Table 2, the length of the syntax element list_entry_l0/l1[i] is Ceil( Log2( NumPocTotalCurr) ). The variable NumPocTotalCurr corresponds to the number of reference pictures that could be used to decode the current picture.

Table 1: slice_header syntax (JCTVC-K0030_v3)

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( !dependent_slice_flag ) { |  |
| ... |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| ... |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| ... |  |
| } |  |
| ... |  |
| } |  |
| } |  |

Table 2: ref_pic_list_modification syntax (JCTVC-K0030_v3)

| ref_pic_list_modification( ) { | Descriptor |
| --- | --- |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| list_entry_l0[ i ] | u(v) |
| if( slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| list_entry_l1[ i ] | u(v) |
| } |  |
| } |  |

## Proposed change 1: Redundancy of RPL modification present flag

When NumPocTotalCurr equals to 1, there is no need to modify RPL since there is only one picture that could be used. In this case, ref_pic_list_modification_flag_l0/l1 in ref_pic_list_modification() is redundant and may lead to design confusion.

Table 3 shows the suggested text. Note that the variable NumPocTotalCurr is anyway needed to parse list_entry_l0/l1 syntax elements in ref_pic_list_modification().

Enforcing list_modification_present_flag to be 0 could be also used to avoid the redundant modification flag signaling. But we prefer the other solution since list_modification_present_flag is signaled in SPS and NumPocTotalCurr may differ from picture to picture.

Table 3: slice_header syntax (Proposed change)

| slice_header( ) { | Descriptor |
| --- | --- |
| ... | u(1) |
| if( !dependent_slice_flag ) { |  |
| ... |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| ... |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_list_modification( ) |  |
| ... |  |
| } |  |
| ... |  |
| } |  |
| ... |  |
| } |  |

# Text of RPL initialization process

## Recent proposed text for RPL initialization (JCTVC-K0030_v3)

The L0 RPL initialization process are described as follows:

8.3.4.2. Initialization process for reference picture lists

This process is invoked when decoding a P or B slice header.

The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ] 
	for( i = 0;  i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
}

The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l0_active_minus1; rIdx++)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? 
			RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
			RefPicListTemp0[ rIdx ]

In this process, a list called RefPicListTemp0 is constructed. Although it is called something like reference picture list, its length is not always equal to the length of the L0 reference picture list (num_ref_idx_l0_active_minus1 + 1). This resulted in the current complicated loop for deriving list RefPicListTemp0. In addition, it may prevent understanding the meaning of list_entry_l0[rIdx].

## Proposed editorial change

In order to make text simpler and more understandable, we suggest following text.

8.3.4.2. Initialization process for reference picture lists

This process is invoked when decoding a P or B slice header.

The list RpsCurrList0 is constructed as follows:

for( i = 0, k=0; i < NumPocStCurrBefore; i++, k++)
		RpsCurrList0[ k ] = RefPicSetStCurrBefore[ i ] 
	for( i= 0;  i < NumPocStCurrAfter; i++, k++ ) 
		RpsCurrList0[ k ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr; i++, k++ )
		RpsCurrList0[ k ] = RefPicSetLtCurr[ i ]

The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l0_active_minus1; rIdx++)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? 
			RpsCurrList0[ list_entry_l0[ rIdx ] ] :
			RpsCurrList0[ rIdx % NumPocTotalCurr ]

In this description, the list RpsCurrList0 is used instead of RefPicListTemp0. RpsCurrList0 is the list of reference pictures available for the current picture with its length equal to NumPocTotalCurr. The order of the pictures are based on the priority for use in reference picture list L0. Note that, with using RpsCurrList0, it is clearer that the value of list_entry_l0[rIdx] is from 0 to (NumPocTotalCurr – 1).

# Conclusion

This contribution proposes changes to remove redundancy on reference picture list modification signaling. In addition, the change of the text of reference picture list initialization is proposed. It is recommended to adopt these changes in the latest HEVC draft.

# Patent rights declaration

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).