| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai CN, 10–19 October 2012 | Document: JCTVC-K0256 |
| --- | --- |

| Title: | Cleanup of Slice Header |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu Jian Lou Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2251 yyu@motorola.com +1-858-404-3427 lou@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

In the current slice header, there are some syntax having dependency each other. To reduce the redundant bits, we propose to use conditional coding for those syntaxes, including slice type and some syntaxes related to motion compensation. Furthermore, some semantics changes are added for no_output_of_prior_pics_flag, pic_output_flag, and pic_order_cnt_lsb syntax to prevent possible crash of decoder.

# Introduction

## RAP related information

In the current HEVC spec [1], random access point (RAP) picture is defined as a coded picture containing only I slices that has nal_unit_type in the range of 7 to 12, inclusive.

A variable, RapPicFlag, defined as

RapPicFlag = ( nal_unit_type>=7  &&  nal_unit_type  <=  12 )

is used to indicate if a current picture is a RAP or not.

Clearly, if a coded picture is a RAP picture, all the slices within the RAP picture are I slice. Therefore, it is no need to specify the slice type in slice header if a current slice is in a RAP picture. Saving in bits at slice header is possible.

In this proposal, we propose that the slice type information for a current slice will be sent only if RapPicFlag is 0. Otherwise (if RapPicFlag is 1), the current slice is inferred to be I slice.

The corresponding syntax change is as follows, where the yellow color highlights the changes and the red color with strikethrough indicates the removed lines from the current HEVC [1].

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| if(!RapPicFlag ) |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| …………………. |  |
| byte_alignment( ) |  |
| } |  |

The corresponding semantics change for slice_type is as follows.

slice_type specifies the coding type of the slice according to Table 1-1.

Table 1-1 – Name association to slice_type

| - slice_type | - Name of slice_type |
| --- | --- |
| - 0 | - B (B slice) |
| - 1 | - P (P slice) |
| - 2 | - I (I slice) |

If slice_type is not present, it is inferred to be equal to 2.

When nal_unit_type has a value in the range of 7 to 12, inclusive (RAP picture), slice_type shall be equal to 2.

When sps_max_dec_pic_buffering[ TemporalId ] is equal to 0, slice_type shall be equal to 2.

## IDR related information

In the current HEVC spec [1], the variable IdrPicFlag is specified as

IdrPicFlag = ( nal_unit_type  = =  IDR_W_DLP  | |  nal_unit_type  = =  IDR_N_LP )

If the current slice is not in a IDR picture, short term and long term reference buffer syntax are needed to send. However, if a slice is an I slice, but not in an IDR(IdrPicFlag is 0), the syntax related to motion compensation is not needed to send. The corresponding syntax change is as follows.

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| No_output_of_prior_pics_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| ………………………….. |  |
| byte_alignment( ) |  |
| } |  |

## Some semantics related changes

In the current HEVC spec [1], syntax no_output_of_prior_pics_flag, pic_output_flag, and pic_order_cnt_lsb are defined at slice header. Because these three syntaxes are used for a whole picture, instead of a slice only, there may be confusion for multiple-slice pictures with the current HEVC spec [1].

For example, in the current HEVC spec [1], there are no restrictions on the value of pic_order_cnt_lsb for slices within a picture. That is, it is allowed that pic_order_cnt_lsb can take different values for different slices within a picture in the current HEVC [1]. According to 8.3.1 of decoding process for picture order count in the HEVC[1], different pic_order_cnt_lsb will results in different PicOrderCntVal, the picture order count, for a current picture. Apparently decoder will be confused, and decoder crashing is therefore possible. Similar problems will happen for no_output_of_prior_pics_flag and pic_output_flag.

In this proposal, we propose to add constraint on no_output_of_prior_pics_flag, pic_output_flag and pic_order_cnt_lsb for slices within a picture. That is, the values of no_output_of_prior_pics_flag, pic_output_flag and pic_order_cnt_lsb for slices within a picture should be identical. The suggested semantics changes for no_output_of_prior_pics_flag, pic_output_flag and pic_order_cnt_lsb are as follows.

no_output_of_prior_pics_flag specifies how the previously-decoded pictures in the decoded picture buffer are treated after decoding of an IDR or a BLA picture, see Annex C. When the IDR or BLA picture is the first picture in the bitstream, the value of no_output_of_prior_pics_flag has no effect on the decoding process. When the IDR or BLA picture is not the first picture in the bitstream and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] derived from the sequence parameter set active for the preceding picture, no_output_of_prior_pics_flag equal to 1 may (but should not) be inferred by the decoder, regardless of the actual value of no_output_of_prior_pics_flag.  The values of no_output_of_prior_pics_flag for different slices within a picture should be identical.

pic_output_flag affects the decoded picture output and removal processes, as specified in Annex C. When pic_output_flag is not present, it is inferred to be equal to 1. The values of pic_output_flag for different slices within a picture should be identical.

pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When pic_order_cnt_lsb is not present, pic_order_cnt_lsb is inferred to be equal to 0. The values of pic_order_cnt_lsb for different slices within a picture should be identical.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 8,” Document of Joint Collaborative Team on Video Coding, JCTVC-J1003_d8, 10th Meeting: Stockholm, SE, 11–20 July 2012.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).