| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai CN, 10–19 October 2012 | Document: JCTVC-K0257 |
| --- | --- |

| Title: | Modification of Enabling Weighted Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu Jian Lou Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2251 yyu@motorola.com +1-858-404-3427 lou@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

In the current HEVC, the weighted prediction flags are coded at PPS, but the specific weighted prediction parameters are coded at slice header. In this proposal, we propose two possible methods to enable weighted prediction. First method is to use one flag instead of current two flags at PPS because it seems that there is no necessary to enable/disable P and B slice within a same picture to have weighted prediction separately. The second method is to move current two flags at PPS to slice header so that weighted predication can be enabled/disabled at slice level.

# Introduction

In the current HEVC [1], there are two syntaxes at PPS, weighted_pred_flag and weighted_bipred_flag, that are used to enable/disable weighted preditcion. More specifically, weighted_pred_flag equal to 0 specifies that weighted prediction shall not be applied to P slices and weighted_pred_flag equal to 1 specifies that weighted prediction shall be applied to P slices. weighted_bipred_flag equal to 0 specifies that the default weighted prediction is applied to B slices and weighted_bipred_flag equal to 1 specifies that weighted prediction is applied to B slices.

If the weighted prediction is enabled, the specific weighted prediction parameters are coded at slice header.

Because there are two separate weighted prediction enable flags for P and B slices, it is assumed that P and B slices within a same picture may enable/disable weighted predictions separately. However, slices with same slice type within a same picture must have same weighted prediction on/off while these slices may have different weighted prediction parameters when weighted prediction is on according to the current HEVC [1].

# The modification of enabling weighted prediction

In this proposal, we propose two methods to enable/disable weighted prediction. The red color with strikethrough indicates the removed lines from the current HEVC [1] and the yellow color highlights the changes.

Method 1:

It seems that there is no reason why we need to enable/disable P and B slices within a same picture weighted prediction separately. If we want to cover all cases, we need to prepare four sets of PPS (00, 01, 10, 11). Therefore, we propose to use one flag, weighted_pred_enable_flag, instead of two flags in the current HEVC, to enable/disable weighted prediction for both P and B slices. By using this new flag, we can save 1 bit at PPS. Moreover, only two sets of PPS (0, 1) are needed to cover all cases. The new PPS syntax is shown as follows.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| ……………………….. |  |
| pic_slice_level_chroma_qp_offsets_present_flag | u(1) |
| weighted_pred_enable_flag | u(1) |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| output_flag_present_flag | u(1) |
| transquant_bypass_enable_flag | u(1) |
| ……………………………. |  |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

weighted_pred_enable_flag equal to 0 specifies that weighted prediction shall not be applied to P slices and the default weighted prediction is applied to B slices. weighted_pred_enable_flag equal to 1 specifies that weighted prediction shall be applied to both P and B slices.

weighted_pred_flag equal to 0 specifies that weighted prediction shall not be applied to P slices. weighted_pred_flag equal to 1 specifies that weighted prediction shall be applied to P slices.

weighted_bipred_flag equal to 0 specifies that the default weighted prediction is applied to B slices. weighted_bipred_flag equal to 1 specifies that weighted prediction is applied to B slices.

The corresponding change for slice header is shown as follows.

#### General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| ………………….. |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| if( weighted_pred_enable_flag ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| ………………………… |  |
| } |  |
| ……………………….. |  |
| byte_alignment( ) |  |
| } |  |

Method 2:

In the current HEVC [1], there are two separate weighted prediction enable flags for P and B slices which may enable/disable weighted prediction for P and B slices separately. If it is really a need to enable/disable P and B slices separately, it is more reasonable to enable/disable individual slice instead of slices with the same slice type. However, the current design cannot enable/disable weighted prediction separately for those slices with same slice type. In this proposal, we propose to remove the weighted prediction flags, weighted_pred_flag and weighted_pred_flag from PPS, and use a new weighted preduiction enabling flag, weighted_pred_enable_flag, at slice header. By using this way, we may enable/disable weighted prediction for each slice instead of P and B slices separately. When  a slice is disabled to use weighted prediction, it may also save some bits for weighted prediction parameters.

The new PPS syntax and slice header are shown as follows.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| ……………………….. |  |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| output_flag_present_flag | u(1) |
| transquant_bypass_enable_flag | u(1) |
| ……………………. |  |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| ………………………. |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| weighted_pred_enable_flag | u(1) |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| …………………… |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| if( weighted_pred_enable_flag ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pic_slice_level_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| ……………………… |  |
| byte_alignment( ) |  |
| } |  |

weighted_pred_enable_flag equal to 0 specifies that weighted prediction shall not be applied to P slices and the default weighted prediction is applied to B slices. weighted_pred_enable_flag equal to 1 specifies that weighted prediction shall be applied to current P and B slice. When not present, the value of weighted_pred_enable_flag shall be inferred to be equal to 0.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 8,” Document of Joint Collaborative Team on Video Coding, JCTVC-J1003_d8, 10th Meeting: Stockholm, SE, 11–20 July 2012.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).