| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0259r1 |
| --- | --- |

| Title: | Scaling list selection for transform skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Yue Yu Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-2251 yyu@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

In the current HEVC specification, if transform skip mode is not enabled, default Human Visual System (HVS) based quantization matrices/scaling lists are used for all the TU sizes including 4x4. When transform skip mode is enabled, the default quantization matrices for 4x4 TUs are set to flat ones with all the values equal to 16.

When the transform skip mode is enabled, there will likely be a mixture of 4x4 TUs coded in traditional mode and transform skip mode. Under the current constraint, only one flat quantization matrix can be used for 4x4 TUs coded in the transform skip mode and the traditional mode. If the default flat quantization matrix is not good for all the 4x4 TUs, the traditional HVS quantization matrix need to be loaded, causing a lot of bits to represent.

It is proposed to introduce syntax for selecting the default quantization matrices/scaling lists when transform skip mode is enabled.

# Introduction

Transform skip is adopted as an extra mode for Intra and Inter 4x4 blocks [1,2]. In the current HEVC specification, if transform skip mode is not enabled, default Human Visual System (HVS) based quantization matrices/scaling lists (Table 1 and Table 2) are used for all the TU sizes including 4x4. When transform skip mode is enabled, the default quantization matrices for 4x4 TUs are set to flat ones with all the values equal to 16.

Table 1 Default quantization matrix for 4x4 Intra transform blocks in HEVC

| 16 | 16 | 17 | 21 |
| --- | --- | --- | --- |
| 16 | 17 | 20 | 25 |
| 17 | 20 | 30 | 41 |
| 21 | 25 | 41 | 70 |

Table 2 Default quantization matrix for 4x4 Inter transform blocks in HEVC

| 16 | 16 | 17 | 21 |
| --- | --- | --- | --- |
| 16 | 17 | 21 | 24 |
| 17 | 21 | 24 | 36 |
| 21 | 24 | 36 | 57 |

When the transform skip mode is enabled, there will likely be a mixture of 4x4 TUs coded in traditional mode and transform skip mode. Under the current constraint, only one flat quantization matrix can be used for 4x4 TUs coded in the transform skip mode and the traditional mode. Table 3 gives the collected statistics of 4x4 TUs coded with the transform skip mode and the traditional mode for all the sequences and Table 4 gives the collected statistics of 4x4 TUs coded with the transform skip mode and the traditional mode for class F sequences.

Table 3 Statistics of 4x4 TUs coded with the transform skip mode and the traditional mode for all the sequences

|  | All Intra Main |  |  |  |  |  | All Intra HE10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TS_Y | NTS_Y | TS_U | NTS_U | TS_V | NTS_V | TS_Y | NTS_Y | TS_U | NTS_U | TS_V | NTS_V |
| Class A | 0.50% | 99.50% | 0.02% | 99.98% | 0.02% | 99.98% | 0.51% | 99.49% | 0.02% | 99.98% | 0.02% | 99.98% |
| Class B | 0.87% | 99.13% | 0.01% | 99.99% | 0.02% | 99.98% | 0.84% | 99.16% | 0.01% | 99.99% | 0.02% | 99.98% |
| Class C | 2.50% | 97.50% | 0.08% | 99.92% | 0.06% | 99.94% | 2.50% | 97.50% | 0.08% | 99.92% | 0.06% | 99.94% |
| Class D | 2.88% | 97.12% | 0.07% | 99.93% | 0.08% | 99.92% | 2.87% | 97.13% | 0.07% | 99.93% | 0.08% | 99.92% |
| Class E | 0.31% | 99.69% | 0.01% | 99.99% | 0.02% | 99.98% | 0.30% | 99.70% | 0.01% | 99.99% | 0.02% | 99.98% |
| Class F | 25.92% | 74.08% | 2.61% | 97.39% | 3.67% | 96.33% | 26.00% | 74.00% | 2.58% | 97.42% | 3.66% | 96.34% |
| Overall | 5.52% | 94.48% | 0.47% | 99.53% | 0.64% | 99.36% | 5.53% | 94.47% | 0.46% | 99.54% | 0.64% | 99.36% |
|  | Random Access Main |  |  |  |  |  | Random Access HE10 |  |  |  |  |  |
|  | TS_Y | NTS_Y | TS_U | NTS_U | TS_V | NTS_V | TS_Y | NTS_Y | TS_U | NTS_U | TS_V | NTS_V |
| Class A | 0.44% | 99.56% | 0.02% | 99.98% | 0.02% | 99.98% | 0.44% | 99.56% | 0.02% | 99.98% | 0.02% | 99.98% |
| Class B | 1.28% | 98.72% | 0.02% | 99.98% | 0.02% | 99.98% | 1.24% | 98.76% | 0.02% | 99.98% | 0.02% | 99.98% |
| Class C | 2.58% | 97.42% | 0.10% | 99.90% | 0.08% | 99.92% | 2.57% | 97.43% | 0.10% | 99.90% | 0.08% | 99.92% |
| Class D | 4.23% | 95.77% | 0.16% | 99.84% | 0.15% | 99.85% | 4.24% | 95.76% | 0.15% | 99.85% | 0.15% | 99.85% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Class F | 24.78% | 75.22% | 2.98% | 97.02% | 3.71% | 96.29% | 24.89% | 75.11% | 2.99% | 97.01% | 3.75% | 96.25% |
| Overall | 6.41% | 93.59% | 0.63% | 99.37% | 0.76% | 99.24% | 6.42% | 93.58% | 0.63% | 99.37% | 0.77% | 99.23% |
|  | Low delay B Main |  |  |  |  |  | Low delay B HE10 |  |  |  |  |  |
|  | TS_Y | NTS_Y | TS_U | NTS_U | TS_V | NTS_V | TS_Y | NTS_Y | TS_U | NTS_U | TS_V | NTS_V |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.68% | 99.32% | 0.01% | 99.99% | 0.01% | 99.99% | 0.63% | 99.37% | 0.01% | 99.99% | 0.01% | 99.99% |
| Class C | 1.30% | 98.70% | 0.03% | 99.97% | 0.03% | 99.97% | 1.29% | 98.71% | 0.03% | 99.97% | 0.02% | 99.98% |
| Class D | 3.24% | 96.76% | 0.10% | 99.90% | 0.07% | 99.93% | 3.25% | 96.75% | 0.11% | 99.89% | 0.07% | 99.93% |
| Class E | 0.23% | 99.77% | 0.01% | 99.99% | 0.01% | 99.99% | 0.26% | 99.74% | 0.02% | 99.98% | 0.01% | 99.99% |
| Class F | 21.89% | 78.11% | 1.39% | 98.61% | 1.94% | 98.06% | 22.01% | 77.99% | 1.38% | 98.62% | 1.89% | 98.11% |
| Overall | 5.49% | 94.51% | 0.31% | 99.69% | 0.41% | 99.59% | 5.51% | 94.49% | 0.31% | 99.69% | 0.40% | 99.60% |

Table 4 Statistics of 4x4 TUs coded with the transform skip mode and the traditional mode for class F

|  |  | All Intra Main |  |  |  |  |  | All Intra HE10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP | TS_Y | NTS_Y | TS_U | NTS_U | TS_V | NTS_V | TS_Y | NTS_Y | TS_U | NTS_U | TS_V | NTS_V |
| BasketballDrillText | 22 | 5.97% | 94.03% | 2.14% | 97.86% | 2.20% | 97.80% | 5.99% | 94.01% | 2.17% | 97.83% | 2.23% | 97.77% |
|  | 27 | 5.65% | 94.35% | 1.85% | 98.15% | 1.84% | 98.16% | 5.70% | 94.30% | 1.86% | 98.14% | 1.86% | 98.14% |
|  | 32 | 5.70% | 94.30% | 1.44% | 98.56% | 1.37% | 98.63% | 5.64% | 94.36% | 1.44% | 98.56% | 1.37% | 98.63% |
|  | 37 | 3.58% | 96.42% | 0.45% | 99.55% | 0.39% | 99.61% | 3.59% | 96.41% | 0.46% | 99.54% | 0.41% | 99.59% |
| ChinaSpeed | 22 | 33.26% | 66.74% | 5.49% | 94.51% | 8.02% | 91.98% | 33.51% | 66.49% | 5.61% | 94.39% | 8.21% | 91.79% |
|  | 27 | 32.53% | 67.47% | 2.73% | 97.27% | 6.54% | 93.46% | 32.55% | 67.45% | 2.62% | 97.38% | 6.60% | 93.40% |
|  | 32 | 30.97% | 69.03% | 0.49% | 99.51% | 5.11% | 94.89% | 30.95% | 69.05% | 0.50% | 99.50% | 5.11% | 94.89% |
|  | 37 | 29.63% | 70.37% | 0.20% | 99.80% | 5.68% | 94.32% | 29.61% | 70.39% | 0.20% | 99.80% | 5.66% | 94.34% |
| SlideEditing | 22 | 50.58% | 49.42% | 21.93% | 78.07% | 21.00% | 79.00% | 50.95% | 49.05% | 21.78% | 78.22% | 20.91% | 79.09% |
|  | 27 | 51.64% | 48.36% | 3.70% | 96.30% | 4.52% | 95.48% | 51.87% | 48.13% | 3.32% | 96.68% | 4.24% | 95.76% |
|  | 32 | 50.89% | 49.11% | 0.30% | 99.70% | 1.08% | 98.92% | 51.02% | 48.98% | 0.31% | 99.69% | 1.08% | 98.92% |
|  | 37 | 46.25% | 53.75% | 0.09% | 99.91% | 0.34% | 99.66% | 46.28% | 53.72% | 0.09% | 99.91% | 0.32% | 99.68% |
| SlideShow | 22 | 20.76% | 79.24% | 0.58% | 99.42% | 0.41% | 99.59% | 20.98% | 79.02% | 0.58% | 99.42% | 0.37% | 99.63% |
|  | 27 | 20.01% | 79.99% | 0.22% | 99.78% | 0.15% | 99.85% | 20.03% | 79.97% | 0.18% | 99.82% | 0.13% | 99.87% |
|  | 32 | 16.18% | 83.82% | 0.08% | 99.92% | 0.04% | 99.96% | 15.89% | 84.11% | 0.07% | 99.93% | 0.03% | 99.97% |
|  | 37 | 11.20% | 88.80% | 0.02% | 99.98% | 0.01% | 99.99% | 11.40% | 88.60% | 0.01% | 99.99% | 0.00% | 100.00% |
|  |  | Random Access Main |  |  |  |  |  | Random Access HE10 |  |  |  |  |  |
|  | QP | TS_Y | NTS_Y | TS_U | NTS_U | TS_V | NTS_V | TS_Y | NTS_Y | TS_U | NTS_U | TS_V | NTS_V |
| BasketballDrillText | 22 | 3.60% | 96.40% | 0.87% | 99.13% | 0.94% | 99.06% | 3.67% | 96.33% | 0.88% | 99.12% | 0.95% | 99.05% |
|  | 27 | 3.32% | 96.68% | 0.78% | 99.22% | 0.81% | 99.19% | 3.39% | 96.61% | 0.78% | 99.22% | 0.80% | 99.20% |
|  | 32 | 3.56% | 96.44% | 0.81% | 99.19% | 0.78% | 99.22% | 3.50% | 96.50% | 0.77% | 99.23% | 0.75% | 99.25% |
|  | 37 | 3.06% | 96.94% | 0.58% | 99.42% | 0.52% | 99.48% | 3.22% | 96.78% | 0.60% | 99.40% | 0.57% | 99.43% |
| ChinaSpeed | 22 | 28.21% | 71.79% | 3.76% | 96.24% | 5.44% | 94.56% | 28.42% | 71.58% | 3.90% | 96.10% | 5.60% | 94.40% |
|  | 27 | 30.99% | 69.01% | 2.61% | 97.39% | 4.23% | 95.77% | 31.22% | 68.78% | 2.57% | 97.43% | 4.29% | 95.71% |
|  | 32 | 30.94% | 69.06% | 0.77% | 99.23% | 3.46% | 96.54% | 30.91% | 69.09% | 0.74% | 99.26% | 3.37% | 96.63% |
|  | 37 | 29.48% | 70.52% | 0.31% | 99.69% | 4.35% | 95.65% | 29.25% | 70.75% | 0.33% | 99.67% | 4.34% | 95.66% |
| SlideEditing | 22 | 48.66% | 51.34% | 24.53% | 75.47% | 23.53% | 76.47% | 49.35% | 50.65% | 25.09% | 74.91% | 24.21% | 75.79% |
|  | 27 | 50.78% | 49.22% | 9.64% | 90.36% | 10.50% | 89.50% | 51.05% | 48.95% | 9.15% | 90.85% | 10.14% | 89.86% |
|  | 32 | 50.78% | 49.22% | 1.26% | 98.74% | 3.02% | 96.98% | 50.76% | 49.24% | 1.28% | 98.72% | 3.02% | 96.98% |
|  | 37 | 47.21% | 52.79% | 0.32% | 99.68% | 0.89% | 99.11% | 47.20% | 52.80% | 0.30% | 99.70% | 0.90% | 99.10% |
| SlideShow | 22 | 18.06% | 81.94% | 0.79% | 99.21% | 0.60% | 99.40% | 18.48% | 81.52% | 0.87% | 99.13% | 0.66% | 99.34% |
|  | 27 | 18.20% | 81.80% | 0.38% | 99.62% | 0.25% | 99.75% | 18.26% | 81.74% | 0.40% | 99.60% | 0.29% | 99.71% |
|  | 32 | 16.40% | 83.60% | 0.16% | 99.84% | 0.08% | 99.92% | 16.29% | 83.71% | 0.15% | 99.85% | 0.08% | 99.92% |
|  | 37 | 13.24% | 86.76% | 0.04% | 99.96% | 0.02% | 99.98% | 13.28% | 86.72% | 0.05% | 99.95% | 0.02% | 99.98% |
|  |  | Low delay B Main |  |  |  |  |  | Low delay B HE10 |  |  |  |  |  |
|  | QP | TS_Y | NTS_Y | TS_U | NTS_U | TS_V | NTS_V | TS_Y | NTS_Y | TS_U | NTS_U | TS_V | NTS_V |
| BasketballDrillText | 22 | 1.34% | 98.66% | 0.07% | 99.93% | 0.11% | 99.89% | 1.29% | 98.71% | 0.07% | 99.93% | 0.10% | 99.90% |
|  | 27 | 0.94% | 99.06% | 0.03% | 99.97% | 0.04% | 99.96% | 0.98% | 99.02% | 0.03% | 99.97% | 0.04% | 99.96% |
|  | 32 | 0.69% | 99.31% | 0.02% | 99.98% | 0.02% | 99.98% | 0.65% | 99.35% | 0.02% | 99.98% | 0.03% | 99.97% |
|  | 37 | 0.48% | 99.52% | 0.01% | 99.99% | 0.02% | 99.98% | 0.52% | 99.48% | 0.01% | 99.99% | 0.01% | 99.99% |
| ChinaSpeed | 22 | 28.68% | 71.32% | 2.47% | 97.53% | 3.67% | 96.33% | 28.86% | 71.14% | 2.53% | 97.47% | 3.79% | 96.21% |
|  | 27 | 31.37% | 68.63% | 0.98% | 99.02% | 1.84% | 98.16% | 31.90% | 68.10% | 1.00% | 99.00% | 1.91% | 98.09% |
|  | 32 | 32.45% | 67.55% | 0.31% | 99.69% | 0.92% | 99.08% | 32.63% | 67.37% | 0.28% | 99.72% | 0.88% | 99.12% |
|  | 37 | 31.13% | 68.87% | 0.11% | 99.89% | 0.91% | 99.09% | 31.91% | 68.09% | 0.09% | 99.91% | 0.88% | 99.12% |
| SlideEditing | 22 | 45.40% | 54.60% | 13.85% | 86.15% | 15.63% | 84.37% | 45.69% | 54.31% | 13.91% | 86.09% | 15.18% | 84.82% |
|  | 27 | 46.52% | 53.48% | 3.40% | 96.60% | 5.72% | 94.28% | 46.58% | 53.42% | 3.23% | 96.77% | 5.12% | 94.88% |
|  | 32 | 43.66% | 56.34% | 0.35% | 99.65% | 1.57% | 98.43% | 43.06% | 56.94% | 0.38% | 99.62% | 1.55% | 98.45% |
|  | 37 | 35.00% | 65.00% | 0.11% | 99.89% | 0.40% | 99.60% | 35.70% | 64.30% | 0.13% | 99.87% | 0.45% | 99.55% |
| SlideShow | 22 | 15.66% | 84.34% | 0.34% | 99.66% | 0.20% | 99.80% | 15.77% | 84.23% | 0.31% | 99.69% | 0.19% | 99.81% |
|  | 27 | 14.07% | 85.93% | 0.09% | 99.91% | 0.05% | 99.95% | 14.19% | 85.81% | 0.09% | 99.91% | 0.05% | 99.95% |
|  | 32 | 12.02% | 87.98% | 0.02% | 99.98% | 0.01% | 99.99% | 11.84% | 88.16% | 0.02% | 99.98% | 0.01% | 99.99% |
|  | 37 | 10.84% | 89.16% | 0.01% | 99.99% | 0.00% | 100.00% | 10.64% | 89.36% | 0.01% | 99.99% | 0.01% | 99.99% |

It is shown in Table 3 and Table 4 that

- For class A, B, C, D, E sequences, the percentage of TS mode is very small for both Luma and Chroma

- For class F Chroma, the percentage of TS mode is still quite small.

- Even for class F Luma, the percentage of TS mode varies and the maximum is about 50%.

Table 5 shows the number of bit needed for loading the default HVS quantization matrices and the flat quantization matrices. If the default flat quantization matrix is not good for all the 4x4 TUs, the traditional HVS quantization matrix need to be loaded, causing a lot of bits to represent.

Table 5 Loading the default HVS quantization matrices

| Intra 4x4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[0][0..2][ i ] | 16 | 16 | 16 | 17 | 17 | 17 | 21 | 20 | 20 | 21 | 25 | 30 | 25 | 41 | 41 | 70 |
| scaling_list_dc_coef_minus8 | 0 | 0 | 0 | 1 | 0 | 0 | 4 | -1 | 0 | 1 | 4 | 5 | -5 | 16 | 0 | 20 |
| Number of bits needed | 1 | 1 | 1 | 3 | 1 | 1 | 7 | 3 | 1 | 3 | 7 | 7 | 7 | 11 | 1 | 11 |
| Table number of bits | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inter 4x4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[0][3..5][ i ] | 16 | 16 | 16 | 17 | 17 | 17 | 21 | 21 | 21 | 21 | 24 | 24 | 24 | 36 | 36 | 57 |
| scaling_list_dc_coef_minus8 | 0 | 0 | 0 | 1 | 0 | 0 | 4 | 0 | 0 | 0 | 3 | 0 | 0 | 8 | 0 | 19 |
| Number of bits needed | 1 | 1 | 1 | 3 | 1 | 1 | 7 | 1 | 1 | 1 | 5 | 1 | 1 | 9 | 1 | 11 |
| Table number of bits | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flat 4x4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[0][0..5][ i ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| scaling_list_dc_coef_minus8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Number of bits needed | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Table number of bits | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Syntax for selecting default quantization matrices when transform skip mode is enabled

It is proposed to add syntax for selecting the default flat quantization matrices when transform skip is enabled. The syntax is used to indicate whether the default flat quantization matrix or the HVS non-flat quantization matrix is selected when transform skip is enabled.

The SPS change is listed in Table 6.

Table 6 Proposed SPS change

| scaling_list_enable_flag | u(1) |
| --- | --- |
| if( scaling_list_enable_flag ) { |  |
| transform_skip_flat_quantization_matrix_flag | u(1) |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| } |  |

transform_skip_flat_quantization_matrix_flag equal to 1 specifies that
 ScalingList[ 0 ][ matrixId ][ i ] is set equal to 16 for i = 0..15 when transform_skip_enabled_flag is euqal to 1. transform_skip_flat_quantization_matrix_flag euqal to 0 specifies that ScalingList[ 0 ][ matrixId ] is specified in Table 7-5 and Table 7-6. When not present,  the value of transform_skip_flat_quantization_matrix_flag is inferred to be equal to  0.

The PPS change is listed in Table 7.

Table 7 Proposed PPS change

| …… |  |
| --- | --- |
| pps_transform_skip_enabled_flag | u(1) |
| …… |  |
| pps_scaling_list_data_present_flag | u(1) |
| if (pps_transform_skip_enabled_flag && !pps_scaling_list_data_present_flag) |  |
| pps_transform_skip_flat_quantization_matrix_flag | u(1) |
| if( pps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |

pps_transform_skip_flat_quantization_matrix_flag equal to 1 specifies that
 ScalingList[ 0 ][ matrixId ][ i ] is set equal to 16 for i = 0..15. pps_transform_skip_flat_quantization_matrix_flag euqal to 0 specifies that ScalingList[ 0 ][ matrixId ] is specified in Table 7-5 and Table 7-6. When not present,  the value of pps_transform_skip_flat_quantization_matrix_flag is inferred to be equal to  0.

–	If transform_skip_enabled_flag is equal to 1 and transform_skip_flat_quantization_matrix_flag is equal to 1 or pps_transform_skip_flat_quantization_matrix_flag is equal to 1 and sizeId is equal to 0, ScalingList[ sizeId ][ matrixId ][ i ] is set equal to 16 for i = 0..15

–	Otherwise, ScalingList[ sizeId ][ matrixId ] is specified in Table 7-5 and Table 7-6.

pic_scaling_list_data_present_flag equal to 1 specifies that parameters are present to modify the scaling lists specified in the sequence parameter set. pic_scaling_list_data_present_flag equal to 0 and pps_transform_skip_flat_quantization_matrix_flag is equal to 0 specifies that the scaling lists used for the pictures referring to the picture parameter set is inferred to be equal to those specified by the sequence parameter set. When scaling_list_enable_flag is equal to 0, the value of pic_scaling_list_data_present_flag shall be equal to 0.

# References

[1] C. Lan, J. Xu, G. J. Sullivan, and F. Wu, “Intra transform skipping,” JCTVC-I0408, Apr. 2012

[2] X. Peng, C. Lan, J. Xu, and G. J. Sullivan, “Inter transform skipping,” JCTVC-J0237, Jul. 2012

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).