| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0260r1 |
| --- | --- |

| Title: | Syntax to support use Chroma filter for Luma interpolation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Koohyar Minoo Yue Yu Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-2521 kminoo@motorola.com +1-858-404-2251 yyu@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

In the current HEVC, 8-tap/7-tap FIR filters are used for Luma component interpolation and 4-tap FIR filters are used for Chroma component interpolation. Generally, 4-tap filters need much less computational complexity and memory bandwidth than the 8-tap filters. It is identifid that for some high resolution sequences, replacing the 8-tap Luma interpolation filters with the 4-tap Chroma filters does not introduce coding efficincy degradation, but greatly reducing the computational and memory complexity. It is proposed to introduce one flag in SPS to indicate if the Chroma interpolation filters are used for Luma interpolation. Crosscheck will be provided by Samsung.

# Introduction

In the current HEVC [1], 8-tap/7-tap FIR filters are used for Luma component interpolation and 4-tap FIR filters are used for Chroma component interpolation. Table 1 and Table 2 summarize the filter coefficients.

Table 1 Luma interpolation filter set coefficient

| Position | Coefficients |
| --- | --- |
| ¼ | {-1, 4, -10, 58, 17, -5, 1, 0} |
| H | {-1, 4, -11, 40, 40, -11, 4, -1} |
| ¾ | {0, 1, -5, 17, 58, -10, 4, -1} |

Table 2 Chroma interpolation filter set coefficient

| Position | Coefficients |
| --- | --- |
| FL0 | {-2, 58, 10, -2} |
| FL1 | {-4, 54, 16, -2} |
| FL2 | {-6, 46, 28, -4} |
| H | {-4, 36, 36, -4} |
| FR2 | {-4, 28, 46, -6} |
| FR1 | {-2, 16, 54, -4} |
| FR0 | {-2, 10, 58, -2} |

Generally, 4-tap filters need much less computational complexity and memory bandwidth than the 8-tap filters. Table 3 shows the worst case complexity comparison between 4-tap and 8-tap filters.

Table 3 Worst case complexity comparison between 4-tap and 8-tap filters

|  | Without Chroma |  |  | With Chroma |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 8-tap | 4-tap | % | 8-tap | 4-tap | % |
| Mults | 46.00 | 19.00 | -58.70% | 90.00 | 63.00 | -30.00% |
| Adds | 40.25 | 14.25 | -64.60% | 73.25 | 47.25 | -35.49% |
| Memband (4x2) | 10.00 | 6.00 | -40.00% | 14.00 | 10.00 | -28.57% |
| Memband (8x2) | 12.00 | 9.00 | -25.00% | 18.00 | 15.00 | -16.67% |
| Memband (4x1) | 11.28 | 5.52 | -51.11% | 16.50 | 10.75 | -34.85% |
| Memband (8x1) | 11.28 | 8.24 | -26.67% | 16.50 | 13.50 | -18.18% |

Longer interpolation filters with more filter taps could achieve higher compression efficiency than shorter interpolation filters. However, it is not aways true. In some cases, shorter interpolation filters could achieve higher compression efficiency than the longer ones. In some other cases, the performance loss from the shorter interpolation filter is negligible.

# Syntax to support use Chroma filter for Luma interpolation

It is proposed to introduce one flag use_chroma_filter_for_luma_interpolation in SPS. Table 4 shows the partial modified SPS syntax.

Table 4 Partial modified SPS syntax

| …… |  |
| --- | --- |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| use_chroma_filter_for_luma_interpolation | u(1) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| …… |  |

use_chroma_filter_for_luma_interpolation equal to 1 specifies that chroma interpolations are used for luma interpolation. use_chroma_filter_for_luma_interpolation equal to 0 specifies that chroma interpolations are not used for luma interpolation, i.e. the normal luma interpolation filters are used.

–	The samples labelled a0,0, b0,0, c0,0, d0,0, h0,0, and n0,0 shall be derived by applying the 8-tap filter to the nearest integer position samples if use_chroma_filter_for_luma_interpolation is equal to 0:

–	The samples labelled a0,0, b0,0, c0,0, d0,0, h0,0, and n0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples if use_chroma_filter_for_luma_interpolation is equal to 1:

a0,0 = ( − 4*A−1,0 + 54*A0,0 + 16*A1,0 − 2*A2,0 ) >> shift1	(8-XXX)

b0,0 = ( − 4*A−1,0 + 36*A0,0 + 36*A1,0 − 4*A2,0 ) >> shift1	(8-XXX)

c0,0 = ( − 2*A−1,0 + 16*A0,0 + 54*A1,0 − 4*A2,0 ) >> shift1	(8-XXX)

d0,0 = ( − 4*A0,−1 + 54*A0,0 + 16*A0,1 − 2*A0,2 ) >> shift1	(8-XXX)

h0,0 = ( − 4*A0,−1 + 36*A0,0 + 36*A0,1 − 4*A0,2 ) >> shift1	(8-XXX)

n0,0 = ( − 2*A0,−1 + 16*A0,0 + 54*A0,1 − 4*A0,2 ) >> shift1	(8-XXX)

–	The samples labelled e0,0, i0,0, p0,0, f0,0, j0,0, q0,0, g0,0, k0,0 and r0,0 shall be derived by applying the 8-tap filter to the samples a0,i, b0,i and c0,i where i = −3..4 in vertical direction samples if use_chroma_filter_for_luma_interpolation is equal to 0:

–	The samples labelled e0,0, i0,0, p0,0, f0,0, j0,0, q0,0, g0,0, k0,0 and r0,0 shall be derived by applying the 4-tap filter to the samples a0,i, b0,i and c0,i where i = −3..4 in vertical direction samples if use_chroma_filter_for_luma_interpolation is equal to 0:

e0,0 = ( − 4*a0,−1 + 54*a0,0 + 16*a0,1 − 2*a0,2 ) >> shift2	(8-XXX)

i0,0 = ( − 4*a0,−1 + 36*a0,0 + 36*a0,1 − 4*a0,2 ) >> shift2	(8-XXX)

p0,0 = ( − 2*a0,−1 + 16*a0,0 + 54*a0,1 − 4*a0,2 ) >> shift2	(8-XXX)

f0,0 = ( − 4*b0,−1 + 54*b0,0 + 16*b0,1 − 2*b0,2 ) >> shift2	(8-XXX)

j0,0 = ( − 4*b0,−1 + 36*b0,0 + 36*b0,1 − 4*b0,2 ) >> shift2	(8-XXX)

q0,0 = ( − 2*b0,−1 + 16*b0,0 + 54*b0,1 − 4*b0,2 ) >> shift2	(8-XXX)

g0,0 = ( − 4*c0,−1 + 54*c0,0 + 16*c0,1 − 2*c0,2 ) >> shift2	(8-XXX)

k0,0 = ( − 4*c0,−1 + 36*c0,0 + 36*c0,1 − 4*c0,2 ) >> shift2	(8-XXX)

r0,0 = ( − 2*c0,−1 + 16*c0,0 + 54*c0,1 − 4*c0,2 ) >> shift2	(8-XXX)

# Change the Chroma half-pel interpolation filter

It is also identified that when 4-tap Chroma interpolation filters are used for Luma interpolation, changing the original [-4, 36, 36, -4] half-pel filter to [-7, 39, 39, -7] could lead to some coding efficiency gain [2].

ae0,0 = ( −7*B−1,0 + 39*B0,0 + 39*B1,0 − 7*B2,0 ) >> shift1	(8-195)

ae0,0 = ( −4*B−1,0 + 36*B0,0 + 36*B1,0 − 4*B2,0 ) >> shift1	(8-195)

# Simulation environment

## Software

The software used to run the simulations is HM8.0. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [3]. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

All the simulation experiments were conducted on the PCs with identical settings, i.e., Microsoft Windows 7 64bits, Intel Xeon 3.2GHz with 14G memory. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The experimental results of the rate-distortion performance and speed comparison are summarized in Table 5-6. Detailed results are included in the attachments (JCTVC-K0260r1(Old filter).xls and JCTVC-K0260r1(New filter).xls).

Table 5 Rate-distortion performance, computational complexity and memory bandwidth complexity comparison of the old 4-tap filter and the 8-tap filter

| RAMain |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BDY | BDU | BDV | Mults | Adds | 2DMem | 1DMem | Encoding | Decoding |
| Traffic | 1.2% | -0.3% | -0.2% | -37.0% | -43.5% | -6.6% | -6.7% | 89% | 90% |
| PeopleOnStreet | -1.0% | -0.8% | -0.5% | -32.0% | -39.0% | -7.7% | -8.0% | 93% | 96% |
| Nebuta | -2.1% | -1.4% | -2.0% | -28.7% | -36.1% | -6.1% | -6.2% | 94% | 93% |
| SteamLocomotive | -1.1% | -0.7% | -0.8% | -30.5% | -37.7% | -4.5% | -4.6% | 93% | 92% |
| Kimono | 0.1% | 0.1% | -0.2% | -33.2% | -40.1% | -7.0% | -7.2% | 91% | 89% |
| ChinaSpeed | -0.3% | -0.8% | -0.7% | -28.9% | -36.0% | -4.4% | -4.5% | 94% | 101% |
| SlideEditing | -0.1% | -0.2% | -0.2% | -13.0% | -19.3% | -0.3% | -0.3% | 90% | 101% |
| SlideShow | 0.2% | 0.3% | 0.3% | -32.0% | -38.7% | -1.6% | -1.6% | 91% | 102% |
| RAHE10 |  |  |  |  |  |  |  |  |  |
|  | BDY | BDU | BDV | Mults | Adds | 2DMem | 1DMem | Encoding | Decoding |
| Traffic | 1.4% | -0.2% | -0.3% | -37.3% | -43.8% | -6.7% | -6.8% | 90% | 90% |
| PeopleOnStreet | -1.0% | -0.6% | -0.4% | -33.1% | -40.0% | -8.8% | -9.1% | 92% | 95% |
| Nebuta | -2.3% | -1.4% | -1.5% | -28.5% | -35.9% | -6.8% | -6.9% | 94% | 95% |
| SteamLocomotive | -1.1% | -0.5% | -1.3% | -32.2% | -39.2% | -5.6% | -5.7% | 92% | 95% |
| Kimono | 0.4% | 0.1% | -0.2% | -35.2% | -41.9% | -7.9% | -8.0% | 90% | 89% |
| ChinaSpeed | -0.3% | -0.8% | -0.7% | -31.3% | -38.2% | -5.2% | -5.3% | 93% | 97% |
| SlideEditing | 0.2% | 0.1% | 0.1% | -11.9% | -18.6% | 0.1% | 0.1% | 89% | 101% |
| SlideShow | 0.3% | 0.4% | 0.4% | -32.9% | -39.5% | -1.8% | -1.8% | 91% | 98% |
| LBMain |  |  |  |  |  |  |  |  |  |
|  | BDY | BDU | BDV | Mults | Adds | 2DMem | 1DMem | Encoding | Decoding |
| Traffic | 4.3% | 0.1% | -0.3% | -40.4% | -46.6% | -11.7% | -11.7% | 89% | 90% |
| PeopleOnStreet | -1.2% | -0.6% | -0.6% | -34.3% | -41.1% | -11.4% | -11.8% | 92% | 95% |
| Nebuta | -2.1% | -1.3% | -1.3% | -29.7% | -37.0% | -6.8% | -6.9% | 93% | 95% |
| SteamLocomotive | -0.6% | 0.2% | 0.5% | -30.8% | -37.9% | -6.3% | -6.5% | 92% | 94% |
| Kimono | 0.3% | 0.7% | -0.3% | -34.3% | -41.1% | -10.9% | -11.2% | 91% | 90% |
| ChinaSpeed | -0.1% | -0.7% | -0.6% | -28.7% | -35.6% | -6.0% | -6.2% | 93% | 98% |
| SlideEditing | -0.1% | 0.0% | -0.1% | -5.0% | -12.0% | 0.2% | 0.2% | 89% | 100% |
| SlideShow | 1.3% | 1.0% | 0.0% | -31.5% | -38.1% | -2.9% | -2.9% | 90% | 98% |
| LBHE10 |  |  |  |  |  |  |  |  |  |
|  | BDY | BDU | BDV | Mults | Adds | 2DMem | 1DMem | Encoding | Decoding |
| Traffic | 5.3% | 0.3% | 0.4% | -42.1% | -48.1% | -11.9% | -11.8% | 89% | 89% |
| PeopleOnStreet | -1.1% | -0.3% | -0.1% | -36.0% | -42.6% | -12.8% | -13.2% | 92% | 95% |
| Nebuta | -2.1% | -1.7% | -1.4% | -29.6% | 36.9% | -7.5% | -7.7% | 93% | 95% |
| SteamLocomotive | 0.0% | 0.9% | 1.5% | -33.2% | -40.1% | -7.6% | -7.8% | 92% | 93% |
| Kimono | 0.9% | 0.6% | 0.0% | -37.0% | -43.5% | -11.3% | -11.6% | 91% | 91% |
| ChinaSpeed | -0.2% | -0.7% | -0.3% | -31.5% | -38.3% | -7.1% | -7.3% | 93% | 97% |
| SlideEditing | -0.5% | -0.7% | -0.8% | -9.0% | -15.5% | 0.0% | 0.0% | 88% | 101% |
| SlideShow | 1.2% | 2.8% | -1.7% | -31.9% | -38.6% | -3.0% | -3.0% | 90% | 97% |
| LPMain |  |  |  |  |  |  |  |  |  |
|  | BDY | BDU | BDV | Mults | Adds | 2DMem | 1DMem | Encoding | Decoding |
| Traffic | 0.2% | -2.0% | -2.0% | -35.2% | -41.9% | -7.2% | -7.2% | 93% | 94% |
| PeopleOnStreet | -3.6% | -2.0% | -1.8% | -29.4% | -36.7% | -6.8% | -7.2% | 95% | 97% |
| Nebuta | -6.7% | 0.5% | -0.6% | -28.9% | -36.3% | -2.9% | -2.9% | 95% | 97% |
| SteamLocomotive | -5.7% | -1.6% | 0.0% | -27.0% | -34.5% | -4.8% | -5.0% | 95% | 97% |
| Kimono | -1.8% | -0.6% | -0.8% | -30.4% | -37.6% | -7.2% | -7.4% | 94% | 94% |
| ChinaSpeed | -0.6% | -0.9% | -1.0% | -23.3% | -30.8% | -3.7% | -3.9% | 97% | 98% |
| SlideEditing | -0.2% | 0.0% | 0.0% | -12.9% | -18.9% | 0.0% | 0.0% | 98% | 100% |
| SlideShow | 0.2% | 0.9% | 2.7% | -28.4% | -35.3% | -1.9% | -1.9% | 92% | 99% |
| LPHE10 |  |  |  |  |  |  |  |  |  |
|  | BDY | BDU | BDV | Mults | Adds | 2DMem | 1DMem | Encoding | Decoding |
| Traffic | 0.8% | -2.2% | -2.1% | -37.8% | -44.2% | -8.3% | -8.3% | 92% | 93% |
| PeopleOnStreet | -3.7% | -2.2% | -2.1% | -30.9% | -38.1% | -8.2% | -8.7% | 95% | 96% |
| Nebuta | -6.6% | 0.8% | -0.3% | -28.5% | -36.0% | -3.5% | -3.6% | 95% | 98% |
| SteamLocomotive | -6.0% | -2.6% | -4.9% | -31.5% | -38.6% | -6.3% | -6.5% | 95% | 97% |
| Kimono | -1.4% | -0.1% | -0.8% | -33.8% | -40.7% | -8.5% | -8.8% | 94% | 93% |
| ChinaSpeed | -0.8% | -1.2% | -0.7% | -27.8% | -34.9% | -4.9% | -5.1% | 96% | 98% |
| SlideEditing | -0.5% | -0.3% | -0.6% | -9.6% | -15.5% | 0.0% | 0.0% | 93% | 100% |
| SlideShow | 0.0% | 0.2% | 3.0% | -30.1% | -37.0% | -2.1% | -2.1% | 92% | 98% |

Table 6 Rate-distortion performance, computational complexity and memory bandwidth complexity comparison of the new 4-tap filter and the 8-tap filter

| RAMain |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BDY | BDU | BDV | Mults | Adds | 2DMem | 1DMem | Encoding | Decoding |
| Traffic | -0.6% | -0.4% | -0.6% | -35.5% | -42.1% | -5.6% | -5.7% | 89% | 91% |
| PeopleOnStreet | -1.1% | -0.7% | -0.5% | -32.3% | -39.3% | -7.7% | -8.0% | 93% | 96% |
| Nebuta | -1.7% | -0.8% | -0.4% | -29.8% | -37.1% | -6.5% | -6.6% | 94% | 93% |
| SteamLocomotive | -1.0% | -0.8% | -0.4% | -31.2% | -38.3% | -4.6% | -4.7% | 92% | 92% |
| Kimono | -0.4% | 0.0% | -0.4% | -33.3% | -40.2% | -6.9% | -7.1% | 90% | 87% |
| ChinaSpeed | -0.3% | -0.7% | -0.5% | -29.5% | -36.5% | -4.4% | -4.5% | 96% | 96% |
| SlideEditing | -0.1% | -0.1% | -0.1% | -13.0% | -19.4% | 0.0% | 0.0% | 93% | 95% |
| SlideShow | 0.0% | -0.3% | -0.1% | -30.8% | -37.6% | -1.4% | -1.4% | 93% | 93% |
| RAHE10 |  |  |  |  |  |  |  |  |  |
|  | BDY | BDU | BDV | Mults | Adds | 2DMem | 1DMem | Encoding | Decoding |
| Traffic | -0.4% | -0.2% | -0.5% | -35.5% | -42.4% | -5.8% | -5.9% | 89% | 92% |
| PeopleOnStreet | -1.0% | -0.5% | -0.3% | -33.3% | -40.2% | -8.7% | -9.0% | 92% | 96% |
| Nebuta | -1.8% | -3.2% | -3.1% | -29.8% | -37.0% | -7.1% | -7.3% | 94% | 96% |
| SteamLocomotive | -0.8% | -0.3% | -0.7% | -33.1% | -40.0% | -5.7% | -5.8% | 92% | 95% |
| Kimono | -0.1% | 0.2% | -0.3% | -35.0% | -41.8% | -7.6% | -7.8% | 90% | 90% |
| ChinaSpeed | -0.3% | -0.7% | -0.5% | -31.2% | -38.1% | -5.1% | -5.2% | 93% | 97% |
| SlideEditing | -0.1% | -0.1% | -0.1% | -15.1% | -21.4% | 0.1% | 0.1% | 90% | 101% |
| SlideShow | 0.0% | -0.3% | 0.0% | -31.6% | -38.3% | -1.6% | -1.5% | 91% | 100% |
| LBMain |  |  |  |  |  |  |  |  |  |
|  | BDY | BDU | BDV | Mults | Adds | 2DMem | 1DMem | Encoding | Decoding |
| Traffic | 1.1% | 0.0% | -0.5% | -36.9% | -43.4% | -7.6% | -7.7% | 89% | 91% |
| PeopleOnStreet | -1.4% | -1.3% | -1.0% | -32.8% | -39.8% | -9.5% | -9.9% | 92% | 95% |
| Nebuta | -1.8% | -3.0% | -1.5% | -29.9% | -37.2% | -6.8% | -7.0% | 93% | 95% |
| SteamLocomotive | -0.9% | 0.2% | 0.0% | -30.2% | -37.4% | -5.6% | -5.8% | 92% | 94% |
| Kimono | -0.2% | 0.7% | -0.8% | -32.8% | -39.8% | -8.2% | -8.5% | 91% | 91% |
| ChinaSpeed | -0.2% | -0.3% | -0.4% | -28.2% | -35.2% | -5.5% | -5.7% | 93% | 99% |
| SlideEditing | 0.4% | 0.4% | 0.2% | -14.7% | -20.8% | 0.0% | -0.1% | 89% | 102% |
| SlideShow | 0.5% | 0.0% | 0.2% | -30.0% | -36.7% | -2.5% | -2.5% | 90% | 100% |
| LBHE10 |  |  |  |  |  |  |  |  |  |
|  | BDY | BDU | BDV | Mults | Adds | 2DMem | 1DMem | Encoding | Decoding |
| Traffic | 2.2% | 0.8% | 1.2% | -37.9% | -44.4% | -7.5% | -7.5% | 89% | 91% |
| PeopleOnStreet | -1.2% | -0.3% | 0.2% | -34.2% | -41.0% | -10.8% | -11.2% | 92% | 95% |
| Nebuta | -1.8% | -3.2% | -1.5% | -30.0% | -37.2% | -7.5% | -7.7% | 93% | 95% |
| SteamLocomotive | -0.2% | -0.3% | -1.1% | -33.2% | -40.2% | -7.0% | -7.2% | 92% | 94% |
| Kimono | 0.3% | 1.4% | 0.3% | -34.9% | -41.7% | -8.4% | -8.7% | 91% | 92% |
| ChinaSpeed | -0.2% | -0.1% | 0.4% | -30.4% | -37.2% | -6.5% | -6.6% | 93% | 98% |
| SlideEditing | -0.3% | -0.1% | -0.3% | -5.1% | -11.9% | 0.1% | 0.1% | 88% | 104% |
| SlideShow | 0.4% | 1.6% | -0.6% | -30.7% | -37.5% | -2.7% | -2.7% | 90% | 99% |
| LPMain |  |  |  |  |  |  |  |  |  |
|  | BDY | BDU | BDV | Mults | Adds | 2DMem | 1DMem | Encoding | Decoding |
| Traffic | -1.5% | -1.3% | -1.4% | -34.0% | -40.8% | -6.5% | -6.6% | 93% | 95% |
| PeopleOnStreet | -3.0% | -1.2% | -1.0% | -29.5% | -36.8% | -7.0% | -7.5% | 95% | 97% |
| Nebuta | -5.2% | 1.5% | 1.6% | -28.8% | -28.8% | -4.9% | -4.9% | 95% | 98% |
| SteamLocomotive | -4.5% | 1.0% | 0.9% | -28.2% | -28.2% | -5.0% | -5.2% | 95% | 97% |
| Kimono | -1.4% | -0.1% | -0.2% | -30.8% | -30.8% | -7.3% | -7.5% | 94% | 96% |
| ChinaSpeed | -0.5% | -0.6% | -0.2% | -24.0% | -24.0% | -4.0% | -4.2% | 96% | 100% |
| SlideEditing | 0.2% | 0.2% | 0.3% | -11.8% | -11.8% | 0.0% | 0.0% | 91% | 102% |
| SlideShow | -0.2% | 0.6% | 2.5% | -27.6% | -27.6% | -1.8% | -1.8% | 92% | 100% |
| LPHE10 |  |  |  |  |  |  |  |  |  |
|  | BDY | BDU | BDV | Mults | Adds | 2DMem | 1DMem | Encoding | Decoding |
| Traffic | -0.5% | 0.0% | 0.6% | -35.9% | -42.5% | -7.6% | -7.7% | 93% | 94% |
| PeopleOnStreet | -3.0% | -0.4% | 0.1% | -30.7% | -37.9% | -8.4% | -8.8% | 95% | 96% |
| Nebuta | -5.1% | 4.4% | 4.3% | -28.5% | -36.0% | -5.4% | -5.4% | 95% | 98% |
| SteamLocomotive | -4.6% | 1.4% | 1.0% | -32.2% | -39.3% | -6.4% | -6.6% | 95% | 97% |
| Kimono | -0.8% | 2.9% | 1.3% | -33.8% | -40.7% | -8.5% | -8.8% | 94% | 95% |
| ChinaSpeed | -0.6% | 0.2% | 0.5% | -27.5% | -34.7% | -5.0% | -5.2% | 96% | 98% |
| SlideEditing | -0.1% | 0.4% | 0.2% | -10.1% | -16.0% | 0.0% | 0.0% | 90% | 103% |
| SlideShow | -0.3% | 0.7% | -1.1% | -29.2% | -36.2% | -2.0% | -2.0% | 92% | 100% |

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 8,” Document of Joint Collaborative Team on Video Coding, JCTVC-J1003_d8, 10th Meeting: Stockholm, SE, 11–20 July 2012.

[2] J. Lou, K. Minoo, L. Wang, and A. Luthra, “On Chroma interpolation filter,” JCTVC-G698

[3] F. Bossen, “HM 8 Common Test Conditions and Software Reference Configurations,” JCTVC-J1100

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).